3GPP TSG-RAN WG4 Meeting #98-bis-e	                               R4-2104658

Electronic Meeting, 12 Apr – 20 Apr, 2021

Source: 	Murata Manufacturing Co Ltd.

Title: 	REFSENS of n2 for 30_40MHz channel bandwidth

Agenda Item:	7.24.2.1 Reference sensitivity [NR_bands_R17_BWs -Core]

Document for:	Approval

Introduction

In this contribution, we provide Tx noise measurement results and MSD estimation for REFSENS specifying of n2 with 30MHz and 40MHz CBW.

WF Review

- The following comment was described in WF [1].

  - Agree to UL config and REFSENS values above for n2 25MHz

  - MSD for n2 30 and 40MHz to be finalized in next meeting

  - Companies encouraged to bring UL config proposal and analysis for REFSENS at next meeting for 30 and 40MHz

Since the frequency range and duplex offset of n2 is almost same with n25, the UL configuration of n25 can be reused to n2.

Proposal 1:

Use the UL configuration as highlighted in Table 7.3.2-3 for n2 REFSENS.

Table 7.3.2-3: Uplink configuration for reference sensitivity

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | Duplex Mode |
| n2 | 15 | 25 | 501 | 501 | 501 | [501] | 481 | 401 |  | FDD |
|  | 30 | 101 | 24 | 241 | 241 | [241] | 241 | 201 |  |  |
|  | 60 |  | 101 | 101 | 101 | [101] | 101 | 101 |  |  |
| n25 | 15 | 25 | 501 | 501 | 501 | 501 | 481 | 401 |  | FDD |
|  | 30 |  | 24 | 241 | 241 | 241 | 241 | 201 |  |  |
|  | 60 |  | 101 | 101 | 101 | 101 | 101 | 101 |  |  |
| NOTE 1: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1). |  |  |  |  |  |  |  |  |  |  |

Tx noise measurement and MSD Estimation

Power Amplifier Calibration and Test Waveforms

PA calibration conditions:

The power amplifier (PA) is calibrated to meet 1 dB MPR for Power Class 3 using a 100 RB QPSK DFT-s-OFDM SCS 15kHz with RBs allocated at the channel edge.

Test waveforms:

- QPSK DFT-s-OFDM SCS 15kHz with the UL RBs be located as close as possible to the DL operating band but confined within the transmission bandwidth configuration for the CBW;

- Image and Local Oscillator (LO) rejection is set to -28 dBc;

- Image and Local Oscillator (LO) rejection is set to -28 dBc;

- Counter-IM3 (C-IM3) set to -60 dBc;

- Counter-IM5 (C-IM5) set to -70 dBc;

Tx noise measurement results

The Rx band Noise (RxBN) of PA measurement results are shown in Table 1.

Table 1: PA RxBN measurement results

| UL/DL BW | Tx freq. | Rx freq. | Tx/Rx band gap | Tx RBs Allocation | RxBN |
| --- | --- | --- | --- | --- | --- |
| (MHz) | (MHz) | (MHz) | (MHz) | (-) | (dBm/BW) |
| 30/30 | 1895 | 1975 | 50 | 48@112 | -40.9 |
| 40/40 | 1890 | 1970 | 40 | 40@176 | -34.6 |

Power Amplifier Calibration and Test Waveforms

The link-budget calculation assumptions and MSD estimation are shown in Table 2.

Table 2: MSD Estimation for n2

| Parameter | Unit | n2 |  |  |  |
| --- | --- | --- | --- | --- | --- |
| UL/DL BW | MHz | 30/30 |  | 40/40 |  |
| Antenna isolation | dB | 10.0 |  | 10.0 |  |
| Front-end loss | dB | 4.0 |  | 4.0 |  |
| PA output power | dBm | 27.0 |  | 27.0 |  |
| PA RxBN | dBm | -40.9 |  | -34.6 |  |
| DPX_ISO_from Tx to Rx | dB | 50.0 |  | 50.0 |  |
| DPX ATT RX band | dB | 40.0 |  | 40.0 |  |
| Div_filter_Tx rejection | dB | 40.0 |  | 40.0 |  |
| Transceiver effective phase noise | dBc/Hz | -144.0 |  | -144.0 |  |
| Thermal noise at PRx/DRX ANT port | dBm/Hz | -164.0 |  | -164.0 |  |
| SNR requirement for QPSK | dB | -1.0 |  | -1.0 |  |
| TX noise power at PRX/DRx ANT port | dBm | -86.9 | -86.9 | -80.6 | -80.6 |
| TX IM2 due to wider CBW at PRx/DRx ANT port | dBm | -87.0 | -95.0 | -87.0 | -95.0 |
| Thermal noise at PRx/DRx ANT Port | dBm | -86.4 | -86.4 | -85.1 | -85.1 |
| Total noise level at PRx/DRx ANT Port | dBm | -83.0 | -84.3 | -79.6 | -80.1 |
| Desense. | dB | 2.7 |  | 5.2 |  |
| REFSENS | dBm | -86.7 |  | -82.9 |  |

Proposal 2:  REFSENS for 30MHz and 40MHz CBW operation are proposed highlighted as Table 7.3.2-1.

Table 7.3.2-1: Two antenna port reference sensitivity QPSK PREFSENS

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | Duplex Mode |
| n2 | 15 | -98.0 | -94.8 | -93.0 | -91.8 | [-90.8] | -86.7 | -82.9 |  | FDD |
|  | 30 |  | -95.1 | -93.1 | -92.0 | [-91.0] | -86.8 | -83.0 |  |  |
|  | 60 |  | -95.5 | -93.4 | -92.2 | [-91.1] | -86.9 | -83.1 |  |  |

Conclusion

In this contribution, we present PA Rx band noise of n2 with 30_40MHz CBW and propose reference sensitivity levels as well as UL configurations.

Proposal 1:

Use the UL configuration as highlighted in Table 7.3.2-3 for n2 REFSENS.

Table 7.3.2-3: Uplink configuration for reference sensitivity

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | Duplex Mode |
| n2 | 15 | 25 | 501 | 501 | 501 | [501] | 481 | 401 |  | FDD |
|  | 30 | 101 | 24 | 241 | 241 | [241] | 241 | 201 |  |  |
|  | 60 |  | 101 | 101 | 101 | [101] | 101 | 101 |  |  |
| n25 | 15 | 25 | 501 | 501 | 501 | 501 | 481 | 401 |  | FDD |
|  | 30 |  | 24 | 241 | 241 | 241 | 241 | 201 |  |  |
|  | 60 |  | 101 | 101 | 101 | 101 | 101 | 101 |  |  |
| NOTE 1: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1). |  |  |  |  |  |  |  |  |  |  |

Proposal 2:

REFSENS for 30MHz and 40MHz CBW operation are proposed highlighted as Table 7.3.2-1.

Table 7.3.2-1: Two antenna port reference sensitivity QPSK PREFSENS

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | Duplex Mode |
| n2 | 15 | -98.0 | -94.8 | -93.0 | -91.8 | [-90.8] | -86.7 | -82.9 |  | FDD |
|  | 30 |  | -95.1 | -93.1 | -92.0 | [-91.0] | -86.8 | -83.0 |  |  |
|  | 60 |  | -95.5 | -93.4 | -92.2 | [-91.1] | -86.9 | -83.1 |  |  |

References

[1]		R4-2103184, Way forward on the introduction of 25, 30 and 40MHz in band n2, AT&T