3GPP TSG-RAN WG4 Meeting #98-bis-e                                                         R4-2104729

Electronic Meeting, 12 - 20 Apr., 2021

Source: 	CATT

Title: 	Discussion on the UE RX RF requirements for 1024QAM for NR FR1

Agenda item:	8.15.2

Document for:	Approval

- Introduction

In RAN #90-e, the revised WID on Introduction of DL 1024QAM for NR FR1was approved. The main objectives for RAN4 in revised WID [1] are shown as follows:

- Specify corresponding UE and BS RF core requirements [RAN4] - UE and BS RF core requirements are specified for stationary wireless scenarios with up to 2 layer DL MIMO - The cell size(s) and type of stationary wireless scenarios for which UE and BS RF core requirements are defined will be studied and decided by RAN4.

In this contribution, we provide our views on UE RX RF requirements for 1024QAM for NR FR1.

- Discussion

2.1 Specifications Impacts for NR FR1 1024QAM

The impacted UE RF requirement related to modulation is UE maximum input level.

2.1.1 Maximum input level

For maximum input level requirement, the introduction of 1024QAM for NR FR1 will impact the following parts in TS 38.101-1[3]:

- 7.4	Maximum input level

- 7.4A.1	Maximum input level for Intra-band contiguous CA

For 1024QAM, since the required SNR is increased, the maximum input level need to be relaxed in theory to accommodate the required linearity of the Rx chain. However taking a look at LTE UE [2], the maximum input level for 1024QAM is not relaxed compared to 256QAM for both single carrier and intra-band CA. So we propose 0dB relaxation for NR UE as well for 1024QAM compared for both single carrier and intra-band CA.

Proposal 1: 0dB relaxation is proposed for 1024QAM compared with 256QAM for both single carrier and intra-band CA.

Table 1: Maximum input level

| Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| Power in Transmission Bandwidth Configuration | dBm | -252 |  |  |  | -242 | -232 | -222 | -212 | -202 |  |  |  |  |
|  |  | -273 |  |  |  | -263 | -253 | -243 | -233 | -223 |  |  |  |  |
|  |  | -274 |  |  |  | -264 | -254 | -244 | -234 | -224 |  |  |  |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum uplink configuration specified in Table 7.3.2-3 with PCMAX_L,f,c as defined in clause 6.2.4. NOTE 2: Reference measurement channel is A.3.2.3 or A.3.3.3 for 64 QAM. NOTE 3: Reference measurement channel is A.3.2.4 or A.3.3.4 for 256 QAM. NOTE 4: Reference measurement channel is FFS for1024 QAM. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 2: Maximum input level for Intra-band contiguous CA

| Rx Parameter | Units | NR CA Bandwidth Class |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | B | C | D |  |
| Power in largest transmission bandwidth configuration CC, Plargest BW | dBm | -232 | -232 | -252 |  |
|  |  | -253 | -253 | -273 |  |
|  |  | -254 | -254 | -274 |  |
| Power in each other CC | dBm | Plargest BW +10*log{(NRB,c*SCSc)/(NRB,largest BW*SCSlargest BW)} |  |  |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum uplink configuration specified in Table 7.3.2-3 with PCMAX_L,f,c as defined in clause 6.2.4. NOTE 2: Reference measurement channel is A.3.2.3 or A.3.3.3 for 64 QAM. NOTE 3: Reference measurement channel is A.3.2.4 or A.3.3.4 for 256 QAM. NOTE 4: Reference measurement channel is FFS for 1024 QAM. |  |  |  |  |  |

Proposal 2: It is proposed to adopt the changes as in Table 1 and Table 2 for 38.101 for 1024QAM.

- Conclusions

This contribution provides analysis on maximum input level for NR FR1 1024QAM. The following proposals are concluded as follows:

Proposal 1: 0dB relaxation is proposed for 1024QAM compared with 256QAM for both single carrier and intra-band CA.

Proposal 2: It is proposed to adopt the changes as in Table 1 and Table 2 for 38.101 for 1024QAM.

- References

- RP-202886,Revised WID on Introduction of DL 1024QAM for NR FR1,Nokia,RAN#90-e

- 3GPP TS 36.101 V17.0.0 (2020-12)

- 3GPP TS 38.101-1 V17.0.0 (2020-12)