3GPP TSG-RAN WG4 Meeting # 98-bis-e 		                       	        R4-2104780

Electronic Meeting, Apr. 12-20, 2021

Title:	LS on synchronous operation between Uu and SL in TDD band n79

Response to:

Release:	Rel-17

Work Item:	NR_SL_enh-Core

Source:	RAN4

To:	RAN1

Cc:

Contact person:

Name:			Yuan Gao

E-mail Address	gaoyuan@catt.cn

Attachments:	None

1. Overall Description:

Partially used SL with Uu in TDD band n79 irrespective of TDM or FDM is being discussed for Rel-17 SL enhancement in RAN4. In such a case, timing alignment between SL and Uu is an important issue, in which transmission timing between SL and Uu and synchronization reference source of SL are involved.

In Rel-16 NR V2X, SL transmission timing is aligned with DL timing of Uu based on RAN1 agreements. The same SL transmission timing if applied to Rel-17 SL in the scenario of partially used SL with Uu in TDD band n79 will give rise to interference problem between SL and Uu. On the other band, Uu uses network as synchronization reference source, while SL can use GNSS, network and SyncRef UE as synchronization reference source. Probably different synchronization reference sources between SL and Uu will also lead to timing misalignment between SL and Uu.

Therefore, RAN4 respectfully request RAN1 to clarify the following questions regarding partially used SL with Uu in TDD band n79:

Question 1: If RAN4 specify SL transmission timing to be aligned with Uu UL timing, does it have any impact or conflict from RAN1 perspective?

Question 2: If RAN4 specify only network as synchronization reference source of SL in case of partially used SL with Uu in TDD band n79, does it have any impact or conflict from RAN1 perspective?

2. Actions:

To RAN WG1:

RAN4 respectfully request RAN1 to clarify the above questions regarding partially used SL with Uu in TDD band n79.

3. Date of Next TSG-RAN WG4 Meetings:

TSG-RAN WG4 Meeting #99-e	May. 19 – May. 27, 2021			Online

TSG-RAN WG4 Meeting #100-e	Oct. 11 – Oct. 15, 2021		             Online