3GPP TSG-RAN WG4 Meeting # 98-bis-e 	R4-2104800

Electronic Meeting, Apr. 12-20, 2021

Title: 	Discussion on other requirements for FR2 NR repeater

Source: 	CATT

Agenda item:	8.11.3.3

Document for:	Approval

- Introduction

The radiated requirements were discussed in last RAN4 meeting and all of the issues were open. There’s no conclusion on whether radiated requirements for FR1 is defined, this contribution provides our further analysis for FR2 NR repeater. The power, emission requirements are discussed in [4] and [5].

- Discussion

As there’s no agreement on what requirement will be defined for NR repeater. E-UTRA FDD repeater and BS requirements and are taken as reference to review how to define them for NR repeater and if there’s any other requirement needed.

  - Frequency stability

Our view on frequency stability doesn’t change that E-UTRA repeater requirement, i.e. 0.01 ppm is reused for both FDD and TDD NR repeater [3].

Proposal 1: E-UTRA repeater requirement, i.e. 0.01 ppm is reused for both access link and backhaul link.

  - EVM

EVM requirement in theory is the SNR requirement for the Tx signals. IAB EVM requirements are the same as BS from SNR perspective for both downlink and uplink. The system performance can be thought as the same as BS to UE or UE to BS because IAB can regenerate signal with good SNR. Repeater’s EVM requirements need some discussion because repeater just retransmits the signals and extra noise is added to the signals after repeater amplifies and forwards the signal from donor BS. In order not to degrade the signal too much, keeping the noise level from repeater as low as possible is important. However, the contribution from RF hardware part especially is usually much larger than BB part in the BS design, it’s difficult to limit repeater’s EVM requirement to a very low level considering the PA efficiency and power consumption. LTE repeater EVM requirement is 8%. According to the following equation,

EVMtotal=EVM12+EVM22

8% or 6% repeater EVM performance impact on the total EVM can be calculated as Table 1,

Table 1: Repeater EVM’s impact to total EVM

| EVM_received signal(%) | 10 | 10 | 8 | 8 | 6 | 6 |
| --- | --- | --- | --- | --- | --- | --- |
| EVM_repeater(%) | 8 | 6 | 8 | 6 | 8 | 6 |
| EVM_total(%) | 12.8 | 11.7 | 11.3 | 10.0 | 10.0 | 8.5 |

As shown in Table 1, 8% EVM requirement impact to the final EVM is a little large. So we propose to define the requirement as 6%.

Proposal 2: EVM requirements for FR2 NR repeater are defined as 6% for both access link and backhaul link.

  - Out of band gain

Out of band gain is a little complicated that E-UTRA repeater requirement was derived from co-existence analysis and simulation. How to derive NR repeater requirement may be discussed.

Observation 1: Out of band gain requirement may need co-existence analysis or simulation.

  - Input intermodulation

In IAB-MT discussion, the understanding is that no narrow band system exists in mmWave band then no Rx IMD requirement for IAB-MT. For FR2 repeater backhaul link, we think that understanding also applies.

Observation 2: Input intermodulation may not be needed for backhaul link.

The input IMD for access link needs more discussion because BS requirements can’t be reused.

  - Output intermodulation

There’s no output intermodulation requirement for FR2 BS considering the large coupling loss in FR2. The analysis also applies to FR2 repeater access link and backhaul link.

Observation 3: Output intermodulation may not be needed for both backhaul link and access link.

  - Others

In last meeting, there were proposals that REFSENS should be defined. REFSENS needs the whole BS or UE receiver which is not necessarily implemented in repeater because repeater just retransmits the analog signal. So REFSENS can’t be defined. Usually, noise figure can be considered as the SNR degradation performance for repeater. But noise figure was not defined and tested for BS/UE/IAB/repeater before. It may not be easy to define it.

- Conclusion

This contribution provides our further analysis for the FR2 NR repeater requirements, we have the following observations and proposals.

Proposal 1: E-UTRA repeater requirement, i.e. 0.01 ppm is reused for both access link and backhaul link.

Proposal 2: EVM requirements for FR2 NR repeater are defined as 6% for both access link and backhaul link.

Observation 1: Out of band gain requirement may need co-existence analysis or simulation.

Observation 2: Input intermodulation may not be needed for backhaul link.

Observation 3: Output intermodulation may not be needed for both backhaul link and access link.

Reference

[1] R4-2103950, Email discussion summary for [98e][312] NR_Repeater_General

[2] R4-2103951,	Email discussion summary for [98e][313] NR_Repeater_RF

[3] R4-2100378, Discussion on NR repeater EVM and frequency stability for FR1/FR2, CATT

[4] R4-2104798, Discussion on power requirement for FR2 NR repeater, CATT

[5] R4-2104799, Discussion on emission requirements for FR2 NR repeater, CATT