3GPP TSG-RAN4 Meeting #98bis-e	R4-2104831

# Online, 12 – 20 April, 2021

Agenda item:	8.4.2.1

Source:	Apple

Title:	On SRS antenna port switching

Document for:	Discussion

# Introduction

In last RAN4 meeting, the RRM requirement for SRS antenna port switching has been discussed and the conclusions were captured in the WF[1]. However, there are couple of open issues from last meeting, and in this contribution, we continue discussing the RRM requirement for SRS antenna port switching.

# Scope of SRS antenna switching requirement

In last meeting, some agreements on scope of SRS antenna switching requirement were,

- Issue 1-1-1: whether interruption requirement would be defined in RRM for SRS antenna port switching - Agreement: RAN4 to define interruption requirements for SRS antenna port switching - Issue 1-1-2: whether delay requirement would be defined in RRM for SRS antenna port switching - Agreements - RRM delay requirement for SRS antenna port switching is FFS - Option 1: Do not define SRS antenna port switching delay requirement in RRM. - Option 2: Define SRS antenna port switching delay requirement same as RF retuning time. - Option 3: Define SRS antenna port switching delay requirement. FFS for the value. At least RF retuning time shall be included. - Issue 1-1-3: Impact of SRS antenna port switching to other RRM requirements - Agreements - Further identify impact of SRS antenna port switching on RRM requirements, e.g. - Timing measurements and corresponding measurement requirements - Other RRM requirements - Issue 1-1-4: RAN4 defines the requirement only for SRS antenna port switching in FR1 or in both FR1 and FR2 - Agreements - Define the RRM requirements for SRS antenna port switching for FR1. - FFS for FR2 SRS antenna port switching requirements: - Further identify the applicability of the existing RF transient period for SRS antenna port switching.

In current RAN4 TS38.133, we have defined interruption requirement for SRS carrier-based switching and some of the clarifications are added for the impacting to other RRM requirements, which is like the following wording:

And the transient period for SRS antenna port switching is defined in the time mask requirement in TS38.101-1, as duplicated below,

The switching delay time is clearly specified in the above RF requirement. In RRM, the granularity of the delay requirement is in slot level, and it’s much coarser than RF requirement. Thus, it’s no need to define the SRS antenna port switching delay requirement in RRM spec.

Proposal 1: Do not define SRS antenna port switching delay requirement in RRM.

Regarding the impact of SRS antenna port switching to other RRM requirement, we think the same methodology in LTE spec could be re-used. In LTE TS36.133, RAN4 has added some clarification in the impacted RRM requirements, e.g.,

So, we could clarify the relaxation/applicability in those RRM requirements whose DL RS or UL RS could be interrupted by SRS antenna port switching, e.g., the delay requirement could be extended, or the requirement only applies when SRS antenna port switching is not colliding with the reference signal.

Proposal 2: Regarding the impact of SRS antenna port switching to other RRM requirements, RAN4 would clarify the relaxation or applicability in those RRM requirements whose wanted DL RS or UL RS could be interrupted by SRS antenna port switching, e.g., the delay requirement could be extended if SRS antenna port switching happens during the UE procedure, or the requirement only applies when SRS antenna port switching is not colliding with the reference signal.

In current RF spec TS38.101-1 for FR1 case, there is very clear time mask definition for SRS antenna port switching, however, in TS38.101-2 for FR2 case, no antenna port switching related time mask is mentioned. In RAN1 spec TS38.214, the guard period for antenna port switching covers both FR1 and FR2 cases, and that’s a different assumption from RAN4 RF. In order to mitigate the mismatch between RAN1 and RAN4, we think a LS to RAN1 may help to clarify since the SRS antenna port switching design was initially discussed in RAN1. Otherwise, RAN4 shall focus on FR1 case only.

Proposal 3: RAN4 defines the RRM requirements for SRS antenna port switching for FR1.

# Interruption requirement applicability

In last meeting, some agreements on interruption requirement applicability were,

- Issue 1-2-1: Interruption requirement applicability - FFS: - Option 1 (MTK, Huawei, OPPO, NEC, Xiaomi): The interruption requirement should base on the band combination capability reporting by UE. - Option 1a: The interruption requirement should base on the band combination capability (indicated by txSwitchImpactToRx or txSwitchWithAnotherBand) reporting by UE. - Option 2 (Huawei (observation 1), Nokia (proposal 5), NEC, Apple, vivo, CATT, QC): For the interruption cause by the switching period, whether the victim CCs are same as those indicated by the IEs (txSwitchImpactToRx or txSwitchWithAnotherBand) needs further discussion. - Option 3 (Ericsson): Needs further checking and discussion. - Issue 1-2-2: Victim cell type impacted by SRS antenna port switching - Agreement: - In the R17 FeRRM WI only RRM requirement of NR SRS antenna port switching would be discussed, including following cases (but all the victim CCs shall be determined by the conclusion in issue 1-2-1): - NR SRS antenna port switching impacting LTE CC - NR SRS antenna port switching impacting NR CC - Issue 1-2-3: whether same interruption requirement applies to different SRS antenna port switching patterns - FFS: - Option 1 (vivo, OPPO, NEC, Apple, Xiaomi, Intel, CATT, QC): use same set of requirements for different SRS antenna switch patterns - Option 2 (LGE): Depending on UE capability supportedSRS-TxPortSwitch, the interruption requirements should be defined. - Option 3 (Ericsson, MTK, Nokia): further discussion is needed.

In RAN2 TS38.306 spec, it was defined that,

When UE supports SRS antenna port switching for DL CSI acquisition, it could indicate the capability of the impact to other Rx. And in RAN4 RF spec TS38.101, the SRS time mask has been defined, in which the uplink transmission on other uplink CCs would also be impacted since the SRS antenna switching without involving carrier frequency change needs to conduct the PA ramp down/up and antenna switching from one port to the other port. Since in TS38.306 txSwitchImpactToRx and txSwitchWithAnotherBand are used to indicate such impacts for BC respectively, it does not need to use other capability indication to determine the interruption applicability, e.g., per-FR MG or per-UE MG capability is not needed to determine whether interruption from one CC to other CC is needed or not.

Proposal 4: The interruption requirement of SRS antenna port switching should base on the band combination capability (indicated by txSwitchImpactToRx or txSwitchWithAnotherBand) reporting by UE.

Regarding the issue 1-2-3 “whether same interruption requirement applies to different SRS antenna port switching patterns”, even though UE may have different switching pattern for different capability of “supportedSRS-TxPortSwitch” and the number of port switching could be different, the interruption requirement shall be defined based on a generic manner rather than in a pattern-specific manner. So, we propose,

Proposal 5: RAN4 uses same interruption requirement applies to different SRS antenna port switching patterns.

# Interruption requirement design for SRS antenna port switching

In last meeting, some agreements on interruption requirement design were,

- Issue 1-3-1: The interruption requirement is defined based on slot level or symbol level - FFS: - Option 1 (Apple, QC, MTK, Huawei, OPPO, Xiaomi, vivo, CATT, Intel): based on slot level - Option 2 (LGE, NEC, Huawei, Ericsson): based on symbol level - Option 3 (Nokia, CMCC): FFS - Issue 1-3-2: The components within interruption time of SRS antenna port switching in FR1 - FFS: - Option 1 (Xiaomi, vivo, Huawei, OPPO, Intel, CATT): includes antenna switching time and SRS transmission time - Option 1a (QC, NEC, LG, MTK): includes antenna switching time and SRS transmission time; and transient periods before and after SRS transmission slot have to be taken into consideration. - Option 1b (Ericsson, Xiaomi, NEC, LG): includes antenna switching time and SRS transmission time; and other components can be further discussed. - Option 2 (Ericsson): FFS on followings - FFS: time to switch to transmit, - FFS: time to switch back, - FFS: guard symbol(s), - FFS: SRS transmission. - Option 3 (Apple, Nokia): only antenna switching time - If the transient period (15us) can be captured within the guard period in FR1, the UL interruption at SRS antenna switching shall be defined based on the minimum guard period specified in RAN1

The guard period in RAN1 TS38.214 is defined as below,

In our understanding, RAN1 has chosen to use the most conservative way to define the guard period and no signals would be expected to transmit in guard period. We propose to use the RAN1 guard period as baseline to determine the interruption requirement. However, similar as interruption requirements for SRS carrier-based switching, even though the GP is at symbol level, but the interruption requirement shall still be defined in unit of slot for NR and in unit of subframe for LTE. In last meeting, some companies thought the interrupting requirement may be defined in unit of symbols since transient time for SRS antenna port switching is 15us from RF spec, but we have different understanding: with the MTTD/MRTD and TA, we cannot precisely determine which DL symbols or UL symbols on other CCs are interrupted by SRS antenna port switching, and therefore using slot level interruption requirement would be a reasonable solution.

Proposal 6: Interruption requirement of SRS antenna port switching shall be defined based on slot level for NR victim CC and based on subframe level for LTE victim CC respectively.

Regarding the components within interruption time of SRS antenna port switching in FR1, the following figure illustrates the SRS antenna port switching in a UL slot. Based on proposal 5, we think the interruption requirement shall apply for all possible switching patterns, and therefore the minimum requirement in the worst case shall consider following components in the SRS antenna port switching:

- antenna port switching time (transient time)

- SRS transmission time

- Transient time before and after SRS transmission occasion (10us as shown in figure 1)

Figure 1. Illustration of SRS antenna port switching in a uplink slot

Thus, we propose,

Proposal 7: The components within interruption time of SRS antenna port switching in FR1 include:

- SRS antenna port switching time (transient time)

- SRS transmission time

- Transient time before and after SRS transmission occasion

With proposal 7, the total interruption time due to SRS antenna port switching in one UL slot could be:

- 6 symbols, accounts for SRS GP and SRS transmission symbols, that is up to 6 symbols, and

- 2*10us, accounts for transient time before and after SRS transmission occasion

Even though some SRS resource sets for antenna switching may take two slots (one slot can only be used for 3 SRS symbols if those 3 SRS symbols are with different antenna port), RAN4 does not need to specialize this case and the interruption could be checked slot by slot. For instance, in T1R4 case, UE may use 2 slots for 4 times of SRS antenna port switching, but the interruption requirement in each slot could be same, and we only need to determine the interruption length in each slot. So, the interruption time due to SRS antenna port switching in one UL slot is: 6 symbols + 20us.

Proposal 8: total interruption time due to SRS antenna port switching in one UL slot could be 6 symbols + 20us.

In last meeting the other open issues for interruption requirement design are as below,

- Issue 1-3-5: Would the interruption requirement based on different SCS? - FFS: - Option 1 (Apple): Interruption requirement is based on the victim CC SCS. - Option 2 (QC, MTK, vivo, OPPO, LG, CATT): Interruption requirement is based on the aggressor CC and victim CC SCS. - Option 3 (Ericsson, Apple, Xiaomi, Nokia, Intel, CATT, Huawei): - The interruption requirement depends at least on the SCS of the victim cell. - FFS: the interruption requirement depends on the SCS of aggressor cell. - Issue 1-3-6: Would the interruption requirement differentiate between sync and async cases? - FFS: - Option 1 (Apple, QC, MTK, OPPO, Xiaomi): No; based on the async case for the minimum requirement. - Option 2 (LGE, CATT, Ericsson, Nokia): Yes. - Option 3 (Huawei): depends on whether to have slot level interruption or symbol level interruption. Need FFS. - Issue 1-3-7: Interruption requirement for UE with or without per-FR MG capability - FFS: - Option 1 (Apple): - For UE capable of per-FR MG, the interruption is only allowed to a victim serving CC if NR SRS antenna port switching happens in the same FR as this victim serving CC. - For UE not capable of per-FR MG, the interruption is always allowed to a victim serving CC regardless of whether NR SRS antenna port switching happens in the same FR as this victim serving CC. - Option 2 (MTK, Huawei, Xiaomi, CATT, Apple): No need to define the UE (not) capable of per-FR gaps requirement for SRS antenna port switching in RAN4. - Option 3 (OPPO, QC, Ericsson, Nokia): FFS after addressing other issues

Since the symbol length in proposal 8 is associated with aggressor UL carrier SCS, the interruption requirement shall be based on both aggressor CC and victim CC SCS.

Proposal 9: Interruption requirement is based on the aggressor CC and victim CC SCS.

We also need to consider the TA between DL and UL as well as the time difference between CCs in CA. Regarding the different CA scenarios, in R16 carrier based SRS switching, we have conclusion in R4-2012155 that,

- Interruption requirements for CA case 1, case 2 and case 3

  - The same interruption requirements as for async case in the spec shall apply

Where,

  - Case 1: CA is co-location deployed

  - Case 2: Single TAG CA, or carriers in the same TAG for multiple TAG CA

  - Case 3: uplink time difference does not exceed a threshold X

    - X = [5] us

So here, we think the same principle shall be applied.

Proposal 10: Interruption requirement is based on the async case for the minimum requirement.

As analyzed for proposal 4 in this paper, there is no need to define the UE (not) capable of per-FR gaps requirement for SRS antenna port switching in RAN4. But the indication of txSwitchImpactToRx or txSwitchWithAnotherBand is not allowed to indicate any band combination cross FR1 and FR2 if UE is capable of per-FR MG, and RAN4 could add such clarification in the requirement applicability condition.

Proposal 11: No need to differentiate the requirement for the UE with or without capability of per-FR gap for SRS antenna port switching in RAN4. But in the interruption requirement applicability condition, RAN4 shall clarify that the indication of txSwitchImpactToRx or txSwitchWithAnotherBand is not allowed to indicate any band combination cross FR1 and FR2 if UE is capable of per-FR MG.

Based on all of the above analysis, the interruption requirement could be defined as below,

Proposal 12: the interruption requirement of SRS antenna port switching is summarized as:

| Victim CC SCS(kHz) | Aggressor CC SCS (kHz) |  |  |
| --- | --- | --- | --- |
|  | 15 | 30 | 60 |
| 15 (NR or LTE) | 2 | 2 | 2 |
| 30 | 2 | 2 | 2 |
| 60 | 3 | 2 | 2 |

Unit of interruption requirement is slot for NR and subframe for LTE.

# Conclusion

In this contribution we discuss the preliminary RRM requirements for SRS antenna port switching.

Proposal 1: Do not define SRS antenna port switching delay requirement in RRM.

Proposal 2: Regarding the impact of SRS antenna port switching to other RRM requirements, RAN4 would clarify the relaxation or applicability in those RRM requirements whose wanted DL RS or UL RS could be interrupted by SRS antenna port switching, e.g., the delay requirement could be extended if SRS antenna port switching happens during the UE procedure, or the requirement only applies when SRS antenna port switching is not colliding with the reference signal.

Proposal 3: RAN4 defines the RRM requirements for SRS antenna port switching for FR1.

Proposal 4: The interruption requirement of SRS antenna port switching should base on the band combination capability (indicated by txSwitchImpactToRx or txSwitchWithAnotherBand) reporting by UE.

Proposal 5: RAN4 uses same interruption requirement applies to different SRS antenna port switching patterns.

Proposal 6: Interruption requirement of SRS antenna port switching shall be defined based on slot level for NR victim CC and based on subframe level for LTE victim CC respectively.

Proposal 7: The components within interruption time of SRS antenna port switching in FR1 include:

- SRS antenna port switching time (transient time)

- SRS transmission time

- Transient time before and after SRS transmission occasion

Proposal 8: total interruption time due to SRS antenna port switching in one UL slot could be 6 symbols + 20us.

Proposal 9: Interruption requirement is based on the aggressor CC and victim CC SCS.

Proposal 10: Interruption requirement is based on the async case for the minimum requirement.

Proposal 11: No need to differentiate the requirement for the UE with or without capability of per-FR gap for SRS antenna port switching in RAN4. But in the interruption requirement applicability condition, RAN4 shall clarify that the indication of txSwitchImpactToRx or txSwitchWithAnotherBand is not allowed to indicate any band combination cross FR1 and FR2 if UE is capable of per-FR MG.

Proposal 12: the interruption requirement of SRS antenna port switching is summarized as:

| Victim CC SCS(kHz) | Aggressor CC SCS (kHz) |  |  |
| --- | --- | --- | --- |
|  | 15 | 30 | 60 |
| 15 (NR or LTE) | 2 | 2 | 2 |
| 30 | 2 | 2 | 2 |
| 60 | 3 | 2 | 2 |

Unit of interruption requirement is slot for NR and subframe for LTE.

# References

[1] R4-2103672, WF on further RRM enhancement for NR and MR-DC - SRS antenna port switching, Apple, RAN4 #98e meeting