3GPP TSG-RAN WG4 Meeting # 98-bis-e	R4-2104839

# Electronic Meeting, Apr. 12-20, 2021

Agenda item:	5.1.4.2

Source:	Apple Inc.

Title:	Simulation results for PDSCH demodulation requirements in NR-U

Release:	Rel-16

Document for:	Discussion

# 1. Introduction

In RAN4#98e UE demodulation requirements for NR-U was discussed and way forward [1] was agreed. In this contribution we present simulation results for the agreed simulation assumptions.

# 2. Simulation Results

In RAN4#98e the simulation assumptions were agreed and captured in Appendix. Additional simulation parameters used in the results are provided below.

Probability of LBT Failure PLBT :0.25, 0.5

Duration of the Downlink portion of the COT (S1): {2,3,5,6} slots

Length of last slot of the Downlink portion of the COT (S2): {6,9,12,14} symbols

Table 1: Results with PLBT =0.25 with 2RX antenna

| Test num. | CBW (MHz) / SCS (kHz) | MCS | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | 20/30 | 13 | TDLA30-10 | 2x2, ULA Low | 70 | 12.5 |
| 1-2 | 40/30 | 13 | TDLA30-10 | 2x2, ULA Low | 70 | 12.6 |
| 1-3 | 60/30 | 13 | TDLA30-10 | 2x2, ULA Low | 70 | 12.5 |
| 1-4 | 80/30 | 13 | TDLA30-10 | 2x2, ULA Low | 70 | 12.9 |

Table 2: Results with PLBT =0.5 with 2RX antenna

| Test num. | CBW (MHz) / SCS (kHz) | MCS | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | 20/30 | 13 | TDLA30-10 | 2x2, ULA Low | 70 | 12.4 |
| 1-2 | 40/30 | 13 | TDLA30-10 | 2x2, ULA Low | 70 | 12.6 |
| 1-3 | 60/30 | 13 | TDLA30-10 | 2x2, ULA Low | 70 | 12.5 |
| 1-4 | 80/30 | 13 | TDLA30-10 | 2x2, ULA Low | 70 | 13.0 |

Table 3: Results with PLBT =0.25 with 4RX antenna

| Test num. | CBW (MHz) / SCS (kHz) | MCS | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | 20/30 | 13 | TDLA30-10 | 2x4, ULA Low | 70 | 8.5 |
| 1-2 | 40/30 | 13 | TDLA30-10 | 2x4, ULA Low | 70 | 8.7 |
| 1-3 | 60/30 | 13 | TDLA30-10 | 2x4, ULA Low | 70 | 8.6 |
| 1-4 | 80/30 | 13 | TDLA30-10 | 2x4, ULA Low | 70 | 8.8 |

Table 4: Results with PLBT =0.5 with 4RX antenna

| Test num. | CBW (MHz) / SCS (kHz) | MCS | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | 20/30 | 13 | TDLA30-10 | 2x4, ULA Low | 70 | 8.5 |
| 1-2 | 40/30 | 13 | TDLA30-10 | 2x4, ULA Low | 70 | 8.8 |
| 1-3 | 60/30 | 13 | TDLA30-10 | 2x4, ULA Low | 70 | 8.7 |
| 1-4 | 80/30 | 13 | TDLA30-10 | 2x4, ULA Low | 70 | 8.9 |

# 3. Conclusion

In this paper we present simulation results for PDSCH demodulation requirements in NR-U.

# Reference

- R4-2103987, “Way Forward on NR-U UE demodulation requirements”, Qualcomm.

# Appendix

Simulation parameters for PDSCH demodulation requirements in NR-U.

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Active DL BWP index |  |  | 1 |
| Slot Pattern |  |  | [TBD, according to decision in the WF] |
| LBT failure probability |  |  | [0.50, 0.25] |
| Common serving cell parameters | Physical Cell ID |  | 0 |
|  | SSB position in burst |  | The first SSB |
|  | SSB Q factor |  | 8 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | According to DL Transmission Model |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| RB Allocation |  |  | Full BW |
| MIMO Layers |  |  | 2 |
| Number of HARQ Processes |  |  | 8 |
| TX EVM |  |  | 6% |
| DL Transmission Model | Maximum COT Duration | ms | [4] |
|  | Idle Time after COT | ms | [1] |
|  | DL Transmission Model Period/ Fixed Frame Period (Note 1) | ms | 5 |
|  | Probability of LBT Failure pLBT |  | [0.25, 0.5] |
|  | Guard Symbols |  | [TBD] |
|  | UL Symbols |  | [TBD] |
|  | Number of slots between PDSCH and corresponding HARQ-ACK information |  | [TBD, HARQ-ACK is sent in any case within the FFP and minimum 1 slot after the end of the allocated Downlink portion] |
|  | Duration of the Downlink portion of the COT | Slots | [{2,3,5,6} (baseline), {2,4,6,7}] |
|  | PDSCH Allocation in the last Slot of the Downlink portion of the COT | Symbols | [{6,9,12,14} (baseline), {5-14}] |
| Notes: - The Fixed Frame Period denomination applies only for ChannelAccessType-r16 = ‘semistatic’. For ChannelAccessType-r16 = ‘dynamic’ this parameter is identified only as DL Transmission Model Period. |  |  |  |