3GPP TSG-RAN WG4 Meeting # 98-bis-e	R4-2104845

# Electronic Meeting, Apr. 12-20, 2021

Agenda item:	8.6.3.3

Source:	Apple Inc.

Title:	On Enhanced transmission schemes for HST

Release:	Rel-17

Document for:	Discussion

# 1. Introduction

In RAN4#98e UE demodulation for FR1 HST was discussed and way forward [1] was agreed. Agreements in [1] related to enhanced transmission schemes for HST:

- Whether to specify PDSCH requirements for transmission scheme 2 - Firstly, focus on the study and evaluate the performance benefits of transmission scheme 2 in HST-SFN deployment comparing to other transmission schemes. - Channel model - Set Ds=700m and Dmin=150m as baseline - Take maximum Doppler frequency “870Hz for 15KHz SCS, 1667Hz for 30KHz SCS” as baseline - Further check whether Doppler/timing offset may exceed maximum UE capability with the assumption of Non-ideal synchronization - Time/frequency errors and assumptions on UE synchronization - Single FFT operation can be assumed as baseline UE implementation for HST scenario - Non-ideal synchronization between RRHs should be considered as a baseline assumption - PDSCHs allocations between different RRHs - Baseline assumption for evaluation: reuse the same PRB allocation as Rel-16 eMIMO multi-DCI based transmission, i.e., overlapped in time domain but not overlapped in frequency domain

In this contribution we present our views on enhanced transmission schemes for HST.

# 2. Discussion

For the simulation assumptions agreed in [1], we evaluated the performance in HST with multi-DCI transmission scheme.

Simulation assumptions:

Channel model: HST-DPS with visibility -Ds to Ds

Antenna configuration: 2x2

MCS: 17

Rank: Rank 2

- DMRS ports 1000/1001 from TRP #(2i)

- DMRS ports 1002/1003 from TRP #(2i+1)

DMRS type: Type 1

Number of DMRS symbols: DMRS 1+1+1

TRS periodicity: 10ms

PDSCH Allocation: Non-overlapping between 2 TRP

Table 1: Performance with Multi-DCI transmission scheme for HST

| Transmission Scheme | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Number of active PDSCH TCI states | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| Multi-DCI | 10 / 15 | 64QAM, 0.43 | HST-DPS-2 | 1 | 2x2 | 70 | 11.21 |
| DPS | 10 / 15 | 64QAM, 0.43 | HST-DPS | 1 | 2x2 | 70 | 10.96 |

Based on the results in table 1, the performance between DPS and multi-DCI transmission scheme is comparable.

Observation #1: The performance with multi-DCI transmission scheme is comparable to DPS for HST.

Given that multi-DCI transmission scheme is subject to UE capability indication and performance is comparable to HST-DPS with no additional UE capability requirement, we don’t see a strong reason to introduce requirements with multi-DCI transmission in HST.

Proposal #1: Do not introduce requirements with enhanced transmission scheme for HST.

Also, the performance evaluation is based on the HST-DPS channel from R16. As we discussed in our companion paper [2], the channel model might need to be evaluated for HST with multi-TRP transmission scheme as well, in case there is consensus to introduce requirements with multi-DCI transmission scheme for HST.

Proposal #2: RAN4 further discuss the channel model for multi-TRP transmission in HST.

# 3. Conclusion

In this paper, we provide our views on enhanced transmission schemes for HST. Our observations and proposals are captured below:

Observation #1: The performance with multi-DCI transmission scheme is comparable to DPS for HST.

Proposal #1: Do not introduce requirements with enhanced transmission scheme for HST.

Proposal #2: RAN4 further discuss the channel model for multi-TRP transmission in HST.

# Reference

- R4-2103876, “WF on FR1 HST demodulation”, CMCC.

- R4-2104844, “On PDSCH CA Requirements in HST”, Apple.