3GPP TSG-RAN WG4 Meeting # 98-bis-e	R4-2104846

# Electronic Meeting, Apr. 12-20, 2021

Agenda item:	8.14.2.1

Source:	Apple Inc.

Title:	On PDSCH requirements in intercell interference scenarios

Release:	Rel-17

Document for:	Discussion

# 1. Introduction

In RAN#89e the Work Item for Further enhancement on NR demodulation performance was approved. The WID [1] includes an objective for MMSE-IRC receiver for suppressing inter-cell interference.  In this contribution we present our views on introducing requirements for PDSCH demodulation in the presence of inter-cell interference.

# 2. Discussion

In [1] the objective for requirements with intercell interference are:

- MMSE-IRC receiver for suppressing inter-cell interference - Type of requirements: - Define PDSCH demodulation requirements - Further decide whether to introduce the corresponding CQI reporting requirements during the WI - SCS and slot duration - Scenario 1: Slot-based transmission and aligned SCS among cells (first priority) - Scenario 2: Non-slot-based transmission and aligned SCS among cells (second priority) - Further discuss the assumptions for requirements definition - Note: Requirements for scenario 1 will first be stabilized before commencing scenario 2. - Reference receiver: - MMSE-IRC with DMRS based interference covariance estimation - Note: use the DRMS for target UE’s PDSCH. - Target frequency: FR1 - Rx antenna number: 2Rx and 4Rx - Interference profile - Reuse LTE interference profiles as a starting point - Other interference profiles are not precluded

We present some initial evaluation results for PDSCH demodulation in the presence of inter-cell interference. The simulation parameters are provided in table 1.

Table 1: Simulation parameters for PDSCH demodulation with ICI

| Parameter | Cell1 | Interferer Cell |
| --- | --- | --- |
| Duplex mode | FDD |  |
| DIP | N/A | -1.73dB |
| Active DL BWP index | 1 | N/A |
| CBW/SCS | 10 MHz/ 15 KHz |  |
| RB allocation | Full bandwidth |  |
| MIMO layer | Rank 1,2 | Rank 1,2 |
| Antenna Config and Correlation | 2x2, ULA Low |  |
| Channel | TDLA30-10 |  |
| PDSCH configuration |  |  |
| Mapping type | Type A | Type A |
| Starting symbol (S) | 2 | 2 |
| Length (L) | 12 | 12 |
| PDSCH DMRS configuration | Type 1 1+1 | Same Type/ Position as Cell1 – On reserved CDM grp of Cell1, 2 CDM Grps without data Non-overlapping with cell1 (Different Type/ position) |
| Precoding Granularity | Wideband Randomly generated per slot |  |
| HARQ Parameters |  |  |
| Number of HARQ Processes | 8 | N/A |
| Maximum HARQ transmission | 4 | N/A |

We evaluated performance with rank 1,2 transmission on both the serving cell and interferer. Slot based transmission is considered for both the serving and interferer cell. There are no DTXed slots for the interferer. With slot-based transmission, 2 cases for DMRS between serving cell and interferer are considered:

Case1: DMRS of Cell 2 is the same Type/ position as Cell 1, but on different CDM grp compared to serving cell and configured with 2 CDM grps without data. For this case serving cell DMRS is “blind” to interference.

Case2: DMRS of serving cell is different Type/ position compared to serving cell. Serving cell DMRS has interference from either PDSCH or DMRS of interferer.

Results are presented for QPSK, 16QAM, 64QAM and 256QAM with combinations of Rank 1 and 2 on serving and interferer respectively.

Figure 1: PDSCH performance in ICI for QPSK

Figure 2: PDSCH performance in ICI for 16QAM

Figure 3: PDSCH performance in ICI for 64QAM

Figure 4: PDSCH performance in ICI for 256QAM

Based on the performance evaluation results, we observe that for the case where interference is blind to serving cell DMRS, MMSE-IRC with DMRS based covariance estimation suffers degradation in performance.

Observation #1: DMRS based interference covariance estimation suffers degradation when there is no interference on DMRS REs.

For requirements with DMRS based interference covariance estimation, we propose to limit the scenarios to where interference can be rejected with DMRS based covariance estimation.

Proposal #1: For requirements with DMRS based interference covariance estimation, limit scenarios to cases where interference can be rejected with DMRS based covariance estimation.

Higher order modulation like 64QAM and 256QAM are more suitable for noise limited scenarios and hence propose to only consider introducing requirements for QPSK and 16QAM with ICI.

Proposal #2: Introduce requirements with ICI for QPSK and 16QAM.

RAN4 should further discuss the interference model used especially for the % DTX slots, distribution of rank and MCS for PDSCH of interference.

Proposal #3: Further discuss interference model for NR.

# 3. Conclusion

In this paper, we provide our views on requirements for PDSCH demodulation in the presence of inter-cell interference. Our observations and proposals are captured below:

Observation #1: DMRS based interference covariance estimation suffers degradation when there is no interference on DMRS REs.

Proposal #1: For requirements with DMRS based interference covariance estimation, limit scenarios to cases where interference can be rejected with DMRS based covariance estimation.

Proposal #2: Introduce requirements with ICI for QPSK and 16QAM.

Proposal #3: Further discuss interference model for NR.

# Reference

- RP-210920, “WID on Further Enhancement on NR Demodulation Performance”, China Telecom.