3GPP TSG-RAN WG4 Meeting # 98-bis-e	R4-2104848

# Electronic Meeting, Apr. 12-20, 2021

Agenda item:	12.1

Source:	Apple Inc.

Title:	Discussion on reply LS on TCI State Update for L1/L2-Centric Inter-Cell Mobility

Release:	Rel-17

Document for:	Discussion

# 1. Introduction

RAN4 has received an LS from RAN1 on TCI State Update for L1/L2-Centric Inter-Cell Mobility [1]. In this contribution we discuss the questions raised by RAN1 and propose replies to those. The draft LS reply is provided in the Appendix.

# 2. Discussion

RAN1 discussed the support of L1/L2-centric inter-cell mobility and made the following agreement.

Agreement On Rel.17 enhancements for L1/L2-centric inter-cell mobility, - Discuss whether to support at least the source RS types already agreed for intra-cell mobility for the purpose of referencing to non-serving cell(s). Note: This implies the following source RS(s): - CSI-RS for BM configured for non-serving cell(s) for DL QCL and UL TX spatial references - CSI-RS for tracking (TRS) configured for non-serving cell(s) for DL QCL and UL TX spatial references - SSB configured for non-serving cell(s) for UL TX spatial references - SRS for BM configured for non-serving cell(s) for UL TX spatial references - FFS: whether to support CSI-RS for mobility - FFS: whether to support other source RS(s) potentially agreed later for intra-cell mobility - FFS: whether to support CSI-RS for BM and tracking configured for non-serving cell(s) and without non-serving cell SSB as QCL-TypeD source - Send an LS to RAN2 on TCI state update (beam indication) using source RS configured for non-serving cell(s) for DL reception and UL transmission. The following topics are considered for the LS: - RRC configuration issues - Serving cell issues - C-RNTI issues - Issues related to CU-DU split - Inter-band CA issues - Inter-frequency issues

RAN1 seeks input from RAN4 on inter-band CA and inter-frequency issues in the context of L1/L2-centric inter-cell mobility and inter-cell multi-TRP operations.

Question 5: In regard of CA issues, RAN1 is discussing whether the operation is supported only for intra-band CA scenario (i.e. UE is configured to operate with serving and non-serving cells that belong to the same frequency band) or for both intra-band CA and inter-band CA scenarios. Note that one common TCI state ID associated with a non-serving cell, if supported, may be optionally applied for CCs in a band. - Are there specific RAN2/4 issues (including higher-layer impact) that need to be considered for deciding between the two alternatives?

Observation #1: From RAN4 viewpoint we don’t see a restriction in supporting the operation in inter-band CA.

[Proposed Response]

From RAN4 viewpoint we don’t see a restriction in supporting L1/L2-centric inter-cell mobility and inter-cell multi-TRP operations in inter-band CA.

Question 6: In regard of inter-frequency issues, from RAN2/4 perspective, what would be the higher-layer and RRM impact assuming inter-frequency scenarios as opposed to intra-frequency scenarios? For intra-frequency scenario, it is assumed that SSBs of non-serving cells have the same center frequency and SCS as the SSBs of the serving cell. - Note: RAN1 has agreed to support intra-frequency scenarios, whereas the support for inter-frequency scenarios is still for further study.

In RAN1 there is agreement that inter-cell multi-TRP operation is only within the same active BWP, hence this question is applicable to L1/L2-centric inter-cell mobility alone. In order to support L1/L2-centric inter-cell mobility, there has been an agreement in RAN1 to configure neighbour cell L1-RSRP measurements. In order to support such measurements for inter-frequency operation, a measurement gap would need to be configured. With a measurement gap configured for inter-frequency neighbor cell measurements, our understanding is that the operation can be supported for inter-frequency scenario.

Observation #2: With measurement gaps configured for inter-frequency L1-RSRP measurements, L1/L2-centric inter-cell mobility can be supported for inter-frequency scenario.

In case L1/L2-centric inter-cell mobility is supported for inter-frequency operation, the TCI state switching delay would need to consider an active BWP switch delay, resulting in a longer switching delay.

Observation #3: A longer TCI state switching delay would need to be considered if inter-frequency scenario is considered for L1/L2-centric inter-cell mobility.

Also, we would like to understand the deployment need for inter-frequency L1/L2-centric inter-cell mobility from operator’s viewpoint. Inter-frequency L1/L2-centric inter-cell mobility should be considered only if there is a deployment need.

Proposal #1: Discuss further the deployment need for inter-frequency scenario for L1/L2-centric inter-cell mobility.

[Proposed Response]

With the assumption that there is deployment need for inter-frequency scenarios for L1/L2-centric inter-cell mobility, from RRM perspective the following aspects need to be considered to support it:

- Configuration of measurement gap to support L1-RSRP measurements on neighbor cell

- Additional delay for TCI state switch for inter-frequency scenario involving active BWP switch

# 3. Conclusion

In this contribution we discuss the response to RAN1 LS on TCI State Update for L1/L2-Centric Inter-Cell Mobility. The proposed response to RAN1 is presented and also the draft LS reply is captured in the Appendix.

# Reference

- R1-2102248, “LS on TCI State Update for L1/L2-Centric Inter-Cell Mobility”, RAN1, Samsung.

# Appendix

The draft reply LS to RAN1 is captured below.

3GPP TSG-RAN WG4 Meeting # 98-bis-e 	R4-210XXXX

# Electronic Meeting, Apr. 12-20, 2021

Title:	[Draft] LS Reply on TCI State Update for L1/L2-Centric Inter-Cell Mobility

Work Item:	NR_feMIMO-Core

Source:	RAN WG4

To:	RAN WG1

Cc:

Contact Person:

#### Name:	Manasa Raghavan

E-mail Address:	manasa[dot]raghavan[at]apple[dot]com

Attachments:

- Overall Description:

RAN4 thanks RAN1 for their enquiry on the topic of TCI State Update for L1/L2-Centric Inter-Cell Mobility and would like to provide the below responses to the questions asked.

Question 5: In regard of CA issues, RAN1 is discussing whether the operation is supported only for intra-band CA scenario (i.e. UE is configured to operate with serving and non-serving cells that belong to the same frequency band) or for both intra-band CA and inter-band CA scenarios. Note that one common TCI state ID associated with a non-serving cell, if supported, may be optionally applied for CCs in a band.

- Are there specific RAN2/4 issues (including higher-layer impact) that need to be considered for deciding  between the two alternatives?

[RAN4] From RAN4 viewpoint we don’t see a restriction in supporting L1/L2-centric inter-cell mobility and inter-cell multi-TRP operations in inter-band CA.

Question 2: In regard of inter-frequency issues, from RAN2/4 perspective, what would be the higher-layer and RRM impact assuming inter-frequency scenarios as opposed to intra-frequency scenarios? For intra-frequency scenario, it is assumed that SSBs of non-serving cells have the same center frequency and SCS as the SSBs of the serving cell.

- Note: RAN1 has agreed to support intra-frequency scenarios, whereas the support for inter-frequency scenarios is still for further study.

[RAN4] With the assumption that there is deployment need for inter-frequency scenarios for L1/L2-centric inter-cell mobility, from RRM perspective the following aspects need to be considered to support it:

- Configuration of measurement gap to support L1-RSRP measurements on neighbor cell

- Additional delay for TCI state switch for inter-frequency scenario involving active BWP switch

2. To RAN WG1 group.

ACTION: RAN4 requests RAN1 to take the above responses into consideration.

3. Date of Next TSG-RAN WG4 Meetings:

TSG RAN WG4 Meeting #99-e                  May 19 – 27, 2021            	 	 Online