3GPP TSG-RAN WG4 Meeting #98-bis-e	R4-2104851

Electronic Meeting, Apr. 12 – Apr.20, 2021

Agenda Item:	8.7.4.1

Source:	Apple

Title:	Discussion on RRM requirement – General

Document for:	Discussion

# 1	Introduction

In Rel-17 NR support for high-speed train scenario in FR2 [1], potential RRM enhancement is proposed:

  - Study and identify RRM requirements impact and possible enhancement for

    - Idle/inactive mode cell reselection requirements enhancement

    - Connected mode requirements

      - Handover delay requirement

      - Measurement requirements including both L1 and SSB based L3 measurement

      - Beam management requirements including beam failure detection, candidate beam detection performance requirements

      - Other requirements if identified

[2] captures the deployment scenario discussion and [3] captures the RRM related discussion in RAN4 98e.

In this paper, we provide initial discussion on the general aspects on RRM requirement.

# 2	Discussion

The deployment scenarios discussed in [2] are listed in Table I. The key difference between scenario A and B is the Dmin, where a larger Dmin implies higher nLOS channel probability and more complicated design.

Table I Deployment scenario for HST FR2 [2]

| Scenario | Ds (meter) | Dmin (meter) | Prioritization | Note |
| --- | --- | --- | --- | --- |
| A | 700 | 10 | Prioritized | Changed from original scenario-2 |
| B | 700 | 150 | Prioritized | New scenario (same as FR1 deployment) |

The scope of HST FR2 has been discussed in RAN4 98-e and captured in way forward in [3]. Here we share our view on the scope.

- Idle/inactive mode requirement: since the target device is CPE mounted on the roof top of the train, and the CPE service all the UEs inside the carriage when the train is in service, the chance of CPE goes to idle can be very small. Existing Rel-16 requirements can be reused, and enhancement for idle/inactive mode can be lower priority.

- Inter-frequency measurement: In FR1 HST up to rel-16, inter-frequency measurement is not enhanced mainly due to deployment scenario where one dedicated carrier is used for HST. Since FR2 HST is limited to single carrier, unless there is an explicit request from operator, inter-frequency measurement enhancement can be lower priority.

- Inter-RAT measurements: the WI scope is “NR SA single carrier scenario in FR2”. Whether the CPE has LTE connection is not clearly defined. We think inter-RAT measurement can be low priority unless there is an explicit request from operator to support it.

- RRC connected mode requirements for DRX: Even though power consumption is less of a concern for roof top mounted CPE comparing to normal UEs, we think it may not be ideal to completely ignore the issue. Therefore, short DRX configuration, such as limit to up to 80ms can be considered.

Proposal 1: Reuse existing R-16 requirement for Idle/inactive mode.

Proposal 2: Inter-frequency measurement and inter-RAT measurement can be lower priority for HST FR2, unless requested explicitly by operators.

Proposal 3: Short DRX configuration can be considered for RRC connected mode requirements.

From RRM perspective, maximum speed is tightly related to deployment scenarios, particularly the number of UE Rx beams needed for beam sweeping, and how much network beam overlaps for measurement. In the deployment scenario A with unidirectional SFN, and the network antenna panel boresight direction is pointed to the next RRH location, one or two network beams are enough with large beam overlapping. For the CPE with 4x4 or 4x2 antenna panel, 1 Rx beam can used when the train move along the track. In this case, RRM performance is quite similar to FR1 case and 350Km/hour speed is feasible from RRM perspective. However, in scenario B, when Dmin is 150m, UE is expected to experience more NLoS channel compared to scenario A, resulting in higher number of Rx beams to sweep. In this case, support of 350Km/hour need further study.

Observation 1: Number of Rx beams are key parameter for maximum speed analysis. Number of Rx beams can be different from different deployment

# 3	Summary

In this paper, we provide our view on general aspect of RRM enhancement for HST FR2.

Observation 1: Number of Rx beams are key parameter for maximum speed analysis. Number of Rx beams can be different from different deployment

mode RRM enhancement.

Proposal 1: Reuse existing R-16 requirement for Idle/inactive mode.

Proposal 2: Inter-frequency measurement and inter-RAT measurement can be lower priority for HST FR2, unless requested explicitly by operators.

Proposal 3: Short DRX configuration can be considered for RRC connected mode requirements.

# References

- RP-202118, New WID on NR support for high speed train scenario in FR2, Samsung, Nokia, Sep 2020

- R4-2103240, Way forward on deployment scenario and UE RF requirement for FR2 HST, Samsung, Jan 2021

- R4-2103679, WF on Rel-17 NR HST FR2 enhancements, RRM requirements, Nokia, Nokia Shanghai Bell, Jan 2021