3GPP TSG-RAN4 Meeting #98-bis-e	R4-2104853

# Online, 12 - 20 April 2021

Agenda item:	12.1

Source:	Apple

Title:	Discussion on RAN1 LS on UE transmit timing error for R17 URLLC

Document for:	Discussion

# Introduction

In RAN1#104-e an LS was approved in [1] to RAN4 on UE transmit timing error for R17 URLLC:

In this contribution, we provide our view on the question RAN1 asked.

# Discussion

Te requirements have been widely discussed at NR early phase. A lot of companies provided comprehensive analysis on how the Te shall be designed. Final agreement was made in RAN4#85 meeting. There are some agreement captured in the chairman note implying that downlink timing estimation has been taken into account when defining Te requirements.

#### 9.7.6 UE timing (38.133/36.133) [NR_newRAT] ##### 9.7.6.1 UE transmit timing, UE timer accuracy and timing advanced [NR_newRAT] Tx timing and timing adjustment accuracy Agreement: - Initial UE transmit timing error (Te): - Principle: - For Te values, the downlink bandwidth of SSB and/or TRS and uplink SCS should be taken into account. - The different RF margins apply to different frequency ranges. - The margin decrease with the increasing SCS - The lower boundary of Te for each bandwidth/SCS should be expected - Autonomous timing adjustment accuracy (Tq): - Principle: - The fixed number of Maximum Autonomous Time Adjustment Step and maximum aggregate adjustment rate will be used for different SCS-es. - The values for FR2 should be smaller that the value for FR1.

Observation 1: downlink timing detection error is included in Te requirement.

Proposal 1: RAN4 is to reply RAN1 that downlink timing detection error is included in UE transmit timing error (Te) requirements.

# Conclusion

In this contribution, we provide background of Te requirements defined in TS38.133. After that, the following conclusions are provided:

Observation 1: downlink timing detection error is included in Te requirement.

Proposal 1: RAN4 is to reply RAN1 that downlink timing detection error is included in UE transmit timing error (Te) requirements.

# References

- R4-1801401, RAN4#85 Meeting report