3GPP TSG-RAN4 Meeting #98-bis-e	R4-2104858

# Online, 12 - 20 April 2021

Agenda item:	6

Source:	Apple

Title:	On R16 NR HST UE capabilities

Document for:	Discussion

# Introduction

According to latest agreement (R4-2016850 in RAN4#97e), three UE capabilities have been introduced in R16. 10-1 and 10-2 are captured in TS38.306 while 10-3 is captured in TS36.306.

| R16 HST UE capability | Index | Feature group | Components | Mandatory/Optional |
| --- | --- | --- | --- | --- |
|  | 10-1 | RRM enhanced requirements specified within NR and NR-E-UTRAN inter-RAT measurement for NR HST | The enhanced RRM requirements specified within NR and NR-E-UTRAN inter-RAT measurement to support high speed up to 500 km/h, as specified in TS 38.133 | Optional with capability signalling |
|  | 10-2 | Demodulation enhancement for HST-SFN joint transmission scheme | The enhanced demodulation processing for HST-SFN joint transmission scheme with velocity up to 500km/h, as specified in TS 38.101-4 | Optional with capability signalling |
|  | 10-3 | RRM enhancement for E-UTRAN-NR inter-RAT measurement for NR HST | The enhanced RRM requirements specified for E-UTRAN-NR inter-RAT measurement to support high speed up to 500 km/h, as specified in TS 36.133 | Optional with capability signalling |

We found that 10-1 is used to indicate the support of both intra-NR and inter-RAT NR-LTE HST RRM measurement enhancement, which is problematic. In this contribution we provide our view on this issue and some potential update on the capability.

# Discussion

In principle, NR HST and LTE HST are independent optional features, which should be separately supported. On top of 10-3, there are other capabilities defined in TS36.306 to indicate the support of LTE HST measurement. Therefore, UE is allowed to separately indicate the support of LTE HST measurement and LTE-NR inter-RAT HST measurement.

Observation 1: support of intra-LTE HST RRM and LTE-NR inter-RAT HST RRM are independently indicated.

However, the new capability 10-1 indicates the support of both intra-NR HST and NR-LTE inter-RAT HST measurement.

| Index | Feature group | Components |
| --- | --- | --- |
| 10-1 | RRM enhanced requirements specified within NR and NR-E-UTRAN inter-RAT measurement for NR HST | The enhanced RRM requirements specified within NR and NR-E-UTRAN inter-RAT measurement to support high speed up to 500 km/h, as specified in TS 38.133 |

The consequence is UE is not allowed to support NR HST without supporting LTE HST. Several drawback can be observed:

- Current design results in misalignment of mechanism of defining HST UE capabilities in LTE spec and NR spec.

- Negative impact on the ecosystem can be foreseen

  - NR standalone UE cannot indicate support of NR HST. Neither UE nor network can benefit from this feature.

  - As NR deployment being increased, UE vendor may choose to allocate more resource on NR (for further enhancement) and only keep some basic LTE functionalities for certain type of UE. However, current design is against this principle. NR enhancement for this type of UE would be reduced.

Observation 2: UE is not allowed to support NR HST without supporting LTE HST according 10-1. Neither network nor UE can benefit from such design.

We have two candidate solutions to resolve this problem:

Option 1: follow LTE methodology, i.e. to update 10-1 to indicate support of intra-NR HST and introduce 10-4 to indicate support of NR-LTE inter-RAT HST:

| R16 HST UE capability | Index | Feature group | Components | Mandatory/Optional |
| --- | --- | --- | --- | --- |
|  | 10-1 | RRM enhanced requirements specified within NR and NR-E-UTRAN inter-RAT measurement for NR HST | The enhanced RRM requirements specified within NR and NR-E-UTRAN inter-RAT measurement to support high speed up to 500 km/h, as specified in TS 38.133 | Optional with capability signalling |
|  | 10-2 | Demodulation enhancement for HST-SFN joint transmission scheme | The enhanced demodulation processing for HST-SFN joint transmission scheme with velocity up to 500km/h, as specified in TS 38.101-4 | Optional with capability signalling |
|  | 10-3 | RRM enhancement for E-UTRAN -NR inter-RAT measurement for NR HST | The enhanced RRM requirements specified for E-UTRAN-NR inter-RAT measurement to support high speed up to 500 km/h, as specified in TS 36.133 | Optional with capability signalling |
|  | 10-4 | RRM enhanced for NR-E-UTRAN inter-RAT measurement for NR HST | The enhanced RRM requirements specified for NR-E-UTRAN inter-RAT measurement to support high speed up to 500 km/h, as specified in TS 38.133 | Optional with capability signalling |

Option 2: add two new capabilities to

  - 1) indicate the support of intra-NR HST RRM measurement with speed up to 500km/h

  - 2) indicate the support of NR-LTE inter-RAT RRM measurement with speed up to 500km/h

  - Note: UE can indicate support of 10-4 or 10-5 only if 10-1 is NOT supported.

| R16 HST UE capability | Index | Feature group | Components | Mandatory/Optional |
| --- | --- | --- | --- | --- |
|  | 10-1 | RRM enhanced requirements specified within NR and NR-E-UTRAN inter-RAT measurement for NR HST | The enhanced RRM requirements specified within NR and NR-E-UTRAN inter-RAT measurement to support high speed up to 500 km/h, as specified in TS 38.133 | Optional with capability signalling |
|  | 10-2 | Demodulation enhancement for HST-SFN joint transmission scheme | The enhanced demodulation processing for HST-SFN joint transmission scheme with velocity up to 500km/h, as specified in TS 38.101-4 | Optional with capability signalling |
|  | 10-3 | RRM enhancement for E-UTRAN -NR inter-RAT measurement for NR HST | The enhanced RRM requirements specified for E-UTRAN-NR inter-RAT measurement to support high speed up to 500 km/h, as specified in TS 36.133 | Optional with capability signalling |
|  | 10-4 | RRM enhanced requirements specified within NR HST | The enhanced RRM measurement requirements specified within NR to support high speed up to 500 km/h, as specified in TS 38.133 | Optional with capability signalling |
|  | 10-5 | RRM enhanced requirements specified within NR and NR-E-UTRAN inter-RAT measurement for NR HST | The enhanced NR-E-UTRAN inter-RAT RRM measurement to support high speed up to 500 km/h, as specified in TS 38.133 | Optional with capability signalling |

Both options can resolve this issue. Option 2 is slightly preferred since it can avoid further update on existing capability 10-1.

Proposal 1: UE shall be allowed to separately support NR HST RRM and NR-LTE inter-RAT HST RRM.

Proposal 2: RAN4 shall discuss and down select the options mentioned in this contribution. Option 2 is preferred from Apple.

# Conclusion

In this contribution, we provide analysis on existing R16 NR HST RRM UE capabilities. After discussion the following conclusions are made:

Observation 1: support of intra-LTE HST RRM and LTE-NR inter-RAT HST RRM are independently indicated.

Observation 2: UE is not allowed to support NR HST without supporting LTE HST according 10-1. Neither network nor UE can benefit from such design.

Proposal 1: UE shall be allowed to separately support NR HST RRM and NR-LTE inter-RAT HST RRM.

Proposal 2: RAN4 shall discuss and down select the options mentioned in this contribution. Option 2 is preferred from Apple.

Option 2:

| Index | Feature group | Components | Mandatory/Optional |
| --- | --- | --- | --- |
| 10-1 | RRM enhanced requirements specified within NR and NR-E-UTRAN inter-RAT measurement for NR HST | The enhanced RRM requirements specified within NR and NR-E-UTRAN inter-RAT measurement to support high speed up to 500 km/h, as specified in TS 38.133 | Optional with capability signalling |
| 10-2 | Demodulation enhancement for HST-SFN joint transmission scheme | The enhanced demodulation processing for HST-SFN joint transmission scheme with velocity up to 500km/h, as specified in TS 38.101-4 | Optional with capability signalling |
| 10-3 | RRM enhancement for E-UTRAN -NR inter-RAT measurement for NR HST | The enhanced RRM requirements specified for E-UTRAN-NR inter-RAT measurement to support high speed up to 500 km/h, as specified in TS 36.133 | Optional with capability signalling |
| 10-4 | RRM enhanced requirements specified within NR HST | The enhanced RRM measurement requirements specified within NR to support high speed up to 500 km/h, as specified in TS 38.133 | Optional with capability signalling |
| 10-5 | RRM enhanced requirements specified within NR and NR-E-UTRAN inter-RAT measurement for NR HST | The enhanced NR-E-UTRAN inter-RAT RRM measurement to support high speed up to 500 km/h, as specified in TS 38.133 | Optional with capability signalling |

# References

- R16 UE feature list.