3GPP RAN WG4 Meeting #98bis-e		R4-2104883

Electronic meeting, 12 – 20th April 2021

Agenda item:	7.38.4

Source:	Apple Inc.

Title:	On simultaneous low power and very low power operation

WI/SI:	NR_6GHz_unlic_EU

Release:	Rel-17

Document for:	Decision

# 1	Introduction

3GPP Rel-16 NR-U WI [1] specified how the NR technology can be used on the unlicensed spectrum thus offering more resources in frequency bands, such as 5GHz and 6GHz.  While 5GHz is a well-known band for the unlicensed operation, 6GHz is a relative new band usage of which was approved recently in different regulatory regions. Referring to the latest 6GHz regulatory status update [2], FCC approved license-exempt usage in the frequency range of 5925-7125MHz, which has been already reflected by the corresponding 3GPP band n96. In addition to that, the regulatory framework for the EU/CEPT countries was completed in November covering the 5945-6425MHz frequency range. There are several more countries/regions, such as South Korea, Brazil and Saudi Arabia, that have announced recently the corresponding regulatory parameters for the 6GHz band.

Since the 6GHz regulatory framework for the CEPT countries is completed, 3GPP RAN#90 meeting decided to resume the corresponding WI [3] that aims at adding 3GPP support for the 6GHz band in that regulatory region. During the RAN4#98 meeting, companies discussed how to support EU/CEPT regulatory requirements, whereupon an issue was raised on so-called very low power (VLP) mode and whether it can be used with NR-U deployments. It was preliminary concluded that the VLP mode can be supported at least by UEs, while it was not entirely clear whether EU/CEPT regulatory framework allows VLP base stations. Furthermore, it is not entirely clear either how all the regulatory requirements will be met in case of mixed deployment cases that involve both low power indoor (LPI) and VLP modes. In this discussion paper we elaborate further on potential mixed deployment cases asking RAN WG4 to consider potential issue that can emerge in this case.

# 2	LPI and VLP modes

As already mentioned in the Introduction part, while some countries have allocated the full range of 5925-7125MHz to the license-exempt operation, CEPT countries will use a smaller portion of the 6GHz band for the unlicensed operation. Another important aspect that almost most Administrations differentiate between indoor mode and outdoor mode, whereupon the indoor mode usually allows higher transmission power, while outdoor mode. And even though actual technical parameters slightly vary from region to region, the main principles are the same. A device operating outdoors shall not exceed the maximum EIRP and PSD limits, which are more stringent when compared to the indoor requirements.

Figure 2-1: Overview of the 6GHz unlicensed band with LPI and VLP modes.

If there is an NR-U base station deployed indoors, then it can follow the LPI regulatory parameters of the corresponding region and configure accordingly client devices, UEs, with a higher transmission power and PSD. However, even though the NR-U base station physically resides indoors, there can be an outdoor UE that still can detect and camp on this cell. It should be noted that most regulatory requirements just indicate that the LPI access point must be powered directly by the electric power network and battery power is not allowed. Not all regions/countries even state that the access point physical structures cannot be protected against bad weather (for instance EU/CEPT rules do not have such a condition). Thus, since the indoor base station allows higher transmission power and PSD, the outdoor UE can effectively violate the regulatory requirements when it starts transmitting at the maximum transmission power allowed for the indoor operation. This situation is depicted in Figure 2-2 that shows an outdoor UE connecting to the indoor base station. Of course, if a particular UE supports only the VLP mode, then it will always meet the requirements. However, as there can be UEs supporting both VLP and LPI transmission power, then the LPI device can end up in the aforementioned situation potentially failing the regulatory requirements.

Figure 2-2: Several deployment scenarios with LPI and VLP.

Our general anticipation is that NR-U deployments on the 6GHz band will consider LPI transmission power, at least for the indoor deployment. However, since it is impossible to prevent outdoor UEs from connecting to the LPI base stations, we ask RAN WG4 to consider this issue. Referring to the summary of the regulatory parameters presented in Annex A, this problem can occur at least in the following countries/regions.

-	EU/CEPT: Both LPI and VLP modes are allowed in 5945 – 6425 MHz;

-	South Korea: Both LPI and VLP modes are allowed in 5945 – 6425 MHz;

-	Brazil: Both LPI and VLP modes are allowed in 5945 – 7125 MHz;

-	US: LPI is mode is allowed for 5945 – 7125 MHz. On the one hand, there is no indoor/outdoor VLP modes similar to other countries because outdoor operation requires AFC. On the other hand, AFC may limit the maximum transmission power down to 23dBm, which is lower than the maximum LPI transmission power of 24dBm.

Proposal:	We ask RAN WG4 to consider a scenario when an outdoor UE is connected to the indoor LPI base station in order to ensure that no regulatory requirements are violated in this case.

# 3	Conclusions

In this discussion paper we have presented a scenario with the indoor NR-U deployment that configures higher transmission power that is allowed in several regulatory regions. However, since there can be an outdoor UE connecting to this indoor cell, the former will violate the regulatory requirements.

Proposal:	We ask RAN WG4 to consider a scenario when an outdoor UE is connected to the indoor LPI base station in order to ensure that no regulatory requirements are violated in this case.

# 4	References

RP-1912926, "WID on NR-based Access to Unlicensed Spectrum ", Qualcomm Inc.

3GPP TR 37.890, "Feasibility Study on 6 GHz for LTE and NR in Licensed and Unlicensed Operations", v0.10.0

RP‐202592, "WID on introduction of lower 6GHz NR unlicensed operation for Europe", Nokia, Nokia Shanghai Bell

Annex A: Summary of the 6GHz band regulatory parameters

| Country | Permissible operation | Frequency range | Maximum mean EIRP for in-band emissions | Maximum mean EIRP density for in-band emissions | Maximum mean EIRP density for out-of-band emissions |
| --- | --- | --- | --- | --- | --- |
| US | Standard power: Outdoor with AFC. | 5925 - 6425MHz 6525 - 6875MHz | 36dBm (AP) 30dBm (CL) | 23dBm/MHz (AP) 17dBm/MHz (CL) | -27 dBm/MHz (outside operational range) |
|  | LPI: The access point devices cannot be weather resistant and shall have integrated antennas (prohibiting connection to other antennas). Operations in moving vehicles (cars, trains, aircraft) and drones are prohibited, with one exception in the U-NII-5 band for large passenger aircraft operating over 10,000 feet. | 5925 - 7125MHz | 30dBm (AP) 24dBm (CL) | 5 dBm/MHz (AP) -1 dBm/MHz (CL) |  |
| EU/CEPT | LPI: Restricted for indoor use only (including trains where metal coated windows are fitted and aircraft). Outdoor use (including in road vehicles) is not permitted. | 5945 - 6425MHz | 23dBm | 10dBm/MHz | -22 dBm/MHz (below 5935MHz) |
|  | VLP: Indoors and outdoors. Use on drones is prohibited. The VLP device is a portable device. |  | 14dBm | 1dBm/MHz | -45 dBm/MHz (below 5935MHz) |

| Country | Permissible operation | Frequency range | Maximum mean EIRP for in-band emissions | Maximum mean EIRP density for in-band emissions | Maximum mean EIRP density for out-of-band emissions |
| --- | --- | --- | --- | --- | --- |
| South Korea | LPI: A device shall use power supply from a wired connection, and a client battery powered device must communicate through the device connected to a wired power supply. This frequency range cannot be used for indoor automotive or airplane deployments. Usage inside moving vehicles is prohibited | 5925 - 7125MHz | 24dBm | 2dBm/MHz | -27 dBm/MHz (outside operational range) |
|  | VLP: Very low power indoor and outdoor usage. | 5925 - 6425MHz | 14dBm | 1dBm/MHz | -34 dBm/MHz (outside operational range) |
| Brazil | LPI: The AP equipment must be used only in an indoor environment. It must be powered directly by the electric power network, and battery power is not allowed. Their physical structures cannot be protected against bad weather. The operation is prohibited on oil extraction platforms, cars, trains, boats and aircraft, with the exception of operation in the 5.925-6.425 GHz band inside large aircraft flying above 3,048m. Client equipment must operate under the control of an indoor AP. | 5925 - 7125MHz | 30dBm (AP) 24dBm (CL) | 5dBm/MHz (AP) -1dBm/MHz (CL) | -27 dBm/MHz (outside operational range) |
|  | VLP: Very low power indoor and outdoor usage |  | 17 dBm | -5 dBm/MHz |  |