3GPP RAN WG4 Meeting #98bis-e		R4-2104884

Electronic meeting, 12 – 20th April 2021

Agenda item:	13.1.1

Source:	Apple Inc.

Title:	Supporting the 6GHz band in other countries/regions

WI/SI:

Release:	Rel-17

Document for:	Decision

# 1	Introduction

3GPP Rel-16 NR-U WI [1] specified how the NR technology can be used on the unlicensed spectrum thus offering more resources in frequency bands, such as 5GHz and 6GHz.  While 5GHz is a well-known band for the unlicensed operation, 6GHz is a relative new band usage of which was approved recently in different regulatory regions. Referring to the latest 6GHz regulatory status update [2], FCC approved license-exempt usage in the frequency range of 5925-7125MHz, which has been already reflected by the corresponding 3GPP band n96. In addition to that, the regulatory framework for the EU/CEPT countries was completed in November covering the 5945-6425MHz frequency range, and for which RAN#90 meeting decided to resume the corresponding WI [3].

However, there are several countries/regions, such as South Korea, Brazil and Saudi Arabia, that have also announced recently the corresponding regulatory parameters for the 6GHz band. And even though their regulatory parameters and principles are somewhat similar to e.g. FCC an EU/CEPT rules, they nevertheless have certain differences that have to be analysed and addressed by 3GPP. Thus, in this discussion paper we present a quick overview of the corresponding parameters and ask 3GPP to consider how they can be supported.

# 2	6GHz band in different countries/regions

Referring to the latest version of TR 37.890, 3GPP has captured information on three countries/regions that have published regulatory parameters for the 6GHz band: US/FCC, EU/CEPT, and Korea/MIST. While support for the 6GHz band in US was already done in the Rel-16 NR-U WI, 3GPP RAN has decided to instantiate a new Rel-17 WI to support the 6GHz band in EU/CEPT region. However, it is not clear in which WI or agenda item other countries should be considered. As already mentioned in the Introduction part, South Korea, Brazil and Saudi Arabia have published the corresponding rules, which are not entirely same as e.g. EU/CEPT (refer to Annex A). In fact, other Administrations have been working on the 6GHz band rules, so even more countries will start publishing the 6GHz band rules in the near future. Based on that we anticipate that non-trivial technical discussions will be needed to understand how to support 6GHz in all countries.

Proposal:	We ask RAN WG4 to consider how to introduce support for the 6GHz band in countries that are not covered by the scope of the existing WIs.

# 3	References

RP-1912926, "WID on NR-based Access to Unlicensed Spectrum ", Qualcomm Inc.

3GPP TR 37.890, "Feasibility Study on 6 GHz for LTE and NR in Licensed and Unlicensed Operations", v0.10.0

RP‐202592, "WID on introduction of lower 6GHz NR unlicensed operation for Europe", Nokia, Nokia Shanghai Bell

Annex A: Summary of the 6GHz band regulatory parameters

| Country | Permissible operation | Frequency range | Maximum mean EIRP for in-band emissions | Maximum mean EIRP density for in-band emissions | Maximum mean EIRP density for out-of-band emissions |
| --- | --- | --- | --- | --- | --- |
| US | Standard power: Outdoor with AFC. | 5925 - 6425MHz 6525 - 6875MHz | 36dBm (AP) 30dBm (CL) | 23dBm/MHz (AP) 17dBm/MHz (CL) | -27 dBm/MHz (outside operational range) |
|  | LPI: The access point devices cannot be weather resistant and shall have integrated antennas (prohibiting connection to other antennas). Operations in moving vehicles (cars, trains, aircraft) and drones are prohibited, with one exception in the U-NII-5 band for large passenger aircraft operating over 10,000 feet. | 5925 - 7125MHz | 30dBm (AP) 24dBm (CL) | 5 dBm/MHz (AP) -1 dBm/MHz (CL) |  |
| EU/CEPT | LPI: Restricted for indoor use only (including trains where metal coated windows are fitted and aircraft). Outdoor use (including in road vehicles) is not permitted. | 5945 - 6425MHz | 23dBm | 10dBm/MHz | -22 dBm/MHz (below 5935MHz) |
|  | VLP: Indoors and outdoors. Use on drones is prohibited. The VLP device is a portable device. |  | 14dBm | 1dBm/MHz | -45 dBm/MHz (below 5935MHz) |

| Country | Permissible operation | Frequency range | Maximum mean EIRP for in-band emissions | Maximum mean EIRP density for in-band emissions | Maximum mean EIRP density for out-of-band emissions |
| --- | --- | --- | --- | --- | --- |
| South Korea | LPI: A device shall use power supply from a wired connection, and a client battery powered device must communicate through the device connected to a wired power supply. This frequency range cannot be used for indoor automotive or airplane deployments. Usage inside moving vehicles is prohibited | 5925 - 7125MHz | 24dBm | 2dBm/MHz | -27 dBm/MHz (outside operational range) |
|  | VLP: Very low power indoor and outdoor usage. | 5925 - 6425MHz | 14dBm | 1dBm/MHz | -34 dBm/MHz (outside operational range) |
| Brazil | LPI: The AP equipment must be used only in an indoor environment. It must be powered directly by the electric power network, and battery power is not allowed. Their physical structures cannot be protected against bad weather. The operation is prohibited on oil extraction platforms, cars, trains, boats and aircraft, with the exception of operation in the 5.925-6.425 GHz band inside large aircraft flying above 3,048m. Client equipment must operate under the control of an indoor AP. | 5925 - 7125MHz | 30dBm (AP) 24dBm (CL) | 5dBm/MHz (AP) -1dBm/MHz (CL) | -27 dBm/MHz (outside operational range) |
|  | VLP: Very low power indoor and outdoor usage |  | 17 dBm | -5 dBm/MHz |  |