3GPP TSG RAN WG4 Meeting #98BIS-e	R4-2104973

Electronic Meeting, 12th – 20th April, 2021

Agenda item:	7.20.2

Source:	LG Electronics

Title:	MSD results by self interference for PC2 NR inter-band CA band combinations

Document for:	Approval

1	Introduction

In the latest RAN Plenary, some new DC band are included to support PC2 DC band combinations in Rel-17 as follow [1].

Table 1: current status on PC2 NR inter-band CA band combinations in Rel-17

| NR CA configuration | Uplink CA configuration | contact name, company | other supporting companies (min. 3) | status (new, ongoing, completed, stopped) |
| --- | --- | --- | --- | --- |
| CA_n3A-n78A | CA_n3A-n78A | Bo Liu, China Telecom | Nokia, Nokia Shanghai Bell, vivo, CATT, OPPO | new |
| CA_n41A-n71A | CA_n41A-n71A | Bill Shvodian, T-Mobile USA | Nokia, Nokia Shanghai Bell, Ericsson | new |
| CA_n41A-n66A | CA_n41A-n66A | Bill Shvodian, T-Mobile USA | Nokia, Nokia Shanghai Bell, Ericsson | new |
| CA_n25A-n41A | CA_n25A-n41A | Bill Shvodian, T-Mobile USA | Nokia, Nokia Shanghai Bell, Ericsson | new |
| CA_n40A-n41A | CA_n40A-n41A | Shao Zhe, CMCC | Huawei, HiSilicon, ZTE, CATT | completed |
| CA_n40A-n79A | CA_n40A-n79A | Shao Zhe, CMCC | Huawei, HiSilicon, ZTE, CATT | new |
| CA_n28A-n41A | CA_n28A-n41A | Shao Zhe, CMCC | Huawei, HiSilicon, ZTE, CATT | completed |
| CA_n28A-n79A | CA_n28A-n79A | Shao Zhe, CMCC | Huawei, HiSilicon, ZTE, CATT | completed |
| CA_n3A-n41A | CA_n3A-n41A | Shao Zhe, CMCC | Huawei, HiSilicon, ZTE, CATT | completed |
| CA_n3A-n79A | CA_n3A-n79A | Shao Zhe, CMCC | Huawei, HiSilicon, ZTE, CATT | new |
| CA_n40A-n78A | CA_n40A-n78A | Vinay Shrivastava, Reliance Jio | CeWIT, Huawei, HiSilicon, Indian Institute of Technology Madras, Indian Institute of Technology Hyderabad, Saankhya Labs Pvt. Ltd., Tejas Networks Ltd. | new |
| CA_n2A-n77A | CA_n2A-n77A | Zheng Zhao, Verizon | Nokia, Samsung, Ericsson, Qualcomm | completed |
| CA_n2A-n77(2A) | CA_n2A-n77A | Zheng zhao, Verizon | Nokia, Samsung, Ericsson, Qualcomm | completed |
| CA_n5A-n77A | CA_n5A-n77A | Zheng Zhao, Verizon | Nokia, Samsung, Ericsson, Qualcomm | completed |
| CA_n5A-n77(2A) | CA_n5A-n77A | Zheng Zhao, Verizon | Nokia, Samsung, Ericsson, Qualcomm | completed |
| CA_n66A_n77A | CA_n66A_n77A | Zheng Zhao, Verizon | Nokia, Samsung, Ericsson, Qualcomm | completed |
| CA_n66A-n77(2A) | CA_n66A-n77 | Zheng Zhao, Verizon | Nokia, Samsung, Ericsson, Qualcomm | completed |
| CA_n25A-n77A | CA_n25A-n77A | Bill Shvodian, T-Mobile USA | Deutsche Telekom, Nokia, Nokia Shanghai Bell | new |
| CA_n41A-n77A | CA_n41A-n77A | Bill Shvodian, T-Mobile USA | Deutsche Telekom, Nokia, Nokia Shanghai Bell | new |
| CA_n71A-n77A | CA_n71A-n77A | Bill Shvodian, T-Mobile USA | Deutsche Telekom, Nokia, Nokia Shanghai Bell | new |

Based on the revised WID [2] and status report [1] at last RAN #91-e meeting, we analyse the self interference problems for the new DC band combinations.

2	Self interference analysis

In this session, we analyze the self interference problems to the own Rx bands as follow

Table 2. Self interference analysis for NR inter-band CA PC2 band combinations

| Downlink band configuration | Uplink DC Configuration | Harmonic relation issues | intermodulation to own rx band | interference due to small frequency separation | MSD |
| --- | --- | --- | --- | --- | --- |
| CA_n3A-n78A | CA_n3A-n78A | - 2nd harmonic from n3 fall into n78. But TDD band only allow Tx or Rx in the same time No issue - Harmonic mixing issue can be considered with 26dBm from n78 into n3 | - 2nd , 4th & 5th IMDs fall into n3 | No cross band isolation issues | Need to study 2nd harmonic mixing from n78 into n3. Follow 31.9 dB/ 18.5 dB MSD by 2nd / 4th IMDs of DC_3A_n78A MSD valus for PC2 in TS38.101-3. |
| CA_n25A-n41A | CA_n25A-n41A | No harmonic problem or harmonic mixing between two band | - No IMD problem into n25 - No IMD problem into n41 | Need to define MSD for cross band isolation issue in n25 | 0.6 dB MSD value for PC3 was defined. follow same MSD values (2.3dB) of PC2 DC_2A_n41A in TS38.101-3 |
| CA_n41A-n71A | CA_n41A-n71A | 4th harmonic from n71 fall into n41 But TDD band only allow Tx or Rx in the same time No issue 4th harmonic mixing issue can be considered with 26dBm from n41 can impact to n71 | - 4th IMD fall into n71 | No cross band isolation issues | - Need to study 4th harmonic mixing impact on the n71 reception for both PC3 and PC2. - 11.0dB was defined for PC3 UE. Need to study MSD value by 4th IMD for PC2 UE |
| CA_n41A-n66A | CA_n41A-n66A | No harmonic problem between two band | - No IMD problem into n66 - 4th IMD fall nto n41. But TDD band only allow Tx or Rx in the same time No issue | Need to define MSD for cross band isolation issue in n66 | 3.5dB MSD value for PC3 was defined. follow same MSD values (5.3dB) of PC2 DC_66A_n41A in TS38.101-3 |
| CA_n40A-n79A | CA_n40A-n79A | - 2nd harmonic from n40 fall into n79. But TDD band only allow Tx or Rx in the same time No issue - 2nd harmonic mixing issue can be considered with 26dBm from n79 fall into n40 | - 2nd, 4th & 5th IMDs fall into n40. Also 4th & 5th IMDs fall into n79. But TDD band only allow Tx or Rx in the same time No issue | No cross band isolation issues | - Need to study 2nd harmonic mixing impact on the n40 reception for both PC3 and PC2. |
| CA_n3A-n79A | CA_n3A-n79A | No harmonic problem between two band | - 5th IMDs fall into n3. - 4th IMD fall into n79. But TDD band only allow Tx or Rx in the same time No issue | No cross band isolation issues | No issue |
| CA_n40A-n78A | CA_n40A-n78A | No harmonic problem between two band | - 4th IMD fall into n40 & n78. But TDD band only allow Tx or Rx in the same time No issue | Need to define MSD for cross band isolation issue in Band 40 | 4.5dB MSD value for PC3 was defined. follow same MSD values (6.5dB) of PC2 DC_7A_n78A in TS38.101-3 |
| CA_n2A-n77A | CA_n2A-n77A | 2nd harmonic from n2 fall into n77. But TDD band only allow Tx or Rx in the same time No issue - 2nd harmonic mixing issue can be considered with 26dBm from n77 fall into n2 | - 2nd, 4th & 5th IMDs fall into n2 - 4th & 5th IMDs fall into n77. But TDD band only allow Tx or Rx in the same time No issue | No cross band isolation issues | - Need to study 2nd harmonic mixing impact on the n2 reception for both PC3 and PC2. - 26.0dB/ 8.0dB MSD values by 2nd/ 4th IMD are specified for PC3 DC_2A_n78A UE. captured MSD requirements in section 5.5.3 in TR38.xxx v0.2.0 |
| CA_n5A-n77A | CA_n5A-n77A | 4th & 5th harmonics from n5 fall into n77. But TDD band only allow Tx or Rx in the same time No issue - 4th harmonic mixing issue can be considered with 26dBm from n77 fall into n5 | - 4th & 5th IMDs fall into n5 - 2nd & 5th IMDs fall into n77. But TDD band only allow Tx or Rx in the same time No issue | No cross band isolation issues | - Need to study 4th harmonic mixing impact on the n5 reception for both PC3 and PC2. - 8.3 dB MSD value by 4th IMD is specified for PC3 DC_5A_n78A UE. captured MSD requirements in section 5.6.3 in TR38.xxx v0.2.0 |
| CA_n66A-n77A | CA_n66A-n77A | 2nd harmonic from n66 fall into n77. But TDD band only allow Tx or Rx in the same time No issue - 2nd harmonic mixing issue can be considered with 26dBm from n77 fall into n66 | - 2nd & 5th IMDs fall into n66. - 4th & 5th IMDs fall into n77. But TDD band only allow Tx or Rx in the same time No issue | No cross band isolation issues | - Need to study 2nd harmonic mixing impact on the n66 reception for both PC3 and PC2. - 5.0 dB MSD value by 5th IMD is specified for PC3 DC_66A_n78A UE. captured MSD requirements in section 5.7.3 in TR38.xxx v0.2.0 |
| CA_n25A-n77A | CA_n25A-n77A | 2nd harmonic from n25 fall into n77. But TDD band only allow Tx or Rx in the same time No issue - 2nd harmonic mixing issue can be considered with 26dBm from n77 fall into n25 | - 2nd, 4th & 5th IMDs fall into n25 - 4th & 5th IMDs fall into n77. But TDD band only allow Tx or Rx in the same time No issue | No cross band isolation issues | - Need to study 2nd harmonic mixing impact on the n25 reception for both PC3 and PC2. - 26.0dB/ 8.0dB MSD values by 2nd/ 4th IMD are specified for PC3 DC_2A_n78A UE. can be follow MSD req. for DC_2A_n77A PC2 UE in TR38.xxx v0.2.0 |
| CA_n41A-n77A | CA_n41A-n77A | No harmonic problem between two band | - 4th IMDs fall into n41. - 3rd & 4th IMDs fall into n77. But TDD band only allow Tx or Rx in the same time No issue | Need to define MSD for cross band isolation issue in n41 and n77 | Fot cross band isolation, - 4.5dB for n41 was defined for PC3 UE - 8.3dB for n77 were defiend for PC3 UE need to define new MSD values for PC2 UE |
| CA_n71A-n77A | CA_n71A-n77A | 5th & 6th harmonic from n71 fall into n77. But TDD band only allow Tx or Rx in the same time No issue - 5th harmonic mixing issue can be considered with 26dBm from n77 fall into n71 | - 5th IMDs fall into n71. - 2nd IMD fall into n77. But TDD band only allow Tx or Rx in the same time No issue | No cross band isolation issues | - Need to study 5th harmonic mixing impact on the n71 reception for both PC3 and PC2. - 5.5dB for DC_71A_n78A by IMD5 was defiend for PC3 UE need to define new MSD values for PC2 UE |

There are harmonic mixing problem, cross band isolation issues and IMD problems to the own Rx band when dual uplink is applied for PC2 NR inter-band CA UE. Hence, RAN4 need to define the MSD requirmeents for PC2 NR inter-band CA combinations in Rel-17.

3. MSD proposal

3.1 Cross band isolation issues

Table 3 show the MSD requirements for cross band isolation issues for power class 3 DC UE with LTE 1 FDD band and NR 1 TDD band combinations in TS38.101-3.

Table 3: MSD requirements due to cross band isolation for PC3 EN-DC in NR FR1 in TS38.101-3

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n13 | 3 | [3] | 2.3 | 2 | 1.8 |  |  |  |  |  |  |  |  |  |
| n1 | 40 | 6.6 | 6.6 | 6.6 | 6.6 |  |  |  |  |  |  |  |  |  |
| 13 | n3 | 3 | 2.2 | 1.9 | 1.7 | 1.6 | 1.5 | [1.4] |  |  |  |  |  |  |
| 1 | n40 | 6.6 | 6.6 | 6.6 | 6.6 | 6.6 | 6.6 | 6.6 | 6.6 | 6.6 |  | 6.6 |  |  |
| 1 | n41 |  | 6.1 | 6.1 | 6.1 |  | [6.1] | 6.1 | 6.1 | 6.1 |  | 6.1 | 6.1 | 6.1 |
| n3 | 11 | 6.4 | 6.1 |  |  |  |  |  |  |  |  |  |  |  |
| 3 | n41 |  | 0.7 | 0.7 | 0.7 |  | [0.7] | 0.7 | 0.7 | 0.7 |  | 0.7 | 0.7 | 0.7 |
| 3 | n51 | 6.4 |  |  |  |  |  |  |  |  |  |  |  |  |
| 30 | n66 | 8.3 | 8.3 | 8.3 | 8.3 | [8.3] | [8.3] | 8.3 |  |  |  |  |  |  |
| n3 | 41 | 0.7 | 0.7 | 0.7 | 0.7 |  |  |  |  |  |  |  |  |  |
| n5 | 28 | 4.5 | 3 | 2.2 | 0.3 |  |  |  |  |  |  |  |  |  |
| 7 | n40 | 3.7 | 3.4 | 3.2 | 3.1 | [3.1] | [3.1] | 3.1 | 3.1 | 3.1 |  | 3.1 |  |  |
| n38 | 1 | 1.9 | 1.9 | 1.9 | 1.9 |  |  |  |  |  |  |  |  |  |
| n38 | 2 | 0.6 | 0.6 | 0.6 | 0.6 |  |  |  |  |  |  |  |  |  |
| n38 | 4 | 1.9 | 1.9 | 1.9 | 1.9 |  |  |  |  |  |  |  |  |  |
| n38 | 66 | 1.9 | 1.9 | 1.9 | 1.9 |  |  |  |  |  |  |  |  |  |
| n40 | 1 | 8.3 | 8.3 | 8.3 | 8.3 |  |  |  |  |  |  |  |  |  |
| n41 | 4 | 3.5 | 3.5 | 3.5 | 3.5 |  |  |  |  |  |  |  |  |  |
| 40 | n1 | 8.3 | 8.3 | 8.3 | 8.3 | [8.3] | [8.3] | [8.3] | [8.3] |  |  |  |  |  |
| n40 | 7 | 3.7 | 3.7 | 3.7 | 3.7 |  |  |  |  |  |  |  |  |  |
| n41 | 1 | 9.1 | 9.1 | 9.1 | 9.1 |  |  |  |  |  |  |  |  |  |
| n41 | 2 | 0.6 | 0.6 | 0.6 | 0.6 |  |  |  |  |  |  |  |  |  |
| n41 | 3 | 0.6 | 0.6 | 0.6 | 0.6 |  |  |  |  |  |  |  |  |  |
| 41 | n3 | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 | [0.6] |  |  |  |  |  |  |
| n41 | 661 | 3.5 | 3.5 | 3.5 | 3.5 |  |  |  |  |  |  |  |  |  |
| n41 | 25 | 0.6 | 0.6 | 0.6 | 0.6 |  |  |  |  |  |  |  |  |  |
| n50 | 3 | 2.5 | 1.9 | 1.6 | 1.5 |  |  |  |  |  |  |  |  |  |
| n77 | 71 | 4.5 | 4.5 | 4.5 | 4.5 |  |  |  |  |  |  |  |  |  |
| n77 | 411 | 4.5 | 4.5 | 4.5 | 4.5 |  |  |  |  |  |  |  |  |  |
| 41 | n77 |  | 8.3 | 8.3 | 8.3 | [7.3] | [6.5] | 6.3 | 5.3 | 4.5 | [4.3] | 4.0 | 3.9 | 3.8 |
| n78 | 71 | 4.5 | 4.5 | 4.5 | 4.5 |  |  |  |  |  |  |  |  |  |
| n78 | 38 | 3.3 | 3.3 | 3.3 | 3.3 |  |  |  |  |  |  |  |  |  |
| n78 | 411 | 4.5 | 4.5 | 4.5 | 4.5 |  |  |  |  |  |  |  |  |  |
| n78 | 46 |  |  |  | 7 |  |  |  |  |  |  |  |  |  |
| 41 | n78 |  | 8.3 | 8.3 | 8.3 | [7.3] | [6.5] | 6.3 | 5.3 | 4.5 | [4.3] | 4.0 | 3.9 | 3.8 |
| n843 | 3 | 3 | 2.3 | 2 | 1.8 |  |  |  |  |  |  |  |  |  |
| NOTE 1: Applicable only when harmonic mixing MSD for this combination is not applied. NOTE 2: The B41 requirements are modified by -0.5dB when carrier frequency of the assigned E-UTRA channel bandwidth is within 2515 – 2690 MHz. NOTE 3: These requirements apply when the uplink is active in Band n1, n84 and the separation between the lower edge of the uplink channel in Band n1, n84 and the upper edge of the downlink channel in Band 3 is < 60 MHz. For each channel bandwidth in Band 3, the requirement applies regardless of channel bandwidth in Band n1, n84. NOTE 4: The DL victim band should be configured using the lowest SCS that is compatible with the highest CBW for which an MSD is specified. NOTE 5: MSD test point can be chosen according to supported BW and lowest SCS supported by the UE. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

In here, RAN4 only allow all power combinations such as 23dBm +23dBm or 26dBm +26dBm for PC2 NR inter-band CA UE.

Based on the specified MSD value in Table 3, we analyse the MSD value for the PC2 CA_n41A-n77A UE with follow simulation assumptions in Table 4.

Table 4.Simulation asummptions for MSD requirements

| n41 reception impact analysis parameters |  |
| --- | --- |
| Band n77 PA noise @NR n41 Rx (dBm/Hz) | -115 |
| RFIC IIP2 (dBm) | 50 |
| n77 front end filter Loss (dB) | 4 |
| n41 front end filter Loss (dB) | 4 |
| PA to antenna and LNA to antenna IL (dB) | 4 |
| n77/n41 diplexer isolation (dB) | 10 |
| n77 filter attenuation @ n41 RX (dB) | 30 |
| Cross-band isolation (dB) (n77 Tx n41 Rx) | 40 |
| Rx Antenna isolation (dB) | 10 |
| n77 reception impact analysis parameters |  |
| Band n41 PA noise @n77 Rx (dBm/Hz) | -115 |
| RFIC IIP2 (dBm) | 50 |
| n41 front end filter Loss (dB) | 4 |
| n77 front end filter Loss (dB) | 4 |
| PA to antenna and LNA to antenna IL (dB) | 4 |
| n77/n41 diplexer isolation (dB) | 10 |
| n41 filter attenuation @ n77 RX (dB) | 27 |
| Cross-band isolation (dB) (n41 Tx --> n77 Rx) | 38 |
| Rx Antenna isolation (dB) | 10 |

Base on the above assumptions, the MSD values are derived as follow by using MRC combining.

Table 5 MSD due to cross band isolation for PC2 for CA band combinations

|  | NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n41 | n25 | 2.3 | 2.3 | 2.3 | 2.3 |  |  |  |  |  |  |  |  |  |
| n41 | n66 | 5.3 | 5.3 | 5.3 | 5.3 |  |  |  |  |  |  |  |  |  |
| n78 | n40 | 6.5 | 6.5 | 6.5 | 6.5 |  |  |  |  |  |  |  |  |  |
| n77 | n41 | 6.5 | 6.5 | 6.5 | 6.5 |  |  |  |  |  |  |  |  |  |
| n41 | n77 |  | 10.5 | 10.5 | 10.5 | 9.5 | 8.6 | 8.3 | 7.2 | 6.3 | 6.0 | 5.7 | 5.6 | [5.6] |

3.2 Inter-modulation problems by dual uplink

In this session, we analyze the required MSD values in the own Rx band by IMD product from dual uplink transmission. In table 6 and Table 7, we show the basic RF simulation assumptions.

Table 6: UE RF Front-end component parameters

| UE ref. architecture Component | Diplexer Architecture w/ single ant. |  |  |  |
| --- | --- | --- | --- | --- |
|  | PC2 for CA_n41A_n71A and CA_n71A_n77A |  |  |  |
|  | IP2 (dBm) | IP3 (dBm) | IP4 (dBm) | IP5 (dBm) |
| Ant. Switch | 112 | 68 | 55 | 55 |
| Triplexer | 110 | 72 | 55 | 52 |
| Diplexer | 115 | 87 | 55 | 55 |
| Duplexer | 100 | 75 | 55 | 53 |
| PA Forward | 28.0 | 32 | 30 | 28 |
| PA Reversed | 40 | 30.5 | 30 | 30 |
| LNA | 10 | 0 | 0 | -10 |

Table 7 show the isolation levels according to the RF component.

Table 7: UE RF Front-end component isolation parameters

| Isolation Parameter | Value (dB) | Comment |
| --- | --- | --- |
| Antenna to Antenna | 10 | Main antenna to diversity antenna |
| PA (out) to PA (in) | 60 | PCB isolation (PA forward mixing) |
| Triplexer | 20 | High/low band isolation |
| Diplexer | 25 | High/low band isolation |
| PA (out) to PA (out) | 60 | L-H/H-L cross-band |
| PA (out) to PA (out) | 50 | H-H cross-band |
| LNA (in) to PA (out) | 60 | L-H/H-L cross-band |
| LNA (in) to PA (out) | 50 | H-H cross-band |
| Duplexer | 50 | Tx band rejection at Rx band |

The existing MSD levels in TS38.101-3 show in Table 8.

Table 8: MSD test requirements for PC3 EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| CA_n41A-n71A | n41 | 2614 | 5 | 25 | 2614 | N/A | N/A |
|  | n71 | 665 | 5 | 25 | 619 | 11 | IMD4 |
| DC_71A_n78A | 71 | 681.5 | 5 | 25 | 635.5 | 5.5 | IMD5 |
|  | n78 | 3361.5 | 10 | 50 | 3582.5 | N/A | N/A |

Based on above assumptions and test configuration, we proposed the MSD levels as below.

Table 9: Proposed MSD test configuration and results by IMD problems for PC2 NR inter-band CA UE

| CA bands | UL band | IMD | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n41A-n71A | n41 | IMD4 ¦3*fn71 -fn41¦ | 2614 | 5 | 25 | 2614 | 5 | 18.2 |
|  | n71 |  | 665 | 5 | 25 | 619 | 5 | N/A |
| CA_n71A-n77A | n71 | IMD5 ¦4*fn71 -fn77¦ | 681.5 | 5 | 25 | 635.5 | 5 | 12.2 |
|  | n77 |  | 3361.5 | 10 | 50 | 3582.5 | 10 | N/A |

4	Conclusion

Above simulation results, we recommend to merged the other interested companies’ simulation results as average manner to derive MSD levels and propose as follow

Proposal 1: For cross-band isolation issue of PC2 NR inter band CA UE, the proposed MSD values in Table 5 shall be considered in TS38.101-1.

Proposal 2: For IMD problem by dual uplink transmission of PC2 NR inter band CA UE, the proposed MSD values in Table 9 shall be considered in TS38.101-1.

Reference

[1] RP-210477 “Status report of WI: Rel-17 High power UE (power class 2) for NR inter-band Carrier Aggregation with 2 bands downlink and 2 bands uplink; rapporteur: China Telecom”,	 RAN WG4

[2] RP-210476 “Revised WID: High power UE for NR inter-band Carrier Aggregation with 2 bands downlink and x bands uplink (x =1,2)”,	 China Telecom

[3] R4-2101125	Draft TR 38.xxx v0.2.0: High power UE for NR inter-band Carrier Aggregation with 2 bands downlink and x bands uplink (x =1,2)”, 	China Telecom

[4] TS38.101-1

[5] TS38.101-3