3GPP TSG-RAN WG4 Meeting#98-bis             	R4-2104990

E-meeting, 12th – 20th April, 2021

Title:	TP for TR 37.826 on PC2 DC_5A_n78A

Source:	SK Telecom, LG Electronics, Murata Manufacturing Co Ltd.

Agenda item:	7.21.2

Document for:	Approval

1	Introduction

This contribution is a text proposal for TR 37.826 [xx] to include DC_5A_n78A.

2	References

[xx]	R4-2100084, 3GPP TR 37.826 V0.2.0: "Rel-17 High power UE (power class 2) for EN-DC with 1 LTE band + 1 NR TDD band".

[yy]	R4-2100286, 3GPP RAN4#98-e, "TP on MSD test results for Power Class 2 UE for DC (1 LTE FDD band +1 NR TDD band) UE", LG Electronics.

3	Text Proposal

***Start of Text Proposal ***

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2] 	3GPP TS 38.101-3: "NR; User Equipment (UE) radio transmission and reception; Part 3:  Range 1 and Range 2 Interworking operation with other radios".

[yy]	R4-2100286, 3GPP RAN4#98-e, "TP on MSD test results for Power Class 2 UE for DC (1 LTE FDD band +1 NR TDD band) UE", LG Electronics.

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

6.x	DC_5A_n78A

6.x.1	Transmitter Characteristics

6.x.1.1	Maximum Output Power

Table 6.x.1.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_5A_n78A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

6.x.1.2	Co-existence study

According to the PC3 DC_5A_n78A study, the 4th IMD products from dual uplink of band 5 and n78 may fall into the band 5 Rx frequency range. Thus, additional MSD for IMD 4 should be considered to mitigate the impact of the interference for PC2 DC_5A_n78A combination.

6.x.2	Receiver Characteristics

6.x.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands

MSD values are provided from a previous contribution [yy] and a new test result (33dBm for PA forward and 65 dB for PCB isolation), the proposed MSD levels are merged in Table 6.x.2.1-1.

Table 6.x.2.1-1: Proposed MSD levels from interested companies for PC2 DC_5_n78

| DC bands | UL DC | IMD |  | LGE | SKT, Murata | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| DC_5A_n78A | 5 | IMD4 | ¦3*fB5 -fn78¦ | 17.7 [yy] | 17.9 | 17.8 |
|  | n78 |  |  |  |  |  |

Based on the merged MSD Table, the test points and required MSD levels are proposed in Table 6.x.2.1-2.

Table 6.x.2.1-2: MSD test points for PCell due to dual uplink operation for PC2 EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_5A_n78A | 5 | 844 | 5 | 25 | 889 | 17.8 | IMD4 |
|  | n78 | 3421 | 10 | 52 | 3421 | N/A | N/A |

***End of Text Proposal ***