3GPP TSG-RAN WG4 Meeting # 98-bis-e	R4-2104992

# Electronic Meeting, 12 - 20 April, 2021

Source:	LG Electronics

Title:	Simulation results for NR V2X single link tests

Agenda item:	5.2.1.2

Document for:	Discussion

Introduction

In last meeting, the WF [1] and simulation assumption [2] for single link tests for NR V2X demodulation was approved. In this contribution, we provide simulation results and impairment results based on [2].

Discussion

Test cases for PSSCH demodulation

Simulation results for PSSCH demodulation are shown in Figure 1, and impairment results are captured in Table 1.

Figure 1 Simulation results for PSSCH demodulation test cases

Table 1 Impairment results for PSSCH demodulation

| Test case | SNR @ 10% BLER |  |
| --- | --- | --- |
|  | w/o impairment | w/ impairment |
| QPSK TDLA30-2700 | 1.23 | 2.73 |
| 16QAM TDLA30-1400 | 5.77 | 7.27 |
| 64QAM TDLA30-180 | 12.03 | 14.03 |

Test cases for PSCCH demodulation

Simulation result for PSCCH demodulation is shown in Figure 2, and impairment result is captured in Table 2.

Figure 2 Simulation results for PSCCH demodulation test case

Table 2 Impairment result for PSCCH demodulation

| Test case | SNR @ 1% BLER |  |
| --- | --- | --- |
|  | w/o impairment | w/ impairment |
| PSCCH | 3.03 | 4.53 |

Test cases for PSBCH demodulation

Simulation result for PSBCH demodulation is shown in Figure 3, and impairment result is captured in Table 3.

Figure 3 Simulation results for PSBCH demodulation test case

Table 3 Impairment result for PSBCH demodulation

| Test case | SNR @ 1% BLER |  |
| --- | --- | --- |
|  | w/o impairment | w/ impairment |
| PSBCH | -3.32 | -1.82 |

Test cases for PSFCH demodulation

Simulation result for PSFCH demodulation is shown in Figure 4, and impairment result is captured in Table 4.

Figure 4 Simulation results for PSFCH demodulation test case

Table 4 Impairment result for PSFCH demodulation

| Test case | SNR @ 1% BLER |  |
| --- | --- | --- |
|  | w/o impairment | w/ impairment |
| PSFCH | 5.98 | 7.48 |

Conclusion

In this contribution, we provide simulation results and impairment results for V2X demodulation single link test cases.

Reference

[1] R4-2103990, “WF on single link tests for NR V2X demodulation performance,” LG Electronics.

[2] R4-2103991, “Updated simulation assumptions for V2X single link test cases,” Huawei, HiSilicon.