3GPP TSG-RAN WG4 Meeting # 98BIS-e	R4-2104994

# Electronic Meeting, 12th – 20th April, 2021

Source: 	LG Electronics

Title: 	MPR initial results for NR PC2 intra-band contiguous CA UE according to RF architecture

Agenda Item:	8.2.2.4

Document for:	Approval

Introduction

In the RAN4#98 e-meeting a way forward on assumptions & RF architectures [1] for PC2 contiguous UL CA MPR/A-MPR was approved as follow.

To achieve PC2 capability the architecture must be able to achieve 26dBm with different allocation in each CC => >23dBm in one CC vs <23dBm in the other CC PC2 contiguous UL CA architecture options. Arch; description; Coherence with PC3; constrains; MPR; prerequisite. #1; 1x26dBm PA + 1LO with 200MHz BW; Equivalent to baseline; need second 26dBm PA to support UL MIMO; baseline; none. #2; 2x26dBm PA + 2LO with 100MHz BW; Equivalent to PC3 option; only for class C no UL MIMO (or 4 PAs); covered by #1 for class C; none. #3; 2x23dBm PA + 1LO with 200MHz BW; Not applicable; must use TxDiv or UL MIMO; need assessment of additional MPR for TxDiv on top of UL CA; single CC TxDiv MPR/signalling. - 1x26dBm PA + 1LO with 200MHz BW is the baseline for MPR/A-MPR and can be defined now - Other architecture options are assessed. a single set of requirement is preferred but will be discussed based evaluation of back-off results for the same allocation definition scenarios (inner/outer from PC3 for contiguous allocation and inner/outer1/2 from PC3 for non-contiguous allocation) - FFS if edge RB allocation are introduced - 2x26dBm PA + 2LO with 100MHz BW is assessed for class C, assumption is that no extra back-off should be defined compared to baseline - 2x23dBm PA + 1LO with 200MHz BW is assessed, need for additional MPR requirement or delta MPR will be assessed, if difference is small a single requirement may be achievable - PA calibration for 20MHz QPSK DFT-s-OFDM 100RB0 waveform based on 4dB post PA losses and 1dB MPR. - 26dBm/antenna: 29dBm at 31dB ACLR - 23dBm/antenna: 26dBm at 30dB ACLR - 2 PA case use 10dB antenna isolation - Equal PSD and Equal back-off power split - For 2 PA (23 or 26dBm): - Measurements is used where two PA are coupled on the output recreating the 10dB antenna isolation assumption - Emission requirements (ACLR/SEM/spurious emissions) are checked by summing the power of the two transmit paths - Since simulation are not available, at least some worst case corners are evaluated for all modulation order Note: companies have already provided results for: - 1x26dBm 200MHz PA for BW class C in [2] - 1x26dBm 100MHz for BW class B and 2x26dBm for BW class C in [3]

In this contribution, we provide initial MPR simulations results for PC2 intra-band UL CA UE.

Discussion

MPR results for PC2 NR intra-band contiguous RB allocation

Way forward [1] also agrees on the MPR and A-MPR evaluation assumptions as follows:

- Inner/outer contiguous allocations definition for PC3 is used to evaluate MPR and AMPR - Inner/outer1/outer2 non-contiguous allocations definition for PC3 is used to evaluate MPR and AMPR - Both CP-OFDM and DFT-s-OFDM waveforms are used. - Channel BW configurations valid for class B and C should be evaluated - Class B: at least 20MHz+20MHz 15kHz SCS and 50MHz+50MHz 15kHz SCS - Class C: at least 60MHz+100MHz 30kHz SCS and 100MHz+100MHz 30kHz SCS - MPR based on ACLR and SEM agreements in “WF on RF requirements for PC2 intra-band contiguous UL CA” are evaluated based on a number of inner/outer contiguous and inner/outer1/2 non-contiguous allocation cases - NS-04 A-MPR for -13dBm/MHz IM3 and -25dBm/MHz IM3 are evaluated especially for 1RB+1RB non-contiguous and Full+Full allocation cases - Additionally to [1] the need for additional edge RB allocation definition for class B is evaluated as discussed in [3] Contiguous allocation - Proponents should clarify for which architecture the table is valid for - If multiple architecture are valid which architecture sets the worst case - Measurements in [2] show that PC3 class C table is fine for PC2 1PA case while [3] discusses introduction of Edge allocation for class C - Further discussion is needed with input for other architectures and companies - Consistency with PC3 should be checked

Firstly, we measured the required MPR results for intra-band NR contiguous RB allocation with CA class B (CC1 (20MHz) + CC2 (20MHz)) using 15 kHz SCS and CA class C (CC1 (100MHz) + CC2 (100MHz)) using 30 kHz SCS based on the candidate RF architecture considering 2x23dBm, 2LOs RF architecture. The measured MPR results are provided in Table 1 as follow

Table 1: Measured MPR results (2x23dBm, 2LOs) based on waveforms, modulation order and contiguous RB allocations

| CC1 20 MHz |  | CC2 20 MHz |  | DFT-S-OFDM |  |  |  | CP-OFDM |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SCS | allocated RB | SCS | allocated RB | MPR |  |  |  | MPR |  |  |  |
|  |  |  |  | QPSK | 16-QAM | 64-QAM | 256-QAM | QPSK | 16-QAM | 64-QAM | 256-QAM |
| 15KHz | 001R105 | 15KHz | 001R000 | TBD | TBD | TBD | TBD | 1.0 | 2.0 | 3.0 | 5 |
| 15KHz | 003R103 | 15KHz | 001R000 | TBD | TBD | TBD | TBD | 1.5 | 3.0 | 4.5 | 6 |
| 15KHz | 007R099 | 15KHz | 001R000 | TBD | TBD | TBD | TBD | 1.5 | 3.0 | 4.0 | 6 |
| 15KHz | 010R096 | 15KHz | 003R000 | TBD | TBD | TBD | TBD | 1.0 | 3.0 | 4.0 | 6 |
| 15KHz | 020R086 | 15KHz | 007R000 | TBD | TBD | TBD | TBD | 1.5 | 3.0 | 4.0 | 6 |
| 15KHz | 030R076 | 15KHz | 010R000 | TBD | TBD | TBD | TBD | 1.5 | 3.0 | 4.0 | 6 |
| 15KHz | 045R061 | 15KHz | 010R000 | TBD | TBD | TBD | TBD | 1.5 | 3.0 | 4.0 | 6 |
| 15KHz | 060R046 | 15KHz | 010R000 | TBD | TBD | TBD | TBD | 1.5 | 3.0 | 4.0 | 6 |
| 15KHz | 075R031 | 15KHz | 020R000 | TBD | TBD | TBD | TBD | 2.5 | 3.0 | 4.0 | 6 |
| 15KHz | 090R016 | 15KHz | 030R000 | TBD | TBD | TBD | TBD | 3.0 | 3.5 | 4.0 | 6 |
| 15KHz | 106R000 | 15KHz | 050R000 | TBD | TBD | TBD | TBD | 3.5 | 4.0 | 4.0 | 6 |
| 15kHz | 050R056 | 15kHz | 050R000 | TBD | TBD | TBD | TBD | 1.0 | 3.0 | 4.0 | 5.5 |
| 15kHz | 070R036 | 15kHz | 070R000 | TBD | TBD | TBD | TBD | 2.5 | 3.0 | 4.0 | 6 |
| 15KHz | 106R000 | 15KHz | 106R000 | TBD | TBD | TBD | TBD | 4.0 | 4.5 | 4.5 | 6.5 |
| NR CC1 100 MHz |  | NR CC2 100 MHz |  | DFT-S-OFDM |  |  |  | CP-OFDM |  |  |  |
| SCS | allocated RB | SCS | allocated RB | MPR |  |  |  | MPR |  |  |  |
|  |  |  |  | QPSK | 16-QAM | 64-QAM | 256-QAM | QPSK | 16-QAM | 64-QAM | 256-QAM |
| 30KHz | 001R272 | 30KHz | 001R000 | TBD | TBD | TBD | TBD | 1.0 | 2.5 | 3.5 | 5.5 |
| 30KHz | 005R268 | 30KHz | 001R000 | TBD | TBD | TBD | TBD | 1.0 | 2.5 | 4.0 | 6.0 |
| 30KHz | 010R263 | 30KHz | 003R000 | TBD | TBD | TBD | TBD | 1.5 | 2.5 | 4.0 | 6.0 |
| 30KHz | 020R253 | 30KHz | 007R000 | TBD | TBD | TBD | TBD | 1.0 | 3.0 | 4.0 | 6.0 |
| 30KHz | 035R238 | 30KHz | 010R000 | TBD | TBD | TBD | TBD | 1.5 | 3.0 | 4.0 | 6.0 |
| 30KHz | 055R218 | 30KHz | 015R000 | TBD | TBD | TBD | TBD | 1.5 | 3.0 | 4.0 | 6.0 |
| 30KHz | 075R198 | 30KHz | 015R000 | TBD | TBD | TBD | TBD | 1.5 | 3.0 | 4.0 | 6.0 |
| 30KHz | 100R173 | 30KHz | 020R000 | TBD | TBD | TBD | TBD | 1.5 | 3.0 | 4.0 | 6.0 |
| 30KHz | 130R140 | 30KHz | 030R000 | TBD | TBD | TBD | TBD | 1.5 | 3.0 | 4.0 | 6.0 |
| 30KHz | 160R113 | 30KHz | 050R000 | TBD | TBD | TBD | TBD | 1.5 | 3.0 | 4.0 | 6.0 |
| 30KHz | 190R83 | 30KHz | 070R000 | TBD | TBD | TBD | TBD | 1.5 | 3.0 | 4.0 | 6.0 |
| 30KHz | 220R053 | 30KHz | 100R000 | TBD | TBD | TBD | TBD | 2.5 | 3.0 | 4.0 | 6.0 |
| 30KHz | 250R023 | 30KHz | 150R000 | TBD | TBD | TBD | TBD | 3.5 | 3.5 | 4.0 | 7.0 |
| 30KHz | 273R000 | 30KHz | 200R000 | TBD | TBD | TBD | TBD | 4.0 | 4.0 | 4.5 | 6.0 |
| 30KHz | 273R000 | 30KHz | 273R000 | TBD | TBD | TBD | TBD | 4.5 | 4.5 | 5.5 | 6.5 |

Secondly, we measured the required MPR results for intra-band NR contiguous RB allocation with CA class B (CC1 (20MHz) + CC2 (20MHz)) using 15 kHz SCS and CA class C (CC1 (100MHz) + CC2 (100MHz)) using 30 kHz SCS based on the candidate RF architecture considering 1x26dBm, 1LOs RF architecture. The measured MPR results are provided in Table 2 as follow

Table 2: Measured MPR results (1x26dBm, 1LO) based on waveforms, modulation order and contiguous RB allocations

| CC1 20 MHz |  | CC2 20 MHz |  | DFT-S-OFDM |  |  |  | CP-OFDM |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SCS | allocated RB | SCS | allocated RB | MPR |  |  |  | MPR |  |  |  |
|  |  |  |  | QPSK | 16-QAM | 64-QAM | 256-QAM | QPSK | 16-QAM | 64-QAM | 256-QAM |
| 15KHz | 001R105 | 15KHz | 001R000 | TBD | TBD | TBD | TBD | 1.0 | 1.5 | 3.0 | 5.0 |
| 15KHz | 003R103 | 15KHz | 001R000 | TBD | TBD | TBD | TBD | 1.0 | 2.5 | 4.0 | 5.5 |
| 15KHz | 007R099 | 15KHz | 001R000 | TBD | TBD | TBD | TBD | 1.5 | 2.5 | 4.0 | 6.0 |
| 15KHz | 010R096 | 15KHz | 003R000 | TBD | TBD | TBD | TBD | 1.0 | 2.5 | 4.0 | 6.0 |
| 15KHz | 020R086 | 15KHz | 007R000 | TBD | TBD | TBD | TBD | 1.0 | 2.5 | 4.0 | 6.0 |
| 15KHz | 030R076 | 15KHz | 010R000 | TBD | TBD | TBD | TBD | 1.5 | 3.0 | 4.0 | 6.0 |
| 15KHz | 045R061 | 15KHz | 010R000 | TBD | TBD | TBD | TBD | 2.0 | 3.0 | 4.0 | 6.0 |
| 15KHz | 060R046 | 15KHz | 010R000 | TBD | TBD | TBD | TBD | 2.0 | 3.0 | 4.0 | 6.0 |
| 15KHz | 075R031 | 15KHz | 020R000 | TBD | TBD | TBD | TBD | 3.0 | 3.5 | 4.0 | 6.0 |
| 15KHz | 090R016 | 15KHz | 030R000 | TBD | TBD | TBD | TBD | 4.0 | 4.0 | 5.0 | 6.0 |
| 15KHz | 106R000 | 15KHz | 050R000 | TBD | TBD | TBD | TBD | 4.0 | 4.5 | 5.0 | 6.0 |
| 15kHz | 050R056 | 15kHz | 050R000 | TBD | TBD | TBD | TBD | 1.5 | 2.5 | 4.5 | 5.5 |
| 15kHz | 070R036 | 15kHz | 070R000 | TBD | TBD | TBD | TBD | 3.5 | 3.5 | 4.0 | 5.5 |
| 15KHz | 106R000 | 15KHz | 106R000 | TBD | TBD | TBD | TBD | 4.5 | 4.5 | 5.0 | 6.0 |
| NR CC1 100 MHz |  | NR CC2 100 MHz |  | DFT-S-OFDM |  |  |  | CP-OFDM |  |  |  |
| SCS | allocated RB | SCS | allocated RB | MPR |  |  |  | MPR |  |  |  |
|  |  |  |  | QPSK | 16-QAM | 64-QAM | 256-QAM | QPSK | 16-QAM | 64-QAM | 256-QAM |
| 30KHz | 001R272 | 30KHz | 001R000 | TBD | TBD | TBD | TBD | 1.0 | 1.5 | 3.5 | 5.0 |
| 30KHz | 005R268 | 30KHz | 001R000 | TBD | TBD | TBD | TBD | 1.5 | 2.5 | 4.0 | 5.5 |
| 30KHz | 010R263 | 30KHz | 003R000 | TBD | TBD | TBD | TBD | 1.0 | 2.5 | 4.0 | 5.5 |
| 30KHz | 020R253 | 30KHz | 007R000 | TBD | TBD | TBD | TBD | 1.0 | 2.5 | 4.0 | 5.5 |
| 30KHz | 035R238 | 30KHz | 010R000 | TBD | TBD | TBD | TBD | 1.5 | 3.0 | 4.0 | 6.0 |
| 30KHz | 055R218 | 30KHz | 015R000 | TBD | TBD | TBD | TBD | 1.5 | 3.0 | 4.0 | 6.0 |
| 30KHz | 075R198 | 30KHz | 015R000 | TBD | TBD | TBD | TBD | 1.5 | 3.0 | 4.5 | 6.0 |
| 30KHz | 100R173 | 30KHz | 020R000 | TBD | TBD | TBD | TBD | 1.5 | 3.0 | 4.0 | 6.0 |
| 30KHz | 130R140 | 30KHz | 030R000 | TBD | TBD | TBD | TBD | 1.5 | 3.0 | 4.0 | 6.0 |
| 30KHz | 160R113 | 30KHz | 050R000 | TBD | TBD | TBD | TBD | 1.5 | 3.0 | 4.0 | 6.0 |
| 30KHz | 190R83 | 30KHz | 070R000 | TBD | TBD | TBD | TBD | 3.5 | 3.5 | 4.0 | 6.0 |
| 30KHz | 220R053 | 30KHz | 100R000 | TBD | TBD | TBD | TBD | 4.0 | 4.0 | 4.0 | 6.0 |
| 30KHz | 250R023 | 30KHz | 150R000 | TBD | TBD | TBD | TBD | 4.5 | 4.5 | 5.0 | 6.0 |
| 30KHz | 273R000 | 30KHz | 200R000 | TBD | TBD | TBD | TBD | 4.5 | 4.5 | 4.5 | 6.0 |
| 30KHz | 273R000 | 30KHz | 273R000 | TBD | TBD | TBD | TBD | 4.5 | 4.5 | 5.0 | 6.0 |

Based on the above MPR results for PC2 intra-band contiguous RB allocation, we propose as follow

Proposal 1: Based on the MPR results, we propose following MPR Table for PC2 NR intra-band contiguous CA UE with contiguous RB allocation.

Table 3: MPR for contiguous RB allocation for Power Class 2

| Modulation |  | MPR for bandwidth class B(dB) |  | MPR for bandwidth class C(dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | inner | outer | inner | outer |
| DFT-s-OFDM | Pi/2 BPSK | 1.0 | 3.5 | 2.5 | 7 |
|  | QPSK | 1.0 | 3.5 | 2.5 | 7 |
|  | 16QAM | 1.5 | 3.5 | 2.5 | 7 |
|  | 64QAM | 3.0 | 4.0 | 5 | 7 |
|  | 256QAM | 5.5 | 6.0 | 7 | 7.5 |
| CP-OFDM | QPSK | 2.0 | 4.0 4.5 | 3.5 | 8 |
|  | 16QAM | 2.5 | 4.0 4.5 | 3.5 | 8 |
|  | 64QAM | 3.5 | 4.0 4.5 | 5 | 8 |
|  | 256QAM | 6.5 | 6.5 | 7 | 8 |

MPR results for PC2 NR intra-band non-contiguous RB allocation

Way forward [1] also agrees on the MPR and A-MPR evaluation assumptions as follows:

- Inner/outer contiguous allocations definition for PC3 is used to evaluate MPR and AMPR - Inner/outer1/outer2 non-contiguous allocations definition for PC3 is used to evaluate MPR and AMPR - Both CP-OFDM and DFT-s-OFDM waveforms are used. - Channel BW configurations valid for class B and C should be evaluated - Class B: at least 20MHz+20MHz 15kHz SCS and 50MHz+50MHz 15kHz SCS - Class C: at least 60MHz+100MHz 30kHz SCS and 100MHz+100MHz 30kHz SCS - MPR based on ACLR and SEM agreements in “WF on RF requirements for PC2 intra-band contiguous UL CA” are evaluated based on a number of inner/outer contiguous and inner/outer1/2 non-contiguous allocation cases - NS-04 A-MPR for -13dBm/MHz IM3 and -25dBm/MHz IM3 are evaluated especially for 1RB+1RB non-contiguous and Full+Full allocation cases - Additionally to [1] the need for additional edge RB allocation definition for class B is evaluated as discussed in [3] Non-Contiguous allocation - Non-contiguous allocation - Proponents should clarify for which architecture the table is valid for - Is multiple architecture are valid which architecture sets the worst case - Measurements in [2] show that additional back-off is needed vs PC3, [3] merges inner and outer1,outer2 - Further discussion is needed with input for other architectures and companies - Note 1 and 2 may be revised for PC2 - Consistency with PC3 should be checked

Firstly, we measured the required MPR results for intra-band NR non-contiguous RB allocation with CA class B (CC1 (20MHz) + CC2 (20MHz)) using 15 kHz SCS and CA class C (CC1 (100MHz) + CC2 (100MHz)) using 30 kHz SCS based on the candidate RF architecture considering 2x23dBm, 2LOs RF architecture. The measured MPR results are provided in Table 4 as follow

Table 4: Measured MPR results (2x23dBm, 2LOs) based on waveforms, modulation order and non-contiguous RB allocations

| CC1 (20 MHz) |  | CC2 (20 MHz) |  | DFT-S-OFDM |  |  |  | CP-OFDM |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SCS | allocated RB | SCS | allocated RB | MPR |  |  |  | MPR |  |  |  |
|  |  |  |  | QPSK | 16-QAM | 64-QAM | 256-QAM | QPSK | 16-QAM | 64-QAM | 256-QAM |
| 15KHz | 001R000 | 15KHz | 001R105 | TBD | TBD | TBD | TBD | 9.5 | 9.5 | 10.0 | 10.0 |
| 15KHz | 003R000 | 15KHz | 001R105 | TBD | TBD | TBD | TBD | 8.5 | 8.5 | 8.5 | 8.5 |
| 15KHz | 007R000 | 15KHz | 001R105 | TBD | TBD | TBD | TBD | 7.5 | 7.5 | 7.5 | 7.5 |
| 15KHz | 010R000 | 15KHz | 003R103 | TBD | TBD | TBD | TBD | 6.0 | 6.0 | 6.0 | 6.0 |
| 15KHz | 020R000 | 15KHz | 007R099 | TBD | TBD | TBD | TBD | 4.5 | 4.5 | 4.5 | 5.5 |
| 15KHz | 030R000 | 15KHz | 010R096 | TBD | TBD | TBD | TBD | 4.0 | 4.0 | 4.0 | 5.5 |
| 15KHz | 045R000 | 15KHz | 010R096 | TBD | TBD | TBD | TBD | 4.0 | 4.0 | 4.0 | 5.5 |
| 15KHz | 060R000 | 15KHz | 010R096 | TBD | TBD | TBD | TBD | 4.0 | 4.0 | 4.0 | 5.5 |
| 15KHz | 075R000 | 15KHz | 020R086 | TBD | TBD | TBD | TBD | 5.0 | 5.0 | 5.0 | 6.5 |
| 15KHz | 090R000 | 15KHz | 030R076 | TBD | TBD | TBD | TBD | 4.5 | 4.5 | 4.5 | 6.0 |
| 15KHz | 106R000 | 15KHz | 050R056 | TBD | TBD | TBD | TBD | 4.5 | 4.5 | 4.5 | 6.0 |
| 15kHz | 050R000 | 15kHz | 050R056 | TBD | TBD | TBD | TBD | 4.0 | 4.0 | 4.0 | 5.5 |
| 15kHz | 070R000 | 15kHz | 070R036 | TBD | TBD | TBD | TBD | 4.0 | 4.0 | 4.0 | 6.0 |
| 15kHz | 001R000 | 15kHz | 001R000 | TBD | TBD | TBD | TBD | 6.5 | 6.5 | 6.5 | 7.0 |
| NR CC1 (100 MHz) |  | NR CC2 (100 MHz) |  | DFT-S-OFDM |  |  |  | CP-OFDM |  |  |  |
| SCS | allocated RB | SCS | allocated RB | MPR |  |  |  | MPR |  |  |  |
|  |  |  |  | QPSK | 16-QAM | 64-QAM | 256-QAM | QPSK | 16-QAM | 64-QAM | 256-QAM |
| 30KHz | 001R000 | 30KHz | 001R272 | TBD | TBD | TBD | TBD | 7.0 | 7.0 | 7.0 | 7.0 |
| 30KHz | 005R000 | 30KHz | 001R272 | TBD | TBD | TBD | TBD | 6.0 | 6.5 | 6.5 | 6.5 |
| 30KHz | 010R000 | 30KHz | 003R270 | TBD | TBD | TBD | TBD | 5.0 | 5.0 | 5.0 | 6.0 |
| 30KHz | 020R000 | 30KHz | 007R268 | TBD | TBD | TBD | TBD | 4.5 | 4.5 | 4.5 | 6.0 |
| 30KHz | 035R000 | 30KHz | 010R263 | TBD | TBD | TBD | TBD | 4.5 | 4.5 | 5.0 | 6.0 |
| 30KHz | 055R000 | 30KHz | 015R258 | TBD | TBD | TBD | TBD | 5.0 | 5.0 | 5.0 | 6.5 |
| 30KHz | 075R000 | 30KHz | 015R258 | TBD | TBD | TBD | TBD | 4.5 | 4.5 | 4.5 | 6.5 |
| 30KHz | 100R000 | 30KHz | 020R253 | TBD | TBD | TBD | TBD | 4.5 | 4.5 | 4.5 | 6.0 |
| 30KHz | 130R000 | 30KHz | 030R243 | TBD | TBD | TBD | TBD | 4.5 | 4.5 | 4.5 | 6.0 |
| 30KHz | 160R000 | 30KHz | 050R223 | TBD | TBD | TBD | TBD | 4.5 | 4.5 | 4.5 | 6.5 |
| 30KHz | 190R000 | 30KHz | 070R203 | TBD | TBD | TBD | TBD | 4.5 | 4.5 | 4.5 | 6.5 |
| 30KHz | 220R000 | 30KHz | 100R173 | TBD | TBD | TBD | TBD | 5.0 | 5.0 | 5.0 | 6.5 |
| 30KHz | 250R000 | 30KHz | 150R123 | TBD | TBD | TBD | TBD | 4.5 | 4.5 | 4.5 | 6.5 |
| 30KHz | 273R000 | 30KHz | 200R073 | TBD | TBD | TBD | TBD | 5.0 | 5.0 | 5.0 | 6.5 |
| 30kHz | 001R000 | 30kHz | 001R000 | TBD | TBD | TBD | TBD | 6.0 | 6.0 | 6.5 | 7.0 |

Secondly, we measured the required MPR results for intra-band NR non-contiguous RB allocation with CA class B (CC1 (20MHz) + CC2 (20MHz)) using 15 kHz SCS and CA class C (CC1 (100MHz) + CC2 (100MHz)) using 30 kHz SCS based on the candidate RF architecture considering 1x26dBm, 1LO RF architecture. The measured MPR results are provided in Table 5 as follow

Table 5: Measured MPR results (1x26dBm, 1LO) based on waveforms, modulation order and non-contiguous RB allocations

| CC1 (20 MHz) |  | CC2 (20 MHz) |  | DFT-S-OFDM |  |  |  | CP-OFDM |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SCS | allocated RB | SCS | allocated RB | MPR |  |  |  | MPR |  |  |  |
|  |  |  |  | QPSK | 16-QAM | 64-QAM | 256-QAM | QPSK | 16-QAM | 64-QAM | 256-QAM |
| 15KHz | 001R000 | 15KHz | 001R105 | TBD | TBD | TBD | TBD | 12.0 | 11.5 | 11.5 | 12.0 |
| 15KHz | 003R000 | 15KHz | 001R105 | TBD | TBD | TBD | TBD | 10.0 | 10.0 | 10.0 | 10.0 |
| 15KHz | 007R000 | 15KHz | 001R105 | TBD | TBD | TBD | TBD | 8.0 | 8.0 | 8.0 | 8.0 |
| 15KHz | 010R000 | 15KHz | 003R103 | TBD | TBD | TBD | TBD | 7.0 | 7.0 | 7.0 | 7.0 |
| 15KHz | 020R000 | 15KHz | 007R099 | TBD | TBD | TBD | TBD | 6.5 | 6.5 | 6.5 | 6.5 |
| 15KHz | 030R000 | 15KHz | 010R096 | TBD | TBD | TBD | TBD | 5.5 | 5.5 | 5.5 | 6.0 |
| 15KHz | 045R000 | 15KHz | 010R096 | TBD | TBD | TBD | TBD | 5.0 | 5.0 | 5.0 | 6.0 |
| 15KHz | 060R000 | 15KHz | 010R096 | TBD | TBD | TBD | TBD | 5.0 | 5.0 | 5.0 | 6.0 |
| 15KHz | 075R000 | 15KHz | 020R086 | TBD | TBD | TBD | TBD | 5.0 | 5.0 | 5.0 | 6.0 |
| 15KHz | 090R000 | 15KHz | 030R076 | TBD | TBD | TBD | TBD | 5.0 | 5.0 | 5.0 | 6.0 |
| 15KHz | 106R000 | 15KHz | 050R056 | TBD | TBD | TBD | TBD | 5.0 | 5.0 | 5.0 | 6.0 |
| 15kHz | 050R000 | 15kHz | 050R056 | TBD | TBD | TBD | TBD | 4.0 | 4.5 | 4.5 | 6.0 |
| 15kHz | 070R000 | 15kHz | 070R036 | TBD | TBD | TBD | TBD | 4.0 | 4.5 | 4.5 | 6.0 |
| 15kHz | 001R000 | 15kHz | 001R000 | TBD | TBD | TBD | TBD | 7.0 | 7.0 | 7.0 | 7.0 |
| NR CC1 (100 MHz) |  | NR CC2 (100 MHz) |  | DFT-S-OFDM |  |  |  | CP-OFDM |  |  |  |
| SCS | allocated RB | SCS | allocated RB | MPR |  |  |  | MPR |  |  |  |
|  |  |  |  | QPSK | 16-QAM | 64-QAM | 256-QAM | QPSK | 16-QAM | 64-QAM | 256-QAM |
| 30KHz | 001R000 | 30KHz | 001R272 | TBD | TBD | TBD | TBD | 7.0 | 7.0 | 7.0 | 7.0 |
| 30KHz | 005R000 | 30KHz | 001R272 | TBD | TBD | TBD | TBD | 6.5 | 7.0 | 7.0 | 7.0 |
| 30KHz | 010R000 | 30KHz | 003R270 | TBD | TBD | TBD | TBD | 5.5 | 5.5 | 5.5 | 6.0 |
| 30KHz | 020R000 | 30KHz | 007R268 | TBD | TBD | TBD | TBD | 5.0 | 5.0 | 5.0 | 6.0 |
| 30KHz | 035R000 | 30KHz | 010R263 | TBD | TBD | TBD | TBD | 4.5 | 4.5 | 4.5 | 6.0 |
| 30KHz | 055R000 | 30KHz | 015R258 | TBD | TBD | TBD | TBD | 4.5 | 4.5 | 4.5 | 6.0 |
| 30KHz | 075R000 | 30KHz | 015R258 | TBD | TBD | TBD | TBD | 4.5 | 4.5 | 4.5 | 6.5 |
| 30KHz | 100R000 | 30KHz | 020R253 | TBD | TBD | TBD | TBD | 4.5 | 4.5 | 4.5 | 6.0 |
| 30KHz | 130R000 | 30KHz | 030R243 | TBD | TBD | TBD | TBD | 4.5 | 4.5 | 4.5 | 6.0 |
| 30KHz | 160R000 | 30KHz | 050R223 | TBD | TBD | TBD | TBD | 4.5 | 4.5 | 4.5 | 6.0 |
| 30KHz | 190R000 | 30KHz | 070R203 | TBD | TBD | TBD | TBD | 4.5 | 4.5 | 4.5 | 6.0 |
| 30KHz | 220R000 | 30KHz | 100R173 | TBD | TBD | TBD | TBD | 4.5 | 4.5 | 4.5 | 6.0 |
| 30KHz | 250R000 | 30KHz | 150R123 | TBD | TBD | TBD | TBD | 5.0 | 4.5 | 4.5 | 6.5 |
| 30KHz | 273R000 | 30KHz | 200R073 | TBD | TBD | TBD | TBD | 4.5 | 4.5 | 4.5 | 6.0 |
| 30kHz | 001R000 | 30kHz | 001R000 | TBD | TBD | TBD | TBD | 6.5 | 6.5 | 6.5 | 7.0 |

Based on the above MPR results for PC2 intra-band contiguous CA UE with non-contiguous RB allocation and refer the other companies MPR results [2] as follow

Proposal 2. We propose MPR Table for PC2 NR intra-band contiguous CA UE with non-contiguous RB allocation.

Table 6: MPR for non-contiguous RB allocation for Power Class 2

| Modulation |  | MPR for bandwidth class B(dB) |  |  | MPR for bandwidth class C(dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | inner | Outer11 | Outer22 | inner | Outer11 | Outer22 |
| DFT-s-OFDM | Pi/2 BPSK | 2 3 | 5.5 6.5 | 11.5 13.0 | 2.53 | 6 6.5 | 13 |
|  | QPSK | 23 | 5.5 6.5 |  | 2.53 | 6 6.5 |  |
|  | 16QAM | 2.53 | 5.5 6.5 |  | 3 | 6 6.5 |  |
|  | 64QAM | 4.5 | 6 6.5 |  | 5 | 6 6.5 |  |
|  | 256QAM | 6 | 6.5 |  | 6.5 | 6.5 |  |
| CP-OFDM | QPSK | 2.5 3 | 6.5 7.0 | 12 14.0 | 3.5 | 7 | 14 |
|  | 16QAM | 3 | 7 |  | 3.5 | 7 |  |
|  | 64QAM | 5 | 7 |  | 5 | 7 |  |
|  | 256QAM | 7.5 | 7.5 |  | 7.5 | 7.5 |  |
| NOTE 1: Outer 1 MPR for Pi/2 BPSK and QPSK is reduced by 2dB for aggregated allocation bandwidth > 10MHz NOTE 2: Outer 2 MPR is reduced by 4.5dB for aggregated allocation bandwidth > 10MHz |  |  |  |  |  |  |  |

Conclusion

In this contribution, we propose MPR requirements for PC2NR intra-band contiguous CA UE according to RF architecture as follow

.

Proposal 1: Based on the MPR results, we propose following MPR Table for PC2 NR intra-band contiguous CA UE with contiguous RB allocation.

Table 3: MPR for contiguous RB allocation for Power Class 2

| Modulation |  | MPR for bandwidth class B(dB) |  | MPR for bandwidth class C(dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | inner | outer | inner | outer |
| DFT-s-OFDM | Pi/2 BPSK | 1.0 | 3.5 | 2.5 | 7 |
|  | QPSK | 1.0 | 3.5 | 2.5 | 7 |
|  | 16QAM | 1.5 | 3.5 | 2.5 | 7 |
|  | 64QAM | 3.0 | 4.0 | 5 | 7 |
|  | 256QAM | 5.5 | 6.0 | 7 | 7.5 |
| CP-OFDM | QPSK | 2.0 | 4.0 4.5 | 3.5 | 8 |
|  | 16QAM | 2.5 | 4.0 4.5 | 3.5 | 8 |
|  | 64QAM | 3.5 | 4.0 4.5 | 5 | 8 |
|  | 256QAM | 6.5 | 6.5 | 7 | 8 |

Proposal 2. We propose MPR Table for PC2 NR intra-band contiguous CA UE with non-contiguous RB allocation.

Table 6: MPR for non-contiguous RB allocation for Power Class 2

| Modulation |  | MPR for bandwidth class B(dB) |  |  | MPR for bandwidth class C(dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | inner | Outer11 | Outer22 | inner | Outer11 | Outer22 |
| DFT-s-OFDM | Pi/2 BPSK | 2 3 | 5.5 6.5 | 11.5 13.0 | 2.53 | 6 6.5 | 13 |
|  | QPSK | 23 | 5.5 6.5 |  | 2.53 | 6 6.5 |  |
|  | 16QAM | 2.53 | 5.5 6.5 |  | 3 | 6 6.5 |  |
|  | 64QAM | 4.5 | 6 6.5 |  | 5 | 6 6.5 |  |
|  | 256QAM | 6 | 6.5 |  | 6.5 | 6.5 |  |
| CP-OFDM | QPSK | 2.5 3 | 6.5 7.0 | 12 14.0 | 3.5 | 7 | 14 |
|  | 16QAM | 3 | 7 |  | 3.5 | 7 |  |
|  | 64QAM | 5 | 7 |  | 5 | 7 |  |
|  | 256QAM | 7.5 | 7.5 |  | 7.5 | 7.5 |  |
| NOTE 1: Outer 1 MPR for Pi/2 BPSK and QPSK is reduced by 2dB for aggregated allocation bandwidth > 10MHz NOTE 2: Outer 2 MPR is reduced by 4.5dB for aggregated allocation bandwidth > 10MHz |  |  |  |  |  |  |  |

- References

[1]	R4-2103232 “WF on MPR&AMPR simulation assumption for PC2 intra-band contiguous UL CA,” Skyworks Solutions Inc.,

[2] R4-2100544 “PC2 Class C UL CA UE Architecture and MPR/A-MPR evaluation, Skyworks Solutions Inc.

[3] R4-2101160 “HPUE TDD+TDD MPR and AMPR,” Qualcomm

[4] R4-2101103	“Discussion on HP UE for TDD intra-band contiguous UL CA,” Xiaomi