3GPP TSG-RAN WG4 Meeting #98-bis-e	R4-2105006

# Electronic meeting, April 12th – April 16th, 2021

Agenda Item:	7.24.2.1

Source: 	MediaTek Inc.

Title: 	MSD due to cross band isolation evaluation for n40 supporting 90/100MHz CBW

Document for:	Approval

- 1. Introduction

Following the WF in [1], the contribution discuss the impact on increasing n40 UL CBW as an aggressor to close band proximity MSD

- 2. Discussion

To evaluate impact on increasing n40 UL CBW, we calculate the ACP order for adjacent bands with existing CA/EN-DC combos as below table:

| Band | F_gap/BW ratio (90Mhz) | F_gap/BW ratio (100Mhz) | Aggressor ACP order |
| --- | --- | --- | --- |
| B1/n1 | 1.444 | 1.3 | ACP2 (5th order IMD) |
| B7/n7 | 2.444 | 2.2 | ACP3 (7th order IMD) |
| B41/n41 | 1.067 | 0.96 | ACP2/ACP1 |

Table 1 ACP order of aggressor impact on adjacent bands receiving range

For impact on B7/n7, since the RX range is far enough (up to 7th IMD), the impact of n40 supporting 90/100MHz is almost no difference with n40 existing CBWs. For CA_n40-n41 or DC_B41_n40, in our understanding the combination does not support mandatory simultaneous RX/TX capability. Discussion for MSD due to supporting simultaneous RX/TX for the combination is out of the scope of the contribution. For impact on B1/n1, it will suffer ACP2 of n40 uplink. With above reasons, we would focus on the impact on B1/n1 receiving range.

CA_1A-40A has been specified in LTE and it was intuitively to copy the requirement of MSD due to cross band isolation to same band combination in NR CA. However, since the CBW is much wider than that of LTE, test condition and MSD needs to be updated accordingly.

The REFSENS calculation assumptions are listed in table 2 and REFSENS analysis is in table3. From discussion in [2] and from our vendor pool, we are fine with the 30dB filter rejection assumption for B1/n1 and n40. We also assume the ACP2 is 50dBc below the wanted signal and the TX noise profile is flat within the ACP2 range.

| Parameter | Value | Unit |
| --- | --- | --- |
| Antenna isolation | 10 | dB |
| n40 filter rejection in n1 RX range | 30 | dB |
| Front-end loss | 4 | dB |
| Thermal noise at n1 RX ANT port | -167 | dBm/Hz |
| PA noise for UL=90MHz | -106.5 | dBm/Hz |
| PA noise for UL=100MHz | -107 | dBm/Hz |
| Transceiver effective phase noise due to wider BW | -150 | dBc/Hz |
| SNR requirement for QPSK | -1 | dB |

Table 2 Typical receiver performance parameters for MSD analysis

| Direct Signal Path |  |  |  |
| --- | --- | --- | --- |
| Parameter | Main | Diversity |  |
| n40 TX power at antenna port (Primary path) | 23 | 23 | dBm |
| n40 PA output noise power at RX freq | -106.5 | -106.5 | dBm/Hz |
| n40 TX noise power at PRX LNA port | -83.9 | NA | dBm/25MHz |
| n40 TX noise power at DRX LNA port | NA | -83.9 | dBm/25MHz |
| n40 receiver reciprocal mixing noise at LNA port | -101.5 | -101.5 | dBm/25MHz |
| Thermal noise at RX ant port | -167 |  | dBm/Hz |
| Total noise level refer to receiver LNA input port | -83.5 | -83.5 | dBm |
| Combined |  |  |  |
| MSD (5MHz BW) | 16.5 |  | dB |

Table 3 Link analysis for n5 SCS=15 KHz, 25 MHz REFSENS calculation

From our analysis, the PA noise of UL=90MHz is slightly worse(higher) than that of 100MHz CBW, thus we would propose the worst test case only which is n40 UL=90MHz.

Proposal 1: Reference sensitivity exceptions (MSD) due to cross band isolation for NR CA FR1 as below:

| NR Band / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n1 | n40 | 6.6 | 6.6 | 6.6 | 6.6 | 6.6 | 6.6 | 6.6 | 6.6 | 6.6 |  | 6.6 | 6.6 | 6.6 |
| n40x | n1 | 16.5 | 16.5 | 16.5 | 16.5 |  |  |  |  |  |  |  |  |  |
| NOTE x: UL CBW = 90MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4 Reference sensitivity exceptions (MSD) due to cross band isolation

| NR Band / SCS / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| n1 | n40 | 15 | 25 | 50 | 75 | 100 | 100 | 100 | 100 | 100 | 100 |  | 100 | 100 | 100 |
| n40 | n1 | 30 | 245 | 245 | 245 | 245 |  |  |  |  |  |  |  |  |  |

Table 5 RB allocations for MSD due to cross band isolation

The MSD value looks large that may make performance of the combination worse. An alternative thinking is to restrict aggressor uplink bandwidth. This was also mentioned by a company in [3]. We are open to further discuss if restriction of aggressor bandwidth is acceptable by RAN4 companies.

Conclusion

Proposal 1: Reference sensitivity exceptions (MSD) due to cross band isolation for NR CA FR1 as below:

| NR Band / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n1 | n40 | 6.6 | 6.6 | 6.6 | 6.6 | 6.6 | 6.6 | 6.6 | 6.6 | 6.6 |  | 6.6 | 6.6 | 6.6 |
| n40x | n1 | 16.5 | 16.5 | 16.5 | 16.5 |  |  |  |  |  |  |  |  |  |
| NOTE x: UL CBW = 90MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4 Reference sensitivity exceptions (MSD) due to cross band isolation

| NR Band / SCS / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| n1 | n40 | 15 | 25 | 50 | 75 | 100 | 100 | 100 | 100 | 100 | 100 |  | 100 | 100 | 100 |
| n40 | n1 | 30 | 245 | 245 | 245 | 245 |  |  |  |  |  |  |  |  |  |

Table 5 RB allocations for MSD due to cross band isolation

Reference:

- R4-2103181	Way forward on adding 90 and 100MHz channel BW for UE in band n40, RAN4#98-e

- R4-2103303	Email discussion summary for [98e][113] NR_R16_Maintenance, RAN4#98-e

- R4-2103308	Email discussion summary for [98e][120] NR_bands_R17_BWs