3GPP TSG-RAN WG4 Meeting # 98-bis-e                                                      R4-2105014

# Electronic Meeting, Apr. 12-20, 2021

Title: 	Discussion on the PC2 UE RF requirements of NR n34

Source: 	CMCC

Agenda item:	7.41.2

Document for:	Approval

Introduction

The new WI on High power UE (power class 2) for NR band n34 was approved in RAN#91-meeting [1]. The purpose is to develop a new feature to enable HPUE (power class 2) for NR band n34.

In this contribution, we would like to discuss how to define the UE RE requirements of PC2 26dBm for NR band n34.

Discussion

As stated in WID [1],

“The core objectives of the WI are:

- Introduction of NR band n34 to support high power UE (Power class 2)

- Specify RF characteristics for n34, including UE maximum output power, Tx power tolerance and UL-MIMO for PC2 n34.

- Release independent issue is to be considered for PC2 n34”

We present our proposals for PC2 n34 UE RF requirements.

PC2 n34 UE RF requirements

- UE MOP and Tx power tolerance - UL MIMO - MPR - AMPR - Release independent issue is to be considered for PC2

- UE MOP and Tx power tolerance

The following UE power classes in table 6.2.1-1 define the maximum output power and Tx power tolerance for n34. The MOP and Tolerance are to be specified as 26dBm ±2 dB for band n34 of power class 2.

# Table 6.2.1-1: UE Power Class

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n34 |  |  |  |  | 26 | ±2 | 23 | ±2 |

Proposal 1:  The MOP and Tolerance are to be specified as 26dBm ±2 dB for band n34 of power class 2.

- UL-MIMO

The following Table 6.2D.1-1 defines UE Power Class for UL MIMO for n34. The MOP and Tolerance are to be specified as 26dBm +2/-3 dB for power class 2 n34 UL MIMO.

# Table 6.2D.1-1: UE Power Class for UL MIMO in closed loop spatial multiplexing scheme

| NR band | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n34 |  |  | 26 | +2/-3 | 23 | +2/-3 |  |  |

Proposal 2:  The MOP and Tolerance for UL MIMO are to be specified as 26dBm +2/-3 dB for n34 of power class 2.

- MPR

No changes to PC2 MPR requirements.

- A-MPR

Since no A-MPR issue for PC2 n34, No changes to A-MPR requirements.

- Release independent issue is to be considered for PC2

Referring to the description of NR UE power class in TS38.307, the feature of TDD power class 2 can be supported form Rel-15 by release independent manner.

Table 5.1-2: NR UE power class

| Feature | Duplex-mode | Release independent from | Requirements to be fulfilled (see TS 38.307 of the release in which the power class was introduced) |
| --- | --- | --- | --- |
| Power Class 1 | FDD | Rel-15 | Table B.4.1-1, Table B.4.3-1 |
| Power Class 1.5 | TDD | Rel-15 | Table B.4.1-1, Table B.4.3-1 |
| Power Class 2 | TDD | Rel-15 | Table B.4.1-1, Table B.4.3-1 |
| Power Class 3 | FDD, TDD, SUL | Rel-15 | Table B.4.1-1, Table B.4.3-1 |

Proposal 3:  The n34 power class 2 can be supported form Rel-15 by release independent manner.

Conclusions

In this paper, we discuss the RF requirements to be specified in order to accomplish introduction of n34 Power class 2. We propose:

Proposal 1:  The MOP and Tolerance are to be specified as 26dBm ±2 dB for band n34 of power class 2.

Proposal 2:  The MOP and Tolerance for UL MIMO are to be specified as 26dBm +2/-3 dB for n34 of power class 2.

Proposal 3:  The n34 power class 2 can be supported form Rel-15 by release independent manner.

References

[1] RP-210844, WID on High power UE (power class 2) for NR band n34, CMCC