3GPP TSG-RAN WG4 Meeting #98-bis-e                                                         R4-2105024

Electronic Meeting, Apr. 12-20, 2021

Source:	Samsung

Title:	Discussion on FR2 HST Deployment Scenario-B

Agenda item:			8.7.2.2

Document for:	Discussion

# 1. Introduction

In RAN Plenary #89-e, the RAN4-led work item of NR support for high speed train scenario in FR2 has been approved [1, RP-202118] (which has been further revised to [2, RP-210800]). The deployment scenarios for HST in FR2 have been further discussed in last RAN4 meeting (RAN4#98-e), with the following two scenarios agreed [5, R4-2103240]:

| Scenario | Ds (meter) | Dmin (meter) | Prioritization | Note |
| --- | --- | --- | --- | --- |
| A | 700 | 10 | Prioritised | Changed from Original Scenario-2 |
| B | 700 | 150 | Prioritised | New Scenario (same as FR1 deployment) |

In this contribution, we would like to further provide our viewpoints on deployment Scenario-B, along with the accompanying discussion paper on Scenario-A [6].

# 2. Uni-directional RRH Deployment for Scenario-B

## 2.1 Assumption for Analysis

By following the assumption agreed in WF [5, R4-2103240], the assumption for following analysis is provided in the following table, with the highlighted ones be the selected option or the alternative not captured in WF.

Table 2.1-1 Assumption for Uni-directional RRH Deployment

| Parameter | Value |
| --- | --- |
| Carrier frequency | 28 GHz |
| Ds and Dmin | Scenario-B: Ds = 700m and Dmin = 150m |
| RRH parameters |  |
| RRH height | 15 m |
| RRH Tx Power | 31 dBm |
| RRH antenna array model | [Mg, Ng, M, N, P]=[1, 1, 4, 8, 2] [Mg, Ng, M, N, P]=[1, 1, 8, 8, 2] [Mg, Ng, M, N, P]=[1, 1, 8, 16, 2] Newly updated beamforming parameters in WF. |
| RRH panel orientation | Option-1: RRH panel boresight pointed to the railway at the distance of Ds (projection of the neighboring RRH on the railway) Accordingly, for Scenario-B: - Azimuth angle: 12.1 degree - Down-titling: 1.2degree |
| Number of RRH sites per BBU | 4 |
| Number of RRH panels per RRH sites | 1 (i.e., uni-directional) |
| Number of Analog Beams per RRH | 1 or 2 Note1 |
| Propagation model | RMa LOS |
| UE parameters |  |
| UE antenna height | 5m |
| UE(PC4) antenna array model | [Mg, Ng, M, N, P]=[1, 1, 4, 4, 2] 5dBi per element antenna gain |
| UE panel orientation | Direction is opposite to RRH boresight direction, i.e., Scenario-B: Azimuth angle: 12.1 degree Down-titling: 1.2degree |

## 2.2 Analyitical Evalution

#### 2.2.1 Single-Beam-per-Panel

By adopting one beam per RRH panel, i.e., the beam direction is the same as RRH boresight direction, the received power’s trends along the HST track are provided in Figure 2.2.1-1/2/3. It should be noted that only one RX beam is considered in the below analysis.

| Rx Power without UE beamforming | Rx Power with UE beamforming |
| --- | --- |

Figure 2.2.1-1 RX power w./without UE RX beamforming, Single-Beam-per-Panel, [Mg, Ng, M, N, P]=[1, 1, 4, 8, 2]

| Rx Power without UE beamforming | Rx Power with UE beamforming |
| --- | --- |

Figure 2.2.1-2 RX power w./without UE RX beamforming, Single-Beam-per-Panel, [Mg, Ng, M, N, P]=[1, 1, 8, 8, 2]

| Rx Power without UE beamforming | Rx Power with UE beamforming |
| --- | --- |

Figure 2.2.1-3 RX power w./without UE RX beamforming, Single-Beam-per-Panel, [Mg, Ng, M, N, P]=[1, 1, 8, 16, 2]

For Scenario-B uni-directional RRH deployment, without UE beamforming, it is observed that around 20dB higher than QPSK Reference for all three RRH panel configurations, illustrated by Figure 2.2.1-1, 2 and 3. To compared with Scenario-A for around 30dB margin as demonstrated in our accompanying discussion paper, Scenario-B is more challenging in terms of coverage.

Particularly, if we also adopt that Option-1 for RRH panel orientation, i.e., RRH panel boresight pointed to the railway at the distance of Ds (projection of the neighboring RRH on the railway), it can be observed that the main lobe’s coverage region is smaller compared with Scenario-A, which will push the beam switching point away from the serving RRH. For example, if we look at RRH panel model [Mg, Ng, M, N, P]=[1, 1, 8, 16, 2], the RRH coverage region by RRH-1 at origin is ~457 meters from the RRH site, which means the region from origin to ~457m needs to be covered by its neighboring RRH.

Observation 1: For uni-directional RRH deployment in Scenario-B, with single beam per RRH utilized, there is around 20dB margin compared against PC4 REFSENS requirement, which is 10dB lower than Scenario-A.

#### 2.2.2 Dual-Beam-per-Panel

Similar to Scenario-A, we want to evaluate how much enhancement we can obtained if one additional beam is implemented on each RRH panel, with other parameters not changed. Based on the results from Section 2.2.1, the direction of the newly added beam needs to point to the region near to RRH, where the coverage is problematic.

For instance, if the newly add beam (in below figure denoted by beam-1) is directed to the point near to its RRH, e.g., with θbeam1=2 degree and φbeam1=20 degree relative to boresight direction for [M, N] = [4, 8]. The beam-2 in the below figure denotes the same beam used in single-beam-per-Panel deployment, with θbeam2=0 degree and φbeam2=0 degree, i.e., the same as boresight direction.

| Rx power without UE beamforming | Rx power with UE beamforming |
| --- | --- |

Figure 2.2.2-1 RX power w./without UE RX beamforming, Dual-Beam-per-Panel, [Mg, Ng, M, N, P]=[1, 1, 4, 8, 2]

| Rx power without UE beamforming | Rx power with UE beamforming |
| --- | --- |

Figure 2.2.2-2 RX power w./without UE RX beamforming, Dual-Beam-per-Panel, [Mg, Ng, M, N, P]=[1, 1, 8, 8, 2]

| Rx power without UE beamforming | Rx power with UE beamforming |
| --- | --- |

Figure 2.2.2-3 RX power w./without UE RX beamforming, Dual-Beam-per-Panel, [Mg, Ng, M, N, P]=[1, 1, 8, 16, 2]

For Scenario-B, with uni-directional RRH deployment two-beam-per-panel, the newly added beam (i.e., beam-2) helps to provide the signal coverage for the region near to RRH site. From the left figure to the right one, because the fixed RX beamforming is oppositely directed toward the RRH panel boresight direction, beam-2 obtain more RX beamforming gain compared with beam-1. However, even without UE Rx beam sweeping, Rx power for RRH’s beam-1 is not worse than beam-2, so UE boresight direction is fixed to RRH’s beam-2 is reasonable configuration.

Observation 2: For uni-directional RRH deployment in Scenario-B, with 2 beams used per RRH panel, the cellular coverage over the region near to RRH site can be significantly enhanced, even for one fixed RX beam used in UE side.

## 2.3 Beam Switching Point and Beam Dwelling Time

Similar to the counterpart for Scenario-A in our accompanying paper [6], here we provide the simulation results for the switching points between RRHs for single-beam-per-panel, described in section 2.3.1, and for dual-beam-per-panel deployments, the similar method is followed, “Ds_offset” value for dual-beam-per-panel deployments is omitted here. More detailed interpretation for “Switch Point” and “Ds_offset” referring to section 2.3.1 in [6].

#### 2.3.1 Switch Point Determination for HST-DPS Channel Modeling

Table 2.3.1-1 Beam Switch Point for Single-Beam-per-Panel, Alt-1: UE moving away from serving beam

|  | RRH antenna array model |  |  |
| --- | --- | --- | --- |
|  | [Mg, Ng, M, N, P] =[1, 1, 4, 8, 2] | [Mg, Ng, M, N, P] =[1, 1, 8, 8, 2] | [Mg, Ng, M, N, P] =[1, 1, 8, 16, 2] |
| Ds_offset (meter) (refer to Figure 2.3-1 in [6]) | 370 + Ds | 373 + Ds | 457 + Ds |

Table 2.3.1-2 Beam Switch Point for Single-Beam-per-Panel, Alt-2: UE moving away from serving beam

|  | RRH antenna array model |  |  |
| --- | --- | --- | --- |
|  | [Mg, Ng, M, N, P] =[1, 1, 4, 8, 2] | [Mg, Ng, M, N, P] =[1, 1, 8, 8, 2] | [Mg, Ng, M, N, P] =[1, 1, 8, 16, 2] |
| Ds_offset (meter) (refer to Figure 2.3-2 in [6]) | 370 | 373 | 457 |

#### 2.3.2 Beam Dwelling Time

Table 2.3.2-1 Beam Dwelling Time for Dual-Beam-per-Panel, Uni-directional RRH Deployment

|  |  |  | RRH antenna array model |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | [Mg, Ng, M, N, P]=[1, 1, 4, 8, 2] | [Mg, Ng, M, N, P]=[1, 1, 8, 8, 2] | [Mg, Ng, M, N, P]=[1, 1, 8, 16, 2] |
| Uni-directional RRH Two-beam-per-panel | RRH beam1 dwelling distance(m) |  | 157 | 155 | 223 |
|  | RRH beam1 dwelling time(s) | speed = 260km/h | 2.71 | 2.15 | 3.09 |
|  |  | speed = 350km/h | 1.61 | 1.59 | 2.29 |
|  | RRH beam2 dwelling distance(m) |  | 543 | 545 | 477 |
|  | RRH beam2 dwelling time(s) | speed = 260km/h | 7.52 | 7.55 | 6.60 |
|  |  | speed = 350km/h | 2.59 | 5.61 | 4.91 |

Observation 3: For uni-directional RRH deployment for Scenario-B, the smallest beam dwelling time can be in the range of [1.61, 2.29] seconds for UE maximum speed of 350kmph.

# 3. Bi-directional RRH Deployment for Scenario-B

## 3.1 Two Schemes for Bi-directional Deployment

As analyzed in our discussion in last RAN4 meeting, for bi-directional RRH deployment, the cellular coverage could be a problem for the region near RRH, i.e., the nearest RRH can’t direct its beam into that region and thus making a coverage hole under each RRH site.

To solve this issue of coverage hole under RRH site, we consider two possible schemes.

- Scheme-1: Connecting to 2nd-Nearest RRH;

- Scheme-2: Connecting to Nearest RRH except Coverage Hole.

And the detailed definition of Scheme-1 and 2 can be found in our accompanying discussion paper for Scenario-A [6].

## 3.2 Assumption for Analysis

By following the assumption agreed in WF [5, R4-2103240], the assumption for following analysis is provided in the following table, with the highlighted ones be the selected option or the alternative not captured in WF.

| Parameter | Value |
| --- | --- |
| Carrier frequency | 28 GHz |
| Ds and Dmin | Scenario-B: Ds = 700m and Dmin = 150m |
| RRH parameters |  |
| RRH height | 15 m |
| RRH Tx Power | 31 dBm |
| RRH antenna array model | [Mg, Ng, M, N, P]=[1, 1, 4, 8, 2] [Mg, Ng, M, N, P]=[1, 1, 8, 8, 2] [Mg, Ng, M, N, P]=[1, 1, 8, 16, 2] Newly updated beamforming parameters in WF. |
| RRH panel orientation | Option-1: RRH panel boresight pointed to the railway in the middle point between 2 RRHs Scenario-A:Azimuth angle: 23.2 degree Down-titling: 2.3 degree (used for Scheme-2, described below) Option-2: RRH panel boresight pointed to the railway at the distance of Ds (projection of the neighboring RRH on the railway) Scenario-A: Azimuth angle: 12.1 degree Down-titling: 1.2 degree (used for Scheme-1, described below) |
| Number of RRH sites per BBU | 4 |
| Number of RRH panels per RRH sites | 2 (i.e., Bi-directional) |
| Number of Analog Beams per RRH | 1, 2 |
| Propagation model | RMa LOS |
| UE parameters |  |
| UE antenna height | 5m |
| UE(PC4) antenna array model | [Mg, Ng, M, N, P]=[1, 1, 4, 4, 2] 5dBi per element antenna gain |
| UE panel orientation | Direction is opposite to RRH boresight direction, i.e., Scenario-B:Azimuth angle: 23.2 degree Down-titling: 2.3 degree |

## 3.3 Analyitical Evalution

#### 3.3.1 Scheme-1: Connecting to 2nd-Nearest RRH, Single-Beam-per-Panel

By applying the scheme-1 illustrated in Section 3.1.1, the received power’s trends along the HST track are provided in Figure 3.3.1-1/2/3.

| Rx Power without UE beamforming | Rx Power with UE beamforming (Scheme-1) |
| --- | --- |

Figure 3.3.1-1 RX power w./without UE RX beamforming, Single-Beam-per-Panel, Scheme-1, 
[Mg, Ng, M, N, P]=[1, 1, 4, 8, 2]

| Rx Power without UE beamforming |  | Rx Power with UE beamforming (Scheme-1) |
| --- | --- | --- |

Figure 3.3.1-2 RX power w./without UE RX beamforming, Single-Beam-per-Panel, Scheme-1, 
[Mg, Ng, M, N, P]=[1, 1, 8, 8, 2]

| Rx Power without UE beamforming | Rx Power with UE beamforming (Scheme-1) |
| --- | --- |

Figure 3.3.1-3 RX power w./without UE RX beamforming, Single-Beam-per-Panel, Scheme-1, 
[Mg, Ng, M, N, P]=[1, 1, 8, 16, 2]

For Scenario-B, if scheme-1 used for bi-directional RRH deployment, around 20-25dB margin above PC4 REFSENS for [Mg, Ng, M, N, P]=[1, 1, 4, 8, 2] and [1, 1, 8, 8, 2], but significant performance loss is observed for narrower beam option, i.e., [Mg, Ng, M, N, P]=[1, 1, 8, 16, 2], especially around the middle area between two RRHs.

Observation 4: For Scenario-B, if scheme-1 used for bi-directional RRH deployment, around 20-25dB margin above PC4 REFSENS for [Mg, Ng, M, N, P]=[1, 1, 4, 8, 2] and [1, 1, 8, 8, 2], but significant performance loss is observed for narrower beam option, i.e., [Mg, Ng, M, N, P]=[1, 1, 8, 16, 2].

Accordingly, the following observation can be obtained:

Observation 5: For Scenario-B, if scheme-1 bi-directional RRH deployment is used，

- For RRH panel configuration [Mg, Ng, M, N, P]=[1, 1, 8, 16, 2], only single beam per RRH panel can’t provide satisfactory coverage.

#### 3.3.2 Scheme-1: Connecting to 2nd-Nearest RRH, Dual-Beam-per-Panel

Based on the above Observation, it is straightforward to introduce additional beam per RRH panel to improve cellular coverage for the RRH track. Here we evaluate the performance enhancement by 2 beams per RRH panel.

| Rx Power without UE beamforming | Rx Power with UE beamforming (Scheme-1) |
| --- | --- |

Figure 3.3.2-1 RX power w./without UE RX beamforming, Dual-Beam-per-Panel, Scheme-1, 
[Mg, Ng, M, N, P]=[1, 1, 4, 8, 2]

| Rx Power without UE beamforming | Rx Power with UE beamforming (Scheme-1) |
| --- | --- |

Figure 3.3.2-2 RX power w./without UE RX beamforming, Dual-Beam-per-Panel, Scheme-1, 
[Mg, Ng, M, N, P]=[1, 1, 8, 8, 2]

| Rx Power without UE beamforming | Rx Power with UE beamforming (Scheme-1) |
| --- | --- |

Figure 3.3.2-3 RX power w./without UE RX beamforming, Dual-Beam-per-Panel, Scheme-1, 
[Mg, Ng, M, N, P]=[1, 1, 8, 16, 2]

By introducing the newly add beam (in below figure denoted by beam-1) to cover the region between two RRH sites (where the coverage is problematic based on the analysis in Section 3.3.1), the intended coverage for the region between two RRH sites can be much improved.

Observation 6: For Scenario-B, if scheme-1 used for bi-directional RRH deployment, two beams per RRH panel can provide satisfactory coverage.

#### 3.3.3 Scheme-2: Connecting to Nearest RRH except Coverage Hole

As provided in Section 3.1 for Scheme-2, i.e., UE connects to the nearest RRH except coverage hole region, it can be seen that Scheme-2 is not suitable for Scenario-B, becuase the mainlobe of RRH’s boresight beam for Scenario-B is far from the serving RRH, see section 3.3.1,  UE would receive the sidelobe of the RRH’s beams if scheme-2 is used,the loss of energy is obvious.

Observation 7: For bi-directional RRH deployment in Scenario-B, the scheme-2 (connecting to nearest RRH except coverage hole) is not recommended to be used.

## 3.4 Beam Dwelling Time

Similar to uni-directional RRH deployment, the coverage region by each RRH is provided as below.

Table 3.4-1 Switch Point and Beam Dwelling Time for two-beam-per-panel

|  |  |  | RRH antenna array model |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | [Mg, Ng, M, N, P]=[1, 1, 4, 8, 2] | [Mg, Ng, M, N, P]=[1, 1, 8, 8, 2] | [Mg, Ng, M, N, P]=[1, 1, 8, 16, 2] |
| Bi-directional RRH Two-beam-per-Panel | RRH beam1 dwelling distance(m) |  | 187 | 183 | 166 |
|  | RRH beam1 dwelling time(s) | speed = 260km/h | 2.59 | 2.53 | 2.30 |
|  |  | speed = 350km/h | 1.92 | 1.88 | 1.71 |
|  | RRH beam2 dwelling distance(m) |  | 163 | 167 | 184 |
|  | RRH beam2 dwelling time(s) | speed = 260km/h | 2.26 | 2.30 | 2.55 |
|  |  | speed = 350km/h | 1.68 | 1.72 | 1.89 |

Observation 8: For bi-directional RRH deployment for Scenario-B, the beam dwelling time can be in the range of [1.68, 1.92] seconds for UE maximum speed of 350kmph.

# 3. Conclusion

In this contribution, we further provided our discussion and viewpoint on Scenario-B for high speed train deployment in FR2. The following observations and proposals are provided accordingly:

Observation 1: For uni-directional RRH deployment in Scenario-B, with single beam per RRH utilized, there is around 20dB margin compared against PC4 REFSENS requirement, which is 10dB lower than Scenario-A.

Observation 2: For uni-directional RRH deployment in Scenario-B, with 2 beams used per RRH panel, the cellular coverage over the region near to RRH site can be significantly enhanced, even for one fixed RX beam used in UE side.

Observation 3: For uni-directional RRH deployment for Scenario-B, the smallest beam dwelling time can be in the range of [1.61, 2.29] seconds for UE maximum speed of 350kmph.

Observation 4: For Scenario-B, if scheme-1 used for bi-directional RRH deployment, around 20-25dB margin above PC4 REFSENS for [Mg, Ng, M, N, P]=[1, 1, 4, 8, 2] and [1, 1, 8, 8, 2], but significant performance loss is observed for narrower beam option, i.e., [Mg, Ng, M, N, P]=[1, 1, 8, 16, 2].

Observation 5: For Scenario-B, if scheme-1 bi-directional RRH deployment is used，

- For RRH panel configuration [Mg, Ng, M, N, P]=[1, 1, 8, 16, 2], only single beam per RRH panel can’t provide satisfactory coverage.

Observation 6: For Scenario-B, if scheme-1 used for bi-directional RRH deployment, two beams per RRH panel can provide satisfactory coverage.

Observation 7: For bi-directional RRH deployment in Scenario-B, the scheme-2 (connecting to nearest RRH except coverage hole) is not recommended to be used.

Observation 8: For bi-directional RRH deployment for Scenario-B, the beam dwelling time can be in the range of [1.68, 1.92] seconds for UE maximum speed of 350kmph.

# 4. Reference

[1] RP-202118, “New WID on NR support for high speed train scenario in FR2”, Samsung, Nokia.

[2] RP-210800, “Revised WID on NR support for high speed train scenario in frequency range 2 (FR2)”, Samsung,

[3] TR 36.878, “Study on performance enhancements for high speed scenario in LTE”, 3GPP.

[4] R4-2100915, “Further discussion on high speed train deployment scenario in FR2”, Samsung.

[5] R4-2103240, “Way forward on Deployment Scenario and UE RF Requirement for FR2 HST”, Samsung.

[6] R4-2105023, “Discussion on FR2 HST Deployment Scenario-A”, Samsung.