3GPP TSG-RAN WG4 Meeting #98-bis-e

R4-2105046

Electronic Meeting, April. 12-April. 20, 2021

Agenda Item:   8.8.2

Source:        Samsung

Title:           Initial simulation results of some NR-NTN co-ex scenarios

Document for:  Discussion

# Introduction

This document proposed the initial results of NTN DL to TN DL co-existence studies in 2 GHz band for discussion by applying certain assumptions and methodologies in R4-2105045.

# Discussions

### Scenarios

Referring to the session report of RAN4#98-e, the NR-NTN study in 2GHz band will consider scenarios in the following table for calibration.

Table1: NR-NTN Scenarios

| No. | Frq. | TN | TN scenario | NTN |
| --- | --- | --- | --- | --- |
| 1 | 2GHz | NR | Rural | GEO |
| 2 | 2GHz | NR | Rural | LEO 600km |
| 3 | 2GHz | NR | Rural | LEO 1200km |
| 4 | 2GHz | NR | Urban macro | GEO |
| 5 | 2GHz | NR | Urban macro | LEO 600km |
| 6 | 2GHz | NR | Urban macro | LEO 1200km |
| 25 | 2GHz | NR | Rural | HAPS |
| 26 | 2GHz | NR | Urban macro | HAPS |

In this document, we provided our initial simulation results and observations for the co-existence study between NTN DL to TN DL of scenarios 1 to 6 listed above.

### Results (2GHz)

This sub-clause captures the NTN DL to TN DL co-existence simulation results of each scenario.

Scenario 1: NTN GEO DL to TN Rural DL

Scenario 2: NTN LEO 600km DL to TN Rural DL

Scenario 3: NTN LEO 1200km DL to TN Rural DL

Scenario 4: NTN GEO DL to TN Urban DL

Scenario 5: NTN LEO 600km DL to TN Urban DL

Scenario 6: NTN LEO 1200km DL to TN Urban DL

### Scenario 1 and 4: 2GHz NTN GEO DL to TN DL

Table 1: Simulation results for throughput loss (%)

| ACIR [dB] | Urban |  |  |  | Rural |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5 | 10 | 15 | 20 | 5 | 10 | 15 | 20 | 25 |
| Average | 0.1295 | 0.0413 | 0.0131 | 0.0041 | 9.0136 | 3.6092 | 1.2629 | 0.4139 | 0.1324 |
| 5%-tile | 0.0515 | 0.0171 | 0.0050 | 0.0014 | 18.9291 | 6.9751 | 2.3157 | 0.7376 | 0.2349 |

Figure 1: Simulation results for throughput loss (%)

### Scenario 2 and 5: 2GHz NTN LEO-600 DL to TN DL

Table 2: Simulation results for throughput loss (%)

| ACIR [dB] | Urban |  |  |  | Rural |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5 | 10 | 15 | 20 | 5 | 10 | 15 | 20 | 25 |
| Average | 1.2813 | 0.4397 | 0.1433 | 0.0457 | 31.9384 | 19.0795 | 9.3582 | 3.7877 | 1.3359 |
| 5%-tile | 0.5967 | 0.1821 | 0.0590 | 0.0182 | 69.3480 | 42.7759 | 19.9901 | 7.4139 | 2.4434 |

Figure 2: Simulation results for throughput loss (%)

### Scenario 3 and 6: 2GHz NTN LEO-1200 DL to TN DL

Table 3: Simulation results for throughput loss (%)

| ACIR [dB] | Urban |  |  |  | Rural |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5 | 10 | 15 | 20 | 5 | 10 | 15 | 20 | 25 |
| Average | 1.3412 | 0.4619 | 0.1508 | 0.0480 | 33.2767 | 20.0916 | 9.9803 | 4.0754 | 1.4454 |
| 5%-tile | 0.6245 | 0.1910 | 0.0610 | 0.0191 | 70.9918 | 44.6005 | 21.1825 | 7.9459 | 2.6519 |

Figure 3: Simulation results for throughput loss (%)

### Summary

This sub-clause captures the summary of the co-existence studies. Based on the results above, we have the following observations.

Considering the 5% throughput loss criteria, the linearly interpolate ACIR required from NTN DL to TN DL for different scenarios is summarized in the table below.

Table 4: Summary of linearly interpolate ACIR required for NTN DL to TN DL

| Scenarios | NTN | GEO |  | LEO-600 |  | LEO-1200 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | TN | Urban | Rural | Urban | Rural | Urban | Rural |
| ACIR (dB) | Average | 0 | 8.7133 | 0 | 18.9119 | 0 | 19.2171 |
|  | 5%-tile | 0 | 12.1195 | 0 | 22.4282 | 0 | 22.7823 |