3GPP TSG-RAN WG4 Meeting #98-bis-e                               R4-2105063

Online, 12th -20th Apr, 2021

Source:	Samsung, KDDI

Title:	TP for TR 38.717-02-01: CA_n3-n41

Agenda item:			7.2.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 38.717-02-01 to include CA_n3-n41 according to the request in [1].

2. Reference

- R4-2102222, Revised WID on Rel-17 NR Inter-band Carrier Aggregation/Dual Connectivity for 2 bands DL with x bands UL (x=1,2).

# 3. Text Proposal

<Start of Text Proposal>

## 6.x	CA_n3-n41

### 6.x.1		Common for 1 band UL and 2 bands UL CA

#### 6.x.1.1		Operating bands for CA

Table 6.x.1.1-1: Inter-band CA operating bands involving FR1 (two bands)

| NR CA Band | NR Band (Table 5.2-1) | DL interruption allowed (Note 8) |
| --- | --- | --- |
| CA_n3-n41 | n3, n41 |  |

#### 6.x.1.2		Channel bandwidths per operating band for CA

Table 6.x.1.2-1: Supported bandwidths per CA band combination of band n3+n41

| NR CA Configuration | UL Configuration | NR Band | Channel bandwidth (MHz) (NOTE 3) |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n3A-n41A | CA_n3A-n41A | n3 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 2 |
|  |  | n41 |  | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 |  |

#### 6.x.1.3		Co-existence studies

The studies for 1 band UL for the CA band combination of band n3 + n41 have been already completed and captured into TR 38.716-02-00.

6.X.1.4		∆TIB and ∆RIB values

For CA_n1-n41 , the ∆TIB,c and ∆RIB,c values are already specified in TS 38.101-1.

6.X.1.5   REFSENS requirements

For this combinations, sensitivity degradation is allowed for a band if it is impacted by UL of another band part of the same CA configuration due to cross band isolation issues. Reference sensitivity exceptions are specified below for BCS2.

Table 6.X.1.5-1: Reference sensitivity exceptions (MSD) due to cross band isolation for NR CA FR1

| UL band | DL band | NR Band / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) |
| n3 | n41 |  | 0.7 | 0.7 | 0.7 |  | 0.7 | 0.7 | 0.7 | 0.7 | 0.7 | 0.7 | 0.7 |

Table 6.X.1.5-2: Uplink configuration for reference sensitivity exceptions due to cross band isolation for NR CA FR1

| Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| n3 | n41 | 15 |  | 50 | 50 | 50 |  | 50 | 50 | 50 | 50 | 50 | 50 | 50 |

6.X.1.6	OOB blocking exception requirements

There is no OOB blocking exception for this CA band combination.

6.X.2	Specific for 2 bands UL CA

6.X.2.1	Maximum output power for inter-band CA

Table 6.X.2.2-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n3A-n41A | 23 | +2/-32 |
| NOTE 2: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |

6.X.2.2	UE co-existence studies

The studies for 2UL for the CA band comnination of band n3+n41 have been already completed and captured into TR38.716-02-00.

6.X.2.3	REFSENS requirements

MSD exception requirement have been already specified in TS38.101-1.

<End of Text Proposal>