3GPP TSG-RAN WG4 Meeting #98-bis-e                              R4-2105067

Online, 12th -20th Apr, 2021

Source:	Samsung, KDDI

Title:	TP for TR 38.717-02-01: CA_n41-n74

Agenda item:			7.2.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 38.717-02-01 to include CA_n41-n74 according to the request in [1].

2. Reference

- R4-2102222, Revised WID on Rel-17 NR Inter-band Carrier Aggregation/Dual Connectivity for 2 bands DL with x bands UL (x=1,2).

# 3. Text Proposal

<Start of Text Proposal>

## 6.x	CA_n41-n74

### 6.x.1		Common for 1 band UL and 2 bands UL CA

#### 6.x.1.1		Operating bands for CA

Table 6.x.1.1-1: Inter-band CA operating bands involving FR1 (two bands)

| NR CA Band | NR Band (Table 5.2-1) | DL interruption allowed (Note 8) |
| --- | --- | --- |
| CA_n41-n74 | n41, n74 |  |

#### 6.x.1.2		Channel bandwidths per operating band for CA

Table 6.x.1.2-1: Supported bandwidths per CA band combination of band n41+n74

| NR CA Configuration | UL Configuration | NR Band | Channel bandwidth (MHz) (NOTE 3) |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n41A-n74A | CA_n41A-n74A | n41 |  | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 | 0 |
|  |  | n74 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |

#### 6.x.1.3		Co-existence studies

Table 6.X.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n41-n74.

Table 6.X.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n41 | 2496 | 2690 | 2496 | 2690 | 4992 | 5380 | 7488 | 8070 | 9984 | 10760 |
| n74 | 1427 | 1470 | 1475 | 1518 | 2854 | 2940 | 4281 | 4410 | 5708 | 5880 |

Based on above table, there is no harmonic issue for the band combination of n41 and n74.

Table 6.X.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n41 | 2496 | 2690 | 2496 | 2690 | 4992 | 5380 | 7488 | 8070 | 9984 | 10760 |
| n74 | 1427 | 1470 | 1475 | 1518 | 2950 | 3036 | 4425 | 4554 | 5900 | 6072 |

Based on above table, there is no harmonic mixing issue for the band combination of n41 and n74.

6.X.1.4		∆TIB and ∆RIB values

For CA_n41-n74 , the ∆TIB,c and ∆RIB,c values are specified in the table below which refer to TS 36.101 CA_11-41.

Table 6.X.1.4-1: ΔTIB,c due to NR CA (two bands)

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n41-n74 | n41 | 0.3 |
|  | n74 | 0.3 |

Table 6.X.1.4-2: ΔRIB,c due to NR CA (two bands)

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n41-n74 | n41 | 0 |
|  | n74 | 0 |

6.X.1.5     REFSENS requirements

There are no specific REFSENS requirements for 1 band UL.

6.X.1.6	OOB blocking exception requirements

There is no OOB blocking exception for this CA band combination.

6.X.2	Specific for 2 bands UL CA

6.X.2.1	Maximum output power for inter-band CA

Table 6.X.2.2-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n41A-n74A | 23 | +2/-32 |
| NOTE 2: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |

6.X.2.2	UE co-existence studies

Table 6.X.2.1-1 lists Band n41 +Band n74 2UL bands CA, 2nd, 3rd, 4th and 5th order IMD for analysis.

Table 6.X.2.2-1: Band n41 and Band n74 UL IMD products

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 2496 | 2690 | 1427 | 1470 |
| 2nd order IMD products | ¦f2_low – f1_high¦ | ¦f2_high – f1_low¦ | ¦f2_low + f1_low¦ | ¦f2_high + f1_high¦ |
| IMD frequency limit (MHz) | 1026 | 1263 | 3923 | 4160 |
| 3rd order IMD products | ¦f2_low – 2*f1_high¦ | ¦f2_high – 2*f1_low¦ | ¦2*f2_low – f1_high¦ | ¦2*f2_high – f1_low¦ |
| IMD frequency limit (MHz) | 3522 | 3953 | 164 | 444 |
| 3rd order IMD products | ¦2*f1_low + f2_low¦ | ¦2*f1_high + f2_high¦ | ¦2*f2_low + f1_low¦ | ¦2*f2_high + f1_high¦ |
| IMD frequency limit (MHz) | 6419 | 6850 | 5350 | 5630 |
| 4th order IMD products | ¦3*f1_low – f2_high¦ | ¦3*f1_high – f2_low¦ | ¦3*f2_low – f1_high¦ | ¦3*f2_high –f1_low¦ |
| IMD frequency limit (MHz) | 6018 | 6643 | 1591 | 1914 |
| 4th order IMD products | ¦3*f1_low +f2_low¦ | ¦3*f1_high + f2_high¦ | ¦3*f2_low+f1_low¦ | ¦3*f2_high +f1_high¦ |
| IMD frequency limit (MHz) | 8915 | 9540 | 6777 | 7100 |
| 4th order IMD products | ¦2*f1_low –2*f2_high¦ | ¦2*f1_high –2*f2_low¦ | ¦2*f1_low +2*f2_low¦ | ¦2*f1_high +2*f2_high¦ |
| IMD frequency limit (MHz) | 2052 | 2526 | 7846 | 8320 |
| 5th order IMD products | ¦f1_low –4*f2_high¦ | ¦f1_high –4*f2_low¦ | ¦f2_low –4*f1_high¦ | ¦f2_high –4*f1_low¦ |
| IMD frequency limit (MHz) | 3018 | 3384 | 8514 | 9333 |
| 5th order IMD products | ¦f1_low +4*f2_low¦ | ¦f1_high +4*f2_high¦ | ¦f2_low+4*f1_low¦ | ¦f2_high +4*f1_high¦ |
| IMD frequency limit (MHz) | 8204 | 8570 | 11411 | 12230 |
| 5th order IMD products | ¦2*f1_low –3*f2_high¦ | ¦2*f1_high –3*f2_low¦ | ¦2*f2_low -3*f1_high¦ | ¦2*f2_high -3*f1_low¦ |
| IMD frequency limit (MHz) | 582 | 1099 | 4548 | 5216 |
| 5th order IMD products | ¦2*f1_low +3*f2_low¦ | ¦2*f1_high +3*f2_high¦ | ¦2*f2_low+3*f1_low¦ | ¦2*f2_high +3*f1_high¦ |
| IMD frequency limit (MHz) | 9273 | 9790 | 10342 | 11010 |

Based on above, there is no IMD issue for band n41 is TDD.

Table 6.X.2.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 6.X.2.2-2: Protected bands for the 2UL bands CA configuration

| UL NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n41-n74 | E-UTRA Band 1, 2, 3, 4, 5, 8, 12, 13, 17, 18, 19, 26, 28, 29, 34, 39, 42, 48, 52, 65, 66, 85 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 1400 | - | 1427 | -32 | 27 | 4, 20 |
|  | Frequency range | 1475 | - | 1488 | -50 | 1 | 21 |
|  | Frequency range | 1488 | - | 1518 | -50 | 1 | 4 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x RBsize kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz. NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. NOTE 20: Applicable for cases and when the lower edge of the assigned NR UL channel bandwidth frequency is greater than or equal to 1427 MHz + the channel BW assigned for 5 and 10 MHz bandwidth, and when the lower edge of the assigned NR UL channel bandwidth frequency is greater than or equal to 1440 MHz for 15 and 20 MHz bandwidth. NOTE 21: Applicable for 5 MHz bandwidth, and when the upper edge of the assigned NR UL channel bandwidth frequency is less than or equal to 1467 MHz assigned for 10 MHz bandwidth, and when the upper edge of the assigned NR UL channel bandwidth frequency is less than or equal to 1463.8 MHz for 15 MHz bandwidth, and when the upper edge of the assigned NR UL channel bandwidth |  |  |  |  |  |  |  |

6.X.2.3	REFSENS requirements

Based on Table 6.X.2.2-1, there is no MSD exception requirement.

<End of Text Proposal>