3GPP TSG-RAN WG4 Meeting # 98-bis-e                                                        R4-2105159

Electronic Meeting, 12th– 20th April, 2021

Source:	Huawei, HiSilicon

Title:	TP for TR 36.717-03-02 to add UL configuration CA_3A-8A for CA_1A-3A-8A-20A and CA_1A-3C-8A-20A

Agenda item:	10.5.3

Document for:	Approval

Background

This contribution provides text proposal on the LTE UL configuration CA_3A-8A for CA_1A-3A-8A-20A and CA_1A-3C-8A-20A as defined in the revised WID [1].

Reference

[1]	RP-210200, “Revised WID on Rel-17 LTE-A inter-band CA for x bands (x= 3, 4, 5) DL with 2 bands UL”, LG Electronics

Text Proposal

---Start of changes---

7.X	LTE-A inter-band CA: Band 1 and Band 3 and Band 8 and Band 20 DL with 2 bands UL

7.X.1	List of specific combination issues

7.X.1.1	Channel bandwidths per operating band for CA

Table 7.X.1.1-1: CA configurations under study

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-3A-8A-20A | CA_3A-8A | 1 |  |  | Yes | Yes | Yes | Yes | 70 | 0 |
|  |  | 3 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_1A-3C-8A-20A | CA_3A-8A | 1 |  |  | Yes | Yes | Yes | Yes | 90 | 0 |
|  |  | 3 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |

7.X.1.2	Co-existence studies for LTE-A UL CA_3A-8A and DL CA_1A-3A-8A-20A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.X.1.2-1

Table 7.X.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 880 | 915 | 1710 | 1785 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1760 | 1830 | 3420 | 3570 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2640 | 2745 | 5130 | 5355 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 795 | 905 | 2590 | 2700 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 25 | 120 | 2505 | 2690 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3470 | 3615 | 4300 | 4485 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 855 | 1035 | 4215 | 4475 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4350 | 4530 | 6010 | 6270 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1810 | 1590 | 5180 | 5400 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6260 | 5925 | 1950 | 1735 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7720 | 8055 | 5230 | 5445 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3595 | 3300 | 675 | 930 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6890 | 7185 | 6060 | 6315 |

Based on Table 7.X.1.2-1, 2nd IMD products by Band 3 and Band 8 falls into the own Rx frequency of Band 20, and 5th IMD products by Band 3 and Band 8 falls into the own Rx frequency of Band 20,and the 2nd harmonic from Band 8 falls into Rx frequency of Band 3.

7.X.1.3	MSD

When uplink CA configurations CA_3A-8A is paired with downlink CA configuration CA_1A-3A-8A-20A there is no additional analysis of MSD because it is solved in fallback band combination.

7.X.1.4	∆TIB and ∆RIB values

The same requirements of lower order combination which are specified in TS36.101 can be applied.

---End of changes---