3GPP TSG-RAN WG4 Meeting #98-bis-e					R4-2105171

# Electronic Meeting, April 12-20, 2021

Source: 	Qualcomm Incorporated

Title: 	n1 35 and 45MHz BW AMPR

Agenda item:	7.25.3

Document for:	Approval

# Introduction

In this document, we present AMPR for n1 for 35 and 45MHz channel BWs.

# Discussion

  - 2.1 Background

WF was presented [1] to evaluate 35MHz and 45MHz AMPR for n1 NS_48 and NS_49.

First since Japanese operators do not have spectrum block larger than 20MHz for n1 for any single operator, AMPR for PHS NS_05 type protection is not required. Protection is still required for Europe and China regions. We use CA_NS_02 and CA_NS_03 requirements for B33/B34 and B39/B34 protection respectively. NS signaling value for the protection regions for 25MHz, 30MHz, and 40MHz channel BWs is yet to be decided.

For B34 protection, filter rejection values from various vendors range from 7dB to 13dB over extreme temperature conditions and 20dB over a limited temperature range. In this contribution, we choose 11dB rejection, rendering the effective emission limit for B34 from -50dBm/MHz to -39dBm/MHz. So, undoubtedly, AMPR would be required under these conditions for B34 protection. Tables 1 and 2 summarize the requirements for AMPR analysis.

Table 1: Additional requirements for “NS_48”

| Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| --- | --- | --- | --- | --- | --- | --- |
| E-UTRA band 34 – NR band n34 | FDL_low | - | FDL_high | -50 | 1 |  |
| Frequency range | 1900 | - | 1915 | -15.5 | 5 | 1 |
| Frequency range | 1915 | - | 1920 | +1.6 | 5 | 1 |
| NOTE 1: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. |  |  |  |  |  |  |

Table 2: Additional requirements for “NS_49”

| Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| --- | --- | --- | --- | --- | --- | --- |
| E-UTRA band 34 - NR band n34 | FDL_low | - | FDL_high | -50 | 1 |  |
| Frequency range | 1880 | - | 1895 | -40 | 1 |  |
| Frequency range | 1895 |  | 1915 | -15.5 | 5 | 1 |
| Frequency range | 1915 | - | 1920 | 1.6 | 5 | 1 |
| NOTE 1: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. |  |  |  |  |  |  |

  - 2.2 Simulations

Simulations are in section 4 with the following simulation assumptions

Calibration is 1dB MPR: DFT-s-OFDM QPSK 20MHz, 100RB

LO leakage = -28dBc

Image = -28dBc

CIM3 = -60dBc

CIM5 = -70dBc.

Simulations and measurements are done with carrier channel BW at low band edge at 1920MHz and high band edge at 1980MHz for both DFT-s-OFDM and CP-OFDM waveforms.

Simulation plots are shown below, and proposed spec is shown in section 2.3 and 2.4.

  - 2.3 AMPR specification for NS_48

Table 3: A-MPR regions for NS_48

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR | Meas. A-MPR DFT/CP |
| --- | --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz |  |  |
| 25 MHz | 1932.5≤ FC ≤ 1967.5 |  |  |  |  |
|  |  | ≥0 | ≥9.72MHz | A3 |  |
|  |  | ≥18.72 | <1.08MHz | A5 | analysis |
| 30 MHz | 1935 ≤ FC ≤ 1965 |  |  |  |  |
|  |  | ≥0 | ≥9.72MHz | A3 |  |
|  |  | ≥21.6 | <1.08MHz | A5 | analysis |
| 35 MHz | 1937.5 ≤ FC ≤ 1962.5 | ≥0, <1.26 | >0 | A2 |  |
|  |  | ≥1.26, <15.4 | ≥max (0, 12*SCS*RBend - 3.6) | A3 |  |
|  |  | ≥15.4, <25.2 | ≥11.8 | A4 |  |
|  |  | ≥25.2, <31.68 | ≥11.8 | A2 |  |
|  |  | ≥25.2, <31.68 | <1.08 | A5 |  |
|  |  | ≥31.68 | >0 | A1 |  |
| 40 MHz | 1940 ≤ FC ≤ 1960 | ≥0, <2.88 |  | A2 | analysis |
|  |  | ≥2.88, <17.1 | ≥max (0, 12*SCS*RBend - 3.6) | A3 |  |
|  |  | ≥17.1, <27.36 | ≥13.5 | A4 |  |
|  |  | ≥27.36, <34.56 | ≥13.5 | A2 |  |
|  |  | ≥27.36, <34.56 | <1.08 | A5 | analysis |
|  |  | ≥34.56 | >0 | A1 | analysis |
| 45 MHz | 1942.5 ≤ FC ≤ 1957.5 | ≥0, <4.5 | >0 | A2 |  |
|  |  | ≥4.5, <19 | ≥max (0, 12*SCS*RBend - 3.6) | A4 |  |
|  |  | ≥19, <37.44 | ≥15.4 | A2 |  |
|  |  | ≥30.96, <37.44 | <1.08 | A5 |  |
|  |  | ≥37.44 | >0 | A1 |  |
| 50 MHz | 1945 ≤ FC ≤ 1955 | ≥0, <6.12 | >0 | A2 |  |
|  |  | ≥6.12, <20.7 | ≥max (0, 12*SCS*RBend - 3.6) | A4 |  |
|  |  | ≥20.7, <41.04 | ≥17.1 | A2 |  |
|  |  | ≥33.84, <41.04 | <1.08 | A5 |  |
|  |  | ≥41.04 | >0 | A1 |  |

Table 4: A-MPR for modulation and waveform type

| Modulation/Waveform | A1 | A2 | A3 | A4 | A5 |
| --- | --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 10 | ≤ 6 | ≤ 3 | ≤ 4 | ≤ 5 |
| DFT-s-OFDM QPSK | ≤ 10 | ≤ 6 | ≤ 3 | ≤ 4 | ≤ 5 |
| DFT-s-OFDM 16 QAM | ≤ 10 | ≤ 6 | ≤ 3 | ≤ 4 | ≤ 5 |
| DFT-s-OFDM 64 QAM | ≤ 10 | ≤ 6 | ≤ 3 | ≤ 4 | ≤ 5 |
| DFT-s-OFDM 256 QAM | ≤ 10 | ≤ 6 |  |  | ≤ 5 |
| CP-OFDM QPSK | ≤ 11 | ≤ 7 | ≤ 4.5 | ≤ 5.5 | ≤ 5 |
| CP-OFDM 16 QAM | ≤ 11 | ≤ 7 | ≤ 4.5 | ≤ 5.5 | ≤ 5 |
| CP-OFDM 64 QAM | ≤ 11 | ≤ 7 | ≤ 4.5 | ≤ 5.5 | ≤ 5 |
| CP-OFDM 256 QAM | ≤ 11 | ≤ 7 |  | ≤ 5.5 | ≤ 5 |
| NOTE 1: The backoff applied is max(MPR, A-MPR) where MPR is defined in Table 6.2.2-1 NOTE 2: Outer and inner allocations are defined in clause 6.2.2 |  |  |  |  |  |

  - 2.4 AMPR specification for NS_49

Table 5: A-MPR regions for NS_49

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz |  |
| 25 MHz | 1932.5≤ FC ≤ 1967.5 | ≥0 | ≥9.72 | A3 |
|  |  | ≥18.72 | <1.08 | A3 |
|  |  | ≤3.96 | <1.08 | A3 |
| 30 MHz | 1935 ≤ FC ≤ 1965 | ≥0, <3.6 | ≥0 | A1 |
|  |  | ≥3.6, <6.48 | ≥0 | A5 |
|  |  | ≥6.48, <14.4 | ≥max (0,12*SCS* RBend - 3.6) | A3 |
|  |  | ≥14.4, <21.6 | ≥10.8 | A4 |
|  |  | ≥21.6 | ≥10.8 | A2 |
|  |  | ≥21.6 | <1.08 | A5 |
| 35 MHz | 1937.5 ≤ FC ≤ 1962.5 | ≥0, <6.3 | ≥0 | A1 |
|  |  | ≥6.3, <8.28 | <1.08 | A5 |
|  |  | ≥6.3, <16.2 | ≥max (0, 12*SCS*RBend - 3.6) | A4 |
|  |  | ≥16.2, <31.68 | ≥12.6, <25.2 | A2 |
|  |  | ≥25.2, <31.68 | < <1.081.08 | A5 |
|  |  | <31.68 | ≥25.2 | A1 |
|  |  | ≥31.68 | ≥0 | A1 |
| 40 MHz | 1940 ≤ FC ≤ 1960 | ≥0, <7.2 | ≥0 | A1 |
|  |  | ≥7.2, <10.44 | <1.08 | A5 |
|  |  | ≥7.2, <18 | ≥max (0, 12*SCS*RBend - 3.6) | A4 |
|  |  | ≥18, <34.56 | ≥14.4, <28.8 | A2 |
|  |  | ≥27.36, <34.56 | <1.08 | A5 |
|  |  | <34.56 | ≥28.8 | A1 |
|  |  | ≥34.56 | ≥0 | A1 |
| 45MHz | 1942.5 ≤ FC ≤ 1957.5 | ≥6.12, <12.42 | < min [1.08, max(0,12*SCS* RBend-6.12)] | A5 |
|  |  | ≥30.96, <36.72 | <1.08 | A5 |
|  |  | <36.72 | ≥16.2, <max (0, 12*SCS*RBend – 6.12) | A2 |
|  |  | <36.72 | ≥max (0, 12*SCS*RBend – 6.12) | A1 |
|  |  | ≥36.72 | >0 | A1 |
| 50MHz | 1945 ≤ FC ≤ 1955 | ≥7.74, <14.4 | < min [1.08, max(0,12*SCS* RBend-7.74)] | A5 |
|  |  | ≥33.84, <39.6 | <1.08 | A5 |
|  |  | <39.6 | ≥18, <max (0, 12*SCS*RBend – 7.74) | A2 |
|  |  | <39.6 | ≥max (0, 12*SCS*RBend – 7.74) | A1 |
|  |  | ≥39.6 | >0 | A1 |

Table 6: A-MPR for modulation and waveform type

| Modulation/Waveform | A1 | A2 | A3 | A4 | A5 |
| --- | --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 10 | ≤ 6 | ≤ 3 | ≤ 4 | ≤ 5 |
| DFT-s-OFDM QPSK | ≤ 10 | ≤ 6 | ≤ 3 | ≤ 4 | ≤ 5 |
| DFT-s-OFDM 16 QAM | ≤ 10 | ≤ 6 | ≤ 3 | ≤ 4 | ≤ 5 |
| DFT-s-OFDM 64 QAM | ≤ 10 | ≤ 6 | ≤ 3 | ≤ 4 | ≤ 5 |
| DFT-s-OFDM 256 QAM | ≤ 10 | ≤ 6 |  |  | ≤ 5 |
| CP-OFDM QPSK | ≤ 11 | ≤ 7 | ≤ 4.5 | ≤ 5.5 | ≤ 5 |
| CP-OFDM 16 QAM | ≤ 11 | ≤ 7 | ≤ 4.5 | ≤ 5.5 | ≤ 5 |
| CP-OFDM 64 QAM | ≤ 11 | ≤ 7 | ≤ 4.5 | ≤ 5.5 | ≤ 5 |
| CP-OFDM 256 QAM | ≤ 11 | ≤ 7 |  | ≤ 5.5 | ≤ 5 |
| NOTE 1: The backoff applied is max(MPR, A-MPR) where MPR is defined in Table 6.2.2-1 NOTE 2: Outer and inner allocations are defined in clause 6.2.2 |  |  |  |  |  |

# Conclusion

Proposal 1: Define n1 AMPR for NS_48 as shown in Table 3 and 4.

Proposal 2: Define n1 AMPR for NS_49 as shown in Table 5 and 6.

# References:

- R4-2103188, “WF on A-MPR for 35MHz and 45MHz”, Apple, 3GPP TSG-WG4 Meeting RAN4#98-e.

[2]

# Simulations

## Simulations for NS_48

### Simulation plots 35MHz

### Simulation plots 45MHz

## Simulations for NS_49

### Simulation plots 35MHz Low Edge and High Edge

### Simulation plots 45MHz Low Edge and High Edge