3GPP TSG-RAN WG4 Meeting # 98-bis-e 												R4-2105188

Electronic Meeting, 12th – 20th April, 2021

Agenda item:			7.27.1

Source:	Hisashi Onozawa (Nokia)

Title:	Email discussion summary for [98-bis-e][115] NR_47GHz_Band

Document for:	Information

# Introduction

UE RF core requirement for NR band n262 is discussed in this email discussion thread.

•	Topic #1: Peak EIRP and EIRP spherical coverage for UE power class 1, 2, and 4

•	Topic #2: REFSENS and EIS spherical coverage for UE power class 1, 2, and 4

•	Topic #3: Side conditions for Beam correspondence

•	Topic #4: Draft CR for UE RF and draft TR 38.847

# Topic #1: Peak EIRP and EIRP spherical coverage for UE power class 1, 2, and 4

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2104492 Power class specific parameters for n262 | Qualcomm Incorporated | n262; Power class. PC1; PC2; PC4. Min. peak EIRP (dBm); 33.6; 22.6; 29.6. REFSENS (dBm), 50 MHz, -1 dB SNR; -92.0; -86.5; -91.5. Performance degradation to spherical coverage %ile point (dB); 8.0; 11.0; 12.0. |
| R4-2104516 Discussion on EIRP for PC1,PC2 and PC4 | vivo | Proposal 1: For PC1, PC2 and PC4, n262 adopt the same percentile value, to ensure the consistent with the single percentile point assigned per power class. Proposal 2: Select the requirements at band n257/258/261 for each power class as the reference, adopt the same gain drop of n262 at PC3, to derive the EIRP and spherical requirements for PC1, PC2 and PC4. Proposal 3: For PC1, PC2 and PC4, min peak EIRP and EIRP spherical coverage requirements for n262 are derived assuming 6.4 dB and 8.6dB degradation, respectively, from n257/258/n261. The final values are listed in the table above. |
| R4-2104695 Peak EIRP and EIRP spherical coverage for PC1, PC2, PC4 for n262 | Sony, Ericsson | Observation 1 A dual-polarized antenna structure with PA for each polarization was assumed when defining the peak EIRP for bands n257 n261. Observation 2 FWA devices (PC1) and vehicle mounted equipment (PC2) are assumed to have less volume constraints and thus, Multi Band Relaxation (MBR) should be significantly less than for PC3. Observation 3 For high power non-handheld UE (PC4), MBR should not be more than for PC3. Observation 4 The spherical coverage performance (delta between peak and specified percentile of EIRP) depends on many factors, and it cannot be concluded that the n262 band must be worse than, e.g., n260 in terms of spherical coverage. Observation 5 The gain drop (delta between peak and specified percentile of EIRP) for band n262 could be no worse than 0.5dB (PC1) to 1dB (PC2) compared to band n258. Proposal 1 For PC1, PC2 and PC4: n262 reuses maximum peak EIRP and maximum TRP from band n258. Proposal 2 Companies shall provide the reference RF architecture they assumed when deriving the peak EIRP link budget. Proposal 3 According to our estimate minimum peak EIRP, n262, shall be: PC1 35 dBm, PC2 24dBm, PC4 29dBm. |
| R4-2104712 EIRP requirements for n262 UE power class 1, 2, and 4 | Nokia, Nokia Shanghai Bell | Proposal 1: Minimum peak EIRP and EIRP spherical coverage requirements for n262 UE power class 1, 2, and 4 are derived assuming 6 dB degradation from n257. |

## Open issues summary

The proposed minimum peak EIRP are listed.

EIRP spherical coverage is calculated from the proposed gain drop in each contribution.

- Sony and Ericsson paper provides the upper bound of the gain drop for PC1 and PC2.

| power class | EIRP | Proposals for n262 [dBm] |  |  |  |  | Average [dBm] |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Qualcomm | Vivo | Sony, Ericsson | Nokia | Intel R4-2102906 RAN4#98 |  |
| PC1 | min peak | 33.6 | 33.6 | 35 | 34 | 29.5 | 33.1 |
| PC1 | spherical | 25.6 | 23.4 | > 26.5 | 26 |  | > 25.4 |
| PC2 | min peak | 22.6 | 22.6 | 24 | 23 |  | 23.1 |
| PC2 | spherical | 11.6 | 9.4 | > 12 | 12 |  | > 11.3 |
| PC4 | min peak | 29.6 | 27.6 | 29 | 28 |  | 28.6 |
| PC4 | spherical | 17.6 | 16.4 | unavailable | 19 |  | 17.7 |

### Sub-topic 1-1 Minimum Peak EIRP for PC1, PC2, and PC4

Issue 1-1: Is the average of the proposed peak EIRP for PC1, PC2, and PC4 acceptable?

- Proposals

  - Option 1: Yes

  - Option 2: No

For Option 2, please provide your counter proposal.

### Sub-topic 1-2 EIRP spherical coverage for PC1, PC2, and PC4

Issue 1-2: Is the average of the proposed EIRP spherical coverage for PC1, PC2, and PC4 acceptable?

- Proposals

  - Option 1: Yes

  - Option 2: No

For Option 2, please provide your counter proposal.

## Companies views’ collection for 1st round

### Open issues

Sub topic 1-1 : Is the average of the proposed peak EIRP for PC1, PC2, and PC4 acceptable?

| Company | Comments |
| --- | --- |
| MediaTek | We don’t have strong view on using average method. However, we’d like to wait for one more meeting (RAN4#99-e) to collect more companies’ evaluation and conclude the discussion, as the WF discussion in RAN4#98-e GTW. |
| Qualcomm | PC2/4: Option 1 PC1: Option 2: Averaging is ok, but the one outlier proposal (from R4-2102906) is based on a 16-element array assumption. There is no physical limitation that prevents arrays bigger than 16 – indeed the 16e array is closer to a PC5 device if it were to be standardized for n262. We think proposals used in averaging should be lower bounded by 33.6 dBm because it is a conservative estimate of PC1 capability in n262, based on body of existing work. 33.6 dBm is derived from 40 dBm expectation in n258 and 6.4 dB reduction in PC3 peak EIRP requirement between n258 and n262. The 33.6 dBm lower bound is pessimistic because according to a good observation in R4-2102906 ‘Compared to PC3, PC1 has a larger form-factor which allows room for more antenna elements. Furthermore, the integration losses are somewhat reduced because of the larger size’. There are 2 options to move forward with averaging while taking this lower bound into account: - Force the lower bound on all peak EIRP proposals prior to averaging. - Scale the 16e 29.5 dBm proposal by 6 dB (to 35.5 dBm) assuming a doubling of elements. Proponent input is requested. |
| Samsung | We are fine to have the mean value, Option 1, for peak EIRP. But if it is not concluded, having one more meeting is also a good option for further evaluation. |
| Sony | Option 1: Averaging is OK however, averaging over mW is the more correct way in this case since the estimation is a deviation (loss) from the ideal case. |
| Huawei, HiSilicon | We have clarification question: What is the assumption on simultaneous active panel number for PC1/2/4 evaluation? And for PC4, the average value seems relative higher compared with PC1/2/3, we would like to have one more meeting round on PC4 evaluation. |
| Ericsson | Averaging can be acceptable but the values from joint contributions should be counted two times. It’s also ok to wait for one more meeting to have more evaluations. |
| vivo | We are OK with the averaging approach to derive final requirements. |
| Intel | Issue 1-1: Is the average of the proposed peak EIRP for PC1, PC2, and PC4 acceptable? Yes, the min peak EIRP averages are acceptable. If needed, we are also ok to further discuss. Regarding PC1, the average should take all proposed values into account. Furthermore, we are not ok to have our 29.5dBm value be scaled by 6dB. Integration is easier given the larger UE size, but doubling the number of antenna elements also has implications in increasing the size of the feed-network and losses associated with it. A min peak EIRP value of 33.6dBm is also acceptable to us. Therefore, for the min peak EIRP of PC1 we are ok to use either one of the options below: - overall average (33.1dBm) 33.6dBm |
| Nokia | For Huawei, PC4 assumption can be found in TR 38.817-01. We are ok with Ericsson and Sony comments. |

Sub topic 1-2 : Is the average of the proposed EIRP spherical coverage for PC1, PC2, and PC4 acceptable?

| Company | Comments |
| --- | --- |
| MediaTek | We don’t have strong view on using average method. However, we’d like to wait for one more meeting (RAN4#99-e) to collect more companies’ evaluation and conclude the discussion, as the WF discussion in RAN4#98-e GTW. |
| Qualcomm | Option 2: We do not think a separate averaging is necessary. The spherical coverage EIRP can be tied to what ever RAN4 decides for peak EIRP requirement and retaining established spherical coverage degradation in the standard. For example, an 8 dB degradation at 85th%ile point for PC1. In case of PC4, there does not seem to be convergence yet on what the gain drop should be: we draw attention to the fact that the closest band that is specified (n260) has a 12 dB gain drop at the spherical coverage specification point. There is also the prospect of repurposing this PC for FR2 HST, so for now we think consistency with existing work would be useful. Gain drop (dB); PC1; PC4. @85th %ile; @20th %ile. Band; n257; 8.0; 9.0. n258; 8.0; 9.0. n259. n260; 8.0; 12.0. n261; 8.0; 9.0. n262. n262 - Proposed; 8.0; 12.0. |
| Samsung | We agree to the averaging as we did for other parameters. However, the discussion should be focused on the gain drop rather than the requirement itself since it does not make sense to have the agreement on spherical coverage before the peak value. As Qualcomm mentioned, considering the previous gain drops is good option as an example for future discussion on the spherical coverage requirement. |
| Sony | Option 2: We do think the information gathered among companies in this meeting is too scarce to make an average. We therefore can accept two options: 2a: Concentrate on peak EIRP in this meeting and wait with EIRP spherical coverage until next meeting when more (hopefully simulation) result is available (preferred). 2b: Go for Qualcomm’s proposal. |
| Huawei, HiSilicon | We have clarification question: What is the assumption on simultaneous active panel number for PC1/2/4 evaluation? And for PC4, the average value seems relative higher compared with PC1/2/3, we would like to have one more meeting round on PC4 evaluation. |
| Ericsson | We would like to wait for one more meeting to have more evaluations. |
| vivo | We share similar view with QC. The spherical coverage requirements can be further defined after making agreements on EIRP. |
| Intel | Issue 1-2: Is the average of the proposed EIRP spherical coverage for PC1, PC2, and PC4 acceptable? No, as companies have already commented above, there is no need to have an average for the spherical requirement. First, we agree on the min peak EIRP requirement and then take X dB drop from the agreed peak requirement. One more comment regarding the drop from peak for spherical coverage. As Qualcomm pointed out, PC4 has different drop from peak values for its 28GHz and 39GHz requirements. We note that this is aligned with PC3, where there is an increase in the drop from peak as we move up in frequency. This is something we may want to discuss further, whether there should be a slight relaxation for band n262 requirements. |

### CRs/TPs comments collection

N/A

## Summary for 1st round

### Open issues

For PC1, PC2 and PC4, the average over mW of the proposed minimum peak EIRP are calculated in the following. The table is used as a baseline for further discussion.

| power class | Qualcomm | Vivo | Sony | Ericsson | Nokia | Intel | Average made over mW [dBm] |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | R4-2104492 | R4-2104517 | R4-2104696 | R4-2104696 | R4-2104712 | R4-2102906 |  |
| PC1 | 33.6 | 33.6 | 35 | 35 | 34 | 29.5 | 33.8 |
| PC2 | 22.6 | 22.6 | 24 | 24 | 23 |  | 23.3 |
| PC4 | 29.6 | 27.6 | 29 | 29 | 28 |  | 28.7 |

|  | Status summary |
| --- | --- |
| Sub-topic #1-1 | Tentative agreements:None Candidate options: - Min peak EIRP value is decided based on the mW average of all inputs in RAN4#99e. Recommendations for 2nd round: Wayforward is prepared. |
| Sub-topic #1-2 | Tentative agreements:None Candidate options: - Gain drop (from min peak to spherical EIRP) is further discussed. - Possible option is reusing an existing band.such as n262 Recommendations for 2nd round: Wayforward is prepared. |

### CRs/TPs

N/A

## Discussion on 2nd round (if applicable)

# Topic #2: REFSENS and EIS spherical coverage for UE power class 1, 2, and 4

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2104492 Power class specific parameters for n262 | Qualcomm Incorporated | n262; Power class. PC1; PC2; PC4. Min. peak EIRP (dBm); 33.6; 22.6; 29.6. REFSENS (dBm), 50 MHz, -1 dB SNR; -92.0; -86.5; -91.5. Performance degradation to spherical coverage %ile point (dB); 8.0; 11.0; 12.0. |
| R4-2104517 Discussion on EIS for PC1,PC2 and PC4 | vivo | Observation 1: For PC3, the gain drops between n257/258/261 and n262 is 5.5 dB and 7.7 dB, for REFSENS and EIS spherical coverage, respectively. Proposal 1: Select the requirements at band n257/258/261 for each power class as the reference, adopt the same gain drop of n262 at PC3, to derive the EIRP and spherical requirements for PC1, PC2 and PC4. Proposal 2: For PC1, PC2 and PC4, min peak EIS and EIS spherical coverage requirements for n262 are derived assuming 5.5 dB and 7.7 dB degradation, respectively, from n257/258/n261. The final values are listed in the table above. |
| R4-2104696 REFSENS and EIS spherical coverage for PC1, PC2, PC4 for n262 | Sony, Ericsson | Observation 1 FWA devices (PC1) and vehicle mounted equipment (PC2) are assumed to have less volume constraints and thus, Multi Band Relaxation (MBR) should be significantly less than for PC3. Observation 2 For high power non-handheld UE (PC4), MBR should not be more than for PC3. Observation 3 The spherical coverage performance (delta between peak and specified percentile of EIRP) depends on many factors, and it cannot be concluded that the n262 band must be worse than, e.g., n260 in terms of spherical coverage. Observation 4 The gain drop (delta between peak and specified percentile of EIRP) for band n262 could be no worse than 0.5dB (PC1) to 1dB (PC2) compared to band n258. Proposal 1 Companies shall provide the reference RF architecture they assumed when deriving the REFSENS link budget. Proposal 2 According to our estimate minimum peak EIRP, n262, shall be: PC1 -91.7 dBm, PC2 -84.5dBm, PC4 -86.7dBm. |
| R4-2104714 EIS requirements for n262 UE power class 1, 2, and 4 | Nokia, Nokia Shanghai Bell | Proposal 1: REFSENS and EIS spherical coverage requirements for n262 UE power class 1, 2, and 4 are derived assuming 6 dB degradation from n257. |

## Open issues summary

The proposed REFSENS values are scaled to 50 MHz channel bandwidth.

EIS spherical coverage is computed from the proposed gain drop.

- Sony and Ericsson paper provides the upper bound of the gain drop for PC1 and PC2.

| power class | EIS | Proposals for n262 [dBm] |  |  |  |  | Average [dBm] |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Qualcomm | Vivo | Sony, Ericsson | Nokia | Intel R4-2102907 RAN4#98 |  |
| PC1 | REFSENS | -92 | -92 | -94.7 | -91.5 | -90.7 | -92.2 |
| PC1 | spherical | -84 | -81.8 | < -86.2 | -83.5 |  | < -83.9 |
| PC2 | REFSENS | -86.5 | -86.5 | -87.7 | -86 |  | -86.7 |
| PC2 | spherical | -75.5 | -73.3 | < -78.7 | -75 |  | < -75.6 |
| PC4 | REFSENS | -91.5 | -91.5 | -89.7 | -91 |  | -90.9 |
| PC4 | spherical | -79.5 | -80.3 | unavailable | -82 |  | -80.6 |

### Sub-topic 2-1

Issue 2-1: Is the average of the proposed REFSENS for PC1, PC2, and PC4 acceptable?

- Proposals

  - Option 1: Yes

  - Option 2: No

For Option 2, please provide your counter proposal.

### Sub-topic 2-2

Issue 2-2: Is the average of the proposed EIS spherical coverage for PC1, PC2, and PC4 acceptable?

- Proposals

  - Option 1: Yes

  - Option 2-a: No. The same gain drop as EIRP shall be applied to EIS. (i.e., Peak to Spherical difference is the same between EIRP and EIS.)

  - Option 2-b: No. Other options.

For Option 2-b, please provide your counter proposal.

## Companies views’ collection for 1st round

### Open issues

Sub topic 2-1 : Is the average of the proposed REFSENS for PC1, PC2, and PC4 acceptable?

| Company | Comments |
| --- | --- |
| MediaTek | We don’t have strong view on using average method. However, we’d like to wait for one more meeting (RAN4#99-e) to collect more companies’ evaluation and conclude the discussion, as the WF discussion in RAN4#98-e GTW. |
| Qualcomm | PC2/4: Option 1 PC1: [option 1] For consistency we could choose to follow a similar method as for peak EIRP requirement for PC1. On the other hand, the values are clustered tightly enough that for the sake of expediency we could use a blind averaging. |
| Samsung | We are fine to have the mean value, Option 1, for REFSENS, but the final method needs to be the same with what we will use for the peak EIRP discussion. As we commented, having one more meeting is also a good option for further evaluation. |
| Sony | Option 1: Averaging is OK. For consistency with peak EIRP it should be averaging over mW. |
| Huawei, HiSilicon | Refsens could be concluded after EIRP is aligned in RAN4. |
| Ericsson | Averaging can be acceptable but the values from joint contributions should be counted two times. It’s also ok to wait for one more meeting to have more evaluations. |
| vivo | We are OK with the averaging approach to derive final requirements. |
| Intel | Issue 2-1: Is the average of the proposed REFSENS for PC1, PC2, and PC4 acceptable? Option 1: using averages is ok |
| Nokia | Support Sony and Ericsson comments. |

Sub topic 2-2 : Is the average of the proposed EIS spherical coverage for PC1, PC2, and PC4 acceptable?

| Company | Comments |
| --- | --- |
| MediaTek | We don’t have strong view on using average method. However, we’d like to wait for one more meeting (RAN4#99-e) to collect more companies’ evaluation and conclude the discussion, as the WF discussion in RAN4#98-e GTW. |
| Qualcomm | Like comment on EIRP spherical coverage, PC4 gain drop in n262 needs to be discussed further. We prefer consistency with gain drop in closest band of specification, n260 (12 dB). |
| Samsung | Prefer Option 2-a as we commented on EIRP spherical coverage. |
| Sony | Option 2a: Same gain drop as for EIRP. The same situation for EIS gain drop as for EIRP gain drop (i.e. wait until next meeting). |
| Huawei, HiSilicon | Refsens could be concluded after EIRP is aligned in RAN4. |
| Ericsson | Needs further discussion. wait for one more meeting for more evaluations. |
| vivo | spherical coverage can be further defined after making conclusions on REFSENS. |
| Intel | Issue 2-2: Is the average of the proposed EIS spherical coverage for PC1, PC2, and PC4 acceptable? Same comment as the one provided in Issue 1-2 for EIRP spherical coverage, first determine min peak EIS and then discuss drop from peak for spherical coverage. We do prefer for the drop to be the same – Option 2a. |

### CRs/TPs comments collection

N/A

## Summary for 1st round

### Open issues

The average over mW of the proposed minimum peak EIRP is calculated in the following. The table is used as a baseline for further discussion.

| power class | Qualcomm | Vivo | Sony | Ericsson | Nokia | Intel | Average made over mW [dBm] |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | R4-2104492 | R4-2104516 | R4-2104695 | R4-2104695 | R4-2104712 | R4-2102907 |  |
| PC1 | -92 | -92 | -94.7 | -94.7 | -91.5 | -90.7 | -92.3 |
| PC2 | -86.5 | -86.5 | -87.7 | -87.7 | -86 |  | -86.8 |
| PC4 | -91.5 | -91.5 | -89.7 | -89.7 | -91 |  | -90.6 |

|  | Status summary |
| --- | --- |
| Sub-topic#2-1 | Tentative agreements:None Candidate options: REFSENS value is decided based on the mW average of all inputs by RAN4#99e. Recommendations for 2nd round: Wayforward is prepared. |
| Sub-topic#2-2 | Tentative agreements:None Candidate options: - Gain drop (from REFSENS to spherical EIS) is further discussed. - Possible option is reusing an existing band.such as n262 Recommendations for 2nd round: Wayforward is prepared. |

### CRs/TPs

N/A

## Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

# Topic #3: Side conditions for Beam correspondence

Side conditions for Beam correspondence is handled.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2104889 Beam correspondence side conditions for SSB and CSI-RS in band n262 | Apple | Proposal 1: RAN4 shall apply the corrected values for the minimum SSB and minimum CSI-RS as provided in Table 1 and Table 2. Proposal 2: RAN4 should discuss this correction for other bands in the RAN4 #99 meeting as part of Rel-15 and Rel-16 maintenance. Table 1: Conditions for SSB based L1-RSRP measurements for beam correspondence Band; Minimum SSB_RP (dBm/SCSSBB) [2]; Correction for Minimum SSB (dBm/SCSSBB). n257; -96.4; -96.2. n258; -96.4; -96.2. n259; -92.1; -90.7. n260; -92.1; -91.9. n261; -96.4; -96.2. n262; -92.1; -88.5. Table 2: Conditions for CSI-RS based L1-RSRP measurements for beam correspondence Band; Minimum SSB_RP (dBm/SCSSBB) [2]; Correction for Minimum CSI-RS (dBm/SCSSBB). n257; -96.4; -96.2. n258; -96.4; -96.2. n259; -92.1; -90.7. n260; -92.1; -91.9. n261; -96.4; -96.2. n262; -92.1; -88.5. |

## Open issues summary

The side conditions for BC is revisited. The corrections are proposed as described above.

It is noted that Previously endorsed value for n262 was [-88.7] dBm in R4-210339.

### Sub-topic 3-1 Side conditions for n262

Issue 3-1: Do you agree to have SSB_RP and Minimum CSI-RS_RP conditions, -88.5 dBm, for n262?

- Proposals

  - Option 1: Yes

  - Option 2: No

### Sub-topic 3-2 Side conditions for other bands

Although the band other than n262 is for Rel-15/16 maintenance topic and cannot be treated in this meeting, but you may provide your views on the proposal 2. No decision will be made in this meeting.

Issue 3-2: Do you agree Proposal 2?

- Proposals

  - Option 1: Yes

  - Option 2: No

## Companies views’ collection for 1st round

### Open issues

Sub topic 3-1 : Do you agree to have SSB_RP and Minimum CSI-RS_RP conditions, -88.5 dBm, for n262?

| Company | Comments |
| --- | --- |
| XXX |  |
| Qualcomm | Thank you for correction |
| Huawei, HiSilicon | It depends on refsens conclusion. |
| Ericsson | We are not ready to relax the requirements. Further analysis is needed. |
| Apple | Thank you for the comments. As shown in our contribution, we have elaborated why the current numbers in the specification have not been calculated correctly. If other companies don’t see the same mistake in the calculation, we encourage them to provide next meeting a clarification that support no correction for the current values in the spec. |
| Nokia | We are ok. |

Sub topic 3-2 : Do you agree Proposal 2?

| Company | Comments |
| --- | --- |
| Ericsson | Option 2: No, this is out of scope of this WI, it should be discussed under TEI agenda. |
| Nokia | We are ok but CR needs to be treated in the maintenance agenda. |

### CRs/TPs comments collection

N/A

## Summary for 1st round

### Open issues

There are split views whether BC side conditions (currently in square bracket in the endorsed CR) should be revised. Further discussion is needed.

|  | Status summary |
| --- | --- |
| Sub-topic#3-1 | Tentative agreements:None Candidate options: - Revise the BC side conditions according to R4-2104889 - Remove the square bracket and keep the values in the endorsed CR in RAN4#98e. - Revise the BC side conditions differently from R4-2104889 Recommendations for 2nd round:Wayforward is prepared. |
| Sub-topic#3-2 | Tentative agreements:None Candidate options: Recommendations for 2nd round:None |

### CRs/TPs

N/A

## Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

# Topic #4: Draft CR for UE RF

Topic #3 is dedicated for revising a draft CR (endorsed for power class 3 in RAN4#98e, R4-2103392) and the latest draft TR 38.847.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2104713 Introduction of n262 UE RF requirements | Nokia, Nokia Shanghai Bell | Power class 1, 2, and 4 requirements are added to the endorsed CR for UE power class 3 in RAN4#98e (R4-2103392). |
| R4-2106889 TR 38.847 Introduction of NR Band n262 (47GHz band) | Ericsson | TR including previously approved TPs. |

## Open issues summary

No specific issues other than Topic#1, #2, #3 are found in the contributions in RAN4#98bis-e.

Draft CR can be revised if agreements are made in these #1, #2 and #3 topics.

If there is any issue in draft CR or TR (which is not related to these topics) can be discussed in this Topic #4.

## Companies views’ collection for 1st round

N/A

### CRs/TPs comments collection

Comments to issues other than discussed in Topic #1, #2 and #3 can be provided here.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2104713 | Company A |
|  | Company B |
|  | Apple: Correction for SSB and min CSI-RS side conditions is required. |
|  | Qualcomm – some parameters require discussion |
| R4-2106889 | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

- PC1/2/4 requirements are still discussed.

- The BC side conditions in square brackets for PC3 is still discussed.

|  | Status summary |
| --- | --- |
| Sub-topic#4 | Tentative agreements:None Candidate options: Recommendations for 2nd round: None. The draft CR revision can be made once the above issues are sorted out. |

### CRs/TPs

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2104713 Introduction of n262 UE RF requirements | Noted. (The previously endorsed CR for PC3 still holds.) |
| R4-2106889 TR 38.847 Introduction of NR Band n262 (47GHz band) | Agreeable. |

## Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

# Recommendations for Tdocs

## 1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on n262 UE RF | Nokia |  |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2104492 | Power class specific parameters for n262 | Qualcomm Incorporated | Noted |  |
| R4-2104516 | Discussion on EIRP for PC1,PC2 and PC4 | vivo | Noted |  |
| R4-2104695 | Peak EIRP and EIRP spherical coverage for PC1, PC2, PC4 for n262 | Sony, Ericsson | Noted |  |
| R4-2104712 | EIRP requirements for n262 UE power class 1, 2, and 4 | Nokia, Nokia Shanghai Bell | Noted |  |
| R4-2104713 | Introduction of n262 UE RF requirements | Nokia, Nokia Shanghai Bell | Noted |  |
| R4-2104889 | Beam correspondence side conditions for SSB and CSI-RS in band n262 | Apple | Noted |  |
| R4-2104517 | Discussion on EIS for PC1,PC2 and PC4 | vivo | Noted |  |
| R4-2104696 | REFSENS and EIS spherical coverage for PC1, PC2, PC4 for n262 | Sony, Ericsson | Noted |  |
| R4-2104714 | EIS requirements for n262 UE power class 1, 2, and 4 | Nokia, Nokia Shanghai Bell | Noted |  |
| R4-2106889 | TR 38.847 Introduction of NR Band n262 (47GHz band) | Ericsson | Agreeable |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

## 2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-210xxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-210xxxx | WF on … | YYY | Agreeable, Revised, Noted |  |
| R4-210xxxx | LS on … | ZZZ | Agreeable, Revised, Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents