3GPP TSG-RAN WG4 Meeting #98-bis-e                              R4-2105059

Online, 12th -20th Apr, 2021

Source:	Samsung, KDDI

Title:	TP for TR 38.717-02-01: CA_n18-n78

Agenda item:	7.2.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 38.717-02-01 to include CA_n18-n78 according to the request in [1].

2. Reference

- R4-2102222, Revised WID on Rel-17 NR Inter-band Carrier Aggregation/Dual Connectivity for 2 bands DL with x bands UL (x=1,2).

# 3. Text Proposal

<Start of Text Proposal>

## 6.x	CA_n18-n78

### 6.x.1		Common for 1 band UL and 2 bands UL CA

#### 6.x.1.1		Operating bands for CA

Table 6.x.1.1-1: Inter-band CA operating bands involving FR1 (two bands)

| NR CA Band | NR Band (Table 5.2-1) | DL interruption allowed (Note 8) |
| --- | --- | --- |
| CA_n18-n7810 | n18, n78 |  |
| NOTE 10: The frequency range in band n78 is restricted for this band combination to 3520 -3560 MHz and 3700– 3800 MHz. |  |  |

#### 6.x.1.2		Channel bandwidths per operating band for CA

Table 6.x.1.2-1: Supported bandwidths per CA band combination of band n18+n78

| NR CA Configuration | UL Configuration | NR Band | Channel bandwidth (MHz) (NOTE 3) |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n18A-n78A | CA_n18A-n78A | n18 | 5 | 10 | 15 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n78 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |

#### 6.x.1.3		Co-existence studies

Table 6.X.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n18-n78.

Table 6.X.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n18 | 815 | 830 | 860 | 875 | 1630 | 1660 | 2445 | 2490 | 3260 | 3320 | 4075 | 4150 |
| n78 | 3300 | 3800 | 3300 | 3800 | 6600 | 7600 | 9900 | 11400 | 13200 | 15200 | 16500 | 19000 |

Based on above table, 4th, harmonic of n18 UL falls into n78 DL.

Table 6.X.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n18 | 815 | 830 | 860 | 875 | 1720 | 1750 | 2580 | 2625 | 3440 | 3500 |
| n78 | 3300 | 3800 | 3300 | 3800 | 6600 | 7600 | 9900 | 11400 | 13200 | 15200 |  |  |

Based on above table, 4th harmonic mixing of n18 DL falls into n78 UL.

#### 6.X.1.4		∆TIB and ∆RIB values

For CA_n18-n78, the ∆TIB,c and ∆RIB,c values are given in the tables below which refer to TR 37.863-01-01 DC_18_n78 relaxation values.

Table 6.X.1.4-1: ΔTIB,c due to NR CA (two bands)

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n18-n78 | n18 | 0.3 |
|  | n78 | 0.8 |

Table 6.X.1.4-2: ΔRIB,c due to NR CA (two bands)

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n18-n78 | n18 | 0 |
|  | n78 | 0.5 |

#### 6.X.1.5     REFSENS requirements

There is no additional MSD issue taking actual spectrum into account.

#### 6.X.1.6	    OOB blocking exception requirements

Table 6.X.1.6-1: CA band combination with exceptions allowed

CA band combination

CA_n18-n78

6.X.2	Specific for 2 bands UL CA

6.X.2.1	Maximum output power for inter-band CA

Table 6.X.2.2-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n18A-n78A | 23 | +2/-32 |
| NOTE 2: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |

6.X.2.2	UE co-existence studies

Table 6.X.2.1-1 lists Band n18 +Band n78 2UL bands CA, 2nd, 3rd, 4th and 5th order IMD for analysis.

Table 6.X.2.2-1: Band n18 and Band n78 UL IMD products

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 815 | 830 | 3300 | 3800 |
| 2nd order IMD products | ¦f2_low – f1_high¦ | ¦f2_high – f1_low¦ | ¦f2_low + f1_low¦ | ¦f2_high + f1_high¦ |
| IMD frequency limit (MHz) | 2470 | 2985 | 4115 | 4630 |
| 3rd order IMD products | ¦f2_low – 2*f1_high¦ | ¦f2_high – 2*f1_low¦ | ¦2*f2_low – f1_high¦ | ¦2*f2_high – f1_low¦ |
| IMD frequency limit (MHz) | 1640 | 2170 | 5770 | 6785 |
| 3rd order IMD products | ¦2*f1_low + f2_low¦ | ¦2*f1_high + f2_high¦ | ¦2*f2_low + f1_low¦ | ¦2*f2_high + f1_high¦ |
| IMD frequency limit (MHz) | 4930 | 5460 | 7415 | 8430 |
| 4th order IMD products | ¦3*f1_low – f2_high¦ | ¦3*f1_high – f2_low¦ | ¦3*f2_low – f1_high¦ | ¦3*f2_high –f1_low¦ |
| IMD frequency limit (MHz) | 810 | 1355 | 9070 | 10585 |
| 4th order IMD products | ¦3*f1_low +f2_low¦ | ¦3*f1_high + f2_high¦ | ¦3*f2_low+f1_low¦ | ¦3*f2_high +f1_high¦ |
| IMD frequency limit (MHz) | 5745 | 6290 | 10715 | 12230 |
| 4th order IMD products | ¦2*f1_low –2*f2_high¦ | ¦2*f1_high –2*f2_low¦ | ¦2*f1_low +2*f2_low¦ | ¦2*f1_high +2*f2_high¦ |
| IMD frequency limit (MHz) | 4940 | 5970 | 8230 | 9260 |
| 5th order IMD products | ¦f1_low –4*f2_high¦ | ¦f1_high –4*f2_low¦ | ¦f2_low –4*f1_high¦ | ¦f2_high –4*f1_low¦ |
| IMD frequency limit (MHz) | 12370 | 14385 | 20 | 540 |
| 5th order IMD products | ¦f1_low +4*f2_low¦ | ¦f1_high +4*f2_high¦ | ¦f2_low+4*f1_low¦ | ¦f2_high +4*f1_high¦ |
| IMD frequency limit (MHz) | 14015 | 16030 | 6560 | 7120 |
| 5th order IMD products | ¦2*f1_low –3*f2_high¦ | ¦2*f1_high –3*f2_low¦ | ¦2*f2_low -3*f1_high¦ | ¦2*f2_high -3*f1_low¦ |
| IMD frequency limit (MHz) | 8240 | 9770 | 4110 | 5155 |
| 5th order IMD products | ¦2*f1_low +3*f2_low¦ | ¦2*f1_high +3*f2_high¦ | ¦2*f2_low+3*f1_low¦ | ¦2*f2_high +3*f1_high¦ |
| IMD frequency limit (MHz) | 11530 | 13060 | 9045 | 10090 |

The 4th IMD of band18 UL and band 78 UL fall into own RX of band 18.

Table 6.X.2.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 6.X.2.2-2: Protected bands for the 2UL bands CA configuration

| UL NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n18-n78 | E-UTRA Band 1, 3, 11, 21, 34, 65, | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 758 | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 |  |
|  | Frequency range | 860 | - | 890 | -40 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz. |  |  |  |  |  |  |  |

6.X.2.3	REFSENS requirements

Although table 6.X.2.2-1 shows that 4th IMD may falls into own Rx of band 18, there is no IMD issue for own Rx range in this combination taking actual spectrum into account.

Table 6.X.2.3-1: MSD due to IMD issue

| Operating band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | Operating band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) |  |
| CA_n18A-n78A9 | n18 | N/A | N/A | N/A | N/A | N/A | IMD4 |
|  | n78 | N/A | N/A | N/A | N/A | N/A | N/A |
| NOTE9: There is no IMD4 product in band n18 downlink for n78 operating in 3520 – 3560MHz and 3700-3800MHz frequency range. |  |  |  |  |  |  |  |

<End of Text Proposal>