3GPP TSG-RAN WG4 Meeting # 98bis-e                                                         R4-2105276

Electronic Meeting, 12– 20 April, 2021

Source:	Huawei, HiSilicon, CMCC

Title:	TP for TR 37.717-00-00 for SUL_n41A-n97A

Agenda item:	7.8.2

Document for:	Approval

Background

The WID for SUL band combinations was updated in RAN #91 meeting. This contribution provides a TP for SUL_n41A-n97A for TR 37.717-00-00.

Discussion

It may have a MSD issue due to cross band isolation for SUL_n41A-n97A. In this paper, we’d like to discuss it. We can use the main/diversity antenna topology as RF front-end architecture in our analysis, as shown in figure 1.

Figure 1 main/diversity antenna topology

We can find the 40dB minimum attenuation value for n97 SAW TX on band n41 frequency range can be easily reached. Thus, table 1 summarize the MSD analysis results for SUL_n41-n97. Since company has concerns on the implementation of diplexer, the corresponding isolation is set to zero. Larger MSD can be observed for small DL channel bandwidths.

Table 1 The MSD analysis results for SUL_n41-n97

| Item | Unit | parameter | Main | Diversity |
| --- | --- | --- | --- | --- |
| n97 TX power at antenna port (Primary path) | dBm | 23 |  |  |
| UL BW (full RB allocation) | MHz | 80 |  |  |
| n97 PA output noise power at B41 freq | dBm/Hz |  | -99 | -99 |
| Antenna isolation Primary <-> Diversity |  | 10 |  |  |
| n41 Rx bandwidth | MHz | 9 |  |  |
| n97 PA output noise power |  |  | -29.45757 | -29.4575749 |
| n97 SAW TX to COM port on B41 RX freq | dB | 40 |  |  |
| Diplexer loss, dB |  | 0 |  |  |
| diplexer out of band isolation | dB | 0 |  |  |
| n97 PA noise power at SAW- COM port |  |  | -69.45757 | -79.4575749 |
| front end loss |  | 4 |  |  |
| n97 PA noise power at ant port |  |  | -65.45757 | -75.4575749 |
| noise figure dB | dB | 11 |  |  |
| Thermal noise at RX ant port(dBm/Hz) |  | -163 |  |  |
| Thermal noise, dBm |  |  | -93.45757 | -93.4575749 |
| n41 filter rejection at 2300~2400MHz | dB | 35 |  |  |
| Blocking signal After n41 filter | dBm |  | -12 | -22 |
| Typical receiver IIP2 | dB | 50 |  |  |
| TX IM2 noise level refer to RX LNA input | dBm |  | -74 | -94 |
| TX IM2 noise level refer to antenna connector | dBm |  | -70 | -90 |
| totle noise level at ANT port, dBm |  |  | -64.14476 | -75.2416083 |
| SNR requirement for QPSK |  | -1 |  |  |
| REFSENSE (referred to antenna)(10MHz BW) |  |  | -65.14476 | -76.2416083 |
| combined REFSENS(10MHz BW), dBm |  |  | -76.56651 |  |
| Margin | dB | 2.5 |  |  |
| combined REFSENS(10MHz BW)after considering Margin, dBm |  |  | -74.06651 |  |
| MSD | dB |  | 20.733489 |  |

Based on the analysis, 20.7dB MSD for 10MHz DL BW can be specified in the spec.

Text Proposal

---Start of changes---

## 5.x	SUL_n41A-n97A

### 5.x.1	Operating bands

Table 5.x.1-1: SUL band combination

| NR Band combination for SUL | NR Band (Table 5.2-1) |
| --- | --- |
| SUL_n41-n972 | n41, n97 |
| NOTE 1: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier is 0 us. NOTE 2: For UE supporting SUL band combination simultaneous Rx/Tx capability is mandatory. |  |

### 5.x.2	Channel bandwidths per operating band

Table 5.x.2-1: Supported bandwidths per SUL band combination

| SUL band combination with CA | UL configuration | NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SUL_n41A-n97A | SUL_n41A-n97A | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  | n97 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |

### 5.x.3	Maximum output power

There is only single UL in uplink so the requirement for each band in clause 6.2.1 from 38.101-1 is applicable.

### 5.x.4	Spurious emission band UE co-existence

There is only single UL in uplink so this requirement specified in clause 6.5.3.2 from 38.101-1 is applicable.

Table 5.x.4-1 summarizes frequency ranges where harmonics and/or harmonics mixing occur for SUL_n41-n97.

Table 5.x.4-1: Impact of UL/DL Harmonic/Harmonic mixing

|  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band |  | Low Band Edge | High Band Edge | Low Band Edge | High Band Edge | Low Band Edge | High Band Edge | Low Band Edge | High Band Edge |
| n41 | UL/DL frequency range | 2496 | 2690 | 4992 | 5380 | 7488 | 8070 | 9984 | 10760 |
| n97 | UL frequency range | 2300 | 2400 | 4600 | 4800 | 6900 | 7200 | 9200 | 9600 |

There is no harmonic/harmonic mixing issue for this band combination. However, it may have MSD due to cross band isolation between band n41 and n97.

### 5.x.5	REFSENS requirements

For SUL operation with CA, the reference receive sensitivity (REFSENS) requirement for downlink bands specified in clause 7.3A.2 from TS 38.101-1 shall be met when supplementary uplink configuration for reference sensitivity are specified as below.

Table 5.x.5-1: Supplementary uplink configuration for reference sensitivity

|  | NR Band / SCS of SUL band / Channel bandwidth of the DL band / NRB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DL band | SUL band | SCS of SUL band (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| n41 | n97 | 30 |  | 216 | 216 | 216 |  | 216 | 216 | 216 | 216 |  | 216 | 216 | 216 |

The MSD exception for SUL_n41-n97 are specified below.

Table 5.X.5-2: Reference sensitivity exceptions due to cross band isolation

| UL band |  | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n97 |  | n41 |  | [20.7] | [18.9] | [17.7] |  | [15.8] | [14.5] | [13.5] | [12.8] | [11.5] | [11.0] | [10.6] |
| NOTE 1: The n41 requirements are modified by -0.5dB when carrier frequency of the assigned NR channel bandwidth is within 2515 – 2690 MHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 5.X.5-3: Uplink configuration for reference sensitivity exceptions due to cross band isolation

| UL band | DL band | 5 MHz (NRB) | 10 MHz (NRB) | 15 MHz (NRB) | 20 MHz (NRB) | 25 MHz (NRB) | 30MHz (NRB) | 40 MHz (NRB) | 50 MHz (NRB) | 60 MHz (NRB) | 80 MHz (NRB) | 90 MHz (NRB) | 100 MHz (NRB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n97X | n41 |  | 216 | 216 | 216 |  | 216 | 216 | 216 | 216 | 216 | 216 | 216 |
| NOTE X: 30 kHz SCS is assumed for UL band. |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 5.x.6	∆TIB and ∆RIB values

For SUL_n41-n97, the TIB,c and RIB,c values are given in the tables below.

Table 5.x.6-1: ΔTIB,c

| SUL Band combination | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| SUL_n41-n97 | n41 | 0.5 |
|  | n97 | 0.5 |

Table 5.x.6-2: ΔRIB,c

| SUL Band combination | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| SUL_n41-n97 | n41 | 0 |

---End of changes---

Reference

[1]	RP-210563, “WID revision: Rel-17 Band combinations for SA NR supplementary uplink (SUL), NSA NR SUL, NSA NR SUL with UL sharing from the UE perspective (ULSUP)”, Huawei, HiSilicon