3GPP TSG-RAN WG4 Meeting#98bis             	R4-2105281

E-meeting, Apr. 12th – 20th, 2021

Title:	TP to TR 38.717-03-02 Addition of CA_n2A-n5A-n66A

Source:	Nokia, Verizon

Agenda item:	7.11.2

Document for:	Approval

# 1	Introduction

This contribution is a TP to TR 38.717-03-02 to introduce CA_n2A-n5A-n66A.

# 2	Text Proposal

*********************** Start of the TP ***************************************************

### 5.1.x	CA_n2-n5-n66

#### 5.1.x.1		Operating bands for CA

Table 5.1.x.1-1: CA band combination

| NR Band | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n2A-n5A-n66A | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

#### 5.1.x.2		Channel bandwidths per operating band for CA

Table 5.1.x.2-1: Supported channel bandwidths per CA configuration

| NR CA configuration | Uplink CA configuration | NR Band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n2A-n5A-n66A | CA_n2A-n5A, CA_n2A-n66A, CA_n5A-n66A | n2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |

#### 5.1.x.3	Co-existence studies

For 3DL/2UL NR CA, only the IMD issues due to dual uplink operation of two bands falling into the DL of the third band shall be verified.

Co-existence studies for dual uplink operation of two bands, i.e. CA_n2A-n5A, CA_n2A-n66A and CA_n5A-n66A have been captured in TR 38.716-02-00, where:

- IMD4 products produced by Band n2 and n5 might fall in Rx of band n66.

- No IMD products produced by Band n2 and n66 might fall in Rx of band n5.

- No IMD products produced by Band n5 and n66 might fall in Rx of band n2.

#### 5.1.x.4	REFSENS requirements

The required MSD is based on already completed EN-DC_2A-66A_n5A.

Table 5.1.x.4-1: MSD for the CA configuration

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n2A-n5A-n66A | n2 | 1900 | 5 | 25 | 1980 | N/A | FDD | N/A |
|  | n5 | 830 | 5 | 25 | 875 | N/A |  | N/A |
|  | n66 | 1740 | 5 | 25 | 2140 | 7.2 |  | IMD4 |

*********************** End of the TP ***************************************************

# 3	References