3GPP TSG-RAN WG4 Meeting # 98-bis-e 	R4-2105358

# Electronic Meeting, 12th – 20th April  2021

Source:		Verizon

Title:			TP for TR 37.xxx for DC_66_n5-n77

Agenda item:		7.23.2

Document for:		Approval

# Introduction

This contribution is a text proposal for TR 37.xxx to include DC_66_n5-n77 according to the request in [1].

Reference

[1] RP-210816, New WID on PC2 EN-DC with x LTE band + y NR band (x= 2, 3, 4, y=1; x=1, 2, y=2)

# Text Proposal

<Start of Text Proposal>

## 6.x	DC_66A_n5A-n77A

### 6.x.1	Transmitter Characteristics

#### 6.x.1.1	Maximum Output Power

Table 6.x.1.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_66A_n77A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

#### 6.x.1.2	Co-existence study

According to the PC3 DC_66A_n5A-n77A study in 37.717-11-21, the Rx impacts are identified as below,

- 3rd 4th and 5th order IMD generated by dual uplink of 66_n5 may fall into own Rx of band n77

Thus, additional MSD for IMD 3, 4 and 5 should be considered to mitigate the impact of the interference for PC2 DC_66A_n5A-n77A combination.

### 6.x.2	Receiver Characteristics

#### 6.x.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands

#### 6.x.2.1.1	Power class 2 Case A

The additional MSD due to intermodulation for PC2 Case A DC_66A_n5A-n77A are defined in table 6.x.2.2.1-1.

Table 6.x.2.1.1-1: MSD test points for PCell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_66A_n5A-n77A DC_66A-66A_n5A-n77A | 66 | 1760 | 5 | 25 | 2160 | N/A | N/A |
|  | n5 | 830 | 5 | 25 | 875 | N/A | N/A |
|  | n77 | 3420 | 10 | 50 | 3420 | 24.9 | IMD3 |
|  | 66 | 1714 | 5 | 25 | 2114 | N/A | N/A |
|  | n5 | 827 | 5 | 25 | 872 | N/A | N/A |
|  | n77 | 4195 | 10 | 50 | 4195 | 24.1 | IMD44 |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |

#### 6.x.2.1.2	Power class 2 Case B

The additional MSD due to intermodulation for PC2 Case B DC_66A_n5A-n77A are the same as the Case A defined in table 6.x.2.1.1-1.

<End of Text Proposal>