3GPP TSG-RAN WG4 Meeting # 98-bis-e 												R4-2105450

Electronic Meeting, 12th – 20th April, 2021

Agenda item:			7.25

Source:	Moderator (Huawei)

Title:	Email discussion summary for [98-bis-e][113] NR_FR1_35MHz_45MHz_BW

Document for:	Information

Introduction

The scope of this email discussion is to discuss the contributions submitted at agenda 7.25.1 and 7.25.3 on introduction of channel bandwidths 35MHz and 45MHz for NR. The following topics are discussed in the email discussion.

- Topic#1: UE RF requirements

  - Sub-topic 1-1: n2, n25 35MHz REFSENS

  - Sub-topic 1-2: n2, n25 45MHz REFSENS

  - Sub-topic 1-3:  n3 35MHz and 45 MHz REFSENS

  - Sub-topic 1-4: n8 and n71 REFSENS

- Topic#2: 38.101-1

  - Sub-topic 2-1: REFSENS table split and simplification

  - Sub-topic 2-2: Draft CR to 38.101-1 for band n1

- Topic#3: A-MPR

  - Sub-topic 3-1: A-MPR for NS_48 and NS_49

Topic #1: UE REFSENS

Companies’ contributions summary

| T-doc number | Company | Title |
| --- | --- | --- |
| R4-2104656 | Murata Manufacturing Co Ltd. | REFSENS of n8, n71, n25 for 35_45MHz channel bandwidth |
| R4-2104811 | Mediatek India Technology Pvt. | REFSENS evaluation of n8 and n71 for 35MHz channel bandwidth |
| R4-2104888 | Apple | UL channel location for MSD calculation for bands n8 and n71 |
| R4-2106483 | Huawei, HiSilicon | UE REFSENS for 35 MHz and 45 MHz |
| R4-2107337 | Skyworks Solutions Inc. | 35-45MHz REFSENS for n2 n3 n8 n25 n71 |
| R4-2107339 | Qualcomm Incorporated | 35MHz and 45MHz REFSENS |

Summary of opinions on UL locations from companies:

| Company | n2 |  |  | n8 |  |  | n25 |  |  | n71 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 45 DL/40 UL |  |  | 35 DL/20 UL |  |  | 45 DL/40 UL |  |  | 35 DL/20 UL |  |  |
|  | worst | Mid | Best | worst | Mid | Best | worst | Mid | Best | worst | Mid | Best |
| Murata |  |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |
| Mediatek |  |  |  | Yes | Yes | Yes |  |  |  | Yes | Yes | Yes |
| Qualcomm |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |
| Huawei |  |  |  |  | Yes |  |  |  |  |  | Yes |  |
| Apple |  |  |  |  | Yes | Yes |  |  |  |  | Yes | Yes |
| Skyworks | Yes |  |  |  | Yes |  | Yes |  |  |  | Yes |  |

| Summary of REFSENS proposals from companies: (15 KHz SCS) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Company | n2 |  | n3 |  | n8 |  |  | n25 |  |  |  | n71 |  |  |
|  | 35 | 45 DL/40 UL | 35 | 45 | 35 DL/20 UL |  |  | 35 | 45 DL/40 UL |  |  | 35 DL/20 UL |  |  |
|  |  |  |  |  | worst | Mid | Best |  | worst | Mid | Best | worst | Mid | Best |
| Murata |  |  | -86 | -84.2 | -64 | -79.5 | -85.2 | -85.4 | -77.4 | -81.5 | -82.8 | -69.1 | -85.2 | -87.7 |
| Mediatek |  |  |  |  | -68.7 | -78 | -86 |  |  |  |  | -69.8 | -78 | -87 |
| Qualcomm |  |  | -85.2 | -80.2 | -70.7 | -78.5 | -84.8 | -81.7 | -76.4 |  |  | -71.2 | -82.7 | -84.9 |
| Huawei |  |  | -86.1 | -82 | -71.5 |  |  |  |  |  |  | -71 |  |  |
| Apple |  |  |  |  |  | -76.7 | -85.1 |  |  |  |  |  | -77 | -85.4 |
| Skyworks | -83.2 | -80.6 | -87.5 | -81.8 |  | -79.2 |  | -81.7 | -79.1 |  |  |  | -80.8 |  |
| Average1 | -83.2 | -80.6 | -86.2 | -81.3 | -68.7 | -78.4 | -85.3 | -81.7 | -77.6 | -81.5 | -82.8 | -70.3 | -80.7 | -86.3 |

Note 1: It is the average of companies REFSENS values larger than the agreed REFSENS value of smaller channel bandwidth.

Open issues summary

Sub-topic 1-1

Issue 1-1:  n2, n25 35MHz REFSENS

- Proposals summary

See clause 1.1.

- Potential agreements

Table 1.2.1-1: Two Antenna Port Reference Sensitivity QPSK PREFSENS for n2 and n25 35 MHz

| Operating Band | SCS kHz | 35 MHz (dBm) |
| --- | --- | --- |
| n2 | 15 | -83.2 |
|  | 30 | -83.3 |
|  | 60 | -83.4 |
| n25 | 15 | -81.7 |
|  | 30 | -81.8 |
|  | 60 | -81.9 |

Table 1.2.1-2: Uplink Configuration for Reference Sensitivity

| Operating Band | SCS kHz | 35 MHz (dBm) | Duplex Mode |
| --- | --- | --- | --- |
| n21 | 15 | 401 | FDD |
|  | 30 | 201 |  |
|  | 60 | 101 |  |
| n251 | 15 | 401 | FDD |
|  | 30 | 201 |  |
|  | 60 | 101 |  |
| NOTE 1: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1) |  |  |  |

- Recommended WF

  - Discussion on the above potential agreements and check whether the REFSENS in Table 1.2.1-1~ Table 1.2.1-2 are acceptable.

Sub-topic 1-2

Issue 1-2:  n2 and n25 45MHz REFSENS

- Proposals summary

See clause 1.1.

- Potential agreements

  - Channel locations 40/45 MHz case

    - Option 1: Worst case

    - Option 2: Middle case

    - Option 3: Best case

  - n2 and n25 45 MHz Refsens

Table 1.2.2-1 Two Antenna Port asymmetric 40MHz Uplink / 45MHz Downlink Reference Sensitivity QPSK PREFSENS and Uplink configuration table

| Operating band / SCS / Channel bandwidth / Channel allocations / REFSENS/ Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | Channel bandwidth (DL) (MHz) | Channel bandwidth (UL) (MHz) | FC (DL) (MHz) | FC (UL) (MHz) | UL allocation (LCRB) | REFSENS (dBm) | Duplex Mode |
| n2 | 15 | 45 | 40 | 1967.5 | 1890.0 | 401 | -80.6 | FDD |
|  | 30 |  |  |  |  | 201 | -80.7 |  |
|  | 60 |  |  |  |  | 101 | -80.8 |  |
| n25 | 15 | 45 | 40 | 1972.5 | 1895.0 | 401 | -77.6 | FDD |
|  | 30 |  |  |  |  | 201 | -77.7 |  |
|  | 60 |  |  |  |  | 101 | -77.8 |  |
| NOTE 1: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1) |  |  |  |  |  |  |  |  |

- Recommended WF

  - Option 1 (Worst case) is recommended for UL channel locations

  - Discussion on the above potential agreements and check whether the REFSENS in Table 1.2.2-1 are acceptable.

Sub-topic 1-3

Issue 1-3: n3 35MHz and 45 MHz REFSENS

- Proposals summary

See clause 1.1.

- Potential agreements

- Table 1.2.3-1: Two Antenna Port Reference Sensitivity QPSK PREFSENS

| Operating Band | SCS kHz | 35 MHz (dBm) | 45 MHz (dBm) |
| --- | --- | --- | --- |
| n3 | 15 | -86.2 | -81.3 |
|  | 30 | -86.3 | -81.4 |
|  | 60 | -86.4 | -81.5 |

- Table 1.2.3-2: Uplink Configuration for Reference Sensitivity

| Operating Band | SCS kHz | 35 MHz (dBm) | 45 MHz (dBm) |
| --- | --- | --- | --- |
| n3 | 15 | 501 | 501 |
|  | 30 | 241 | 241 |
|  | 60 | 101 | 101 |
| NOTE 1: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1). |  |  |  |

- Recommended WF

  - Discussion on the above potential agreements and check whether the REFSENS in Table 1.2.3-1 and  Table 1.2.3-2 are acceptable

Sub-topic 1-4

Issue 3-3: n8 and n71 REFSENS

- Proposals summary

See clause 1.1.

- Potential agreements

  - Channel locations

    - Option 1: Worst case

    - Option 2: Middle case

    - Option 3: Best case

  - Refsens

Table 1.2.4-1: Two Antenna Port Reference Sensitivity QPSK PREFSENS for n8 35MHz CBW.

| Operating Band | SCS kHz | Channel bandwidth (DL) (MHz) | Channel bandwidth (UL) (MHz) | FC (DL) (MHz) | FC (UL) (MHz) | UL allocation (LCRB) | REFSENS (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n8 | 15 | 35 | 20 | 942.5 | 897.5 | 25 | -78.4 |
|  | 30 |  |  |  |  | 10 | -78.5 |
|  | 60 |  |  |  |  |  |  |
| n71 | 15 | 35 | 20 | 634.5 | 680.5 | 25 | -80.7 |
|  | 30 |  |  |  |  | 10 | -80.8 |
|  | 60 |  |  |  |  |  |  |

- Recommended WF

  - Option 2 (Middle case) is recommended for UL channel locations

  - Discussion on the above potential agreements and check whether the REFSENS in Table 1.2.4-1 are acceptable.

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| XXX | Sub-topic 1-1: n2, n25 35MHz REFSENS Sub-topic 1-2: n2, n25 45MHz REFSENS Sub-topic 1-3: n3 35MHz and 45 MHz REFSENS Sub-topic 1-4: n8 and n71 REFSENS Others: |

Sub-topic 1-1: n2, n25 35MHz REFSENS

| Company | Comments |
| --- | --- |
| Qualcomm | QC would like to contribute to the proposed values. WE can bring revised contribution to this meeting or the next meeting. REFSENS numbers are close. n2; n25. 35M; 35M. Skyworks; -83.2; -81.7. QC; -83.6; -81.7. UL config; 40; 40. |
| Skyworks | To QCOM: thank you for sharing this data. Considering the small differences for n2, we could agree to QCOM proposal of -83.6dBm since our proposal includes some margin vs our measured values. |
| AT&T | We can agree to -83.6dBm for n2 35MHz REFSENS as proposed by QC and supported by Skyworks. We would like to confirm this as the n2 35MHz REFSENS value at this meeting. Perhaps, we can align on final values for 30MHz and 40MHz at this meeting also in topic [112]. |

Sub-topic 1-2: n2, n25 45MHz REFSENS

| Company | Comments |
| --- | --- |
| Qualcomm | Sub-topic 1-2: n2, n25 45MHz REFSENS Prefer option 1. n2; n25. 40/45 w; 40/45 w. Skyworks; -80.6; -77.6. QC; -78.6; -76.4. Murata; -77.4. UL config; 40; 40. |
| Skyworks | Option 1 |
| Huawei | Option1 |
| Ericsson | Option 1 |

Sub-topic 1-3:  n3 35MHz and 45 MHz REFSENS

| Company | Comments |
| --- | --- |
| Qualcomm | We are ok with the average of n3 35M and the average of n3 45M of the proposed REFSENS values above the 40MHz REFSENS value, so the numbers proposed are in line with this expectation. |
| Skyworks | Our 35MHz value is based on worst case measurements data, and the 45MHz value is based on the agreed Delta MSD approach of WF. We could compromise to the proposed averaged values since the difference is very small with our proposals. |
| Huawei | Ok with the proposed averaged values |

Sub-topic 1-4: n8 and n71 REFSENS

| Company | Comments |
| --- | --- |
| Qualcomm | Are the numbers proposed the average of all middle cases? Then there should be some consistency for all the bands whether or not to include any IM3 contribution or choose middle/worst for all asymmetric cases. n8; n71. 20/35m; 20/35m. MediaTek; -78; -78. Murata; -79.5; -85.2. Skyworks; -79.2; -80.8. Apple; -76.7; -77. QC; -78.5; -82.7. HW; -77.6; -79.9. Average; -78; -80.1. |
| Skyworks | Option 2. We are Ok to compromise with averaged REFSENS values of Table 1.2.4-1 as there is little difference vs our proposals. For option 2: We have changed our preference from last meeting “best case” to this meeting “middle-case” based on concerns that best case may not always be a level of freedom for cells at the border of a tracking area, and to avoid breaking the default Tx_Rx frequency separation. We note however that deviation from Tx-Rx frequency separation is handled by parameter “DeltaFtx-rx” which is needed since we are looking at UL 20 MHz, DL 35MHz. If we were to adopt carrier frequencies from Table 1.2.4-1, how does DeltaFtx-rx apply in particular for band 71 where UL and DL bands are swapped compared to n8? |
| MediaTek | We are fine with middle case (option2) or worst case. Qualcomm puts relevant companies’ contribution values into averaged value as shown above and the averaged values makes sense to us. |
| Huawei | Option 2. To QC, the averaged values can be found in the clause 1.1 table for summary of REFSENS proposals from companies. We only evaluates the worst case hence no number is counted for middle case. |
| Apple | We are Ok with Option 2, the REFSENS values considering the average of the proposals for the UL location in the Middle case, as shown in Table 1.2.4-1. |

CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic 1-1: n2, n25 35MHz REFSENS | Tentative agreements: Table 1.2.1-1: Two Antenna Port Reference Sensitivity QPSK PREFSENS for n2 and n25 35 MHz Operating Band; SCS kHz; 35 MHz (dBm). n2; 15; -83.6. 30; -83.7. 60; -83.8. n25; 15; -81.7. 30; -81.8. 60; -81.9. Table 1.2.1-2: Uplink Configuration for Reference Sensitivity Operating Band; SCS kHz; 35 MHz (dBm); Duplex Mode. n21; 15; 401; FDD. 30; 201. 60; 101. n251; 15; 401; FDD. 30; 201. 60; 101. NOTE 1: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1). Recommendations for 2nd round: agree on the tentative agreements above and capture the agreements in WF1. |
| Sub-topic 1-2: n2, n25 45MHz REFSENS | Tentative agreements: - Channel locations for 40/45 MHz case - Option 1: Worst case - n2 and n25 45 MHz Refsens Table 1.2.2-1 Two Antenna Port asymmetric 40MHz Uplink / 45MHz Downlink Reference Sensitivity QPSK PREFSENS and Uplink configuration table Operating band / SCS / Channel bandwidth / Channel allocations / REFSENS/ Duplex mode. Operating Band; SCS kHz; Channel bandwidth (DL) (MHz); Channel bandwidth (UL) (MHz); FC (DL) (MHz); FC (UL) (MHz); UL allocation (LCRB); REFSENS (dBm); Duplex Mode. n2; 15; 45; 40; 1967.5; 1890.0; 401; -79.6; FDD. 30; 201; -79.7. 60; 101; -79.8. n25; 15; 45; 40; 1972.5; 1895.0; 401; -77.6; FDD. 30; 201; -77.7. 60; 101; -77.8. NOTE 1: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1). Note: QC values for n2 proposed during 1st round was counted for the average value. Recommendations for 2nd round: further discussion on the tentative agreements above and capture the agreements in WF1. |
| Sub-topic 1-3: n3 35MHz and 45 MHz REFSENS | Tentative agreements: Table 1.2.3-1: Two Antenna Port Reference Sensitivity QPSK PREFSENS Operating Band; SCS kHz; 35 MHz (dBm); 45 MHz (dBm). n3; 15; -86.2; -81.3. 30; -86.3; -81.4. 60; -86.4; -81.5. Table 1.2.3-2: Uplink Configuration for Reference Sensitivity Operating Band; SCS kHz; 35 MHz (dBm); 45 MHz (dBm). n3; 15; 501; 501. 30; 241; 241. 60; 101; 101. NOTE 1: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1).. Recommendations for 2nd round: agree on the tentative agreements above and capture the agreements in WF1. |
| Sub-topic 1-4: n8 and n71 REFSENS | Tentative agreements: - Channel locations - Option 2: Middle case - Refsens Table 1.2.4-1: Two Antenna Port Reference Sensitivity QPSK PREFSENS for n8 35MHz CBW. Operating Band; SCS kHz; Channel bandwidth (DL) (MHz); Channel bandwidth (UL) (MHz); FC (DL) (MHz); FC (UL) (MHz); UL allocation (LCRB); REFSENS (dBm). n8; 15; 35; 20; 942.5; 897.5; 25; -78.4. 30; 10; -78.5. 60. n71; 15; 35; 20; 634.5; 680.5; 25; -80.7. 30; 10; -80.8. 60. Recommendations for 2nd round: further discussion on the tentative agreements above and capture the agreements in WF1. |

Discussion on 2nd round (if applicable)

Companies views’ collection for 2nd round

WF on UE REFSENS for 35 MHz and 45 MHz    Source: Huawei

| Company | Comments |
| --- | --- |
| Qualcomm | For the asymmetric cases for n8, n71, n25, it is recommended to strongly consider following the approach of 1UL/2DL CA REFSENS, where the SCC was further located away from PCC UL. The “effective” duplex offset closely resembles the “best” case as we have identified in the contributions to date. To follow up with consistency, we could specify that REFSENS requirement be met only in the “best case” duplex offset position with a statement in the general REFSENS section. |
| MediaTek | Regarding modification of slide 7 in WF_v02, it make sense to us and we are fine with it. We are fine with WF_v02. |
| Skyworks | Thank you for addressing our comments on missing impact of introducing assymetric UL/DL CBW on the technical specifications sub-clause 5.3.6 Asymetric Channel Bandwidth and need to FFS impact on default Tx-Rx frequency separation. We are also fine with WF_v02. About “mid-case” vs “best-case”: our motivation at this meeting to propose “mid-case” was driven by concerns to minimize impact on the rest of the specifications, such as Tx-Rx Frequency separation. If adopting “mid-case” test point does not alleviate this impact, then we may just as well revert back to best-case scenario as this is the most likely configuration an operator would select. No strong views. |
| Huawei | We agree more discussion on introducing asymmetric UL/DL CBW. In WF v02 slide 7, options are FFS and hope the version is ok to everyone. |

Topic #2: 38.101-1

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2106906 | Apple | Proposal: FR1 REFSENS requirements table is revised to as shown in Table 2-1 and Table 2-2 starting from Rel-17. The details on how to split the table are summarized below: - Group FDD bands and SDL bands in one table with 35MHz and 45MHz columns added. - Group TDD bands in another table. - FDD band REFSENS representation remains unchanged, i.e., the exact REFSENS value is explicitly shown for each channel BW. - TDD band REFSENS use equation-based representation with supported minimum channel BW as reference. - Remove the duplex mode column to further reduce the table width as the table itself can already differentiate the duplex mode. - Keep the existing notes and void the unrelated notes in each table. |
| R4-2106283 | China Telecom | Draft CR to 38.101-1 Introduce band specific requirements for 45MHz CBW for Band n1 |

Open issues summary

Sub-topic 2-1

Issue 2-1: REFSENS table split and simplification

- Proposals

Proposal: FR1 REFSENS requirements table is revised to as shown in Table 2-1 and Table 2-2 starting from Rel-17.

Table 2-1 Proposed REFSENS table for FR1 FDD/SDL bands

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) |
| n1 | 15 | -100.0 | -96.8 | -95.0 | -93.8 | -92.7 | -91.9 |  | -90.6 |  | -89.6 |
|  | 30 |  | -97.1 | -95.1 | -94.0 | -92.8 | -92.0 |  | -90.7 |  | -89.7 |
|  | 60 |  | -97.5 | -95.4 | -94.2 | -93.0 | -92.1 |  | -90.9 |  | -89.7 |
| n2 | 15 | -98.0 | -94.8 | -93.0 | -91.8 |  |  |  |  |  |  |
|  | 30 |  | -95.1 | -93.1 | -92.0 |  |  |  |  |  |  |
|  | 60 |  | -95.5 | -93.4 | -92.2 |  |  |  |  |  |  |
| n3 | 15 | -97.0 | -93.8 | -92.0 | -90.8 | -89.7 | -88.9 |  | -82.3 |  |  |
|  | 30 |  | -94.1 | -92.1 | -91.0 | -89.8 | -89.0 |  | -82.4 |  |  |
|  | 60 |  | -94.5 | -92.4 | -91.2 | -90.0 | -89.1 |  | -82.6 |  |  |
| n5 | 15 | -98.0 | -94.8 | -93.0 | -86.8 |  |  |  |  |  |  |
|  | 30 |  | -95.1 | -93.1 | -88.6 |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |
| n71 | 15 | -98.0 | -94.8 | -93.0 | -91.8 | -90.7 | -89.9 |  | -88.6 |  | -81.5 |
|  | 30 |  | -95.1 | -93.1 | -92.0 | -90.8 | -90.0 |  | -88.7 |  | -81.5 |
|  | 60 |  | -95.5 | -93.4 | -92.2 | -91.0 | -90.1 |  | -88.9 |  | -81.5 |
| n8 | 15 | -97.0 | -93.8 | -91.4 | -85.8 |  |  |  |  |  |  |
|  | 30 |  | -94.1 | -91.7 | -87.2 |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |
| n12 | 15 | -97.0 | -93.8 | -84.0 |  |  |  |  |  |  |  |
|  | 30 |  | -94.1 | -84.1 |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |
| n13 | 15 | -97.0 | -93.8 |  |  |  |  |  |  |  |  |
|  | 30 |  | -94.1 |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |
| n14 | 15 | -97.0 | -93.8 |  |  |  |  |  |  |  |  |
|  | 30 |  | -94.1 |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |
| n18 | 15 | -100.0 | -96.8 | -95.0 |  |  |  |  |  |  |  |
|  | 30 |  | -97.1 | -95.1 |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |
| n20 | 15 | -97.0 | -93.8 | -91.0 | -89.8 |  |  |  |  |  |  |
|  | 30 |  | -94.1 | -91.1 | -90.0 |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |
| n25 | 15 | -96.5 | -93.3 | -91.5 | -90.3 | -89.3 | -82.2 |  | -79.5 |  |  |
|  | 30 |  | -93.6 | -91.6 | -90.5 | -89.4 | -82.3 |  | -79.6 |  |  |
|  | 60 |  | -94.0 | -91.9 | -90.7 | -89.6 | -82.4 |  | -79.7 |  |  |
| n26 | 15 | -97.56 | -94.56 | -92.76 | -87.6 |  |  |  |  |  |  |
|  | 30 |  | -94.86 | -92.76 | -87.7 |  |  |  |  |  |  |
| n28 | 15 | -98.5 | -95.5 | -93.5 | -90.8 |  | -78.5 |  |  |  |  |
|  | 30 |  | -95.6 | -93.6 | -91.0 |  | -78.6 |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |
| n297 | 15 | -97.0 | -93.8 |  |  |  |  |  |  |  |  |
|  | 30 |  | -94.1 |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |
| n30 | 15 | -99.0 | -95.8 |  |  |  |  |  |  |  |  |
|  | 30 |  | -96.1 |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |
| n65 | 15 | -99.5 | -96.3 | -94.5 | -93.3 |  |  |  |  |  | -89.2 |
|  | 30 |  | -96.6 | -94.6 | -93.5 |  |  |  |  |  | -89.3 |
|  | 60 |  | -97.0 | -94.9 | -93.7 |  |  |  |  |  | -89.4 |
| n66 | 15 | -99.5 | -96.3 | -94.5 | -93.3 | -92.2 | -91.4 |  | -90.1 |  |  |
|  | 30 |  | -96.6 | -94.6 | -93.5 | -92.3 | -91.5 |  | -90.2 |  |  |
|  | 60 |  | -97.0 | -94.9 | -93.7 | -92.5 | -91.6 |  | -90.4 |  |  |
| n70 | 15 | -100.0 | -96.8 | -95.0 | -93.8 | -92.7 |  |  |  |  |  |
|  | 30 |  | -97.1 | -95.1 | -94.0 | -92.8 |  |  |  |  |  |
|  | 60 |  | -97.5 | -95.4 | -94.2 | -93.0 |  |  |  |  |  |
| n71 | 15 | -97.2 | -94.0 | -91.6 | -86.0 |  |  |  |  |  |  |
|  | 30 |  | -94.3 | -91.9 | -87.4 |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |
| n743 | 15 | -99.5 | -96.3 | -94.5 | -89.3 |  |  |  |  |  |  |
|  | 30 |  | -96.6 | -94.6 | -89.5 |  |  |  |  |  |  |
|  | 60 |  | -97.0 | -94.9 | -89.6 |  |  |  |  |  |  |
| n757 | 15 | -100 | -96.8 | -95.0 | -93.8 | -92.7 | -91.9 |  | -90.6 |  | -89.6 |
|  | 30 |  | -97.1 | -95.1 | -94.0 | -92.8 | -92.0 |  | -90.7 |  | -89.7 |
|  | 60 |  | -97.5 | -95.4 | -94.2 | -93.0 | -92.1 |  | -90.9 |  | -89.8 |
| n767 | 15 | -100 |  |  |  |  |  |  |  |  |  |
|  | 30 |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |
| n91 | 15 | -100 |  |  |  |  |  |  |  |  |  |
|  | 30 |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |
| n92 | 15 | -100 | -96.8 | -95.0 | -93.8 |  |  |  |  |  |  |
|  | 30 |  | -97.1 | -95.1 | -94.0 |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |
| n93 | 15 | -100 |  |  |  |  |  |  |  |  |  |
|  | 30 |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |
| n94 | 15 | -100 | -96.8 | -95.0 | -93.8 |  |  |  |  |  |  |
|  | 30 |  | -97.1 | -95.1 | -94.0 |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: Four Rx antenna ports shall be the baseline for this operating band except for two Rx vehicular UE. NOTE 2: The transmitter shall be set to PUMAX as defined in clause 6.2.4 NOTE 3: The requirement is modified by -0.5 dB when the assigned NR channel bandwidth is confined within 1475.9 - 1510.9 MHz. NOTE 4: Void NOTE 5: Void NOTE 6: Values are modified by -0.5dB when carrier channel BW is between 865MHz and 894MHz. NOTE 7: For SDL bands, the reference sensitivity requirements shall be verified by inter-band CA combinations with SDL band, which are supported by UE. |  |  |  |  |  |  |  |  |  |  |  |

Table 2-2 Proposed REFSENS table for FR1 TDD bands

| Operating band / SCS / Channel bandwidth / REFSENS |  |  |  |
| --- | --- | --- | --- |
| Operating band | SCS kHz | Channel bandwidth (MHz) | REFSENS (dBm)8 |
| n34 | 15 | 5, 10, 15 | -100 + 10log10(NRB/25) |
|  | 30 | 10, 15 | -97.1 + 10log10(NRB/24) |
|  | 60 | 10, 15 | -97.5 + 10log10(NRB/11) |
| n381 | 15 | 5, 10, 15, 20, 25, 30, 40 | -100 + 10log10(NRB/25) |
|  | 30 | 10, 15, 20, 25, 30, 40 | -97.1 + 10log10(NRB/24) |
|  | 60 | 10, 15, 20, 25, 30, 40 | -97.5 + 10log10(NRB/11) |
| n39 | 15 | 5, 10, 15, 20, 25, 30, 40 | -100 + 10log10(NRB/25) |
|  | 30 | 10, 15, 20, 25, 30, 40 | -97.1 + 10log10(NRB/24) |
|  | 60 | 10, 15, 20, 25, 30, 40 | -97.5 + 10log10(NRB/11) |
| n40 | 15 | 5, 10, 15, 20, 25, 30, 40, 50 | -100 + 10log10(NRB/25) |
|  | 30 | 10, 15, 20, 25, 30, 40, 50, 60, 80 | -97.1 + 10log10(NRB/24) |
|  | 60 | 10, 15, 20, 25, 30, 40, 50, 60, 80 | -97.5 + 10log10(NRB/11) |
| n411 | 15 | 10, 15, 20, 30, 40, 50 | -94.8 + 10log10(NRB/50) |
|  | 30 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | -95.1 + 10log10(NRB/24) |
|  | 60 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | -95.5 + 10log10(NRB/11) |
| n481 | 15 | 5, 10, 15, 20, 40, 505 | -99 + 10log10(NRB/25) |
|  | 30 | 10, 15, 20, 40, 505, 605, 705, 805, 905, 1005 | -96.1 + 10log10(NRB/24) |
|  | 60 | 10, 15, 20, 40, 505, 605, 705, 805, 905, 1005 | -96.5 + 10log10(NRB/11) |
| n50 | 15 | 5, 10, 15, 20, 30, 40, 50 | -100 + 10log10(NRB/25) |
|  | 30 | 10, 15, 20, 30, 40, 50, 60, 80 | -97.1 + 10log10(NRB/24) |
|  | 60 | 10, 15, 20, 30, 40, 50, 60, 80 | -97.5 + 10log10(NRB/11) |
| n51 | 15 | 5 | -100 |
|  | 30 |  |  |
|  | 60 |  |  |
| n53 | 15 | 5, 10 | -100 + 10log10(NRB/25) |
|  | 30 | 10 | -97.1 |
|  | 60 | 10 | -97.5 |
| n771,4 | 15 | 10, 15, 20, 25, 30, 40, 50 | -95.3 + 10log10(NRB/50) |
|  | 30 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | -95.6 + 10log10(NRB/24) |
|  | 60 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | -96.0 + 10log10(NRB/11) |
| n781 | 15 | 10, 15, 20, 25, 30, 40, 50 | -95.8 + 10log10(NRB/50) |
|  | 30 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | -96.1 + 10log10(NRB/24) |
|  | 60 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | -96.5 + 10log10(NRB/11) |
| n791 | 15 | 40, 50 | -89.6 + 10log10(NRB/216) |
|  | 30 | 40, 50, 60, 80, 100 | -89.7 + 10log10(NRB/106) |
|  | 60 | 40, 50, 60, 80, 100 | -89.9 + 10log10(NRB/51) |
| NOTE 1: Four Rx antenna ports shall be the baseline for this operating band except for two Rx vehicular UE. NOTE 2: The transmitter shall be set to PUMAX as defined in clause 6.2.4 NOTE 3: Void NOTE 4: The requirement is modified by -0.5 dB when the assigned UE channel bandwidth is confined within 3300 - 3800 MHz. NOTE 5: For these bandwidths, the minimum requirements are restricted to operation when carrier is configured as a downlink carrier part of CA configuration. NOTE 6: Void NOTE 7: Void NOTE 8: The REFSENS value is rounded to the nearest number down to one decimal point. |  |  |  |

  - Option 1: Agree

  - Option 2: Do not agree

- Recommended WF

  - TBA

Sub-topic 2-2

Issue 2-2: Draft CR to 38.101-1 for band n1

- Proposals

  - Option 1: Agree

  - Option 2: Do not agree

- Recommended WF

  - TBA

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| XXX | Sub-topic 2-1: REFSENS table split and simplification Sub-topic 2-2: Draft CR to 38.101-1 for band n1 …. Others: |

Sub-topic 2-1: REFSENS table split and simplification

| Qualcomm | Seems reasonable since REFSENS values that are not scalable over BW are only FDD bands, but some verification is required to see if formulas are correct. |
| --- | --- |
| Skyworks | We support this initiative, but we would like to offer further improvements with a third option for FDD bands as “Option 3: Adopt FDD band REFSENS equation-based representation with supported minimum channel BW and a maximum sensitivity degradation (MSD) lookup table as a reference”. We believe option 3 will futher reduce table complexity since the MSD lookup table is rather small. The MSD lookup table format and equations could be further discussed in this summary or via a WF. |
| ZTE | Good approach. For FDD band, it seems it is impossible to support up to 100MHz which make separate tables with TDD band feasible. For TDD band, a note may be need to specify NRB of each channel bandwidth defined in Table 5.3.2-1 is used. |

Sub-topic 2-2: Draft CR to 38.101-1 for band n1

| Skyworks | Option 2: [TBD] values are proposed for A-MPR, we should have values to agree upon. |
| --- | --- |

CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic 2-1: REFSENS table split and simplification | Based on 1st round discussion, all companies are positive on the proposal, and some further improvements and some verifications may be required. Recommendations for 2nd round: further discussion on the sub-topic and capture the agreements in WF2. |
| Sub-topic 2-2: Draft CR to 38.101-1 for band n1 | A-MPR values need to be addressed firstly. Recommendations for 2nd round: focus on the A-MPR discussion. |

Discussion on 2nd round (if applicable)

Companies views’ collection for 2nd round

WF on REFSENS table split and simplification    Source: Apple

| Company | Comments |
| --- | --- |
| Skyworks | We support this WF. |
| Huawei | Ok with the WF |
| Ericsson | OK with WF. Regarding REFSENS table split we encourage interested companies to seek feedback from their RAN5 colleagues since RAN5 specs will be needed to be updated accordingly. |

Topic #3: A-MPR

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2105171 | Qualcomm Technologies Int | A-MPR regions for NS_48 Channel Bandwidth, MHz; Carrier Center Frequency, Fc, MHz; Regions; A-MPR. RBend*12*SCS MHz; LCRB*12*SCS MHz. 45 MHz; 1942.5 ≤ FC ≤ 1957.5; ≥0, <4.5; >0; A2. ≥4.5, <19; ≥max (0, 12*SCS*RBend - 3.6); A4. ≥19, <37.44; ≥15.4; A2. ≥30.96, <37.44; <1.08; A5. ≥37.44; >0; A1. A-MPR regions for NS_49 Channel Bandwidth, MHz; Carrier Center Frequency, Fc, MHz; Regions; A-MPR. RBend*12*SCS MHz; LCRB*12*SCS MHz. 45MHz; 1942.5 ≤ FC ≤ 1957.5; ≥6.12, <12.42; < min [1.08, max(0,12*SCS* RBend-6.12)]; A5. ≥30.96, <36.72; <1.08; A5. <36.72; ≥16.2, <max (0, 12*SCS*RBend – 6.12); A2. <36.72; ≥max (0, 12*SCS*RBend – 6.12); A1. ≥36.72; >0; A1. |
| R4-2106282 | China Telecom | Observation 1: For NS_49, A-MPR requirement for 50MHz is a little pessimistic by comparing to that for 40MHz, which needs to be perceived when defining the A-MPR for 45MHz. |
| R4-2106484 | Huawei, HiSilicon | The simulation results are provided and the interpolated values from the data points of 40 MHz and 50 MHz is proposed in Table 2.2-1 and Table2.2-2. Table 2.2-1: A-MPR regions for NS_48 Channel Bandwidth, MHz; Carrier Center Frequency, Fc, MHz; Regions; A-MPR. RBend*12*SCS MHz; LCRB*12*SCS MHz. 45 MHz; 1942.5 ≤ FC ≤ 1957.5; ≥0, <[4.5]; >0; A2. ≥[4.5], <[18.9]; ≥max (0, 12*SCS*RBend - 3.6); A4. ≥[18.9], <[37.8]; ≥[15.3]; A2. ≥[30.6], <[37.8]; <1.08; A5. ≥[37.8]; >0; A1. Table 2.2-2: A-MPR regions for NS_49 Channel Bandwidth, MHz; Carrier Center Frequency, Fc, MHz; Regions; A-MPR. RBend*12*SCS MHz; LCRB*12*SCS MHz. 45 MHz; 1945 ≤ FC ≤ 1955; ≥0, <[7.47]; ≥0; A1. ≥[7.47], <[16.2]; < min [1.08, max(0,12*SCS* RBend-[7.47])]; A5. ≥[31.68], <[37.08]; <1.08; A2. <[37.08]; ≥[23.4]; A1. ≥[37.08]; >0; A1. |
| R4-2100516 from RAN4#98-e | Apple Inc. | Table 2: Regions for NS_48 Channel BW (MHz); Carrier Frequency Fc (MHz); RBend*12*SCS (MHz); LCRB*12*SCS (MHz); A-MPR. 45; 1942.5 ≤ Fc ≤1957.5; >=0, <5.76; >0; A2. >=5.76, <19.44; >=max(0,12*RB_end*SCS-3.6); A4. >=5.76, <19.44; <max(0,12*RB_end*SCS-3.6) >=max(0,12*RB_end*SCS-5.76); A3. >=19.44, <38.16; >=14.4; A2. >=30.24, <38.16; <1.08; A5. >=38.16; >0; A1. Table 3: Regions for NS_49 Channel BW (MHz); Carrier Frequency Fc (MHz); RBend*12*SCS (MHz); LCRB*12*SCS (MHz); A-MPR. 45; 1942.5 ≤ Fc ≤1957.5; >=7.74, <14.4; <max(0,RB_end-7.74); A5. >=30.96, <35.28; <1.08; A5. <35.28; >=15.12 <max(0,RB_end-7.74); A2. <35.28; <15.12 >=11.52 < max(0,RB_end-7.74); A3. <35.28; >=max(0,RB_end-7.74); A1. >=35.28; >0; A1. |

Open issues summary

Sub-topic 3-1

Issue 3-1: regions and A-MPR for NS_48 and NS_49

- Proposals on regions and A-MPR for NS_48

  - Option 1:

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz |  |
| 45 MHz | 1942.5 ≤ FC ≤ 1957.5 | ≥0, <[4.5] | >0 | A2 |
|  |  | ≥[4.5], <[18.9] | ≥max (0, 12*SCS*RBend - 3.6) | A4 |
|  |  | ≥[18.9], <[37.8] | ≥[15.3] | A2 |
|  |  | ≥[30.6], <[37.8] | <1.08 | A5 |
|  |  | ≥[37.8] | >0 | A1 |

  - Option 2:

| Channel BW (MHz) | Carrier Frequency Fc (MHz) | RBend*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 45 | 1942.5 ≤ Fc ≤1957.5 | >=0, <5.76 | >0 | A2 |
|  |  | >=5.76, <19.44 | >=max(0,12*RB_end*SCS-3.6) | A4 |
|  |  | >=5.76, <19.44 | <max(0,12*RB_end*SCS-3.6) >=max(0,12*RB_end*SCS-5.76) | A3 |
|  |  | >=19.44, <38.16 | >=14.4 | A2 |
|  |  | >=30.24, <38.16 | <1.08 | A5 |
|  |  | >=38.16 | >0 | A1 |

  - Option 3:

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz |  |
| 45 MHz | 1942.5 ≤ FC ≤ 1957.5 | ≥0, <4.5 | >0 | A2 |
|  |  | ≥4.5, <19 | ≥max (0, 12*SCS*RBend - 3.6) | A4 |
|  |  | ≥19, <37.44 | ≥15.4 | A2 |
|  |  | ≥30.96, <37.44 | <1.08 | A5 |
|  |  | ≥37.44 | >0 | A1 |

Note: A-MPR for 35 MHz is also proposed but 35 MHz is not requested in the WID.

- Proposals on regions and A-MPR for NS_49

  - Option 1:

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz |  |
| 45 MHz | 1945 ≤ FC ≤ 1955 | ≥0, <[7.47] | ≥0 | A1 |
|  |  | ≥[7.47], <[16.2] | < min [1.08, max(0,12*SCS* RBend-[7.47])] | A5 |
|  |  | ≥[31.68], <[37.08] | <1.08 | A2 |
|  |  | <[37.08] | ≥[23.4] | A1 |
|  |  | ≥[37.08] | >0 | A1 |

  - Option 2:

| Channel BW (MHz) | Carrier Frequency Fc (MHz) | RBend*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 45 | 1942.5 ≤ Fc ≤1957.5 | >=7.74, <14.4 | <max(0,RB_end-7.74) | A5 |
|  |  | >=30.96, <35.28 | <1.08 | A5 |
|  |  | <35.28 | >=15.12 <max(0,RB_end-7.74) | A2 |
|  |  | <35.28 | <15.12 >=11.52 < max(0,RB_end-7.74) | A3 |
|  |  | <35.28 | >=max(0,RB_end-7.74) | A1 |
|  |  | >=35.28 | >0 | A1 |

  - Option 3:

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz |  |
| 45MHz | 1942.5 ≤ FC ≤ 1957.5 | ≥6.12, <12.42 | < min [1.08, max(0,12*SCS* RBend-6.12)] | A5 |
|  |  | ≥30.96, <36.72 | <1.08 | A5 |
|  |  | <36.72 | ≥16.2, <max (0, 12*SCS*RBend – 6.12) | A2 |
|  |  | <36.72 | ≥max (0, 12*SCS*RBend – 6.12) | A1 |
|  |  | ≥36.72 | >0 | A1 |

- Recommended WF

  - Discussion on the options and check whether consensus can be reached at 1st round discussion.

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| XXX | Sub topic 1-1: A-MPR for NS_48 and NS_49 …. Others: |

Subtopic 1-1: A-MPR for NS_48 and NS_49

| Company | Comments |
| --- | --- |
| Qualcomm | NS_48: Prefer option 3, since option1 (37.8MHz) and option 2 (38.16MHz) is > option3 (37.44MHz) do not cover the A1 IM3 region at 60KHz SCS. NS_49: Prefer topology of option 2 or option 3 since for NS_49 because the IM3 regions can occur when the channel BW is at the low edge or high edge of band n1, and so the IM3 regrowth region spills over the proposed boundary in option 1. Option 1 has IM3 spillover into region A5 that is not accounted for. |
| China Telecom | NS_48: Prefer option 1 and option3, as in general Ax distribution looks aligned with 40MHz and 50MHz. NS_49: Wonder why option 1 looks different from option 2 and 3? We slightly prefer option 1. It seems option1 proposed an optimism requirement by comparing option 2 and 3, which is aligned with our observation in contribution R4-2106282. |
| Huawei | NS_48: ok with option 3 since the difference is small compared to our proposal. NS_49: ok to use option 2 or 3 as starting point and further check the simulation results. |
| Apple | NS_48: Option 3 is fine for us. We would like to request to increase the requirement 4.5MHz by 0.36MHz to 4.86MHz. Also, Option 2 is good. NS_49: We could accept Option 3 as is and of course also Option 2. |

CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Subtopic 1-1: A-MPR for NS_48 and NS_49 | Based on 1st round discussion, for NS_48, option 3 is agreeable for companies. More discussion for NS_49 is needed in 2nd round. Recommendations for 2nd round: further discussion on A-MPR for NS_49 and capture the agreements on A-MPR for n1 45 MHz in WF3 |

Discussion on 2nd round (if applicable)

Companies views’ collection for 2nd round

WF on A-MPR for n1 45 MHz    Source: China Telecom

| Company | Comments |
| --- | --- |
| Huawei | Ok with the WF |

Recommendations for Tdocs

1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on UE REFSENS for 35 MHz and 45 MHz | Huawei | WF1 |
| WF on REFSENS table split and simplification | Apple | WF2 |
| WF on A-MPR for n1 45 MHz | China Telecom | WF3 |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2104656 | REFSENS of n8, n71, n25 for 35_45MHz channel bandwidth | Murata Manufacturing Co Ltd. | Noted |  |
| R4-2104811 | REFSENS evaluation of n8 and n71 for 35MHz channel bandwidth | Mediatek India Technology Pvt. | Noted |  |
| R4-2104878 | n1 35 and 45MHz BW AMPR | Qualcomm Technologies Int | Noted |  |
| R4-2104888 | UL channel location for MSD calculation for bands n8 and n71 | Apple | Noted |  |
| R4-2106282 | Discussion on A-MPR for 45MHz CBW for Band n1 | China Telecom | Noted |  |
| R4-2106283 | Draft CR to 38.101-1 Introduce band specific requirements for 45MHz CBW for Band n1 | China Telecom | Noted |  |
| R4-2106483 | UE REFSENS for 35 MHz and 45 MHz | Huawei, HiSilicon | Noted |  |
| R4-2106484 | A-MPR for n1 45MHz | Huawei, HiSilicon | Noted |  |
| R4-2106906 | FR1 REFSENS table split and simplification | Apple | Noted |  |
| R4-2107337 | 35-45MHz REFSENS for n2 n3 n8 n25 n71 | Skyworks Solutions Inc. | Noted |  |
| R4-2107339 | 35MHz and 45MHz REFSENS | Qualcomm Incorporated | Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2105364 | WF on UE REFSENS for 35 MHz and 45 MHz | Huawei | Agreeable |  |
| R4-2105365 | WF on REFSENS table split and simplification | Apple | Agreeable |  |
| R4-2105366 | WF on A-MPR for n1 45 MHz | China Telecom | Agreeable |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents