3GPP TSG-RAN WG4 Meeting #98bis-e	R4-2105457

Electronic Meeting, Apr. 12-20, 2021

Agenda item:			7.34.1

Source:	Moderator (China Telecom)

Title:	Email discussion summary for [98-bis-e][120] DL_intrpt_combos_TxSW_R17

Document for:	Information

Introduction

This email discussion thread is related to Downlink interruption for band combinations basket WI, and will focus on the topic of following aspects:

- Topic #1: Draft TR, draft big CR

- Topic#2: Downlink interruption analysis

  - Issue 2-1-1: DL interruption applicability

  - Issue 2-1-2: TP’s for approval

Note that the tables for collecting comments for sub-topic issues are arranged just below each issue.

Topic #1: Draft and draft big CR

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations/Abstracts |
| --- | --- | --- |
| R4-2104801 | CATT | Draft TR v 0.2.0 for Downlink interruption for band combinations to conduct dynamic Tx Switching |
| R4-2106280 | China Telecom | Big CR to 38101-1: Introduce the downlink interruption clarification for CA conducting Tx Switching |

Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 1-1: Draft TR, draft big CR

This sub-topic will discuss rapporteur input for TR skeleton, work plan and revised WID.

Issue 1-1-1: Draft TR

- Recommended WF

  - It is recommended for email approval for the draft TR of R4-2104801

| Company | Comments on Issue 1-1-1: draft TR |
| --- | --- |

Issue 1-1-2: draft big CR

- Recommended WF

  - It is recommended to endorse the draft big CR of R4-2106280, new modification or combos completed in this meeting could be captured into formal big CR in next meeting.

| Company | Comments on Issue 1-1-2: draft big CR |
| --- | --- |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: No concern on the draft big CR, the draft big CR R4-2106280 is recommended as endorsed Candidate options: Recommendations for 2nd round: Topic #1 is closed |

Recommendations on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2104801 | for email approval |
| R4-2106280 | Endorsed |

Discussion on 2nd round (if applicable)

No need 2nd round, Topic #1 is closed

Summary on 2nd round (if applicable)

Topic #2: Downlink interruption analysis

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations/Abstracts |
| --- | --- | --- |
| R4-2104589 | CMCC | TP on update the note of DL interruption applicability for 37.867 |
| R4-2104590 | CMCC | TP on DL interruption applicability of CA_n3-n40 for 37.867 |
| R4-2104591 | CMCC | Proposal 1: it is proposed that the basket WI on DL interruption for dynamic switching should cover the following cases: - 1 Tx-2Tx switching between carrier 1 and carrier 2 - 2Tx-2Tx switching between carrier 1 and carrier 2 - 1Tx-2Tx and 2Tx-2Tx switching between band A (carrier 1) and band B (carrier 2+3) Proposal 2: The following changes are suggested for the WID and to be captured in TR: - Discuss and specify inter-band uplink CA and EN-DC combinations for which DL interruption is not allowed when dynamic switching is conducted. - Based on the requested band combinations from operators, identify if it is feasible to specify no DL interruption in RAN4 specification. - Discuss and find out an appropriate way to indicate no DL interruption for higher order CA and EN-DC combinations in RAN4 specification. - The dynamic switching includes: - 1 Tx-2Tx switching between carrier 1 and carrier 2 - 2Tx-2Tx switching between carrier 1 and carrier 2 - 1Tx-2Tx and 2Tx-2Tx switching between band A (carrier 1) and band B (carrier 2+3) - For 1Tx-2Tx switching between two uplink carriers, the mandating of no DL interruption for new band combinations is release independent from Rel-16. Proposal 3: change the note in tables in TR 37.867 as follows: - NOTE 3: Applicable when dynamic Tx switching is conducted. The DL interruption requirement is specified in clause 8.2.2.2.10 of 38.133 [13]. |
| R4-2106281 | China Telecom | TP to 37.867: DL interruption clarification for CA_n1-n3-n78 |

Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 2-1: Downlink interruption analysis

This sub-topic will discuss DL interruption applicability for inter-band CA and corresponding TP’s for approval.

Issue 2-1-1: change the note in tables in TR 37.867

- Proposals (R4-2104591, R4-2104589)

  - Proposal: change the note in tables in TR 37.867 as follows:

NOTE 3:	Applicable when dynamic Tx switching is conducted. The DL interruption requirement is specified in clause 8.2.2.2.10 of 38.133 [13].

- Recommended WF

  - Collect views on this proposal

| Company | Comments on Issue 2-1-1: change the note in tables in TR 37.867 |
| --- | --- |
| Qualcomm | There is same proposal in R4-2104593. Do we need to discuss this in two places? Here is our comment toward this issue: The original wording in 38.133 and TR 37.867, 'dynamic switching between two uplink carriers' covers the cases specified in this WID. Therefore, no wording change is needed |
| CMCC | We are OK to discuss this first in email thread#129 and follow the agreements there. We first propose to modify the WID to cover all the dynamic Tx switching cases. Since in last meeting RAN4 agreed that: - There is no need to differentiate the DL interruption applicability between Rel-16 1Tx-2Tx switching and Rel-17 Tx switching scenarios, which means that “DL interruption allowed” specified in existing TS 38.101-1 should also be applied to the Rel-17 Tx switching scenarios including: - 2Tx-2Tx switching between carrier 1 and carrier 2 - 1Tx-2Tx and 2Tx-2Tx switching between band A (carrier 1) and band B (carrier 2+3) If companies agree that this basket WI need to cover all the following cases, then the change is necessary. Otherwise, separate basket need to be created to cover 2Tx switching and switching between band A and band B, which is not preferred, since the interruption applicability are the same according to the above RAN4 agreements. Regarding 38.133, the changes are also needed for Rel-17 in order to cover all switching cases. - 1 Tx-2Tx switching between carrier 1 and carrier 2 - 2Tx-2Tx switching between carrier 1 and carrier 2 - 1Tx-2Tx and 2Tx-2Tx switching between band A (carrier 1) and band B (carrier 2+3) |

Issue 2-1-2: TPs for approval

- Proposed TPs

  - R4-2104589, R4-2104590, R4-2106281

- Recommended WF

  - Collect the comments for proposed TPs. If no comments for certain of TP’s, the TP’s will be recommended as approved.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2104589 | company A: |
| R4-2104590 |  |
| R4-2106281 | Qualcomm: We have a concern on 3+78 case. We may need to change the 3+78 mandatory no interruption so also need to postpone this TP. Reason is band 3 harmonic lands on 78 RX so for duration of any switching event n78 RX is disturbed. |
|  | China Telecom: Ok, However for the reason we are curious to learn more details, as according to the previous discussion, it was identified that TDD will have impact FDD, but FDD disturbing TDD was not mentioned. |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: TP R4-2104589 and R4-2104590 are recommended as revised TP R4-2106281 is recommended as return to Candidate options: Recommendations for 2nd round: Continue discuss the issue Issue 2-1-1: change the note in tables in TR 37.867, and also the Issue 2-1-2: for revised and return to TPs |

Recommendations on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2104589 | revised |
| R4-2104590 | revised |
| R4-2106281 | return to |

Discussion on 2nd round (if applicable)

Sub-topic 2-1: Downlink interruption analysis

Continue discussion on sub-topic 2-1 and focus on the revised and return to TPs

| CR/TP number | Comments collection |
| --- | --- |
| Rev of R4-2104589 | MediaTek: we are not ready to approve the TP before the concern on R4-2106281 is clarified. Not only MSD due to UL harmonic, all MSD mechanisms need to be considered if exists large MSD (ex: > 6dB) |
| Rev of R4-2104590 |  |
| Return to R4-2106281 | MediaTek: Share same concern with Qualcomm and it needs to be clarified. |
|  | Qualcomm: We would appreciate the opportunity to come back to this next meeting. Sorry for late and unable to provide more information, the band combo dependent capabilities are hard to know and forcing a capability in one way or the other is always difficult. |

Summary on 2nd round (if applicable)

|  | Status summary |
| --- | --- |
| Sub-topic 2-1: Downlink interruption analysis | Recommendations for conclusion: According to the discussion on 2nd round, As no further concern to TP R4-2105376(rev of R4-2104590), so this TP is recommended as approved. The concern for R4-2105375(Rev of R4-2104589) is solved according to discussion in reflector, so this TP is recommended as approved. Regarding the endorsed big CR R4-2106280 on 1st round, MediaTek has concern to some of combos on 2nd round. So could we suggest chairman to add the chairman note below the CR? The chairman note: MediaTek has concern to the CA_n3-n40-n41, CA_n3-n41-n79, CA_n8-n39-n41, CA_n8-n41-n79 with TPs approved in last meeting, and will confirm in next meeting if above CAs could be captured in the formal CR. |

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| R4-2106281 | noted |
| R4-2105375 | approved |
| R4-2105376 | approved |
| R4-2106280 | Endorsed Hi Steven, according to the discussion in the reflector, could you add the chairman note below the big CR R4-2106280? The chairman note is suggested: Chairman note: MediaTek has concern to the CA_n3-n40-n41, CA_n3-n41-n79, CA_n8-n39-n41, CA_n8-n41-n79 with TPs approved in last meeting, and will confirm in next meeting if above CAs could be captured in the formal CR. |