3GPP TSG-RAN WG4 Meeting #98-bis-e	R4-2105758

Electronic Meeting, 12 – 20 April, 2021

Title: 	Updated link level simulation assumption for gNB positioning measurement performance

Source: 	Huawei, HiSilicon

Agenda item:	5.5.2.3.1

Document for:	Approval

Introduction

In RAN4#96-e, the link level simulation assumption for gNB positioning measurement was agreed in [1].

In [1] the following combinations of {comb size, symbol size} are simulated:

- {2, 2}, which gives SRS RE number 12

- {4, 4}, which gives SRS RE number 12

- {8, 8}, which gives SRS RE number 12

- {8, 12}, which gives SRS RE number 18

Based on RAN1/2 design, the number of SRS REs can be

- 6 as minimum, e.g. given by {2,1}

- 36 as maximum, given by {4, 12}

To better understand the impact of {comb size, symbol size} on the accuracy performance, and to decide whether requirements should be agnostic to {comb size, symbol size}, more combinations of {comb size, symbol size} are added to the simulation assumption.

Updated simulation assumptions

Table 1 lists the updated link simulation assumptions.

Table 1: Updated link simulation assumptions for gNB positioning measurement

| Parameter |  | Value |  |  |
| --- | --- | --- | --- | --- |
|  |  | FR1 |  | FR2 |
| Cell layout |  | - 2 cells at distinct locations: <cell 1, cell2>, where cell 1 is the serving cell. - 1 UE in cell 1 transmits p-SRS, received by cell1 and by cell2. |  |  |
| Network synchronization |  | - Synchronous with time shifts <0 (cell 1), 3 us (cell 2)> - Asynchronous with time shifts: <0 (cell 1), 7 symbols (cell 2) > |  |  |
| Duplex modes |  | FDD and TDD |  |  |
| TDD specific parameters (TDD configuration is in 38.133, section A.3.1.4) |  | - TDDConf.1.1 (15 kHz) - TDDConf.2.1 (30/60 kHz) |  | - TDDConf.3.1 (60/120 kHz) |
| Load in SRS symbols |  | - No other SRS transmissions from other UEs on the same subcarriers of the SRS symbols |  |  |
| Data and CCH load in non-SRS symbols |  | - 0% RE utilization - 50% RE utilization in time - 100% RE utilization |  |  |
| Cyclic prefix |  | Normal |  |  |
| DRX |  | OFF |  |  |
| Carrier frequency / BW / SCS / duplex mode |  | - 2 GHz - 5 MHz, 10 MHz, 20 MHz, 50 MHz - 15 kHz - FDD, TDD - 4 GHz - 20 MHz, 50 MHz, 100 MHz - 30 kHz, 60kHz - FDD, TDD |  | - 40 GHz - 50 MHz, 100 MHz, 200 MHz - 120 kHz, 60kHz - TDD |
| Propagation conditions [TS 38.101-4] |  | AWGN, |  | AWGN, |
| SRS Ês/Iot [dB] |  | For cell 1: 3dB For cell 2: -13dB |  | For cell 1: 3dB For cell 2: -13dB |
| Number of gNB receive antennas |  | 2 rx (uncorrelated with equal gain, no rx beamforming) |  |  |
| gNB measurement bandwidth |  | Full carrier bandwidth |  |  |
| TA |  | Constant |  |  |
| Number of transmit SRS antennas | 1 |  |  |  |
| Number of SRS Resource sets | 1 |  |  |  |
| Number of SRS resources within one SRS resource set | 1 |  | 1, TBD |  |
| SRS transmission bandwidth (in PRBs) | - 15 kHz: - 24 (5MHz), 52 (10MHz), 104 (20MHz), 264 (50MHz) - 30 kHz: - 48 (20MHz),132 (50MHz), 272 (100MHz) - 60 kHz: - 48 (40MHz), 132 (100MHz) |  | - 120 kHz: - 32(50MHz), 64(100MHz), 132 (200MHz) - 60 kHz: 132 (100MHz), 264 (200MHz) |  |
| SRS comb size | comb-2, comb-4, comb-8 |  |  |  |
| Number of symbols | NsymbSRS=2 for comb-2, NsymbSRS=4 for comb-4, NsymbSRS=8 and NsymbSRS=12 for comb-8, NsymbSRS=1 for comb-2 and NsymbSRS= 12 for comb-4 |  |  |  |
| SRS repetition factor | 1 (TS 38.211, clause 6.4.1.4.3) |  |  |  |
| SRS periodicity | 40 ms, 160 ms |  |  |  |
| Sequence or group hopping | Disabled |  |  |  |
| Starting symbol () | l0=2 for comb-2, l0=4 for comb-4, l0=12 for comb-8 |  |  |  |

Performance Characteristics

At least the following performance characteristics are to be provided for TgNB-RX:

- TgNB-RX error CDFs for each cell

- 90%-ile of the TgNB-RX errors for each cell

In the above,

- TgNB-RX error = abs(estimated TgNB-RX – ideal TgNB-RX ) (based on perfect channel and UE location knowledge).

At least the following performance characteristics are to be provided for SRS-RSRP:

- RSRP error CDFs for each cell

- 90%-ile of the RSRP errors for each cell

In the above,

- SRS-RSRP error = estimated SRS-RSRP – ideal SRS-RSRP (based on perfect channel knowledge).

Reference

- R4-2012142, Link simulation assumptions for deriving positioning SRS configurations, Nokia