3GPP TSG-RAN4 Meeting #98-bis-e															R4-2105815

Electronic Meeting, Apr.12-20, 2021

Agenda item:			5.7.2.3

Source:	Moderator (OPPO)

Title:	Email discussion summary for [98-bis-e][213] NR_CSIRS_L3meas_2

Document for:	Information

Introduction

Briefly introduce background, the scope of this email discussion and provide some guidelines for email discussion if necessary.

Based on the endorsed CR (R4-2101533) for CSI-RS L3 RRM test cases in RAN4#98e, we focus on the discussion of companies’ draft CRs for test cases in this meeting, including

- Update timing offset

- Clarification of SSB index in test requirements

- Update values of measurement delay in test requirements

- Others

List of candidate target of email discussion for 1st round and 2nd round

- 1st round:  agree on open issues and draft CRs for test cases (e.g., intra-f, inter-f measurement, accuracy tests)

- 2nd round: TBA

Topic #1: Test cases

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2106529 | OPPO | Observation 1: [TBD]s for time offset need to clean up: - [0.9] CP length as the time offset between cells can apply uniformly in RSRP/RSRQ/SINR test cases. Observation 2: Clarify that for all tests in FR1 or FR2, - the UE is required to read the neighbour cell SSB index in this test in order to detect associated SSB for the CSI-RS resource of neighbour cell. Observation 3: Test requirement for each test can be updated via maintenance CR. Observation 4: Coordination from Chairman/MCC level would be expected for revision of clause numbers in TS 38.133. |
| R4-2106530 | OPPO | - draft CR reserved to update time offset configuration. - correct values of T2 in Test Requirements and clarification on SSB index for intra-frequency and inter-frequency measurement procedure |
| R4-2104740 | CATT | Draft CR on intra-frequency measurement in FR2: - Update the measurement delay in test requirement of CSI-RS based intra-frequency measurement in FR2 (in 5.6.4.1.2 and 7.6.5.1.2). ##### A.5.6.4.1.2 Test Requirements In the test, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than X ms from the beginning of time period T2, where X is - [2.0]s for a UE supporting power class 1, - [1.2]s for a UE supporting power class 2, 3 and 4 ##### A.7.6.5.1.2 Test Requirements In this test, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than X1 ms from the beginning of time period T2, where X1 is - [4.8]s for a UE supporting power class 1, - [2.88]s for a UE supporting power class 2, 3 and 4 The UE is required to read the neighbour cell SSB index in this test in order to detect associated SSB for the CSI-RS resource of Cell 2. |
| R4-2106413 | Nokia, Nokia Shanghai Bell | The test case for E-UTRAN-NR event triggered reporting without gaps in FR1 cells is updated. The new change is tracked using “RAN4#98bis-e”: - Changing the DRX configuration to DRX.5. - Setting timing offset between PCell and PSCell to 3us assuming synchronous EN-DC. - Updating the test requirements with the value in line with DRX.5. - The timing offset configuration between serving and neighbor cell will be updated based on the conclusion in RAN4#98bis-e. |
| R4-2107172 | Qualcomm CDMA Technologies | Draft CR on Intra-frequency measurement - Update the cell time offset within a CP in table A.5.6.4.1.1-2 - Remove the [TBD] in the test requirements in A.5.6.4.1.2 |
| R4-2106623 | vivo | Draft CR on inter-frequency measurement in FR2 - Specified test parameter time offset between serving and neighbour cells in test A.7.6.6.1 Time offset between serving and neighbour cells; us; Config 1; 0.6; Synchronous cells.. |
| R4-2104579 | MediaTek inc. | Update timing offset in test case for CSI-SINR in SA FR2 Table A.7.7.8.1.2-2: CSI-SINR Intra frequency test parameters Time offset with Cell 1; -; CP/2; -; CP/2. Table A.7.7.8.2.2-2: CSI-SINR Inter frequency general test parameters Time offset with Cell 1; -; CP/2; -; CP/2; -; CP/2. |
| R4-2106414 | Nokia, Nokia Shanghai Bell | Draft CR on TC7 of CSI-RSRP accuracy requirement in EN-DC in FR1 - This CR is reserved to update the timing offset configuration based on the discussion in RAN4#98bis-e |
| R4-2106621 | vivo | Draft CR to 38.133 on test cases for EN-DC CSI-SINR measurement accuracy - Specified test parameter timing offset with Cell 2 in test A.4.7.9.1 Time offset with Cell 2; Config 1,2,4,5; s; -; 4.7; -; -4.7. Config 3,6; s; -; 2.35; -; -2.35. - Specified test parameter timing offset with Cell 2 in test A.4.7.9.2 Time offset with Cell 2; Config 1,2, 4, 5; s; -; 4.7; -; 4.7; -; -4.7. Config 3, 6; s; -; 2.35; -; 2.35; -; -2.35. |
| R4-2106622 | vivo | Draft CR to 38.133 on CSI-RSRQ measurement accuracy for NR neighbor cell in FR2 - Specified test parameter timing offset with Cell 1 in test A.7.7.7.1 Time offset with Cell 1; us; -; 0.6; -; 0.6. - Specified test parameter timing offset with Cell 1 in test A.7.7.7.2 Time offset with Cell 1; us; -; 0.6; -; 0.6. |
| R4-2107026 | Huawei, HiSilicon | Draft CR to update TC3 and TC12 for CSI-RS accuracy test - Update timing offset between the serving and neighbor cell in the test setup to the side condition in accuracy requirements. Table A.6.7.11.1.2-2: CSI-SINR Intra frequency test parameters & Table A.6.7.11.2.2-2: CSI-SINR Inter frequency test parameters Time offset with Cell 1; Config 1; s; -; 4; -; 4; -; 4. Config 2,3; s; -; 2; -; 2; -; 2. Table A.5.7.8.1.2-2: CSI-SINR Intra frequency test parameters & Table A.5.7.8.2.2-2: CSI-SINR Inter frequency general test parameters Time offset with Cell 2; s; -; 0.5; -; 0.5. |
| R4-2107173 | Qualcomm CDMA Technologies | Draft test case of measurement performance for EN-DC CSI-RSRP measurement accuracy for NR neighbour cell in FR2 Remove the square brackets in tables Table A.5.7.6.1.2-2 and Table A.5.7.6.2.2-1 Time offset with Cell 2; s; -; <CP; -; <CP. Time offset between Cell 2 and Cell 3; 1~4; s; <CP; <CP. |

Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 1-1: Time offset configuration

- Proposals

  - Option 1: 1 CP for CSI-RSRP and CSI-RSRQ; CSI-SINR test setting pending on discussion in thread [212](Qualcomm)

  - Option 2: 0.9 CP (OPPO)

  - Option 3: 0.5 CP (MTK)

  - Option 4: Specified different time offsets as below (vivo)

    - A.7.6.6.1& A.7.7.7.1 & A.7.7.7.2:

      - 0.6 us

    - A.4.7.9.1 & A.4.7.9.2:

| Time offset with Cell 2 | Config 1,2,4,5 | s | - | 4.7 | - | -4.7 |
| --- | --- | --- | --- | --- | --- | --- |
|  | Config 3,6 | s | - | 2.35 | - | -2.35 |

  - Option 5: Specified different time offsets as below (Huawei)

    - Table A.6.7.11.1.2-2: CSI-SINR Intra frequency test parameters &

Table A.6.7.11.2.2-2: CSI-SINR Inter frequency test parameters

| Time offset with Cell 1 | Config 1 | s | - | 4 | - | 4 | - | 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Config 2,3 | s | - | 2 | - | 2 | - | 2 |

    - Table A.5.7.8.1.2-2: CSI-SINR Intra frequency test parameters &

Table A.5.7.8.2.2-2: CSI-SINR Inter frequency general test parameters

| Time offset with Cell 2 | s | - | 0.5 | - | 0.5 |
| --- | --- | --- | --- | --- | --- |

  - Option 6: based on the discussion in RAN4#98bis-e (Nokia)

- Recommended WF

  - Companies’ CR is reserved to update the timing offset configuration based on the discussion in RAN4#98bis-e

Sub-topic 1-2: Clarification on test requirements

[Moderator: the options are reorganized as below based on companies’ comments and GTW progress]

Q: Whether UE is required to read/report the neighbor cell SSB index in order to detect associated SSB for the CSI-RS resource of neighbor cell ?

- Option 1(OPPO): YES, for all tests in FR1 and FR2 the UE is required to read the neighbor cell SSB index, because the definitions of intra-f or inter-f measurement for SSB and CSI-RS are decoupled and UE is required to read inter-f SSB index.

- Option 2: clarify the test requirements case by case

  - For FR1 TDD and FR2 with intra-frequency measurement:

    - Option 2a(MTK, QC, CMCC, Nokia, CATT): No, UE is not required to report the SSB index for FR1 TDD and FR2, as agreed in section 9.2.6.2, ”It is assumed that deriveSSB-IndexFromCell is always enabled for FR1 TDD and FR2. ”

  - For intra-frequency FDD and/or inter-frequency test cases

    - Option 2b(CATT): For intra-frequency FDD and inter-frequency tests, both cases (UE is required or is not required to read SSB index) need to be defined.

    - Option 2c(CMCC): for inter-frequency measurement,if deriveSSB-IndexFromCell is set to true, it means UE need to read SSB index for only one cell, and no need to read SSB index for other cells on the same carrier. If deriveSSB-IndexFromCell is set to false, UE need to read SSB index for all the cells on that carrier.

    - Option 2d(MTK): if the inter-frequency measurement is conducted in a sync-ed network (i,e., deriveSSB-IndexFromCell), UE can skip SBI reading. For other cases (inter-freq or async network), time for SBI reading should be provided.

- Option 3(vivo): No need to clarify in tests as already defined in core requirements about how and whether SSB index is detected.

- Recommended WF

  - TBA

Sub-topic 1-3: Measurement delay on test requirements

- Proposal 1(CATT, OPPO):

  - Update the measurement delay in test requirement of CSI-RS based intra-frequency measurement in FR2 (in 5.6.4.1.2 and 7.6.5.1.2).

##### A.5.6.4.1.2 Test Requirements In the test, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than X ms from the beginning of time period T2, where X is - [2.0] s for a UE supporting power class 1, - [1.2] s for a UE supporting power class 2, 3 and 4 The UE is required to read the neighbour cell SSB index in this test in order to detect associated SSB for the CSI-RS resource of Cell 3.

##### A.7.6.5.1.2 Test Requirements In this test, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than X1 ms from the beginning of time period T2, where X1 is - [4.8] s for a UE supporting power class 1, - [2.88] s for a UE supporting power class 2, 3 and 4 The UE is required to read the neighbour cell SSB index in this test in order to detect associated SSB for the CSI-RS resource of Cell 2.

- Recommended WF

  - Option 1 is recommended

Companies views’ collection for 1st round

Open issues

|  | Companies' Comments |
| --- | --- |
| Sub-topic 1-1 | Sub-topic 1-1: Time offset configuration MTK: If RAN4 agrees 2 different timing offset values for RSRP/RSRQ and SINR, then we suggest to also specify 2 differernt values in different test cases Qualcomm: Suggest this topic be postponed and depend on the outcome of GTW. vivo: depending on conclusion in email thread 212. In the test cases timing offset with unit microseconds should be used. CATT: depending on the conclusion of performance part, suggest to discuss together with performance part in GTW. Nokia: If timing offset could be concluded in email thread [212], at least one test case shall be defined with a timing offset configuration within the threshold. OPPO: Agree with CATT to discuss together in GTW. CMCC: we are OK with option 1. And we agree that we can have further discussion in GTW. Huawei: suggest to follow the conclusions on the accuracy requirements from email #212. |
| Sub-topic 1-2 | - Sub-topic 1-2: Clarification on test requirements - MTK: More discussion may be needed. In our understanding, UE can skip SBI reading if the inter-frequency measurement is conducted in a sync-ed network (i,e., deriveSSB-IndexFromCell). For other cases (inter-freq or async network), time for SBI reading should be provided. - Qualcomm: In 9.2.6.2, ”It is assumed that deriveSSB-IndexFromCell is always enabled for FR1 TDD and FR2. ”, but note it is for intra-frequency. As for inter-frequency, can companies/moderator please confirm if deriveSSB-IndexFromCell shall be assumed to be False? For designing the test cases, we may follow above assumptions for intra and inter-frequency scenarios. - vivo: It is based on core requirements how and whether SSB index is detected. No need to further clarify in the test cases. - CATT: In 9.2.6.2, ”It is assumed that deriveSSB-IndexFromCell is always enabled for FR1 TDD and FR2. ”. So for intra-frequency measurement test cases, UE is not required to report the SSB index for FR1 TDD and FR2. For intra-frequency FDD and inter-frequency test case, both cases (UE is required or is not required to read SSB index) need to be defined. Nokia: We also understood SSB index reading is not applicable to FR2. This has been clearly stated in 38.133 section 9.2.5.1. - OPPO: In our understanding, the definitions of intra-f or inter-f measurement for SSB and CSI-RS are de-coupled, which can lead to the case that, even for intra-f (or inter-f) CSI-RS L3 measurement the associated SSB of neighbour cell may be an inter-f (or intra-f) SSB. As commented by companies above, SBI reading should be required. In addition, the agreement ’deriveSSB-IndexFromCell is always enabled for FR1 TDD and FR2’ applies for intra-f SSB measurement, not for inter-f SSB measurement. Thus, to be simplified, we propose option 1 for all tests in FR1/FR2. - BTW, other options are also reorganized as below for further discussion. - Option 1: Clarify that for all tests in FR1 (or FR2) - the UE is required to read the neighbour cell SSB index in order to detect associated SSB for the CSI-RS resource of neighbour cell. - Option 2: For intra-frequency measurement test cases, UE is not required to report the SSB index for FR1 TDD and FR2. For intra-frequency FDD and inter-frequency test case, both cases (UE is required or is not required to read SSB index) need to be defined. - Option 3: No need to clarify in tests as already defined in core requirements about how and whether SSB index is detected. CMCC: for FR1 TDD and FR2 with intra-frequency measurement, according to section 9.2.5.1 in TS38.133, no need to read SSB index. For inter-frequency measurement, according to TS38.331, the deriveSSB-IndexFromCell can be signalled in MO, which means UE may use the timing of any detected cell on that target frequency to derive the SSB index of all neighbour cells on that frequency. In our view, for inter-frequency measurement, if deriveSSB-IndexFromCell is set to true, it means UE need to read SSB index for only one cell, and no need to read SSB index for other cells on the same carrier. If deriveSSB-IndexFromCell is set to false, UE need to read SSB index for all the cells on that carrier. Huawei: we do not think any clarification on UE behaviour is needed in the test case, but the test requirements need to be calculated based on the core requirements. UE needs to know the SBI of the target cell, but it does not necessarily mean UE needs to read SBI. - If the SSB is intra-frequency, then depending on how deriveSSB-IndexFromCell is set in the test, UE may or may not need to read SBI. For FR1 TDD and FR2, deriveSSB-IndexFromCell should be always set to ‘true’, so no need for UE to read SBI. The question is how to set the parameter for FR1 FDD test. We do not have strong view here, but we do not think we need to test both cases (‘true’ and ‘false’). - If the SSB is inter-frequency, then no matter how deriveSSB-IndexFromCell is set, UE needs to read SBI, and the TSSB_time_index should be included in the test requirements. |
| Sub-topic 1-3 | - Sub-topic 1-3: Measurement delay on test requirements - company A: Qualcomm: Assume reading SSB index is not needed for FR2, for A.5.6.4.1.2 PC2 UE, we have TPSS/SSS_sync_intra=24*20=480 ms; TSSB_time_index_intra = 0ms and T CSI-RS_measurement_period_intra : Since 24*20=480>=400 400 *CSSF(i.e. 2)=800ms; So X=1.28s. Suggest this can hold when agreements are made for sub-topic1-2. CATT: Modify our proposal in A.5.6.4.1.2 as below: for A.5.6.4.1.2 PC1 UE(without gap and non-DRX), TPSS/SSS_sync_intra=max{600, 40*20}=800ms; TSSB_time_index_intra = 0ms and T CSI-RS_measurement_period_intra=max{400, 40*20}=800ms; so X=1.6s for A.5.6.4.1.2 PC2 UE(without gap and non-DRX), TPSS/SSS_sync_intra=max{600, 24*20}=600ms; TSSB_time_index_intra = 0ms and T CSI-RS_measurement_period_intra=max{400, 24*20}=480ms; so X=1.08s Keep the proposal in A.7.6.5.1.2： for A.7.6.5.1.2 PC2 UE(without gap and DRX cycle=40ms), TPSS/SSS_sync_intra=max{600, 1.5*24*40}=1440ms; TSSB_time_index_intra = 0ms and T CSI-RS_measurement_period_intra=max{400, 1.5*24*40}=1440ms; so X=2.88s Nokia: We also understood SSB index reading is not applicable to FR2. This has been clearly stated in 38.133 section 9.2.5.1. - OPPO: Companies are encouraged to double check the delay of their CRs if any conclusion were achieved. QC’s proposal looks fine to me as well. Huawei: We think CATT’s calculation for TPSS/SSS_sync_intra and T CSI-RS_measurement_period_intra is correct, but CSSF=2 is missing as mentioned in QC calculation. |

CRs/TPs comments collection

Major close-to-finalize WIs and Rel-16 maintenance, comments collections can be arranged for TPs and CRs. For Rel-17 on-going WIs, suggest to focus on open issues discussion on 1st round.

Intra-frequency measurement [NR_CSIRS_L3meas-Perf]

| CR | Title | Comments collection |
| --- | --- | --- |
| R4-2106530 OPPO | Updated CR on CSI-RS based L3 measurement RRM test cases | Company A: Company B: |
| R4-2104740 CATT | draft CR on test case for intra-frequency CSI-RS based measurement | Company A: Company B: |
| R4-2106413 Nokia, Nokia Shanghai Bell | 38.133 CR on the test case of EN-DC event triggered reporting for intra-frequency CSI-RS based measurements in FR1 | Anritsu: Is it assumed that CSI-RSRP is at the same power as normal SS-RSRP? The values in Table A.4.6.6.1.2-3 for Cell 3 during T1 appear to be inconsistent. Nokia: Thanks for the good comments. We have assumed the UE has been monitoring associatedSSB in T1, hence the SS-RSRP is not -infinite in cell 3 during T1. But CSI-RS can be measured only after T2 in cell 3. With this assumption, the measurement delay does not account the SSB index reading time. CATT: The DRX has been updated in the big CR R4-2101533. The draft CR should be based on the endorsed big CR. Nokia: Thanks for the comments. Will check the endorsed big CR for alignment. |
| R4-2107172 Qualcomm | Draft test case of CSI-RS based intra-frequency test for EN-DC event triggered reporting tests without gap for NR neighbor cell in FR2 | Company A: Company B: |

Inter-frequency measurement [NR_CSIRS_L3meas-Perf]

| R4-2106623 vivo | Draft CR to 38.133 on SA event triggered reporting tests with gap for NR neighbor cell in FR2 | CATT: The draft CR should be based on the endorsed big CR R4-2101533 Company B: |
| --- | --- | --- |

Measurement performance [NR_CSIRS_L3meas-Perf]

| R4-2104579 MediaTek inc. | Draft CR to update timing offset in test case for CSI-SINR in SA FR2 | Company A: Company B: |
| --- | --- | --- |
| R4-2106414 Nokia, Nokia Shanghai Bell | 38.133 CR on the TC of CSI-RSRP accuracy requirement in EN-DC in FR1 | Company A: Company B: |
| R4-2106621 vivo | Draft CR to 38.133 on test cases for EN-DC CSI-SINR measurement accuracy | Company A: Company B: |
| R4-2106622 vivo | Draft CR to 38.133 on CSI-RSRQ measurement accuracy for NR neighbor cell in FR2 | Company A: Company B: |
| R4-2107026 Huawei, HiSilicon | draft CR to update TC3 and TC12 for CSI-RS accuracy test | Company A: Company B: |
| R4-2107173 Qualcomm CDMA Technologies | Draft test case of measurement performance for EN-DC CSI-RSRP measurement accuracy for NR neighbor cell in FR2 | Company A: Company B: |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1-1 | Sub-topic 1-1: Time offset configuration Tentative agreements: N/A Candidate options: - Option 1: 1 CP for CSI-RSRP and CSI-RSRQ; CSI-SINR test setting pending on discussion in thread [212](Qualcomm) - Option 2: 0.9 CP (OPPO) - Option 3: 0.5 CP (MTK) - Option 4: Specified different time offsets as below (vivo) - A.7.6.6.1& A.7.7.7.1 & A.7.7.7.2: - 0.6 us - A.4.7.9.1 & A.4.7.9.2: Time offset with Cell 2; Config 1,2,4,5; s; -; 4.7; -; -4.7. Config 3,6; s; -; 2.35; -; -2.35. - Option 5: Specified different time offsets as below (Huawei) - Table A.6.7.11.1.2-2: CSI-SINR Intra frequency test parameters & Table A.6.7.11.2.2-2: CSI-SINR Inter frequency test parameters Time offset with Cell 1; Config 1; s; -; 4; -; 4; -; 4. Config 2,3; s; -; 2; -; 2; -; 2. - Table A.5.7.8.1.2-2: CSI-SINR Intra frequency test parameters & Table A.5.7.8.2.2-2: CSI-SINR Inter frequency general test parameters Time offset with Cell 2; s; -; 0.5; -; 0.5. - Option 6: based on the discussion in RAN4#98bis-e (Nokia) Recommendations for 2nd round: Pending on the discussion of email thread [212]. |
| Sub-topic#1-2 | Sub-topic 1-2: Clarification on test requirements Tentative agreements: - No more clarification on UE behavior is needed in the test case, but the test requirements need to be calculated based on the core requirements. Candidate options: Q: Whether UE is required to read/report the neighbour cell SSB index in order to detect associated SSB for the CSI-RS resource of neighbour cell ? - Option 2: clarify the test requirements case by case - For FR1 TDD and FR2 with intra-frequency measurement: - Option 2a(MTK, QC, CMCC, Nokia, CATT): No, UE is not required to report the SSB index for FR1 TDD and FR2, as agreed in section 9.2.6.2, ”It is assumed that deriveSSB-IndexFromCell is always enabled for FR1 TDD and FR2. ” - For intra-frequency FDD and/or inter-frequency test cases - Option 2b(CATT): For intra-frequency FDD and inter-frequency tests, both cases (UE is required or is not required to read SSB index) need to be defined. - Option 2c(CMCC): for inter-frequency measurement, if deriveSSB-IndexFromCell is set to true, it means UE need to read SSB index for only one cell, and no need to read SSB index for other cells on the same carrier. If deriveSSB-IndexFromCell is set to false, UE need to read SSB index for all the cells on that carrier. - Option 2d(MTK): if the inter-frequency measurement is conducted in a sync-ed network (i,e., deriveSSB-IndexFromCell), UE can skip SBI reading. For other cases (inter-freq or async network), time for SBI reading should be provided. - Option 3(vivo): No need to clarify in tests as already defined in core requirements about how and whether SSB index is detected. - Option 4(Huawei): - UE needs to know the SBI of the target cell, but it does not necessarily mean UE needs to read SBI. - No more clarification on UE behaviour is needed in the test case, but the test requirements need to be calculated based on the core requirements. Recommendations for 2nd round: - The agreements should be captured in the WF. The test requirements need to be calculated based on the core requirements. - Otherwise, need more discussion (if no agreement was achieved in 1st round). |
| Sub-topic#1-3 | Sub-topic 1-3: Measurement delay on test requirements Tentative agreements: N/A Candidate options: For A.5.6.4.1.2, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than X ms from the beginning of time period T2, where X is: - Option 1(CATT): - [1.6] s for a UE supporting power class 1, - [1.08] s for a UE supporting power class 2, 3 and 4 - Option 2(QC): - [2] s for a UE supporting power class 1, - [1.4] s for a UE supporting power class 2, 3 and 4 For A.7.6.5.1.2, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than X ms from the beginning of time period T2, where X is: - Option 1(CATT): - [4.8] s for a UE supporting power class 1, - [2.88] s for a UE supporting power class 2, 3 and 4 Recommendations for 2nd round: Need more discussion. Companies are encouraged to double check the delay of their CRs. |

Recommendations on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Discussion on 2nd round (if applicable)

Sub-topic 1-1: time offset configuration

Moderator: In the tests, time offset can follow the maximum delta_T agreed in email thread [212], as below.

- Specify CSI-RSRP accuracy requirement with the absolute timing offset between the reference measurement timing and the target CSI-RS in one layer no larger than CP

- Specify CSI-RSRQ accuracy requirement with the absolute timing offset between the reference measurement timing and the target CSI-RS in one layer no larger than CP.

- Specify CSI-SINR accuracy requirement based on one of the following options on timing offset between the reference measurement timing and the target CSI-RS (TΔ) and Es/Iot side condition(s)

    - Side condition #1:

      - |TΔ|≤ CP/2

    - FFS: Side condition #2

      - |TΔ|≤ CP

      - No dedicated test cases will be introduced for Side condition #2 if introduced

      - Side condition #2 is subject to decision in RAN4 #98-bis-e

Q1: How to define time offset

- Option 1: Time offsets follow the maximum delta_T agreed in email thread [212]

- Time offset for CSI-RSRP and CSI-RSRQ accuracy requirement: CP

- Time offset for CSI-SINR accuracy requirement: CP/2

Q2: The unit of time offset

- Option 1: use absolute time, e.g., s

- Option 2: use CP as unit

Recommended WF:

- For Q1:  Option 1 is recommended.

- For Q2:  All time offset in tests should be aligned with same unit regardless of whichever option.

| Sub-topic 1-1 | Companies' Comments |
| --- | --- |
| MTK | Q1: Option 1 is agreeable. Q2: No strong view, but Option 2 is slightly preferred. Since one test case may have multiple test configurations for different SCS. Perhaps using CP as the unit is simpler and more generic. |
| vivo | Q1: Option 1 is agreeable to us. Q2: Option 1. Timing offset should be specified in units of microseconds so that it is clearer to RAN5. |
| OPPO | Q1: Option 1 is agreeable. Q2: Either Option 1 or option 2 is fine, but it is better that all TCs are aligned. |
| CMCC | Q1: we are OK with option 1 |
| CATT | Q1: support option 1. Q2: no strong view. |
| Nokia | Q1: support option 1. Q2: Slightly prefer Option 1 to be clearer on the test parameters. |
| Huawei | Q1: Option 1 is fine. Q2: slightly prefer option 1, to align with existing test cases. |

Sub-topic 1-2: closed

Tentative agreements:  To be captured in WF.

- No more clarification on UE behavior is needed in the test case. The test requirements need to be calculated based on the core requirements.

Sub-topic 1-3: Measurement delay on test requirements

Candidate options:

Case 1: For A.5.6.4.1.2, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than X ms from the beginning of time period T2, where X is:

- Option 1(CATT):

-	[1.6] s for a UE supporting power class 1,

-	[1.08] s for a UE supporting power class 2, 3 and 4

- Option 2(QC):

-	[2] s for a UE supporting power class 1,

-	[1.4] s for a UE supporting power class 2, 3 and 4

Case 2: For A.7.6.5.1.2, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than X ms from the beginning of time period T2, where X is:

- Option 1(CATT):

-	[4.8] s for a UE supporting power class 1,

-	[2.88] s for a UE supporting power class 2, 3 and 4

Recommended WF:

Need more discussion the two cases. Companies are encouraged to double check the delay of their CRs.

| Sub-topic 1-3 | Companies' Comments |
| --- | --- |
| OPPO | Case 1: Option 2 is preferred considering CSSF. Case 2: option 1 is fine without counting time of SSB detection for FR2 intra-frequency measurement. |
| CATT | The measurement delay is modified in CR revision considering CSSF = 2. |

CRs/TPs comments collection on 2nd round

| R4-2106413 Nokia, Nokia Shanghai Bell |  | 38.133 CR on the test case of EN-DC event triggered reporting for intra-frequency CSI-RS based measurements in FR1 |  | Anritsu: following the comments from Nokia in the 1st round In RRM test cases, the Noc and Es/Noc for each cell in each time period are the original key parameters (the minimum necessary to define what the test system transmits), and all other parameters such as Es/Iot, SS-RSRP and Io are derived from them. So if the intention is that for Cell 3 during T1, SS-RSRP is -94dBm/15kHz and CSI-RSRP is -∞dBm/15kHz, that means that Es/Noc for SSB_RP and for CSI_RP are different (the Noc remains at -98dBm/15kHz in all REs). We need to find a clear way to specify this. For example, CSI-RS-based beam failure detection and link recovery test cases such as A.4.5.5.3 refer to “SNR_CSI-RS of set q0“, in Table A.4.5.5.3.1-3: In A.4.5.5.3 we are only interested in CSI-RS_RP (nothing about SSB_RP). I’m not sure if we have existing test cases with both RS and CSI-RS, and different values. Do Nokia know of any, where we could follow an example? By the way, test cases should specify CSI-RS_RP, which is an applied value. CSI-RSRP is a reported value, subject to UE reporting accuracy, so the test system can’t control it directly. Many RRM test cases have this error. Nokia: Probably I can explain our understanding on this TC. For CSI-RS based measurement, there is requirement the UE is expected to monitor CSI-RS only after associatedSSB is detected. That is why in this TC, we have both SSB (i.e. associatedSSB) and CSI-RS. So to reflect this measurement behavior, we may have two alternatives: Alternative 1: During time duration T1, the UE does not have any timing information of Cell 3. From T2, the UE detects associatedSSB and then measure CSI-RS. In this case, SS-RSRP and CSI-RSRP are both -∞dBm/15kHz during T1, and they become -94dBm/15kHz in T2. As a result, in the test requirement, The UE is required to read the SSB index. Alternative 2: During time duration T1, the UE has detected associatedSSB. From T2, it starts measuring CSI-RS in T2 In this case, during T1, SS-RSRP is -94dBm/15kHz and CSI-RSRP is -∞dBm/15kHz so that the UE can detect associatedSSB. And in T2, SS-RSRP and CSI-RSRP are -94dBm/15kHz for CSI-RS measurements. As a result, in the test requirement, The UE is NOT required to read the SSB index. We think either of the alternative can be used for the TC, just the UE behaviour and SS-RSRP/CSI-RSRP setting needs to be consistent. But I am not sure which alternative is easy or even feasible from test point of view? Is there any concern from you side to set both SS-RSRP and CSI-RSRP in one test? Anritsu: From viewpoint of implementation easiness, both implementation cost of alternative 1 and 2 is same, since the test equipment can control channel power of SSB/CSIRS independently on subframe basis. From viewpoint of test purpose, I think Alternative 2 is better. Alternative 2 can evaluate only CSIRS measurement and reporting delay, excluding detection time of PSS/SSS of associated SSB and SSB index reading time (to find associated SSB). SSB detection delay can be tested by A.4.6.1.x. OPPO: As agreed in 1st round, no more clarification on UE behavior is needed in the test case. The test requirements need to be calculated based on the core requirements. In our view, for this specific CR on tests of FR1 intra-frequency CSI-RS measurement, the SSB detection delay can be skipped assuming serving cell timing as reference or deriveSSB-IndexFromCell set to ‘true’. Otherwise, UE needs to read SBI. The general rules provided by Huawei may be referred for all TCs about whether to read the SBI of the target cell: - If the SSB is intra-frequency, then depending on how deriveSSB-IndexFromCell is set in the test, UE may or may not need to read SBI. For FR1 TDD and FR2, deriveSSB-IndexFromCell should be always set to ‘true’, so no need for UE to read SBI. The question is how to set the parameter for FR1 FDD test. We do not have strong view here, but we do not think we need to test both cases (‘true’ and ‘false’). - If the SSB is inter-frequency, then no matter how deriveSSB-IndexFromCell is set, UE needs to read SBI, and the TSSB_time_index should be included in the test requirements. |  |
| --- | --- | --- | --- | --- | --- |
|  |  | CATT: for the test procedure, we are fine with either alternative. But for alternative 2, we are wondering whether the different value for SS-RSRP and CSI-RSRP in T1 is reasonable. Since we think the signals for the same cell should be added at the same time. As for SSB index reading, we are fine with OPPO’s suggestion. For FR1 FDD test, we are fine to test one case and we think ‘deriveSSB-IndexFromCell’ can also be set ‘TRUE’ as same as TDD test. As a result, the intra-frequency requirements are tested without SSB index reading and inter-frequency requirements are tested with SSB index reading. |
|  |  | Huawei: agree with CATT, i.e. to set deriveSSB-IndexFromCell as ‘true’ for all test configurations for intra-frequency tests. |

Recommendations for Tdocs

1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on CSI-RS based L3 measurement performance requirements and tests | CATT, OPPO | It can be merged with WF of [212] |
| LS on … | ZZZ | To: RAN_X; Cc: RAN_Y |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-210xxxx | CR on … | XXX |  |  |
| R4-2106529 | Discussion on remaining issues for CSI-RS L3 measurement tests | OPPO | Noted |  |
| R4-2104740 | draft CR on test case for intra-frequency CSI-RS based measurement | CATT | Revised | Update time offset based on conclusion of [211] and check test requirements |
| R4-2106413 | 38.133 CR on the test case of EN-DC event triggered reporting for intra-frequency CSI-RS based measurements in FR1 | Nokia, Nokia Shanghai Bell | Revised | Check the endorsed big CR for alignment. Update time offset based on conclusion of [211] and check test requirements |
| R4-2106530 | Updated CR on CSI-RS based L3 measurement RRM test cases | OPPO | Revised | It is reserved for draft big CR |
| R4-2107172 | Draft test case of CSI-RS based intra-frequency test for EN-DC event triggered reporting tests without gap for NR neighbor cell in FR2 | Qualcomm CDMA Technologies | Revised | Update time offset based on conclusion of [211] and check test requirements |
| R4-2106623 | Draft CR to 38.133 on SA event triggered reporting tests with gap for NR neighbor cell in FR2 | vivo | Revised | check the endorsed big CR for alignment. Update time offset based on conclusion of [211] and check test requirements |
| R4-2104579 | Draft CR to update timing offset in test case for CSI-SINR in SA FR2 | MediaTek inc. | Merged or Postponed | It can be merged into draft big CR, if any conclusion was achieved on time offset. Otherwise, suggest to be Postponed |
| R4-2106414 | 38.133 CR on the TC of CSI-RSRP accuracy requirement in EN-DC in FR1 | Nokia, Nokia Shanghai Bell | Merged or Postponed | Similar to the above one. |
| R4-2106621 | Draft CR to 38.133 on test cases for EN-DC CSI-SINR measurement accuracy | vivo | Merged or Postponed | Similar to the above one. |
| R4-2106622 | Draft CR to 38.133 on CSI-RSRQ measurement accuracy for NR neighbor cell in FR2 | vivo | Merged or Postponed | Similar to the above one. |
| R4-2107026 | draft CR to update TC3 and TC12 for CSI-RS accuracy test | Huawei, HiSilicon | Merged or Postponed | Similar to the above one. |
| R4-2107173 | Draft test case of measurement performance for EN-DC CSI-RSRP measurement accuracy for NR neighbor cell in FR2 | Qualcomm CDMA Technologies | Merged or Postponed | Similar to the above one. |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2105779 | draft CR on test case for intra-frequency CSI-RS based measurement | CATT | Agreeable |  |
| R4-2105780 | 38.133 CR on the test case of EN-DC event triggered reporting for intra-frequency CSI-RS based measurements in FR1 | Nokia, Nokia Shanghai Bell | Agreeable |  |
| R4-2105781 | Updated CR on CSI-RS based L3 measurement RRM test cases | OPPO | Agreeable |  |
| R4-2105782 | Draft test case of CSI-RS based intra-frequency test for EN-DC event triggered reporting tests without gap for NR neighbor cell in FR2 | Qualcomm CDMA Technologies | Agreeable |  |
| R4-2105783 | Draft CR to 38.133 on SA event triggered reporting tests with gap for NR neighbor cell in FR2 | vivo | Agreeable |  |
| R4-2104579 | Draft CR to update timing offset in test case for CSI-SINR in SA FR2 | MediaTek inc. | Merged | Merged into R4-2105781. |
| R4-2106414 | 38.133 CR on the TC of CSI-RSRP accuracy requirement in EN-DC in FR1 | Nokia, Nokia Shanghai Bell | Merged | Merged into R4-2105781. |
| R4-2106621 | Draft CR to 38.133 on test cases for EN-DC CSI-SINR measurement accuracy | vivo | Merged | Merged into R4-2105781. |
| R4-2106622 | Draft CR to 38.133 on CSI-RSRQ measurement accuracy for NR neighbor cell in FR2 | vivo | Merged | Merged into R4-2105781. |
| R4-2107026 | draft CR to update TC3 and TC12 for CSI-RS accuracy test | Huawei, HiSilicon | Merged | Merged into R4-2105781. |
| R4-2107173 | Draft test case of measurement performance for EN-DC CSI-RSRP measurement accuracy for NR neighbor cell in FR2 | Qualcomm CDMA Technologies | Merged | Merged into R4-2105781. |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents