3GPP TSG-RAN WG4 Meeting # 98-bis-e 												R4-2105989

Electronic Meeting, 12th – 20th April, 2021

Agenda item:			5.2.1.3

Source:	Moderator (Intel Corporation)

Title:	Email discussion summary for [98-bis-e][318] V2X_Demod_Part2

Document for:	Information

# Introduction

The scope of this email thread is Rel-16 V2X multi-link performance requirements.

Email discussion targets for the 1st round and 2nd round

- 1st round:

  - Discussion of open issues for all considered tests.

  - Collection of comments for Draft CRs

- 2nd round:

  - WF preparation

  - Collection of comments for Updated Draft CRs.

# Topic #1: Power imbalance test

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2104902 | Qualcomm Inc. | Proposal 3: Set 2 DMRS symbols for power imbalance test for all slots. |
| R4-2104998 | LGE | Simulation results |
| R4-2104774 | CATT, GOHIGH | Simulation results |
| R4-2106422 | Intel Corporation | Simulation results |
| R4-2106801 | Huawei, HiSilicon | Simulation results |
| R4-2106802 | Huawei, HiSilicon | Rel-16 CR for 38.101-4 with following changes: - Added the requirments for power imbalance with two links. - Added the RMC for power imbalance with two links. - Added the related resource pool. |

## Open issues summary

Issue 1-1: Number of DMRS symbols

- Proposals

  - Option 1 (QC): Set 2 DMRS symbols for all slots

  - Option 2 (previous agreement): Use 2 DMRS symbols for slots with PSFCH and 3 DMRS symbols for slots without PSFCH

- Recommended WF

  - Check companies view on options above

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| XXX | Issue 1-1: Number of DMRS symbols |
| Intel | Issue 1-1: Number of DMRS symbols Both options are fine for us. Option 1 looks more suitable for static propagation conditions. In this meeting we can check alignment results for Option 2 and define requirements in []. If Option 1 will be agreed, then in the next meeting companies can prepare new results and we can update the requirements. |
| QC | Issue 1-1: Number of DMRS symbols We proposed option 1 because it is more reasonable for the given propagation condition. However, we are open to discuss and see if other companies have any concern of revising the previous agreement. |
| LG | Issue 1-1: Number of DMRS symbols We prefer to keep the previous agreement as option 2 if there is no technical issue. But we are fine with Intel’s comments. |
| Huawei, HiSilicon | Issue 1-1: Number of DMRS symbols We prefer to keep the previous agreements as option 2, since the allocated RBs are 10 and first DMRS symbol does not exist and the actual number of DMRS symbol is 2. |
| CATT | Issue 1-1: Number of DMRS symbols We support the previous agreement. |

------------------GTW discussion--------------

Intel: We can define requirements with option 2 assumption in [ ], if option 1 agreed, new results can be collected and update the requirements if any.

Huawei: We prefer to keep previous agreements if no technical issues identified. We expect no much performance difference among option 1 and option 2.

QC: We think option 2 more realistic. And we also agree previous agreement also works well from technical aspect.

Intel: We have impact on TBS calculation. We slightly prefer option 1 with less overhead.

LGE: We have same view as Huawei. We have only one meeting left, we prefer to keep the agreements if no technical issues.

### CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2106802 | Intel: Test parameters structure, RMC and resource pool can be updated based on discussion of issues in Topic #2 of e-mail thread [98-bis-e][317] V2X_Demod_Part1. |
|  | LG: According to Big CR approach in RAN4, draft CR should be based on endorsed Big draft CR in the previous meeting. So, it is better to submit the draft CR with track change based on the endorsed Big draft CR. Generally we are fine, but the requirement and resource pool configuration need to be revised based on conclusion of corresponding discussion. |
|  | Huawei, HiSilicon: Formal CR was submitted for this meeting that should be created based on the current latest specification, if we created it based on the endorsed Big draft CR with track changes, it will be inconsistent with 3GPP drafting rule, this is the reason that we use change mark for the whole CR. Further revision can be done as per the conclusion about the test parameters table, RMC and resource pool configuration. |

## Summary for 1st round

### Open issues

Status summary

Issue 1-1: Number of DMRS symbols GTW agreement: - Option 2: Use 2 DMRS symbols for slots with PSFCH and 3 DMRS symbols for slots without PSFCH

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

# Topic #2: HARQ soft buffer combing test

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2104902 | Qualcomm Inc. | Proposal 2: Use Table 1 FRC for HARQ buffer test. |
| R4-2104903 | Qualcomm Inc. | Placeholder for Draft CR on HARQ buffer test |
| R4-2106423 | Intel Corporation | Observation: No performance difference is observed between cases with CFO 600 Hz/ TO 12 Ts and CFO 0 Hz/ TO 0 Ts for scenarios with HARQ combining functionality. |
| R4-2106803 | Huawei, HiSilicon | Observation 1: 0.93 dB performance gap caused by ICI can be observed between the test with/without 0.1PPM CFO and 12Ts CTO. Proposal 1: Define requirements for soft buffer test considering 0.1PPM CFO and 12Ts CTO at RX side. |

## Open issues summary

Issue 2-1: FRC

- Proposals

  - Option 1 (QC): Use FRC from Table 1 in R4-2104902

  - Option 2 (previous agreement): Use FRC from Table 2.2-2 in R4-2103993

- Way forward

  - Collect companies views on options above with detailed calculation

Issue 2-2: RX assumptions for requirements definition

- Proposals

  - Option 1 (Huawei): Define requirements for soft buffer test considering 0.1PPM CFO and 12Ts CTO at RX side.

- Way forward

  - Collect companies views on option above

Issue 2-3 (New): MCS selection

- Option 1: MSC 27 (QC)

- Option 2: MCS 28 (previous agreement)

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| XXX | Issue 2-1: FRC Issue 2-2: RX assumptions for requirements definition |
| Intel | Issue 2-1: FRC Based previous meetings agreements, PSFCH periodicity 1 will be considered for requirements definition. Therefore, Option 1 is not aligned with previous agreement, because it provides TBS information for slots with and without PSFCH. Same time, our calculations are not aligned with Option 1 or Option 2. NRE'=NscRBNsymbsh-NsymbPSFCH-NohPRB-NREDMRS=12*9-0-12=96 NRE=NRE'∙nPRB-NRESCI,1-NRESCI,2=96*10-240-80=640 NRESCI,1=12*10*2=240 NRESCI,2=OSCI2+LSCI2∙βoffsetSCI2QmSCI2∙R=35+24∙2.52∙948/1024=80 Ninfo=NRE*R*Qm*v=640*9481024*6*1=3555 Ninfo'=2nNinfo/2n=3552 TBS = 3624 We suggest companies to double check the calculation to understand the correct configuration Issue 2-2: RX assumptions for requirements definition Based on agreement from previous meeting, companies can bring analysis for alignment purpose with and without RX CFO/TO modeling. Same time, requirements are defined based on impairments results and it is up to each company which assumption on Rx CFO/TO tacking accuracy is taken into account. Therefore, we think that we do not need to make any agreement of Rx assumptions for requirements definition. |
| QC | Issue 2-1 To Intel’s comment: we use beta = 3.5, hence your calculate is correct, we will update the CR accordingly. PSSCH bits are updated to 3816, too. The no PSFCH part will be removed, too. Issue 2-2 Based on our understanding to Huawei’s proposal, the assumption is for simulation results based on UE model without complete FTL and TTL model. Therefore, we propose the following simulation assumption: For simulations without complete FTL and TTL modeling, use the assumption of 0.1PPM CFO and 12Ts CTO. 0.1ppm CFO is from RAN4 Tx requirement and 12Ts is from RAN4 transmission timing error requirement. Both makes sense to us. |
| Huawei | Issue 2-1 Agree with Intel’s calculation. Issue 2-2: RX assumptions for requirements definition Our understanding is that if the results are not aligned, CFO/CTO may be an important factor affecting the performance. But according to the companies’ simulation results, the results are aligned, we agree that we don’t need to make any agreement of RX assumptions and the affection of CFO/CTO can be up to each company on how to handle them, either by complete FTL/TTL modelling or just directly consider 0.1ppm CFO and 12Ts in the simulation as pointed by QC. |
| QC | Issue 2-1 We checked the code rate with Intel’s calculation, found that it is over 0.95. However, the code rate should be within 0.93. MCS code rate is <0.93 only when number of PSSCH RBs > 40. Since we use 20MHz, we don’t have that much RBs. Therefore, we propose to reduce MCS to MCS 27 to avoid code rate violation. |

--------------------GTW discussion on Issue 2-3--------------

New issue: MCS selection

-Option 1: MSC 27 (QC)

-Option 2: MCS 28 (previous agreement)

QC: We have different calculate method compared to Intel.  We propose to first agree coding rate less 0.95 and further check the calculation among companies.

Huawei: We have double check the coding rate and aligned with QC calculation. 0.95 coding rate captured in core specification. We are fine to change MCS as MCS27. TBS calculation from Intel is correct, for channel bits, the gamma values should be 4 other than 0.

Intel: We are ok to change as MCS 27.

MTK: We also double check coding rate and share similar observation as QC and prefer MCS 27.

### CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2104903 | Intel: Please check our suggestions on editorial corrections in file (draft)R4-2104903 CR_V2X_HARQ_98ebis_Intel.docx in folder Drafts. Test parameters structure and RMC can be updated based on discussion of issues in Topic #2 of e-mail thread [98-bis-e][317] V2X_Demod_Part1. |
|  | QC: Thanks for Intel’s suggestion, we uploaded (draft)R4-2104903 CR_V2X_HARQ_98ebis_v2.docx based on Intel’s version and correct the TBS and coded bits according to comment from issue 2-1. |

## Summary for 1st round

### Open issues

Status summary

Issue 2-1: FRC - Tentative agreements: N/A - Recommendations for 2nd round: Continue discussion in the 2nd round for new MCS. This issue will be handled as a part of Draft CR R4-2104903 discussion Issue 2-2: RX assumptions for requirements definition - Tentative agreements: No need any agreement on RX CFO and TO assumptions for requirements definition Issue 2-3 (New): MCS selection - GTW agreement: Use MCS 27

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

# Topic #3: PSFCH decoding capability test

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2104998 | LGE | Simulation results |
| R4-2104774 | CATT, GOHIGH | Simulation results |
| R4-2106804 | Huawei, HiSilicon | Observation 1: No feedback can be used for ACK/NACK miss statistics in V2X PSFCH decoding capability/performance test so AT command may not be avoid. Proposal 2: Always let tested UE to transmit PSSCH to enable it to detect related PSFCHs. Meanwhile, it is not necessary for TE to decode the PSSCH to transmit ACK or NACK randomly (For decoding capability test) or only NACK (For performance test). Proposal 3: Test the PSFCH decoding capability in group cast scenario. Proposal 4: Use following test setup and parameter configurations for PSFCH decoding capability test: - Test configurations: - PSSCH allocation: 10 subchannels. - Sl-PSFCH-CandidateResourceType-r16: allocSubCH - Sl-PSFCH-RB-Set-r16: 100. - PSFCH periodicity: 1 slot - Source ID for tested UE: 0 - Member ID for feedback UE: 0 for UE 0, 1 for UE 1,…,N-1 for UE N-1 and N equals to PSFCH-RX-Number. - Sl-MinTimeGapPSFCH-r16: 2 - Test setup: - The slots are divided two part: PSFCH transmission slots and DTX occasion slots. In slots for PSFCH transmission, all feedback UEs transmit ACKs or NACKs randomly to tested UE. In slots for DTX occasions, all receiving UEs transmit nothing to the tested UE. The slots for DTX occasions are random. - For PSSCH is scheduled in slot n, tested UE detect the corresponding PSFCHs in PSFCH resources as specified in our proposed configurations in slot n+2. - Test metric: - The DTX to ACK probability shall not exceed 1%: - The ACK miss probability shall not exceed 1%. |
| R4-2106805 | Huawei, HiSilicon | Rel-16 CR for 38.101-4 with following changes: - Added the requirements and test setup for NR V2X PSFCH decoding capability test - Added the related resource pool. |

## Open issues summary

Issue 3-1: Assumptions on PSSCH transmission from tested UE

- Proposals

  - Option 1 (HW): Always let tested UE to transmit PSSCH to enable it to detect related PSFCHs. Meanwhile, it is not necessary for TE to decode the PSSCH to transmit ACK or NACK randomly (For decoding capability test) or only NACK (For performance test).

- Way forward

  - Collect companies views on option above

Issue 3-2: Scenario for test

- Proposals

  - Option 1 (Huawei): Test the PSFCH decoding capability in group cast scenario.

- Way forward

  - Collect companies views on option above

Issue 3-3: Details of Test configuration

- Proposals

  - Option 1 (Huawei):

    - PSSCH allocation:  10 subchannels.

    - Sl-PSFCH-CandidateResourceType-r16: allocSubCH

    - Sl-PSFCH-RB-Set-r16: 100.

    - PSFCH periodicity: 1 slot

    - Source ID for tested UE: 0

    - Member ID for feedback UE: 0 for UE 0, 1 for UE 1,…,N-1 for UE N-1 and N equals to PSFCH-RX-Number.

    - Sl-MinTimeGapPSFCH-r16: 2

- Way forward

  - Collect companies views on option above

Issue 3-4: Details of Test setup

- Proposals

  - Option 1 (Huawei):

    - The slots are divided two part: PSFCH transmission slots and DTX occasion slots. In slots for PSFCH transmission, all feedback UEs transmit ACKs or NACKs randomly to tested UE. In slots for DTX occasions, all receiving UEs transmit nothing to the tested UE. The slots for DTX occasions are random.

    - For PSSCH is scheduled in slot n, tested UE detect the corresponding PSFCHs in PSFCH resources as specified in our proposed configurations in slot n+2.

  - Option 2: (Huawei) TE randomly transmit ACK or DTX on each PSFCH resource.

- Way forward

  - Collect companies views on option above

Issue 3-5: Test Method

Background: All other test cases are conducted by collecting the statistic results of HARQ ACK/NACK feedback transmitted by PSFCH except the PSBCH that is not testable, but how to test PSFCH is unclear without further accompanying feedback.

- Proposals

  - Option 1: AT command

  - Option 2: Other methods

- Way forward

  - Collect companies views on how to calculate the test results of PSFCH single link and multi-link testing

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| XXX | Issue 3-1: Assumptions on PSSCH transmission from tested UE Issue 3-2: Scenario for test Issue 3-3: Details of Test configuration Issue 3-4: Details of Test setup Issue 3-5: Test Method |
| Intel | Issue 3-1: Assumptions on PSSCH transmission from tested UE Proposal is fine for us. Issue 3-2: Scenario for test It’s not clear how this issue affects the requirements definition and whether we need to define any restrictions for requirements. More clarifications are needed. Issue 3-3: Details of Test configuration Option 1 is fine for us Issue 3-4: Details of Test setup More clarifications on Option 1 is needed: What is the purpose to introduce the DTX occasions in the test? Issue 3-5: Test Method If TE will always generate NACK message in the PSFCH then we can count number of PSSCH retransmissions, because if UE detects PSFCH and decode is as NACK message then it will generate PSSCH retransmission. Same time, in this test we consider random ACK/NACK message generation. Therefore, based on our understanding, AT command is the only possible solution. |
| QC | We agree with these clarifications in general, but there is one comment to issue 3-4: NACK is not part of the test metric, then should we have NACK transmission? If it is not part of test metric, we suggest to have DTX and ACK for transmission, to save test time. |
| LG | We are generally fine the proposals, but for Issue 3-2, does it need to define specific scenario for the test case in the specification? |
| Huawei | Issue 3-2: Scenario for test To all: During the test, it is unavoidable that many PSFCHs are corresponding to one PSSCH since the maximum number of PSSCHs is 10 while the maximum number of PSFCHs is 64. Therefore, group cast scenario should be used. Moreover, cast type is specified by “Cast type indicator” in SCI format 2-A transmitted by tested UE: Cast type indicator – 2 bits as defined in Table 8.4.1.1-1. Table 8.4.1.1-1: Cast type indicator Value of Cast type indicator; Cast type. 00; Broadcast. 01; Groupcast when HARQ-ACK information includes ACK or NACK. 10; Unicast. 11; Groupcast when HARQ-ACK information includes only NACK. According to our analysis, the Cast type indicator should be set to “01”, we think it is more clear to define the scenario directly compared to set Cast type indicator to “01”. Issue 3-4: Details of Test setup To Intel: According to the previous agreements, Prob(DTX-> ACK) should be considered, so DTX occasions should be introduced. To prevent UEs from cheating, we propose to change design that “The slots are divided two part: PSFCH transmission slots and DTX occasion slots.” to “TE randomly transmits ACK or DTX on each PSFCH resource,” The related Option 2 (TE randomly transmit ACK or DTX on each PSFCH resource) is listed. To QC, We agree with you that only ACKs are needed since there is no NACK requirements. Issue 3-5: Test Method For PSFCH performance test, if tested UE detect a NACK, it will generate PSSCH retransmission, if tested UE detect DTX, there is no PSSCH retransmission. Therefore we can calculate the results by counting the PSSCH retransmissions. For PSFCH decoding capability test: If the tested UE detect:; Tested UE’s behavior. ACK on all PSFCH resources; No retransmission. NACK on any one PSFCH resource; Retransmissions of PSSCHs to all TX UEs. DTX on any one PSFCH resource; Retransmissions of PSSCHs to all TX UEs. We can see that if a UE receives retransmissions, we can’t calculate how many ACKs are missing detected since many PSFCHs are corresponding to one PSSCH, so maybe AT command is the only choice. |
| MTK | We have one question about PSFCH decoding capability test using AT command. If TE randomly transmit ACK or NACK, and the ACK or NACK information is missed from UE side, how and when doses UE to inform TE the missed information using AT command? |

--------------------GTW discussion on Issue 3-5 -----------------

QC: AT command is simple solution. We are also open to discuss if there is any alternative way.

Intel: AT command is only solution on hand.

MTK: For single link of PSFCH, we have alternative way. For multi-link, do we need TE to send ACK/NACK ?

Intel: AT command will stat the results in receiver side.

### CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2106805 | Intel: We suggest the following clarification for Note 3 in Table 11.1.9.1-1 Note 3: N equals to the number of PSFCH(s) resources that UE can receive in a slot as specified in Clause 4.2.16.1.6 of TS 38.306[14] (IE psfch-RxNumber). Resource pool configuration can be updated based on discussion of issues in Topic #2 of e-mail thread [98-bis-e][317] V2X_Demod_Part1. |
|  | QC: resource pool configuration can be removed |
|  | LG: we prefer to keep the minimum requirement table as previous draft Big CR including CBW and SCS |
|  | Huawei, HiSilicon: Intel’s comments is fine for us; If one general common resource pool configuration is defined in other CR, it is fine for us to remove it from our CR, but some PSFCH specific resource pool configurations need to be included; We will update the CR to include CBW and SCS in the minimum requirement table. |

## Summary for 1st round

### Open issues

Status summary

Issue 3-1: Assumptions on PSSCH transmission from tested UE - Tentative agreements: Always let tested UE to transmit PSSCH to enable it to detect related PSFCHs. Meanwhile, it is not necessary for TE to decode the PSSCH to transmit ACK or NACK randomly (For decoding capability test) or only NACK (For performance test). Issue 3-2: Scenario for test - Tentative agreements: N/A - Recommendations for 2nd round: Discuss whether to specify Cast type indicator as “01” in the test. Issue 3-3: Details of Test configuration - Tentative agreements: - PSSCH allocation: 10 subchannels. - Sl-PSFCH-CandidateResourceType-r16: allocSubCH - Sl-PSFCH-RB-Set-r16: 100. - PSFCH periodicity: 1 slot - Source ID for tested UE: 0 - Member ID for feedback UE: 0 for UE 0, 1 for UE 1,…,N-1 for UE N-1 and N equals to PSFCH-RX-Number. - Sl-MinTimeGapPSFCH-r16: 2 Issue 3-4: Details of Test setup - Tentative agreements: N/A - Recommendations for 2nd round: Check companies view on Option 2 (TE randomly transmit ACK or DTX on each PSFCH resource) Issue 3-5: Test Method - GTW agreement: AT command adopted for this tets case based on current available solution.

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

# Topic #4: PSCCH/PSSCH decoding capability

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2104902 | Qualcomm Inc. | Proposal 1: Use the following feedback scheme and set 40MHz channel bandwidth and 10 subchannels for PSCCH/PSSCH decoding capability test. 1. TE configures min(n,10) high priority PSSCH (in PSCCH), where n is psfch-tx capability. Therefore, if n < 10, UE receivers n PSSCH with higher priority, 10-n PSSCH with lower priority. To avoid UE cheating, TE can randomize the location of higher priority PSSCHs 2. UE PSFCH transmission is selected according to PSSCH priority, hence min(n,10) feedback corresponding to min(n,10) high priority PSSCH is transmitted to TE. 3. TE calculates missing PSCCH decoding probability by the missing probability of higher priority PSSCH feedback. |
| R4-2104774 | CATT, GOHIGH | Simulation results |
| R4-2104996 | LG Electronics | Proposal 1: Use option 3 for PSCCH decoding capability test |
| R4-2106424 | Intel Corporation | Observation #1: The main test purpose is to verify the reception of multiple PSCCHs in a slot. Observation #2: Reception of 10 PSCCHs in a slot is mandatory for Rel-16 V2X UE. Proposal 1: Rename the test from “PSCCH/PSSCH decoding capability requirements” to “PSCCH decoding capability requirements”. Observation #3: Option 2 test design allows to resolve the issue with testing of UEs with different psfch-TxNumber capabilities. However, Option 2 test design does not fulfil the PSCCH decoding capability test purpose (i.e. reception of 10 PSCCHs in a slot) for all possible UEs. Observation #4: Option 1 test design allows directly verify that UE supports reception of 10 PSCCHs in a slot. However, Option 1 test design requires additional test setup (i.e. connection of the SS COM port to the UE COM port) in comparison to other V2X requirements. Observation #5: Option 3 test design allows to verify that UE supports reception of 10 PSCCHs in a slot. However, Option 3 test design does not allow to have direct verification of PSCCH decoding capability and testing time can be different for UEs with different psfch-TxNumber capability. Proposal 2: Define Rel-16 V2X PSCCH decoding capability test with 40 MHz channel bandwidth. Proposal 3: Use testing methodology with AT command (Option 1) or with different PSSCH priority configuration (Option 3) for Rel-16 V2X PSCCH decoding capability test. |
| R4-2106806 | Huawei, HiSilicon | Proposal 1: Use option 3 for PSSCH/PSCCH decoding capability test: - TE sets PSSCH priority (in PSCCH) when PSFCH Tx capability < 10, x PSSCH with higher priority, 10-x PSSCH with lower priority. Note that PSFCH is selected according to PSSCH priority, hence if PSFCH Tx capability = x, the x feedback corresponding to x high priority PSSCH is transmitted to TE. - TE can verify UE PSCCH decoding success or failure by checking whether all the higher priority PSSCH feedback is received. In order to always feedback all the high priority SCHs, UE has to decode all CCH to know the priority. - To avoid UE cheating, TE can randomize the location of higher priority PSSCHs Proposal 2: Test time for PSFCH-TX-Capability=4 should be 2.5 times that of test time with PSFCH-TX-Capability=16, test time for PSFCH-TX-Capability=8 should be 1.25 times that of test time with PSFCH-TX-Capability=16. |
| R4-2106807 | Huawei, HiSilicon | Rel-16 CR for 38.101-4 with following changes: - Added the requirements and test setup for NR V2X PSCCH/PSSCH decoding capability test - Added the related resource pool configurations. |

## Open issues summary

Issue 4-1: CBW and feedback configuration

- Previous meeting agreements

  - Option1: No PSFCH, AT command and 40 MHz

  - Option2: Using PSFCH with PSFCH with periodicity 1 and flexibly CBW depending on psfch-TxNumber capability:

    - 40MHz channel bandwidth is for the UE supporting psfch-TxNumber =n16, 20MHz/30MHz channel bandwidth is for the UE supporting psfch-TxNumber =n8, and 10MHz channel bandwidth is for the UE supporting psfch-TxNumber =n4.

  - Option 3: 40 MHz CBW, PSFCH based feedback and following test methodology

    - TE sets PSSCH priority (in PSCCH) when PSFCH Tx capability < 10, x PSSCH with higher priority, 10-x PSSCH with lower priority. Note that PSFCH is selected according to PSSCH priority, hence if PSFCH Tx capability = x, the x feedback corresponding to x high priority PSSCH is transmitted to TE.

    - TE can verify UE PSCCH decoding success or failure by checking whether all the higher priority PSSCH feedback is received. In order to always feedback all the high priority SCHs, UE has to decode all CCH to know the priority.

    - To avoid UE cheating, TE can randomize the location of higher priority PSSCHs

- Proposals

  - Option 1 (Intel)

  - Option 3 (QC, LGE, HW, Intel)

- Way forward

  - Check whether Option 3 is acceptable for all companies

Issue 4-2: Test title

- Proposals

  - Option 1 (Intel): Rename the test from “PSCCH/PSSCH decoding capability requirements” to “PSCCH decoding capability requirements”.

  - Option 2: Keep current naming

- Way forward

  - Collect companies views on options above

Issue 4-3: Test time (in case Option 3 for Issue 4-1 is agreed)

- Proposals

  - Option 1 (HW): Test time for PSFCH-TX-Capability=4 should be 2.5 times that of test time with PSFCH-TX-Capability=16, test time for PSFCH-TX-Capability=8 should be 1.25 times that of test time with PSFCH-TX-Capability=16.

- Way forward

  - Collect companies views on options above

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| XXX | Issue 4-1: CBW and feedback configuration Issue 4-2: Test title Issue 4-3: Test time |
| Intel | Issue 4-1: CBW and feedback configuration Options 1 or 3 are fine for us. Option 1 is slightly preferred for us. Taking into account that AT command will be potentially used for PSFCH testing, it means that connection of the SS COM port to the UE COM port will be established anyway during V2X testing. Therefore, we don’t see big issue to consider AT command for this test. We would like to see the technical concerns from other companies for Option 1. In case of strong concerns, we are fine to go with option 3. Issue 4-2: Test title Taking into account that the purpose of the test is to verify PSCCH decoding capability test and test metric PSCCH miss detection, we think that it is better to rename the test from “PSCCH/PSSCH decoding capability requirements” to “PSCCH decoding capability requirements” to avoid any confusions. Issue 4-3: Test time Based on our understanding, we think that test time assumptions should be discussed by RAN5 and should be captured in conformance specification (not in performance specification). |
| QC | Issue 4-1: While option 1 is fine for us, option 3 can address the concerns raised in the previous meeting for using AT command. Hence option 3 is preferred. Issue 4-2: We are fine with change the test title. Issue 4-3: Agree with the test time proposal, if it is decided that RAN4 should specify it. |
| LG | Issue 4-1: CBW and feedback configuration Support option 3. But option 1 is also fine for us if companies are supported. Issue 4-2: Test title Based on signaling name (pscch-Rxsidelink), we are fine with using the title as ‘PSCCH decoding capability requirement’ |
| Huawei | Issue 4-1: CBW and feedback configuration Option 3 is fine for us. Issue 4-2: Test title We are fine to change the test title to avoid any confusions. Issue 4-3: Test time The test time is derived based on the specific test method of Option 3 and to get sufficient samples to ensure correct/reliable testing, if should be specified in RAN4. Like did for SDR test in LTE “the TB success rate shall be sustained during at least 300 frames.”. |
| MTK | Issue 4-1: CBW and feedback configuration We support Option 3. Issue 4-2: Test title Regarding the test title, we slight prefer to keep unchanged as LTE V2X used. But, if majority companies want to change it, we are also OK. |
| CATT | Issue 4-1: CBW and feedback configuration Ok with option 3. Issue 4-2: Test title Both options are OK with us. |

------------------GTW discussion for Issue 4-1----------------

Intel: Seems AT command unavoided for PSFCH test cases. Then we prefer option 1 to simplify test procedure.

Huawei: This pending on issue 3-5, if AT command not avoidable, then we can discuss among option 1 and option 3.

### CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2106807 | Intel: This CR can be revised based on outcome of open issues discussion in this e-mail thread and based on discussion of issues in Topic #2 of e-mail thread [98-bis-e][317] V2X_Demod_Part1. |
|  | QC: note 1 in test configuration table and resource pool configuration should be removed. |
|  | LG: same view with Intel |
|  | Huawei, HiSilicon: Thanks for all comments. We will revise the CR as per outcome of the open issues. The resource pool configuration will be removed if it is included in other CR. @QC, “Note 1 in test configuration table should be removed”, could you clarify more? |

## Summary for 1st round

### Open issues

Status summary

Issue 4-1: CBW and feedback configuration - GTW agreement: Option 1 adopted with the consideration that AT command need to be used for some other test case(s). Issue 4-2: Test title - Tentative agreements: Rename the test from “PSCCH/PSSCH decoding capability requirements” to “PSCCH decoding capability requirements”. Issue 4-3: Test time (in case Option 3 for Issue 4-1 is agreed) - Moderator comment: This issue can be closed because Option 1 was agreed for Issue 4-1

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

# Recommendations for Tdocs

## 1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on multiple link tests for NR V2X demodulation performance | Intel Corporation |  |
| Updated simulation assumptions for NR V2X multiple link test case | Huawei, HiSilicon |  |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2106802 | CR for 38.101-4: Introduction of power imbalance requirements for NR V2X. | Huawei, HiSilicon | Revised |  |
| R4-2104903 | CR: Demod HARQ buffer soft combining test cases for NR V2X | Qualcomm Incorporated | Return to | This tdoc was not uploaded. Therefore, we can use reserved tdoc number to prepare final version. |
| R4-2106805 | CR for 38.101-4: Introduction of PSFCH decoding capability test for NR V2X. | Huawei, HiSilicon | Revised |  |
| R4-2106807 | CR for 38.101-4: Introduction of PSCCH/PSSCH capability test for NR V2X. | Huawei, HiSilicon | Revised |  |
| R4-2106425 | Summary of NR V2X Multiple link simulation results | Intel Corporation | Return to |  |

Moderator note: Documents under AI 5.2.1.3 which are not captured in below tables can be Noted.

## 2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-210xxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-210xxxx | WF on … | YYY | Agreeable, Revised, Noted |  |
| R4-210xxxx | LS on … | ZZZ | Agreeable, Revised, Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents