3GPP TSG-RAN WG4 Meeting #98bis-e                                            R4-2106115

Electronic Meeting, 12th– 20th Apr, 2021

Agenda Item:   8.13

Source:        Samsung

Title:           WF on NR Rel-17 eIAB RAN4 scope

Document for:  Approval

Introduction

This Way forward is to capture the tentative agreement and discussion points identified for future study according to preliminary discussion on Rel-17 eIAB as summarized in [1]. This is with goal to facilitate future discussion with refined scope deserved more analysis. However, it is not precluded other discussion if any issue identified.

Note: it’s also observed that PHY enhancement on Rel-17 eIAB includes other aspects such as UL power control, interference measurement of BH link for which analysis on RAN4 impact could not start until concrete progress in RAN1. Hence those items are not listed explicitly in this meeting.

Discussion

Simultaneous operation of IAB-node’s child and parent links

Agreements: Declaration is agreed to be considered for IAB-node to indicate whether to support simultaneous operation and/or new requirement to be introduced in Rel-17. - FFS on details Way forward: There is no agreement on whether the RF core requirement will be impact due to introduction of simultaneous operation between IAB-DU and IAB-MT. While it is identified that RF conformance testing need to be reviewed anyway. - Further input on necessity on RF impact is encouraged in next meeting to justify the necessity. - Include the conformance testing spec(TS38.716-1/2) in updated WID and address perf TU on RF session

IAB-node DC scenario

Agreement: All companies provide feedback on this topic share the same understanding as: Existing rel-16 IAB RF specification is compatible for DC scenario. - This is not precluded further discussion on DC if new feature/scenario agreed in RAN1/2/3. Way forward: Further review needed in next meeting regarding: - DC scenario impact on IAB RRM requirement to be discussed in RRM session - Capability signaling for DC feature to be reviewed in next RAN4 meeting to see whether LS needed

Timing related enhancement

Way forward Further study on RAN4 impact due to timing enhancement to support case 6, case 7 timing and switching between case1/6/7.

DL power control

Agreement No new RF requirement is suggested due to DL power control enhancement on IAB node and gNB.

CLI

Agreement It’s suggested that if no fundamentally change between rel-16 and rel-17 with respect to adjacent channel interference, Rel-17 IAB enhancement can leverage on Rel-16 CLI conclusion for RF perspective. - Further discussion on CLI is not precluded if issue identified.

Reference

[1] R4-2105984, Email discussion summary for [98-bis-e][313] NR_eIAB

[2] RP-210758, WID on Enhancements to Integrated Access and Backhaul