# 3GPP TSG-RAN WG4 Meeting # 98-bis-e 										 R4-2106294

# Electronic Meeting, Apr. 12-20, 2021

Title: 	on frequency range definition for NR 52_71GHz

Source: 	Xiaomi

Agenda item:	8.12.6

Document for:	Approval

Background

During the RAN#91-e meeting, a specific email thread [1] for NR 52.6 to 71GHz has been set and the frequency range definition for NR 52.6 to 71GHz has been discussed with below the agreement captured in the minutes as:

RAN1, RAN2 and RAN4 are asked to provide its analysis or recommendation to RAN#92E (June) on how to introduce the 52.6-71GHz frequency range.

Hence in this paper, we try to give some analysis from RAN4 perspective.

Discussion

As decided by RAN#91-e, each WG to further discuss the frequency range definition. Hence we have a summary table below to further check the current requirements and system parameters on these frequency ranges and table 1 is shown below:

Table 1 Comparison of FR1、FR2 and FRx

|  | FR1 | FR2 | FRx |
| --- | --- | --- | --- |
| Frequency range | 410 MHz – 7125 MHz | 24250 MHz – 52600 MHz | 52600 MHz – 71000 MHz |
| Supported SCS | 15kHz, 30kHz, 60kHz | 60kHz, 120kHz | 120kHz,（480/960kHz） |
| Channel bandwidth | Min:5MHz, Max:100MHz | Min:50MHz, Max:400MHz | Subcarrier spacing [kHz]; Minimum bandwidths [MHz]; Maximum bandwidths [MHz]. 120; 50, 400 (Note); 400. 480; 200; 1600. 960; 400, 2160 (Note); 1600, 2000, 2160, 3200 (Note). Note: for the cases where multiple values are listed as candidates, there is no direct linking among min and max values in this table.. |
| RF requirement set | Conducted | OTA | OTA |

Firstly, The frequency gap between FR1 and FR2 are as large as 17GHz from the FR1 upper end and FR2 lower end. However, as considering the frequency range of 52.6GHz to 71GHz, the frequency range boundary are contiguous at 52.6GHz frequency point.

Observation 1：There is no frequency gap between FR2 and the new 52.6 to 71GHz frequency range.

Secondly, if we track back to the discussion beginning of NR. The FR1 and FR2 mainly difference is the UE RF architecture, especially the antenna architecture. For FR1, digital beamforming is mainly used for easier and more flexible beam setting while for FR2, the hybrid beamforming is used considering larger number of RF chains with the increase of cost and larger volume of base station and UE. Also the antenna physical port is not assumed for FR2 with smaller size of antenna with shorter wavelength coming from larger carrier frequency. With the above consideration, all the FR2 requirements are intended to be OTA. For the FRx, we don’t see the new RF architecture and the antenna architecture at least at this stage comparing to FR2 and hence similar set of OTA requirements are to be defined for the FRx range.

Observation 2: Similar RF architecture for FRx range comparing to FR2.

Thirdly, for SCS, the FRx are supporting higher SCS due to higher frequency are quite different from FR2. With this, we believe that the value of each specific requirement can be different based on the discussion during the work item. However, if we look to the requirement definition in TS 38.101-2, take ACLR as an example, we will find that the requirement are band specific and channel bandwidth specific:

The general ACLR requirement above is captured from table 6.5.2.3-1 from TS 38.101-2. As we can see, the channel bandwidth, ACLR value, measurement bandwidth are all defined specifically. With that, the requirement difference do not result to a new frequency range definition. For new channel bandwidth of 52.6GHz to 71GHz, requirements can be defined as band basis.

Observation 3: RF requirements are defined band specific with different value and measurement bandwidth.

With the above consideration, we believe to extend current FR2 to cover the frequency range 52.6—71GHz is a feasible way and minimize the standardization effort.

Proposal 1: To extend FR2 to cover 52.6—71GHz.

Conclusions

In this contribution, we give discussion on remaining issue on UE operations for licensed bands partially used for SL transmission and the observations are shown as below：

Observation 1：There is no frequency gap between FR2 and the new 52.6 to 71GHz frequency range.

Observation 2: Similar RF architecture for FRx range comparing to FR2.

Observation 3: RF requirements are defined band specific with different value and measurement bandwidth.

Proposal 1: To extend FR2 to cover 52.6—71GHz.

Reference

- RAN [91E][24][52.6_71GHz_WI] NMW discussion,  https://nwm-trial.etsi.org/#/documents/2856