3GPP TSG-RAN WG4 Meeting # 98b-e	R4-2106304

# Electronic Meeting, Apr. 12-20, 2021

Source: 	Skyworks Solutions, Inc.

Title: 	PC2 Class C UL CA UE Architecture and MPR/A-MPR evaluation

Agenda Item:	8.2.2.4 HPUE for TDD intra-band contiguous UL CA [NR_RF_FR1_enh-Core]

Document for:	Approval

# Introduction

In the RAN4#98e, ways forward were agreed on architectures [1] and back-off evaluation assumptions [4] for PC2 contiguous UL CA MPR/A-MPR. Additionally, the associated WI was updated in RAN#91e [5] to include objectives on intra-band contiguous UL CA plus UL MIMO. This contribution discusses the PC2 contiguous UL CA transmitter architecture and associated MPR requirements options and re-uses the measured data from [2] to make proposals for PC2 class B and C UL CA MPR and NS04 A-MPR.

# Discussion

## Architecture Options and Requirements

In RAN#91e, FR1 non-spectrum dedicated work item [5] which covers PC2 contiguous UL CA was updated to introduce an objective on UL CA combined with UL MIMO, the relevant text is copied here.

“In Rel-16, intra-band UL contiguous CA MPR requirement evaluation takes 1PA/1LO architecture as the reference RF architecture, however 2PA/2LO architecture is not precluded in implementation. UL MIMO support can be signaled per Band combination per CC for CA, but there is no RF requirements for UE support both intra-band UL CA and UL MIMO. In Rel-16, emission requirements for UL MIMO is clarified as the sum of the connectors, while it has impact on MPR requirement, i.e. the UE cannot use the same MPR requirement defined for non-MIMO. So it is necessary to define the RF requirements for intra-band UL CA for UL MIMO.

“Specify RF requirements for intra-band UL contiguous CA for UL MIMO. This objective can also apply to UL Tx switching. Consider n41C and n78C as example band combinations”

In RAN4#98e two ways forward [1, 4] were agreed on PC2 contiguous UL CA architecture and MPR/A-MPR evaluation which had the following agreement in [1]:

“Agreement:

- A single TX PC2 PA (200MHz 1LO) is the baseline to develop MPR and A-MPR requirement

- In parallel, also assess the two other cases: 1) 2 100MHz PC2 PA; 2) 2 200MHz PC3 PA

- It is preferred to define 1 set of MPR and A-MPR requirements. Final decisions are FFS”

In [4], the architectures are discussed in further detail and the evaluation setup and PA calibrations are agreed for the different cases.

Regarding the number of  MPR and A-MPR requirements, it was already the case that for PC3 Contiguous UL CA, the same MPR and A-MPR was used for 1x23dBm PA + 1 LO and 2x23dBm PA +2 LO architecture for bandwidth class C.

#### Comparing 1PA+1LO and 2PA+2LO Performances

As previously discussed in [2], when looking in detail into the MPR for the 2xPC3 UL CA for non-contiguous UL CA, it is found that it is worse than for a single PC3 PA contiguous UL CA outer 2 or outer 1 allocation as shown in Table 1. This is inconsistent with our measurements for PC3 in release 16 but also our PC2 measurements presented in this meeting.

Table 1: Comparison of MPR for PC3 single and two PA architecture from 38.101-1

| Case | NS | Req. | PC | Arch. | WC MPR /AMPR | comment |
| --- | --- | --- | --- | --- | --- | --- |
| PC3 Cont UL CA | 01 | -30dBm/MHz IM3 | 3 | 1xPC3 PA | 14 | based on outer 2 class C |
|  | 01-04 | -13dBm/MHz IM3 | 3 | 1xPC3 PA | 7 | based on outer 1 class C |
|  | 04 | -25dBm/MHz IM3 | 3 | 1xPC3 PA | 13 | Based on CA_NS04 |
| PC3 NC UL CA | 01 | -30dBm/MHz IM3 | 3 | 2xPC3 PA | 15 | 2PA 1dB worse than 1PA - should be lower than 1PA |
|  | 01-04 | -13dBm/MHz IM3 | 3 | 2xPC3 PA | 9 | 2PA 2dB worse than 1PA - should be lower than 1PA |
|  | 04 | -25dBm/MHz IM3 | 3 | 2xPC3 PA | 13 | Consistent with 1 PA but should be lower |

Since the use of 2xPC2 PAs is enabled only to facilitate PA architectures that have limited BW for bandwidth class C only, it is not acceptable that those drive the baseline MPR especially since with their intrinsic 3dB margin, limited bandwidth and being subject to reverse IMD compared to forward IMD for the baseline, their emission levels should be lower.

#### Specific case requiring TxDiv or UL MIMO

For the third case however, where UL MIMO or TxDiv is involved to achieve PC2 level based on 2x23dBm 1LO architecture, the single CC MPR for TxDiv and UL MIMO is not yet agreed and uses a separate MPR table. Given the revised WI calls for additional requirements needed and the status of the single CC, it seems better to have a separate MPR table for this architecture that will cover the TxDiv and UL MIMO case like for the single CC case. Moreover, this case has a dedicated agenda.

Proposal 1 on MPR requirements:

- The 2x100MHz PC2 PA+ 2LO architecture uses the same MPR than the baseline 200MHz single PC2 PA + 1LO case, is limited to bandwidth class D and should not drive higher MPR/A-MPR values.

- The 2x200MHz PC3 PA+1LO case has a dedicated MPR table covering both TxDiv and UL MIMO operation and should be treated under the intra-band UL contiguous CA for UL MIMO objective.

## Power Class 2 Single PA Measurements Summary

In this section, we only provide the worst cases back-off out of all the measured results that can be found in [2] the corresponding annex sections. When large MPR/A-MPR numbers are found at maximum PA voltage, a lower voltage was evaluated and results are denoted with HV/LV. This is especially the case for non-contiguous allocations for NS04 requirements.

#### Contiguous Allocations

Table 3 provides the contiguous allocations PC2 QPSK DFT-s-OFDM and CP-OFDM back-off measurements results for contiguous UL CA class B and C for NS01 and NS04. PA maximum power and related MPR are given, and compares them with PC3 specification. Inner+ case is an Outer allocation which is just at the edge of the Inner region.

Table 3: back-off results for NS01 MPR and NS04 AMPR for Contiguous allocations

| CA class and type |  | DFT-s-OFDM |  |  |  |  | CP-OFDM |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Channel and waveform allocation | Pmax [dBm] |  | MPR | AMPR | Channel and waveform allocation | Pmax [dBm] |  | MPR | AMPR |
|  |  |  | NS01 | NS04 | dB | dB |  | NS01 | NS04 | dB | dB |
| Class B | inner/inner+ | 20M0_20M0_054R052_054R000 | 24.3 | 22.1 | 2 | 3.5 | 20M0_20M0_054R052_054R000 | 23.7 | 21 | 2.5 | 5 |
|  |  | 20M0_20M0_054R052_050R000 | 24.5 | 22.2 | 1.5 | 3.5 | 20M0_20M0_053R053_053R000 | 23.8 | 21.1 | 2.5 | 4.5 |
|  |  | 20M0_20M0_050R056_050R000 | 24.7 | 22.4 | 1.5 | 3.5 | 20M0_20M0_052R052_053R000 | 23.7 | 21 | 2.5 | 5 |
|  |  | 50M0_50M0_144R126_128R000 | 23.8 | 22.2 | 2.5 | 3.5 | 50M0_50M0_136R134_136R000 | 23.4 | 21.6 | 3 | 4 |
|  |  | 50M0_50M0_144R126_125R000 | 23.8 | 22.2 | 2.5 | 3.5 | 50M0_50M0_135R135_135R000 | 23.5 | 21.6 | 2.5 | 4 |
|  |  | 50M0_50M0_135R135_135R000 | 24 | 22.3 | 2 | 3.5 | 50M0_50M0_134R136_135R000 | 23.5 | 21.6 | 2.5 | 4 |
|  |  | PC3 Class B MPR |  |  | 1 | dB | PC3 Class B MPR |  |  | 2 | dB |
|  | Outer | 20M0_20M0_100R006_100R000 | 22.2 | 21.9 | 3.5 | 4 | 20M0_20M0_106R000_106R000 | 21.2 | 20.1 | 4.5 | 5.5 |
|  |  | 20M0_20M0_090R016_072R000 | 22.4 | 21.8 | 3.5 | 4 | 20M0_20M0_081R025_080R000 | 21.8 | 20.5 | 4 | 5.5 |
|  |  | 20M0_20M0_080R026_080R000 | 23.2 | 21.8 | 2.5 | 4 | 20M0_20M0_080R026_080R000 | 21.9 | 20.4 | 4 | 5.5 |
|  |  | 20M0_20M0_080R026_072R000 | 23.1 | 21.8 | 2.5 | 4 | 20M0_20M0_079R027_080R000 | 21.9 | 20.4 | 4 | 5.5 |
|  |  | 50M0_50M0_270R000_270R000 | 22.1 | 22.1 | 3.5 | 3.5 | 50M0_50M0_270R000_270R000 | 20.9 | 21 | 5 | 5 |
|  |  | 50M0_50M0_216R054_192R000 | 22.5 | 22 | 3.5 | 4 | 50M0_50M0_203R067_202R000 | 21.6 | 21 | 4 | 5 |
|  |  | 50M0_50M0_216R054_180R000 | 22.5 | 22 | 3.5 | 4 | 50M0_50M0_202R068_202R000 | 21.5 | 21 | 4.5 | 5 |
|  |  | 50M0_50M0_200R070_200R000 | 22.8 | 22.1 | 3 | 3.5 | 50M0_50M0_201R069_202R000 | 21.5 | 21 | 4.5 | 5 |
|  |  | PC3 Class B MPR |  |  | 3.5 | dB | PC3 Class B MPR |  |  | 4 | dB |
| Class C | inner/inner+ | 60M1_100M1_060R102_162R000 | 24 | 22.8 | 2 | 3 | 60M1_100M1_055R107_162R000 | 23.4 | 22 | 3 | 4 |
|  |  | 60M1_100M1_054R108_162R000 | 24.2 | 22.8 | 2 | 3 | 60M1_100M1_054R108_162R000 | 23.4 | 22 | 3 | 4 |
|  |  | 60M1_100M1_050R112_162R000 | 24.4 | 23 | 2 | 3 | 60M1_100M1_053R109_162R000 | 23.5 | 22.1 | 2.5 | 3.5 |
|  |  | 100M1_100M1_144R129_135R000 | 23.7 | 22.7 | 2.5 | 3 | 100M1_100M1_137R136_137R000 | 23.2 | 22.1 | 3 | 3.5 |
|  |  | 100M1_100M1_135R138_135R000 | 24.1 | 23 | 2 | 3 | 100M1_100M1_136R137_136R000 | 23.3 | 22.1 | 3 | 3.5 |
|  |  | 100M1_100M1_135R138_128R000 | 24.5 | 22.9 | 1.5 | 3 | 100M1_100M1_135R138_135R000 | 23.8 | 22 | 2.5 | 4 |
|  |  | PC3 Class C MPR |  |  | 2.5 | dB | PC3 Class C MPR |  |  | 3.5 | dB |
|  | outer | 60M1_100M1_162R000_270R000 | 22.5 | 22.7 | 3.5 | 3.5 | 60M1_100M1_162R000_273R000 | 20.7 | 21.4 | 5 | 5 |
|  |  | 60M1_100M1_135R027_200R000 | 22.8 | 22.7 | 3 | 3 | 60M1_100M1_125R037_209R000 | 21.5 | 21.5 | 4.5 | 4.5 |
|  |  | 60M1_100M1_128R034_200R000 | 22.7 | 22.6 | 3 | 3 | 60M1_100M1_125R037_208R000 | 21.5 | 21.5 | 4.5 | 4.5 |
|  |  | 60M1_100M1_128R034_192R000 | 22.7 | 22.6 | 3 | 3 | 60M1_100M1_125R037_207R000 | 21.3 | 21.4 | 4.5 | 4.5 |
|  |  | 100M1_100M1_270R003_270R000 | 21.3 | 22.4 | 4.5 | 4.5 | 100M1_100M1_273R000_273R000 | 20.5 | 21.4 | 5.5 | 5.5 |
|  |  | 100M1_100M1_216R057_200R000 | 22.2 | 22.4 | 3.5 | 3.5 | 100M1_100M1_205R068_204R000 | 21 | 21.4 | 5 | 5 |
|  |  | 100M1_100M1_216R057_192R000 | 22.2 | 22.4 | 3.5 | 3.5 | 100M1_100M1_204R069_204R000 | 21 | 21.4 | 5 | 5 |
|  |  | 100M1_100M1_200R073_192R000 | 22.5 | 22.5 | 3.5 | 3.5 | 100M1_100M1_203R070_204R000 | 21 | 21.4 | 5 | 5 |
|  |  | PC3 Class C MPR |  |  | 7 | dB | PC3 Class C MPR |  |  | 8 | dB |

Observations on contiguous allocations MPR:

- PC3 MPR is valid for PC2 inner outer for class C as it has higher margins

- For class B, PC3 MPR requires and additional 1dB for PC2 inner and 0.5dB for outer

Proposal 2 on contiguous allocations PC2 class B and C UL CA MPR:

- The following MPR table is adopted for PC2 contiguous allocation MPR (changes from PC3 highlighted in yellow)

| Modulation |  | MPR for bandwidth class B(dB) |  | MPR for bandwidth class C(dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | inner | outer | inner | outer |
| DFT-s-OFDM | Pi/2 BPSK | 2.5 | 3.5 | 2.5 | 7 |
|  | QPSK | 2.5 | 3.5 | 2.5 | 7 |
|  | 16QAM | 2.5 | 3.5 | 2.5 | 7 |
|  | 64QAM | 3.0 | 4.0 | 5 | 7 |
|  | 256QAM | 5.5 | 6.0 | 7 | 7.5 |
| CP-OFDM | QPSK | 3 | 5 | 3.5 | 8 |
|  | 16QAM | 3 | 5 | 3.5 | 8 |
|  | 64QAM | 3.5 | 5 | 5 | 8 |
|  | 256QAM | 6.5 | 6.5 | 7 | 8 |

Observations on contiguous allocations NS04 A-MPR :

- For NS04 AMPR the worst case inner and outer allocation are limited by the -25dBm/MHz limit at 10MHz offset thus with 3dB higher power, compared to PC3, a larger back-off is needed.

- For Class C outer allocations, MPR is sufficient, AMPR = MPR

- For Class C inner allocations, AMPR=MPR+0.5 dB is sufficient

Proposal 3 on contiguous allocations NS04 PC2 class C A-MPR:

- NS04 A-MPR = MPR for outer class C PC2

- NS04 A-MPR = MPR+0.5dB for inner class C PC2 when RBstart ≤ 0.33*BWchannel_CA/0.18MHz

- NS04 A-MPR = MPR for inner class C PC2 when RBstart > 0.33*BWchannel_CA/0.18MHz

#### Non-contiguous Allocations

Table 4 provides the PC2 QPSK MPR/AMPR contiguous UL CA class C results for NS01/NSO4 for non-contiguous allocations, and compares them with PC3 specification. Inner+ case is an Outer1 allocation which is just at the edge of the Inner region and Outer1+ case is an Outer2 allocation which is just at the edge of the Outer1 region. For high back-off values, measurements have been done at a lower voltage to properly represent the lower linearity of APT and ET solutions for large back-off when compared to 3GPP fixed bias assumptions.

Table 4: NS01 MPR and NS04 A-MPR for Non-contiguous allocations

|  | n41B HV/LV [dB] |  | DFT-s-OFDM |  | CP-OFDM |  | n41C HV/LV [dB] |  | DFT-s-OFDM |  | CP-OFDM |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BW | WF | MPR | NS04 | MPR | NS04 | BW | WF | MPR | NS04 | MPR | NS04 |
| Inner | 20+20 | 1R73_1R32 | 3/ | 8/ | 3/ | 8.5/ | 60+100 | 1R110_1R51 | 1.5/ | 7/ | 2/ | 8/ |
|  |  | 1R72_1R33 | 2.5/ | 8/ | 3/ | 8.5/ |  | 1R109_1R52 | 1.5/ | 7/ | 2/ | 8/ |
|  |  | 1R56_1R00 | 1/ | 8/ | 1.5/ | 8.5/ |  | 1R84_1R00 | 1/ | 7/ | 7/ | 7.5/ |
|  |  | 1R55_1R00 | 1/ | 8/ | 1.5/ | 8.5/ |  | 1R83_1R00 | 1/ | 7/ | 1.5/ | 8/ |
|  | 50+50 | 1R183_1R86 | 3/ | 8/ | 3/ | 8.5/ | 100+100 | 1R184_1R88 | 3/ | 7/ | 3/ | 7.5/ |
|  |  | 1R182_1R87 | 3/ | 8/ | 3/ | 8.5/ |  | 1R183_1R89 | 3/ | 7/ | 3/ | 7.5/ |
|  |  | 1R139_1R00 | 2/ | 7.5/ | 2/ | 8.5/ |  | 1R140_1R00 | 1.5/ | 7/ | 2/ | 8/ |
|  |  | 1R138_1R00 | 2/ | 7.5/ | 2/ | 8.5/ |  | 1R139_1R00 | 1.5/ | 7/ | 2/ | 8/ |
|  |  | PC3 MPR | 2 | dB | 2.5 | dB |  | PC3 MPR | 2.5 | dB | 3.5 | dB |
| Outer1 | 20+20 | 1R71_1R34 | 2.5/ | 8/ | 3/ | 8.5/ | 60+100 | 1R108_1R53 | 3/ | 7/ | 4/ | 7.5/ |
|  |  | 1R54_1R00 | 1/ | 8/ | 1.5/ | 8.5/ |  | 1R82_1R00 | 1/ | 7/ | 1.5/ | 7.5/ |
|  |  | 1R42_1R63 | 6.5/ | 14.5/14.5 | 6.5/ | 14.5/15 |  | 1R43_1R118 | 5/ | 14.5/14 | 6/ | 14/15 |
|  |  | 1R41_1R64 | 6.5/ | 14.5/14.5 | 6.5/ | 14.5/15.5 |  | 1R42_1R119 | 5/ | 13.5/14.5 | 6/ | 14/15 |
|  |  | 1R01_1R00 | 6.5/ | 15/14.5 | 7/ | 15/15.5 |  | 1R01_1R00 | 5.5/ | 14.5/14 | 5/ | 14.5/14.5 |
|  |  | 1R00_1R00 | 6.5/ | 7/15 | 7/ | 14/ |  | 1R00_1R00 | 5/ | 13.5/14 | 5/ | 14.5/15 |
|  | 50+50 | 1R181_1R88 | 3/ | 8/ | 3/ | 8.5/ | 100+100 | 1R182_1R90 | 4.5/ | 7/ | 5.5/ | 7.5/ |
|  |  | 1R137_1R00 | 2/ | 7.5/ | 2/ | 8.5/ |  | 1R138_1R00 | 1.5/ | 7/ | 2/ | 8/ |
|  |  | 1R109_1R160 | 5.5/ | 14/14.5 | 6/ | 14.5/15 |  | 1R110_1R162 | 6.5/ | 15.5/14.5 | 7/ | 14.5/15.5 |
|  |  | 1R108_1R161 | 5.5/ | 14/14.5 | 6/ | 14.5/15 |  | 1R109_1R163 | 6.5/ | 14.5/14.5 | 7/ | 15/15.5 |
|  |  | 1R01_1R00 | 6/ | 14/14.5 | 6/ | 14.5/15 |  | 1R01_1R00 | 5.5/ | 14/14.5 | 6/ | 15/15.5 |
|  |  | 1R00_1R00 | 6/ | 14/14.5 | 6/ | 14/15 |  | 1R00_1R00 | 5.5/ | 14.5/14.5 | 6.5/ | 15.5/15.5 |
|  |  | PC3 MPR | 5.5 |  | 6.5 |  |  | PC3 MPR | 6 | dB | 7 | dB |
| Outer2 | 20+20 | 1R40_1R65 | 8.5/12.5 | 15/14.5 | 6.5/13.5 | 15.5/15.5 | 60+100 | 1R41_1R120 | 6.5/12.5 | 14/14 | 6/13.5 | 14.5/15 |
|  |  | 1R00_1R105 | 10/13 | 14.5/15 | 10/14 | 14.5/15.5 |  | 1R00_1R272 | 8/12.5 | 13/14 | 8/13 | 13/15 |
|  |  | 1R00_1R01 | 6.5/13 | 14.5/14.5 | 7/14 | 15/15.5 |  | 1R00_1R01 | 5/12.5 | 14.5/14 | 5/13 | 14.5/15 |
|  | 50+50 | 1R107_1R162 | 5.5/13 | 14/14 | 6/13.5 | 15.5/15 | 100+100 | 1R108_1R164 | 6.5/13 | 14/14.5 | 7/14 | 14/15.5 |
|  |  | 1R00_1R269 | 9/13 | 13.5/15 | 9/14 | 13.5/15.5 |  | 1R00_1R272 | 8/12.5 | 13/14.5 | 8/13.5 | 13/15.5 |
|  |  | 1R00_1R01 | 6/13 | 14.5/14.5 | 6/13.5 | 15/15 |  | 1R00_1R01 | 5.5/13 | 15/14.5 | 6.5/14 | 15/15.5 |
|  |  | PC3 MPR | 11.5 | dB | 12 | dB |  | PC3 MPR | 13 | dB | 14 | dB |

Observations on non-contiguous allocations:

- In general, PC2 allocations requires higher MPR than PC3 and this is justified by the fact that the worst case allocations are limited by IMD3 or IMD5 reaching the SEM mask limit which is, in absolute values, the same as PC3 but with 3dB higher output power. We note that 1dB extra back-off should compensate for this as PC2 has a slightly better linearity at 31 dB ACLR instead of 30 dB.

- There is no significant difference for class B and class C MPR results

- For outer 2 cases the HV results reach up to 10dB A-MPR thus LV values should be used

- This issue is also the same for NS04 where narrow allocations need to meet -25dBm/MHz with their IMD3 or IMD5, at least 2 dB higher A-MPR is needed.

- For NS04 inner, IM3 is in-band thus A-MPR is related to IMD5 in the -25dBm/MHz region (as IMD5 in -13dBm/MHz region is already covered by MPR) and A-MPR is 8.5dB worst case

- For NS04 outer1&2, IM3 may fall in -25dBm/MHz region and A-MPR is 15.5dB worst case

Proposal 4 on non-contiguous allocations MPR:

- PC3 QPSK MPR is adopted for PC2 (1Tx) with additional back-off as in Table 6.2A.2.1-3 below (yellow highlight)

Table 6.2A.2.1-3: non-contiguous RB allocation for Power Class 2

| Modulation |  | MPR for bandwidth class B(dB) |  |  | MPR for bandwidth class C(dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | inner | Outer11 | Outer22 | inner | Outer11 | Outer22 |
| DFT-s-OFDM | Pi/2 BPSK | 3 | 6.5 | 13 | 3 | 6.5 | 13 |
|  | QPSK | 3 | 6.5 |  | 3 | 6.5 |  |
|  | 16QAM | 3 | 6.5 |  | 3 | 6.5 |  |
|  | 64QAM | 4.5 | 6.5 |  | 5 | 6.5 |  |
|  | 256QAM | 6 | 6.5 |  | 6.5 | 6.5 |  |
| CP-OFDM | QPSK | 3 | 7 | 14 | 3.5 | 7 | 14 |
|  | 16QAM | 3 | 7 |  | 3.5 | 7 |  |
|  | 64QAM | 5 | 7 |  | 5 | 7 |  |
|  | 256QAM | 7.5 | 7.5 |  | 7.5 | 7.5 |  |
| NOTE 1: Outer 1 MPR for Pi/2 BPSK and QPSK is reduced by 2dB for aggregated allocation bandwidth > 10MHz NOTE 2: Outer 2 MPR is reduced by 4.5dB for aggregated allocation bandwidth > 10MHz |  |  |  |  |  |  |  |

Proposal 5 on non-contiguous allocations NS04 A-MPR:

- For channels and allocations where IM3 is within the -13dBm/MHz NS04 region, the PC2 MPR is sufficient

- PC2 (1Tx) NS04 A-MPR for outer 1 and outer 2 with IM3 in -25dBm/MHz region is 15.5 for B<2.16

- All SEM limited allocations will see the back-off increase for PC2 vs PC3 but ACLR limited region will stay the same thus the following AMPR curve are proposed: AMPRIM3 to meet -25dBm/MHz

MA = 		15.5; 		0 ≤ B < 2.16

14; 		2.16 ≤ B < 3.24

13;       3.24 ≤ B < 5.04

11.5; 		5.04 ≤ B < 10.08

10; 		10.08 ≤ B < 16.56

8;        16.56 ≤ B < 21.96

6; 	     	21.96 ≤ B

## Need for Inner and Edge Allocation MPR

In RAN4#98e meeting, contribution [3] pointed at the fact in CA configurations and when the lowest valid SCS is used, the resulting edge guard bands can be lower than for the single CC case. This is only true for the case where the bandwidth class B is used and when CCs use the 15kHz SCS, thus edge allocation should not be introduced in all cases. In our understanding the issue only arises when only one CC is allocated with an edge allocation since edge allocation has no meaning for contiguous allocation in both CCs. Furthermore, the outer contiguous allocation MPR values are already equal or higher than the single CC edge allocation case.

Regarding non-contiguous allocation, the removal of inner allocation is introduced only for bandwidth class B and is not well justified as a PC2 PA has increased linearity compared to PC3 cases and in our measurements IMD5 are still well below -13dBm/MHz for inner allocations.

Finally, the increase of inner and outer 1 class C MPR for non-contiguous allocation is introduced based on justifications that pertain to the 2x26dBm architecture which should not drive the baseline MPR.

Proposal 6 on removal of inner for non-contiguous allocation and addition of edge contiguous allocation for Class B MPR:

- The 2x26dBm 2LO architecture should not drive the bandwidth class B MPR nor the baseline MPR for bandwidth class B.

- Inner allocation should not be removed from BW Class B non-contiguous allocation

- Edge allocation addition to BW class B contiguous allocation should be further justified and if introduced restricted to the relevant cases.

# Conclusion

In this contribution, we first discuss implementation options for PC2 contiguous UL AC, resulting in the following proposals on architecture baseline.

Proposal 1 on MPR requirements:

- The 2x100MHz PC2 PA+ 2LO architecture uses the same MPR than the baseline 200MHz single PC2 PA + 1LO case, is limited to bandwidth class D and should not drive higher MPR/A-MPR values.

- The 2x200MHz PC3 PA+1LO case has a dedicated MPR table covering both TxDiv and UL MIMO operation and should be treated under the intra-band UL contiguous CA for UL MIMO objective.

Measurement for 1 PA architecture for SEM, ACLR and NS04 requirements are analyzed for contiguous allocation for MPR and AMPR proposals below.

Proposal 2 on contiguous allocations PC2 class B and C UL CA MPR:

- The following MPR table is adopted for PC2 contiguous allocation MPR (changes from PC3 highlighted in yellow)

| Modulation |  | MPR for bandwidth class B(dB) |  | MPR for bandwidth class C(dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | inner | outer | inner | outer |
| DFT-s-OFDM | Pi/2 BPSK | 2.5 | 3.5 | 2.5 | 7 |
|  | QPSK | 2.5 | 3.5 | 2.5 | 7 |
|  | 16QAM | 2.5 | 3.5 | 2.5 | 7 |
|  | 64QAM | 3.0 | 4.0 | 5 | 7 |
|  | 256QAM | 5.5 | 6.0 | 7 | 7.5 |
| CP-OFDM | QPSK | 3 | 5 | 3.5 | 8 |
|  | 16QAM | 3 | 5 | 3.5 | 8 |
|  | 64QAM | 3.5 | 5 | 5 | 8 |
|  | 256QAM | 6.5 | 6.5 | 7 | 8 |

Proposal 3 on contiguous allocations NS04 PC2 class C A-MPR:

- NS04 A-MPR = MPR for outer class C PC2

- NS04 A-MPR = MPR+0.5dB for inner class C PC2 when RBstart ≤ 0.33*BWchannel_CA/0.18MHz

- NS04 A-MPR = MPR for inner class C PC2 when RBstart > 0.33*BWchannel_CA/0.18MHz

Measurement for 1 PA architecture for SEM, ACLR and NS04 requirements are analyzed for non-contiguous allocation for MPR and AMPR proposals below.

Proposal 4 on non-contiguous allocations MPR:

- PC3 QPSK MPR is adopted for PC2 (1Tx) with additional back-off as in Table 6.2A.2.1-3 below (yellow highlight)

Table 6.2A.2.1-3: non-contiguous RB allocation for Power Class 2

| Modulation |  | MPR for bandwidth class B(dB) |  |  | MPR for bandwidth class C(dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | inner | Outer11 | Outer22 | inner | Outer11 | Outer22 |
| DFT-s-OFDM | Pi/2 BPSK | 3 | 6.5 | 13 | 3 | 6.5 | 13 |
|  | QPSK | 3 | 6.5 |  | 3 | 6.5 |  |
|  | 16QAM | 3 | 6.5 |  | 3 | 6.5 |  |
|  | 64QAM | 4.5 | 6.5 |  | 5 | 6.5 |  |
|  | 256QAM | 6 | 6.5 |  | 6.5 | 6.5 |  |
| CP-OFDM | QPSK | 3 | 7 | 14 | 3.5 | 7 | 14 |
|  | 16QAM | 3 | 7 |  | 3.5 | 7 |  |
|  | 64QAM | 5 | 7 |  | 5 | 7 |  |
|  | 256QAM | 7.5 | 7.5 |  | 7.5 | 7.5 |  |
| NOTE 1: Outer 1 MPR for Pi/2 BPSK and QPSK is reduced by 2dB for aggregated allocation bandwidth > 10MHz NOTE 2: Outer 2 MPR is reduced by 4.5dB for aggregated allocation bandwidth > 10MHz |  |  |  |  |  |  |  |

Proposal 5 on non-contiguous allocations NS04 A-MPR:

- For channels and allocations where IM3 is within the -13dBm/MHz NS04 region, the PC2 MPR is sufficient

- PC2 (1Tx) NS04 A-MPR for outer 1 and outer 2 with IM3 in -25dBm/MHz region is 15.5 for B<2.16

- All SEM limited allocations will see the back-off increase for PC2 vs PC3 but ACLR limited region will stay the same thus the following AMPR curve are proposed: AMPRIM3 to meet -25dBm/MHz

MA = 		15.5; 		0 ≤ B < 2.16

14; 		2.16 ≤ B < 3.24

13;       3.24 ≤ B < 5.04

11.5; 		5.04 ≤ B < 10.08

10; 		10.08 ≤ B < 16.56

8;        16.56 ≤ B < 21.96

6; 	      	21.96 ≤ B

Proposal 6 on removal of inner for non-contiguous allocation and addition of edge contiguous allocation for Class B MPR:

- The 2x26dBm 2LO architecture should not drive the bandwidth class B MPR nor the baseline MPR for bandwidth class B.

- Inner allocation should not be removed from BW Class B non-contiguous allocation

- Edge allocation addition for contiguous to BW class B contiguous allocation should be further justified and if introduced restricted to the relevant cases.

# References

[1] R4-2103231 WF on RF architecture and requirements for PC2 intra-band UL contiguous CA, Huawei, HiSilicon, RAN4#98e

[2] R4-2100544 PC2 Class C UL CA  UE Architecture and MPR&A-MPR, Skyworks Solutions Inc., RAN4#98e

[3] R4-2101160 HPUE TDD+TDD MPR and AMPR, Qualcomm Incorporated, R4#98e

[4] R4-2103232 Way forward on MPR and AMPR for intra-band UL contiguous CA, Skyworks Solutions Inc., Qualcomm, RAN4#98e

[5] RP‑210899, revised WID of RF requirements enhancement for NR frequency range 1 (FR1), Huawei, HiSilicon, RAN4#98be