3GPP TSG-RAN WG4 Meeting #98-bis-e	R4-2106335

Online Meeting, Apr 12 – 20, 2021

Agenda item:	5.5.1.2

Source: 	Qualcomm Incorporated

Title: 	On PRS-RSRP measurement requirements

Document for:	Discussion

# Introduction

In RAN4#98-e meeting, several questions surrounding PRS-RSRP measurement requirements were discussed further, with agreements and open issues captured in a WF [1]. In this paper, we discuss the following remaining question:

- Measurement period when configured with PRS-RSTD or UE Rx-Tx

# Measurement period when configured with PRS-RSTD or UE Rx-Tx

The discussion on this topic in RAN4#98-e considered two scenarios described in the WF [1], with corresponding agreements and options reproduced below for convenience. For scenario #2 there was agreement that measurement periods are independent for different positioning methods. Therefore, we can focus our attention on scenario #1, in which PRS-RSRP is reported for DL-TDOA or Multi-RTT.

- Scenario #1: UE being configured to measure PRS-RSRP for DL-TDOA (or Multi-RTT)

  - UE behavior:

    - Option 1

      - UE continues PRS-RSRP over the entire RSTD/UE Rx-Tx measurement period, when PRS-RSRP is configured together with RSTD/UE Rx-Tx, and the required PRS-RSRP measurement period is shorter than that for RSTD/UE Rx-Tx (without PRS-RSRP)

    - Option 2

      - UE does not continue PRS-RSRP measurement

  - Applicable PRS-RSRP measurement period requirements

    - Option 1:

      - The minimum measurement period is extended to that for RSTD/UE Rx-Tx

    - Option 2:

      - The minimum measurement period is linked with the accuracy and thus does not need to be extended, but the UE shall continue measuring so the PRS-RSRP matches RSTD/UE Rx-Tx

    - Option 3

      - Current requirements in clause 9.9.3 also apply for the case when PRS-RSRP is measured for DL-TDOA or Multi-RTT, the UE stops PRS-RSRP measurement

- Scenario #2: UE being configured to measure PRS-RSRP for different positioning methods

  - If PRS-RSRP measurement is separately configured for each positioning method, the requirements shall be met independently for each positioning method.

For scenario #1, we provided our view in a prior contribution [2].  Briefly, our view is that the measurement period is the same for both the primary measurement (RTSD or UE Rx-Tx) and the secondary measurement (PRS-RSRP) as defined in TS 38.133 sections 9.9.2.5 and 9.9.4.5, respectively, because

- The assistance data is configured for the positioning method (DL-TDOA or Multi-RTT) and it is common to all measurements configured to be reported.

- The number of samples in the measurement period requirement for PRS-RSTD and UE Rx-Tx does not depend on whether PRS-RSRP is reported or not.

Furthermore, RAN4 reached the following agreements in RAN4#96-e [3]:

- When configured with UE Rx-Tx time difference, PRS-RSRP measurement period requirements can be same as that of UE Rx-Tx time difference measurement

- When configured with PRS-RSTD, PRS-RSRP measurement period requirements can be same as that of RSTD measurement

- When not configured with either UE Rx-Tx or RSTD, measurement period requirements of RSTD can be reused for PRS-RSRP measurement.

- Note: The agreements above are valid for the case when the number of samples for PRS-RSRP measurements Nsample = [4]. Agreement can be revisited if more samples are needed for PRS-RSRP measurements.

The first two bullet points in the RAN4#96-e agreement are consistent with our view: there’s only one measurement period requirement for DL-TDOA and Multi-RTT and it’s the same whether or not PRS-RSRP is also measured and reported. The fourth bullet point notes that the number of samples in the agreement may be revisited if more samples are needed for PRS-RSRP but this is the only aspect that should be in question. Therefore, in our view, any further discussion of this topic should focus on whether the number of samples in the measurement period requirement should be revisited.

Proposal 1: The RSTD measurement period requirement is independent of whether or not PRS-RSRP is configured to be reported for DL-TDOA. According to a prior agreement, RAN4 may revise the number of samples (Nsample) in the requirement if more samples are needed to meet PRS-RSRP accuracy requirements.

Proposal 2: The UE Rx-Tx measurement period requirement is independent of whether or not PRS-RSRP is configured to be reported for Multi-RTT. According to a prior agreement, RAN4 may revise the number of samples (Nsample) in the requirement if more samples are needed to meet PRS-RSRP accuracy requirements.

# Conclusions

Proposal 1: The RSTD measurement period requirement is independent of whether or not PRS-RSRP is configured to be reported for DL-TDOA. According to a prior agreement, RAN4 may revise the number of samples (Nsample) in the requirement if more samples are needed to meet PRS-RSRP accuracy requirements.

Proposal 2: The UE Rx-Tx measurement period requirement is independent of whether or not PRS-RSRP is configured to be reported for Multi-RTT. According to a prior agreement, RAN4 may revise the number of samples (Nsample) in the requirement if more samples are needed to meet PRS-RSRP accuracy requirements.

# References

[1] R4-2104076, WF on UE PRS measurement requirements

[2] R4-2102290, On PRS-RSTD measurement requirements

[3] R4-2012263, WF on requirements for PRS-RSRP measurements