3GPP TSG-RAN WG4 Meeting # 98-bis-e                                                       R4-2106383

# Electronic Meeting, Apr. 12-20, 2021

Source:	ZTE Corporation

Title:	TP for TR38.717-03-02_ CA_n8A-n39A-n41A

Agenda Item:	7.11.2

Document for:	Approval

- Introduction

This contribution provides a text proposal to introduce 3DL/2UL CA_n8A-n39A-n41A in TR38.717-03-02.

- Reference

- TR38.717-03-02,Rel-17 NR inter-band Carrier Aggregation for 3 bands DL with 1 band UL,v0.3.0

- Text Proposal

----- Start of TP -----

## 5.1.x	CA_n8-n39-n41

### 5.1.x.1		Operating bands for CA

Table 5.1.x.1-1: 3DL Inter-band CA operating bands

| NR Band | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n8A-n39A-n41A | n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n39 | 1880 MHz | – | 1920 MHz | 1880 MHz | – | 1920 MHz | TDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

### 5.1.x.2		Channel bandwidths per operating band for CA

Table 5.1.x.2-1: Supported channel bandwidths per CA configuration for 3DL inter-band CA

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) (NOTE 3) |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n8A-n39A-n41A | CA_n8A-n39A CA_n8A-n41A CA_n39A-n41A | n8 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n39 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n41 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |

### 5.1.x.3		Co-existence studies

For 3DL/2UL NR CA, only the IMD issues due to dual uplink operation of two bands falling into the DL of the third band shall be verified.

Actually, the co-existence studies for dual uplink operation of two bands, i.e. CA_n8A-n39A, CA_n8A-n41A and CA_n39A-n41A have been captured in TR38.716-02-00, where:

- No IMD products produced  by Band 8 and Band n39 that impact the reference sensitivity of NR band n41.

- No IMD products produced  by Band 8 and Band n41 that impact the reference sensitivity of NR band n39.

- No IMD products produced  by Band 39 and Band n41 that impact the reference sensitivity of NR band n8.

### 5.1.x.4	MSD

According to the co-existence study, no MSD issues need to be specified. Therefore, no additional REFENS requirements need to be specified.

----- End of TP -----