3GPP TSG-RAN WG4 Meeting #98-bis-e		R4-2106415

E-meeting, 12 April – 20 April, 2021

Agenda item:	5.2.1.1

Source:	Intel Corporation

Title:	Discussion on NR V2X requirements structure

Document for:	Discussion

# Introduction

In the RAN4 #98-e meeting, Big Draft CR on V2X requirements was endorsed [1].

In this paper we provide our view on remaining open issues related to NR V2X requirements structure.

# Discussion

## PSSCH RMC

Based on endorsed Big Draft CR [1] PSSCH RMCs are defined in the following format

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PSSCH.2-1.1 | R.PSSCH.2-1.2 | R.PSSCH.2-1.3 |  |
| Channel bandwidth | MHz | 20 | 20 | 20 |  |
| Subcarrier spacing | kHz | 30 | 30 | 30 |  |
| Allocated resource blocks | RB | 20 | 20 | 10 |  |
| CP-OFDM symbols for slot with PSFCH(Note 1) |  | 9 | 9 | 9 |  |
| CP-OFDM symbols for slot without PSFCH |  | 12 | 12 | 12 |  |
| DMRS symbols for slot with PSFCH |  | 3 | 2 | 2 |  |
| DMRS symbols for slot without PSFCH |  | 4 | 3 | 2 |  |
| Modulation order |  | QPSK | 16QAM | 64QAM |  |
| MCS index |  | 4 | 11 | 17 |  |
| Number of MIMO layers |  | 1 | 1 | 1 |  |
| Number of DMRS REs |  |  |  |  |  |
| Transport Block Size for slot with PSFCH | Bits | 704 | 1800 | 984 |  |
| Transport Block Size for slot without PSFCH | Bits | 1128 | 2856 | 1928 |  |
| Transport block CRC | Bits | 24 | 24 | 24 |  |
| Maximum number of HARQ transmissions |  | 1 | 1 | 1 |  |
| Binary Channel Bits for slots with PSFCH |  | 2304 | 5088 | 2232 |  |
| Binary Channel Bits for slots without PSFCH | Bits | 3504 | 7728 | 4392 |  |
| Note 1: OFDM symbols is for PSCCH/PSSCH transmission not including first symbol (AGC), PSFCH symbols, and guard symbols. |  |  |  |  |  |

For definition of PDSCH RMC, we assume that all required parameters for TBS determination should be included in the RMC table. Based on TS 38.214 [2] Section 8.1.3.2 the following TBS determination procedure is defined:

The UE shall first determine the number of REs (NRE) within the slot. - A UE first determines the number of REs allocated for PSSCH within a PRB (NRE') by NRE'=NscRBNsymbsh-NsymbPSFCH-NohPRB-NREDMRS, where … A UE determines the total number of REs allocated for PSSCH ()NRE) by NRE=NRE'∙nPRB-NRESCI,1-NRESCI,2NRE= NRE'* nPRB, where

Number of DMRS REs

Information about number of DMRS symbols for different slots is required to find the number of DMRS REs for TBS determination based on Table 8.1.3.2-1 from TS 38.214 [2]. At current stage, PSSCH RMC table includes fields for both parameters. Therefore, we think that it is redundant, and we can keep only information about number of DMRS REs and remove information about number of DMRS symbols.

Overhead

Information about overhead parameter is missing in current version of PSSCH RMC table. Therefore, it should be included.

Number of SCI REs

For PSSCH TBS determination, information about number of resources elements allocated for SCI1 and SCI2 transmission is required. Number of resource elements allocated for SCI2 transmission can be calculated based on equations from TS 38.212 [3] Section 8.4.4 and parameters which currently are defined in Table 11.1.2.1.1-1 of V2X PSSCH requirements. Therefore, we can just move these parameters from table with PSSCH test configuration to RMC table or just add information about number of resource elements allocated for SCI2 transmission. As for number of resource elements allocated for SCI1 transmission, we can just add a number, i.e. without detailed SCI1 configuration

Proposal 1:	Make the following changes to PSCCH RMC table

- Remove information about number of DMRS symbols and keep only information about number of DMRS REs

- Add information about overhead for TBS determination

- Add information about number of resource elements allocated for SCI1 transmission

- Add information about number of resource elements allocated for SCI2 transmission or add SCI2 configuration which is required for calculation of number of resource elements

## Resource pool

Based on endorsed Big Draft CR [1], resource pool configuration is defined in different formats. Therefore, we suggest to defined unified approach for definition of resource pool configuration.

Based on TS 38.331 [4], SL resource pool has the following configuration

SL-ResourcePool-r16 ::= SEQUENCE { sl-PSCCH-Config-r16 SetupRelease { SL-PSCCH-Config-r16 } OPTIONAL, -- Need M sl-PSSCH-Config-r16 SetupRelease { SL-PSSCH-Config-r16 } OPTIONAL, -- Need M sl-PSFCH-Config-r16 SetupRelease { SL-PSFCH-Config-r16 } OPTIONAL, -- Need M sl-SyncAllowed-r16 SL-SyncAllowed-r16 OPTIONAL, -- Need M sl-SubchannelSize-r16 ENUMERATED {n10, n12, n15, n20, n25, n50, n75, n100} OPTIONAL, Need M dummy INTEGER (10..160) OPTIONAL, -- Need M sl-StartRB-Subchannel-r16 INTEGER (0..265) OPTIONAL, -- Need M sl-NumSubchannel-r16 INTEGER (1..27) OPTIONAL, -- Need M sl-Additional-MCS-Table-r16 ENUMERATED {qam256, qam64LowSE, qam256-qam64LowSE } OPTIONAL, - Need M sl-ThreshS-RSSI-CBR-r16 INTEGER (0..45) OPTIONAL, -- Need M sl-TimeWindowSizeCBR-r16 ENUMERATED {ms100, slot100} OPTIONAL, -- Need M sl-TimeWindowSizeCR-r16 ENUMERATED {ms1000, slot1000} OPTIONAL, -- Need M sl-PTRS-Config-r16 SL-PTRS-Config-r16 OPTIONAL, -- Need M sl-UE-SelectedConfigRP-r16 SL-UE-SelectedConfigRP-r16 OPTIONAL, -- Need M sl-RxParametersNcell-r16 SEQUENCE { sl-TDD-Configuration-r16 TDD-UL-DL-ConfigCommon OPTIONAL, -- Need M sl-SyncConfigIndex-r16 INTEGER (0..15) } OPTIONAL, -- Need M sl-ZoneConfigMCR-List-r16 SEQUENCE (SIZE (16)) OF SL-ZoneConfigMCR-r16 OPTIONAL, -- Need M sl-FilterCoefficient-r16 FilterCoefficient OPTIONAL, -- Need M sl-RB-Number-r16 INTEGER (10..275) OPTIONAL, -- Need M sl-PreemptionEnable-r16 ENUMERATED {enabled, pl1, pl2, pl3, pl4, pl5, pl6, pl7, pl8} OPTIONAL, - Need R sl-PriorityThreshold-UL-URLLC-r16 INTEGER (1..9) OPTIONAL, -- Need M sl-PriorityThreshold-r16 INTEGER (1..9) OPTIONAL, -- Need M sl-X-Overhead-r16 ENUMERATED {n0,n3, n6, n9} OPTIONAL, -- Need S sl-PowerControl-r16 SL-PowerControl-r16 OPTIONAL, -- Need M sl-TxPercentageList-r16 SL-TxPercentageList-r16 OPTIONAL, -- Need M sl-MinMaxMCS-List-r16 SL-MinMaxMCS-List-r16 OPTIONAL, -- Need M ..., [[ sl-TimeResource-r16 BIT STRING (SIZE (10..160)) OPTIONAL -- Need M ]] } SL-PSCCH-Config-r16 ::= SEQUENCE { sl-TimeResourcePSCCH-r16 ENUMERATED {n2, n3} OPTIONAL, -- Need M sl-FreqResourcePSCCH-r16 ENUMERATED {n10,n12, n15, n20, n25} OPTIONAL, -- Need M sl-DMRS-ScrambleID-r16 INTEGER (0..65535) OPTIONAL, -- Need M sl-NumReservedBits-r16 INTEGER (2..4) OPTIONAL, -- Need M ... } SL-PSSCH-Config-r16 ::= SEQUENCE { sl-PSSCH-DMRS-TimePatternList-r16 SEQUENCE (SIZE (1..3)) OF INTEGER (2..4) OPTIONAL, -- Need M sl-BetaOffsets2ndSCI-r16 SEQUENCE (SIZE (4)) OF SL-BetaOffsets-r16 OPTIONAL, -- Need M sl-Scaling-r16 ENUMERATED {f0p5, f0p65, f0p8, f1} OPTIONAL, -- Need M ... } SL-PSFCH-Config-r16 ::= SEQUENCE { sl-PSFCH-Period-r16 ENUMERATED {sl0, sl1, sl2, sl4} OPTIONAL, -- Need M sl-PSFCH-RB-Set-r16 BIT STRING (SIZE (10..275)) OPTIONAL, -- Need M sl-NumMuxCS-Pair-r16 ENUMERATED {n1, n2, n3, n6} OPTIONAL, -- Need M sl-MinTimeGapPSFCH-r16 ENUMERATED {sl2, sl3} OPTIONAL, -- Need M sl-PSFCH-HopID-r16 INTEGER (0..1023) OPTIONAL, -- Need M sl-PSFCH-CandidateResourceType-r16 ENUMERATED {startSubCH, allocSubCH} OPTIONAL, -- Need M ... }

Based on our understanding, resource pool configuration in V2X requirements should include only parameters related to demodulation processing and parameters not included in tables with test configurations. In Table 1 we provide our view on resource pool configuration structure.

Table 1. Example of resource pool configuration definition in Annex

| Information Element |  |  | Value |  |
| --- | --- | --- | --- | --- |
|  |  |  | 20 MHz | 40 MHz |
| SL-ResourcePool-r16 | sl-PSCCH-Config-r16 | sl-TimeResourcePSCCH-r16 | n2 | n2 |
|  |  | sl-FreqResourcePSCCH-r16 | n10 | n10 |
|  | sl-SyncAllowed-r16 |  | gnss-Sync-r16 | gnss-Sync-r16 |
|  | sl-SubchannelSize-r16 |  | n10 | n10 |
|  | sl-TimeResource-r16 |  | ones(1, 160) | ones(1, 160) |
|  | sl-StartRB-Subchannel-r16 |  | 0 | 0 |
|  | sl-NumSubchannel-r16 |  | 5 | 10 |
|  | sl-RB-Number-r16 |  | 51 | 106 |

Another possible option to define the resource pool configuration is to include these parameters in the Table 11.1.1.2-1 with common test parameters in the following format:

Table 2. Example of resource pool configuration definition in common test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Resource pool configuration | PSCCH Time resource | Symbols | 2 |
|  | PSCCH Frequency resource | PRBs | 10 |
|  | Synchronization reference |  | GNSS |
|  | Subchannel size | PRBs | 10 |
|  | Number of sub-channels |  | 5 for 20 MHz and 10 for 40 MHz |
|  | Start PRB for first sub-channel |  | 0 |
|  | Time resource bitmap |  | ones(1, 160) |
|  | Number of PRBs |  | 51 for 20 MHz and 106 for 40 MHz |

Proposal 2:	Define the resource pool configuration in the Annex using example from Table 1 or in the table with common test parameters using example from Table 2.

# Conclusion

In this paper we provided view on remaining open issues related to NR V2X requirements structure and made the following proposals:

Proposal 1:	Make the following changes to PSCCH RMC table

- Remove information about number of DMRS symbols and keep only information about number of DMRS REs

- Add information about overhead for TBS determination

- Add information about number of resource elements allocated for SCI1 transmission

- Add information about number of resource elements allocated for SCI2 transmission or add SCI2 configuration which is required for calculation of number of resource elements

Proposal 2:	Define the resource pool configuration in the Annex using example from Table 1 or in the table with common test parameters using example from Table 2.

# References

- R4-2103976 “Draft Big CR: Introduction of Rel-16 NR V2X demodulation performance requirements”, LGE, RAN4 #98-e, February 2021.

- TS 38.214 “Physical layer procedures for data”

- TS 38.212 “Multiplexing and channel coding”

- TS 38.331 “Radio Resource Control (RRC); Protocol specification”