3GPP TSG-RAN WG4 Meeting # 98-bis-e 	                                  R4-2106455

Electronic Meeting, Apr. 12-20, 2021

Agenda Item:	5.5.2.2.1.3

Source:	Intel Corporation

Title:	Discussion on UE RX-TX time difference measurement accuracy requirements

Document for:		Discussion

# Introduction

In the previous RAN4 meeting, the core requirements for UE RX-TX time difference requirements was completed, and some open issues for the performance requirements were summarized in [1]. For some of requirements can be same as these of PRS RSTD (e.g. number of PRS occasions or PRS repetitions for defining accuracy) [2]. In this contribution we will focus on the discussion of remaining issue below which are unique for UE RX-TX time difference measurement.

- Applicability of accuracy requirements in case of NTA_offset change

- Measurement accuracy requirements

# Discussion

# Applicability of accuracy requirements in case of NTA_offset change

In previous RAN4 meeting, the impacts of Nta_offset on the UE Rx-Tx time difference requirements was addressed.

- Option 1: RAN4 not to capture applicability of UE Rx-Tx time difference accuracy requirements under NTA_offset change during the measurement period - Option 2: Clarify in section 10.1.25.2 in TS 38.133: “UE Rx-Tx time difference accuracy requirements shall not apply if NTA_offset defined in Table 7.1.2-2 in 38.133 changes during the UE Rx-Tx measurement period.”

Thus, from the specification perspective, some necessary clarification or notes is necessary which can also reflect RAN4’s agreement precisely.

Proposal 1 : UE Rx-Tx measurement requirements in TS38.133 shall be applicable unless the NTA_offset changes during the measurement period.

# Measurement accuracy requirements

As agreed in the last meeting, how to define the accuracy requirements of UE Rx-Tx time diffference with the parameters combinations (e.g. PRS BW, DL-PRS-NumSymbols ,DL-PRS_ResourceRepetitionFactor and DL-PRS-CombSizeN, e.t.c) can follow the same principle for RSTD measurement. Thus, base on our discussion for RSTD accuracy requirements [3] and corresponding simulation results provided in Appendix, we can conclude that :

Proposal 2: The accuracy of UE Rx-Tx time difference can be specified by the Table 1 below.

Table 1: UE Rx-Tx difference measurement accuracy requirements (ns)

| FR | PRS BW (PRBs) | PRS_NormLenthPerSlot | SINR= [-3dB] | SINR= -13dB |
| --- | --- | --- | --- | --- |
| FR1 | 52 | 1 | TBD | TBD |
|  |  | ≥2 | TBD | TBD |
|  |  | ≥4 | TBD | TBD |
|  |  | ≥6 | TBD | TBD |
|  | 104 | 1 | TBD | TBD |
|  |  | ≥2 | TBD | TBD |
|  |  | ≥4 | TBD | TBD |
|  |  | ≥6 | TBD | TBD |
|  | 268 | 1 | TBD | TBD |
|  |  | ≥2 | TBD | TBD |
|  |  | ≥4 | TBD | TBD |
|  |  | ≥6 | TBD | TBD |
| FR2 | 32 | 1 | TBD | TBD |
|  |  | ≥2 | TBD | TBD |
|  |  | ≥4 | TBD | TBD |
|  |  | ≥6 | TBD | TBD |
|  | 64 | 1 | TBD | TBD |
|  |  | ≥2 | TBD | TBD |
|  |  | ≥4 | TBD | TBD |
|  |  | ≥6 | TBD | TBD |
|  | 128 | 1 | TBD | TBD |
|  |  | ≥2 | TBD | TBD |
|  |  | ≥4 | TBD | TBD |
|  |  | ≥6 | TBD | TBD |
| Note 1: PRS_NormLenthPerSlot = (DL-PRS-NumSymbols x DL-PRS_ResourceRepetitionFactor) /DL-PRS-CombSizeN |  |  |  |  |

# Conclusion

In this contribution, some considerations on UE RX-TX time difference measurement accuracy requirements are provided and the following observations and proposals can be drawn:

Proposal 1 : UE Rx-Tx measurement requirements in TS38.133 shall be applicable unless the NTA_offset changes during the measurement period.

Proposal 2: The accuracy of UE Rx-Tx time difference can be specified by the Table 1 below.

Table 1: UE Rx-Tx difference measurement accuracy requirements (ns)

| FR | PRS BW (PRBs) | PRS_NormLenthPerSlot | SINR= [-3dB] | SINR= -13dB |
| --- | --- | --- | --- | --- |
| FR1 | ≥52 | ≥1 | TBD | TBD |
|  |  | ≥2 | TBD | TBD |
|  |  | ≥4 | TBD | TBD |
|  |  | ≥6 | TBD | TBD |
|  | ≥104 | ≥1 | TBD | TBD |
|  |  | ≥2 | TBD | TBD |
|  |  | ≥4 | TBD | TBD |
|  |  | ≥6 | TBD | TBD |
|  | ≥268 | 1 | TBD | TBD |
|  |  | ≥2 | TBD | TBD |
|  |  | ≥4 | TBD | TBD |
|  |  | ≥6 | TBD | TBD |
| FR2 | ≥32 | ≥1 | TBD | TBD |
|  |  | ≥2 | TBD | TBD |
|  |  | ≥4 | TBD | TBD |
|  |  | ≥6 | TBD | TBD |
|  | ≥64 | ≥1 | TBD | TBD |
|  |  | ≥2 | TBD | TBD |
|  |  | ≥4 | TBD | TBD |
|  |  | ≥6 | TBD | TBD |
|  | ≥128 | ≥1 | TBD | TBD |
|  |  | ≥2 | TBD | TBD |
|  |  | ≥4 | TBD | TBD |
|  |  | ≥6 | TBD | TBD |
| Note 1: PRS_NormLenthPerSlot = (DL-PRS-NumSymbols x DL-PRS_ResourceRepetitionFactor) /DL-PRS-CombSizeN |  |  |  |  |

# References

- R4-2103586, “WF on requirements on NR Pos performance requirements”

- 3GPP TS 38.133 v16.5.0: "NR: Requirements for support of radio resource management "

- R4-2106454, “Discussion on NR PRS RSTD measurement accuracy requirements”

# Appendix: Link level simulation results

Notes: these results will be collected during R4#98-bis-e meeting.

Table A1: UE Rx-Tx time difference measurement accuracy simulation results in fading channels

| FR | PRS BW (PRBs) | SCS(kHz) | sampling rate (Tc) | Number of sample | DL-PRS-NumSymbols | DL-PRS-CombSizeN | DL-PRS_ResourceRepetitionFactor | Accuracy @ [90%] (Tc), SINR [-3dB] | Accuracy @ [90%] (Tc), SINR [-13dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Intel | Intel |
| FR1 | 24 | 15 | 128 | 4 | 2 | 2 | 1 | 256 | 266 |
|  |  |  |  | 4 | 2 | 2 | 4 | 256 | 246 |
|  | 52 | 15 | 128 | 4 | 4 | 4 | 1 | 128 | 236 |
|  |  |  |  | 4 | 2 | 2 | 2 | 128 | 236 |
|  |  |  |  | 4 | 2 | 2 | 4 | 128 | 236 |
|  |  |  |  | 4 | 2 | 2 | 6 | 128 | 236 |
|  |  |  |  | 4 | 12 | 4 | 1 | 128 | 236 |
|  | 48 | 30 | 64 | 4 | 4 | 4 | 1 | 128 | 192 |
|  |  |  |  | 4 | 2 | 2 | 2 | 128 | 192 |
|  |  |  |  | 4 | 2 | 2 | 4 | 128 | 128 |
|  |  |  |  | 4 | 2 | 2 | 6 | 128 | 64 |
|  |  |  |  | 4 | 12 | 4 | 1 | 128 | 128 |
|  | 104 | 15 | 64 | 4 | 4 | 4 | 1 | 128 | 148 |
|  |  |  |  | 4 | 2 | 2 | 2 | 128 | 84 |
|  |  |  |  | 4 | 2 | 2 | 4 | 128 | 84 |
|  |  |  |  | 4 | 2 | 2 | 6 | 64 | 84 |
|  |  |  |  | 4 | 12 | 4 | 1 |  |  |
|  | 132 | 30 | 32 | 4 | 4 | 4 | 1 | 64 | 320 |
|  |  |  |  | 4 | 2 | 2 | 2 | 64 | 224 |
|  |  |  |  | 4 | 2 | 2 | 4 | 64 | 96 |
|  |  |  |  | 4 | 4 | 4 | 1 |  |  |
|  |  |  |  | 4 | 12 | 4 | 1 | 64 |  |
|  | 268 | 15 | 32 | 4 | 2 | 2 | 1 | 64 | 86 |
|  |  |  |  | 4 | 2 | 2 | 2 | 64 | 86 |
|  |  |  |  | 4 | 2 | 2 | 4 | 64 | 86 |
|  |  |  |  | 4 | 2 | 2 | 6 | 64 | 86 |
|  |  |  |  | 4 | 12 | 4 | 1 | 32 | 54 |
|  | 272 | 30 | 16 | 4 | 4 | 4 | 1 | 32 | 48 |
|  |  |  |  | 4 | 2 | 2 | 2 | 32 | 48 |
|  |  |  |  | 4 | 2 | 2 | 4 | 16 | 48 |
|  |  |  |  | 4 | 2 | 2 | 6 | 16 | 32 |
|  |  |  |  | 4 | 12 | 4 | 1 | 16 | 32 |
| FR2 | 24 | 120 | 16 | 4 | 2 | 2 | 1 | 352 | 608 |
|  | 32 | 120 | 16 | 4 | 4 | 4 | 1 | 128 | 336 |
|  |  |  |  | 4 | 2 | 2 | 2 | 128 | 128 |
|  |  |  |  | 4 | 2 | 2 | 4 | 128 | 128 |
|  |  |  |  | 4 | 2 | 2 | 6 |  |  |
|  | 64 | 120 | 16 | 4 | 4 | 4 | 1 | 144 | 144 |
|  |  |  |  | 4 | 2 | 2 | 2 | 128 | 144 |
|  |  |  |  | 4 | 2 | 2 | 4 | 128 | 144 |
|  |  |  |  | 4 | 2 | 2 | 6 | 128 | 84 |
|  | 128 | 120 | 8 | 4 | 4 | 4 | 1 | 120 | 152 |
|  |  |  |  | 4 | 2 | 2 | 2 | 120 | 84 |
|  |  |  |  | 4 | 2 | 2 | 4 | 120 | 84 |
|  |  |  |  | 4 | 2 | 2 | 6 | 120 | 84 |