3GPP TSG-RAN WG4 Meeting # 98-bis-e                                                        R4-2106476

Electronic Meeting, Apr. 12-20, 2021

Title: 	Simulation assumption for NTN co-existence study

Source: 	CATT

Agenda item:	8.8.2.1

Document for:	Discussion

- Introduction

This document captures initial simulation assumptions for the NTN coexistence study.

- Discussion

2.1 Co-existence simulation scenarios

In [3], the proposed scenarios for coexistence study are duplicated in the following table.

Table 2.1-1 Scenarios for NTN-NTN/TN co-existence

| FR1: 2GHz |  |  | Set 1 |  |  | Set 2 |  |  | HAPS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | GEO | LEO 600km | LEO 1200km | GEO | LEO 600km | LEO 1200km |  |
| NR / NB-IoT | Rural |  | X | X | X | X | X | X | FFS |
|  | Urban macro |  | X | X | X | X | X | X | FFS |
|  | Dense Urban |  | X | X | X | X | X | X | FFS |
|  | Indoor |  | X | X | X | X | X | X |  |
| NTN | GEO | Set 1 | X | X | X | N/A | N/A | N/A | FFS |
|  | LEO 1200km |  | X | X | X | N/A | N/A | N/A | FFS |
|  | LEO 600km |  | X | X | X | N/A | N/A | N/A | FFS |
|  | GEO | Set 2 | N/A | N/A | N/A | X | X | X | FFS |
|  | LEO 1200km |  | N/A | N/A | N/A | X | X | X | FFS |
|  | LEO 600km |  | N/A | N/A | N/A | X | X | X | FFS |
| Note 1: Start with Earth Fixed beam first, Earth Moving Beams could be further discussed Note 2: Set 1 and Set 2 could be found in Table 6.1.1.1-6 of TR 38.821. FFS if one set would be more stringent and so, if all simulations would be needed for both sets. Note 3: GEO and LEO only operate at adjacent channel. |  |  |  |  |  |  |  |  |  |

The aggressor and victim combination is list in Table 2.1-2.

Table 2.1-2 Aggressor and victim

| No. | Combination | Aggressor | Victim | Notes |
| --- | --- | --- | --- | --- |
| 1 | TN with NTN | TN DL | NTN DL |  |
| 2 | TN with NTN | TN UL | NTN UL |  |
| 3 | TN with NTN | NTN DL | TN DL |  |
| 4 | TN with NTN | NTN UL | TN UL |  |
| 5 | TN with NTN | NTN UL | TN DL | Applicable for satellite operating in S band, e.g. coexistence with Band 34 TDD. |
| 6 | TN with NTN | TN DL | NTN UL | Applicable for satellite operating in S band, e.g. coexistence with Band 34 TDD. |
| 7 | NTN with NTN | NTN DL | NTN DL | LEO-LEO or GEO-GEO |
|  |  | NTN UL | NTN UL | LEO-LEO or GEO-GEO |

The proposed frequency and bandwidth are listed as table 2.1-3.

Table 2.1-3.  Proposed frequency and bandwidth for co-existence study

|  | Frequency | Bandwidth | Duplex mode | Frequency reuse factor |
| --- | --- | --- | --- | --- |
| Rural | 2 GHz | TBD | FDD, TDD | [1] |
| Urban macro | 2 GHz | TBD | FDD, TDD | [1] |
| Dense Urban | 2 GHz | TBD | FDD, TDD | [1] |
| GEO | 2 GHz | 30 MHz for FR1 | FDD | [2] or [3] |
| LEO | 2 GHz | 30 MHz for FR1 | FDD | [2] or [3] |
| HAPS | 2 GHz | TBD | FDD | [1] |

2.2. Network layout model

Cellular cell structure is considered for both NTN and TN network layout.

Co-existence between NTN and TN

For co-existence between NTN and TN, it is proposed to consider [TBD] satellite(s) and the layout is FFS. The number of TN IMT BS should be large enough to emulate the interference seen by the satellite from the IMT systems. It is FFS on exact range of TN BS deployment based on simulations.

Co-existence between NTN and NTN

For co-existence between NTN and NTN, the following 2 cases are considered as [candidate options].

- One satellite carries two neighbour carriers, where the footprints of the 2 carriers are the same and coordinated see figure 2.2-1.

- Two satellites (GEO and LEO) operate on two neighbour carriers but at different height, see figure 2.2-2. The number of LEO satellite and footprints are FFS.

Co-existence between HAPS and TN

For co-existence between HAPS and TN, the exact layout is FFS.

Figure2.1-1 Layout for coexistence between NTN and TN (TBD)

Figure 2.2-2 Layout for coexistence between NTN systems

Figure 2.2-3 Layout for coexistence between NTN systems (different height satellites)

2.3. Simulation parameters

Two sets of satellite parameters are proposed in Table 2.3-1 and Table 2.3-2 according to TR 38.821.

Table 2.3-0 Simulation assumptions of TN respectively based on NB-IoT and NR

|  | NB-IoT standalone | NR |
| --- | --- | --- |
| Carrier frequency in GHz | 2 | 2 |
| Size of each nominal channel BW in MHz | 0.2 | 20 |
| Transmission bandwidth in MHz | 0.18 | 9 |
| Environment | Urban macro Sub-urban Rural | Urban macro Sub-urban Rural |
| Network layout | 19-sites [57 sectors] with wrap-around | 19-sites [57 sectors] with wrap-around |
| Inter-site distance in meter | 500 for 2GHz band for UMA | 500 for 2GHz band for UMA |
| System loading and activity | Full buffer 100% | Full buffer 100% |
| Network location | FFS | FFS |
| DL subcarrier spacing | 15kHz | 15kHz |
| UL | See RP-152284 | OFDMA |
| DL power control | No | No |
| UL power control | 36.942 section 5.1.1.6 (set 1) by bandwidth scale, target SNR at BS is 15 dB | 36.942 section 5.1.1.6 (set=1) |
| Frequency reuse | 1 | 1 |
| Number of scheduled UE per cell (DL) | 1 | 1 |
| Number of scheduled UE per cell (UL) | 3 for multi-tone (60kHz per UE), 12 for 15kHz single-tone, 48 for 3.75kHz single-tone | 3 |
| BS antenna height in meter | 30 | 30 |
| BS max TX power in dBm | 43dBm/200kHz | 46 |
| BS antenna gain including feeder loss in dBi | 15 | 15 |
| BS antenna pattern | Horizontal (36.942) | [8x4x2 AAS BS] |
| BS antenna front-back ratio in dB | 20 | 20 |
| UE antenna height in meter | 1.5 | 1.5 |
| UE TX power in dBm | -40 to 23 | -40 to 23 |
| UE antenna gain in dBi | 0 | 0 |
| Building penetration loss | 45.820 Annex D.1 | n/a |
| Cell selection margin in dB | 3 | 3 |
| BS-MS min couple loss in dB | 70 | 70 |
| BS noise figure in dB | 5 | 5 |
| UE noise figure in dB | 9 | 9 |
| BS-UE path-loss model | TR36.942 macro urban | TR38.803 |
| Standard deviation of BS-UE log-normal shadow fading in dB | 10 | 10 |
| Shadowing correlation | Inter-cell 0.5 intra-cell 1 | Inter-cell 0.5 intra-cell 1 |
| Link-level performance model |  | section 2.9 |
| Evaluation metrics | SINR vs ACS (as victim) | SINR and throughput loss vs standalone NB-IoT ACLR (as victim); |

Table 2.3-1 Set-1 satellite parameters for co-existence study

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 1200 km | 600 km |
| Payload characteristics for DL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture |  | 22 m | 2 m | 2 m |
| Satellite EIRP density | 2GHz | 59 dBW/MHz | 40 dBW/MHz | 34 dBW/MHz |
| Satellite Tx max Gain |  | 51 dBi | 30 dBi | 30 dBi |
| 3dB beamwidth |  | 0.4011 deg | 4.4127 deg | 4.4127 deg |
| Satellite beam diameter (Note 2) |  | 250 km | 90 km | 50 km |
| Payload characteristics for UL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note1) | 2 GHz | 22 m | 2 m | 2 m |
| G/T |  | 19 dB K-1 | 1.1 dB K-1 | 1.1 dB K-1 |
| Satellite Rx max Gain |  | 51 dBi | 30 dBi | 30 dBi |
| NOTE 1: This value is equivalent to the antenna diameter in Sec. 6.4.1 of [2]. NOTE 2: This beam size refers to the Nadir pointing of the satellite NOTE 3: All these satellite parameters are applied per beam. NOTE 4: The EIRP density values are considered identical for all frequency re-use factor options. NOTE 5: The EIRP density values are provided assuming the satellite HPA is operated with a back-off of [5] dB. |  |  |  |  |

Table 2.3-2 Set-2 satellite parameters for co-existence study

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 1200 km | 600 km |
| Payload characteristics for DL transmissions |  |  |  |  |
| Satellite EIRP density | 2GHz | 53.5 dBW/MHz | 34 dBW/MHz | 28 dBW/MHz |
| Satellite Tx max Gain |  | 45.5 dBi | 24 dBi | 24 dBi |
| 3dB beamwidth |  | 0.7353 deg | 8.8320 deg | 8.8320 deg |
| Satellite beam diameter |  | 450 km | 190 km | 90 km |
| Payload characteristics for UL transmissions |  |  |  |  |
| G/T | 2 GHz | 14 dB K-1 | -4.9 dB K-1 | -4.9 dB K-1 |
| Satellite Rx max Gain |  | 45.5 dBi | 24 dBi | 24 dBi |

Table 2.3-3 HAPS parameters for co-existence study (TBD)

Table 2.3-4 UE characteristics for co-existence study

| Characteristics | Handheld |
| --- | --- |
| Frequency band | 2 GHz |
| Polarisation | Linear: +/-45°X-pol |
| Rx Antenna gain | 0 dBi per element |
| Antenna temperature | 290 K |
| Noise figure | 7 dB |
| Tx transmit power | 200 mW (23 dBm) |
| Tx antenna gain | 0 dBi per element |

Table 2.3-5 ACLR/ACS for TN  (2GHz)

|  |  | NR | NB-IOT |
| --- | --- | --- | --- |
| BS | ACLR | 45 dB | 40 dB |
|  | ACS | 45 dB | 45 dB |
| UE | ACLR | 30dB (ACLR1) 43dB (ACLR2) | 37 |
|  | ACS | 33 | 28 |

Table 2.3-6: Other simulation parameters for NR

| Parameters | NR | NB-IOT | NTN |
| --- | --- | --- | --- |
| Carrier frequency | 2GHz | 2GHz | 2GHz |
| Channel bandwidth | 20MHz | 200kHz | 5MHz |
| Scheduled channel bandwidth per UE (DL) |  |  |  |
| Scheduled channel bandwidth per UE (UL) |  |  |  |
| The number of active UE (DL) (Note 1) | 1 | 1 | 1 |
| The number of active UE (UL) (Note 1) | 1/3 | 1/3 | 1/3 |
| Traffic model | Full buffer | Full buffer | Full buffer |
| DL power control | NO | NO | NO |
| UL power control | YES | YES | TBD |
| BS max TX power in dBm | 43 | 43 | TBD |
| UE max TX power in dBm | 23 | 23 | 23 |
| UE min TX power in dBm | -33 | -33 | -33 |
| BS Noise figure in dB | 5 (@2GHz) | 5 (@2GHz) | TBD |
| UE Noise figure in dB | 9 | 9 | 7 |
| Handover margin | 3dB | 3dB | 3dB |
| Note 1: Further down scope between 1 and 3 will be done based on later simulations. |  |  |  |

  - Antenna and beam forming pattern modelling

2.4.1 Satellite and UE Antenna and beam forming pattern modelling

Satellite and UE Antenna and beam forming pattern modelling of satellite could be referred to section 6.1.1.1 in TR 38.821 [5].

Satellite antenna pattern

The following normalized antenna gain pattern, corresponding to a typical reflector antenna with a circular aperture, is considered

1

where J1(x) is the Bessel function of the first kind and first order with argument x, a is the radius of the antenna's circular aperture, k = 2f/c is the wave number, f is the frequency of operation, c is the speed of light in a vacuum and  is the angle measured from the bore sight of the antenna's main beam. Note that ka equals to the number of wavelengths on the circumference of the aperture and is independent of the operating frequency.

The normalized gain pattern for a = 10 c/f (aperture radius of 10 wavelengths) is shown in Figure 2.4.1-1.

Figure 2.4.1-1: Satellite antenna gain pattern for aperture radius 10 wavelengths, a=10 c/f

HAPS antenna pattern

Two different antenna patterns are considered:

-	The above antenna pattern defined for satellite scenarios, based on the Bessel function.

-	The 3GPP antenna pattern defined for the base station in Section 7.3 of [12], corresponding to a uniform rectangular panel array with dual linear polarization.

Satellite and UE beam forming pattern

The following table is agreed for the beam layout definition for a single satellite simulation

Table 2.4.1-1: Beam layout definition for single satellite simulation

| Scenario | Scenario A, C2 and D2 |
| --- | --- |
| Beam layout definition | Baseline: Hexagonal mapping of the beam bore sight directions on UV plane defined in the satellite reference frame. Only the 3dB beam width parameters should be used. The beam diameter and beam spacing values can be computed directly from the 3 dB beam width assumptions and should be considered as informative. |
| Number of beams | Baseline: 7-beam layout (i.e. 6 co-frequency beams surrounding the central beam) |
| UV plane illustration (extracted from [19]) |  |
| UV plane convention | U axis is defined as the perpendicular line to the satellite-earth line on the orbital plane as illustrated here after: The straight line being orthogonal to UV plane is pointing towards the Earth centre. UV coordinates of the nadir of the reference satellite is (0,0) |
| Adjacent beam spacing on UV plane | Baseline: Adjacent beam spacing computation based on 3dB beam width of the satellite antenna pattern: ABS = sqrt(3) x sin(HPBW/2 [rad]) |
| Central beam bore sight direction definition | Baseline: Case 1: Central beam center is considered at nadir point |

| Frequency re-use factor | Option 1: 1 Option 2: 3 Option 3: 2 if polarization re-use is enabled |
| --- | --- |
| Polarization re-use | Option 1: Disable Option 2: Enable Note: Polarization re-use should apply only if circular polarization for terminal antenna is considered |
| Channel model | Large scale model of [2] (Note 2) |
| Deployment scenarios | Base-line: Rural Additional deployment scenario results can be provided |
| Propagation conditions | Base-line: Clear Sky Line of sight |
| UEs outdoor/indoor distribution | 100% outdoor distribution for UEs |
| UE distribution | Base-line for calibration: at least X=10 UEs per beam with uniform distribution in all the Voronoi cell area associated to each beam. The cell area associated to a given beam is defined as the Voronoi cell associated with the corresponding beam centers. |
| UE configuration | S-band: Handheld (optional for scenario A) |
| UE orientation | VSAT and Others: Ideal Tracking serving beam; Handheld: Random |
| Handover Margin | 0 dB |
| UE attachment | RSRP |
| Metrics for calibration | Base-line: Coupling loss, Geometry Note: Coupling loss is defined as the signal loss from the antenna port to the antenna port |
| NOTE 1: Typical impairment values (additional frequency error, SNR loss) due to the feeder link except for delay can be considered to be negligible. When available, specific values can be considered in the evaluation and should be reported. NOTE 2: For the calibration purpose, the ionospheric scintillation loss shall be considered equal to zero (i.e., the UEs are located between 20 and 60 degrees of latitude). The atmospheric absorptions loss shall be considered. |  |

The beam layout parameters captured in the following table are adopted as a starting point for single satellite simulations.

Table 2.4.1-2: Beam layout parameters for single satellite simulation

| Scenario | Scenario A | Scenario C2/D2 |
| --- | --- | --- |
| Carrier frequency | S-band: 2 GHz Ka-band: 20 GHz for DL | S-band: 2 GHz Ka-band: 20 GHz for DL |
| Adjacent beam spacing (ABS) on UV plane | S-band: Set 1: ABS = 0.0061 Set 2:ABS = 0.0111 Ka-band: Set 1: ABS = 0.0027 Set 2: ABS = 0.0067 | S-band: Set 1: ABS = 0.0668 Set 2: ABS = 0.1334 Ka-band: Set 1: ABS = 0.0267 Set 2: ABS = 0.0667 |
| Satellite location | Any position on the geostationary orbit | Any position on the LEO orbit |
| Central beam center elevation angle target | Baseline: 45 deg | Baseline: 90 deg |
| Central beam bore sight direction coordinates in UV plane | Baseline: (0.107,0) | Baseline: (0,0) |
| Gateway direction coordinates in UV plane | Baseline: Same as central beam bore sight direction coordinates in UV plane Note: Not needed for calibration |  |

2.4.2 TN BS and UE Antenna and beam forming pattern modelling

Antenna and beam forming pattern modelling of TN BS and UE could be referred to TR 38.803 [6].

BS antennas

,

is the 3dB beam width which corresponds to 65 degrees, and  is the maximum attenuation

Antenna heights and gains for macro cells are given in table 2.4.2-1.

Table 2.4.2-1: Antenna height and gain for Macro Cells

|  | Rural Area | Urban Area |  |
| --- | --- | --- | --- |
|  | 900 MHz | 2000 MHz | 900 MHz |
| BS antenna gain (dBi) (including feeder loss) | 15 | 15 | 12 |
| BS antenna height (m) | 45 | 30 | 30 |

UE antenna

For UE antennas, an omni-directional radiation pattern with antenna gain 0dBi is assumed.The antenna and beam forming pattern modeling for HAPS is FFS.

2.5. Propagation model between NTN and UE

Propagation model between NTN and UE could be referred to section 6.6 in TR 38.811 [5].

2.5.1	LOS probability

Line-Of-Sight (LOS) probability depends on UE environment and elevation angle, and is obtained from Table 2.5.1-1. Reference elevation angles are considered from 10° to 90° with a 10° step. For an UE-to-satellite or UE-to-HAPS link, the LOS probability is taken from the nearest reference elevation angle.

Table 2.5.1-1 LOS probability

| Elevation | Dense urban scenario | Urban scenario | Suburban and Rural scenarios |
| --- | --- | --- | --- |
| 10° | 28.2% | 24.6% | 78.2% |
| 20° | 33.1% | 38.6% | 86.9% |
| 30° | 39.8% | 49.3% | 91.9% |
| 40° | 46.8% | 61.3% | 92.9% |
| 50° | 53.7% | 72.6% | 93.5% |
| 60° | 61.2% | 80.5% | 94.0% |
| 70° | 73.8% | 91.9% | 94.9% |
| 80° | 82.0% | 96.8% | 95.2% |
| 90° | 98.1% | 99.2% | 99.8% |

2.5.2	Path loss and Shadow fading

The signal path between a satellite or HAPS transmitter and an NTN terminal undergoes several stages of propagation and attenuation. The path loss (PL) is composed of components as follows:

,

where	is the total path loss in dB,

is the basic path loss in dB,

is the attenuation due to atmospheric gasses in dB,

is the attenuation due to either ionospheric or tropospheric scintillation in dB,

is building entry loss in dB.

This section specifies the basic path loss model () which accounts for the signal's free space propagation, clutter loss, and shadow fading.

The free space path loss (FSPL) in dB for a separation distance d in meter and frequency fc in GHz is given by

(6.6-2)

For a ground terminal, the distance d (a.k.a. slant range), as shown in Figure 2.5.2-1, can be determined by the satellite/HAPS altitude  and elevation angle α by

,

where  denotes Earth radius.

Figure 2.5.2-1: Slant range d between a satellite and a ground terminal

Clutter loss (CL) models the attenuation of signal power caused by surrounding buildings and objects on the ground. It depends on the elevation angle α, the carrier frequency fc, and the environment. Shadow fading (SF) is modeled by a log-normal distribution, which when expressed in decibel unit, is a zero-mean normal distribution with a standard deviation , i.e., .

The basic path loss in dB unit is modeled as

,

where FSPLd, fc is the free space path loss, is clutter loss, and  is shadow fading loss represented by a random number generated by the normal distribution, i.e.,  ~SF~N0, σSF2. When the UE is in LOS condition, clutter loss is negligible and should be set to 0 dB in the basic path loss model.

The values of σSF and  are given in tables 2.5.2-1 to 2.5.2-3 at reference elevation angles for different scenarios. The UE in a particular scenario should take the values corresponding to the reference angle nearest to its elevation angle α.

Table 2.5.2-1: Shadow fading and clutter loss for dense urban scenario

| Elevation | S-band |  |  | Ka-band |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | LOS | NLOS |  | LOS | NLOS |  |
|  | σSF (dB) | (dB) | (dB) | σSF (dB) | σSF (dB) | (dB) |
| 10° | 3.5 | 15.5 | 34.3 | 2.9 | 17.1 | 44.3 |
| 20° | 3.4 | 13.9 | 30.9 | 2.4 | 17.1 | 39.9 |
| 30° | 2.9 | 12.4 | 29.0 | 2.7 | 15.6 | 37.5 |
| 40° | 3.0 | 11.7 | 27.7 | 2.4 | 14.6 | 35.8 |
| 50° | 3.1 | 10.6 | 26.8 | 2.4 | 14.2 | 34.6 |
| 60° | 2.7 | 10.5 | 26.2 | 2.7 | 12.6 | 33.8 |
| 70° | 2.5 | 10.1 | 25.8 | 2.6 | 12.1 | 33.3 |
| 80° | 2.3 | 9.2 | 25.5 | 2.8 | 12.3 | 33.0 |
| 90° | 1.2 | 9.2 | 25.5 | 0.6 | 12.3 | 32.9 |

Table 2.5.2-2: Shadow fading and clutter loss for urban scenario

| Elevation | S-band |  |  | Ka-band |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | LOS | NLOS |  | LOS | NLOS |  |
|  | σSF (dB) | (dB) | (dB) | σSF (dB) | σSF (dB) | (dB) |
| 10° | 4 | 6 | 34.3 | 4 | 6 | 44.3 |
| 20° | 4 | 6 | 30.9 | 4 | 6 | 39.9 |
| 30° | 4 | 6 | 29.0 | 4 | 6 | 37.5 |
| 40° | 4 | 6 | 27.7 | 4 | 6 | 35.8 |
| 50° | 4 | 6 | 26.8 | 4 | 6 | 34.6 |
| 60° | 4 | 6 | 26.2 | 4 | 6 | 33.8 |
| 70° | 4 | 6 | 25.8 | 4 | 6 | 33.3 |
| 80° | 4 | 6 | 25.5 | 4 | 6 | 33.0 |
| 90° | 4 | 6 | 25.5 | 4 | 6 | 32.9 |

Table 2.5.2-3: Shadow fading and clutter loss for suburban and rural scenarios

| Elevation | S-band |  |  | Ka-band |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | LOS | NLOS |  | LOS | NLOS |  |
|  | σSF(dB) | σSF(dB) | (dB) | σSF(dB) | σSF(dB) | (dB) |
| 10° | 1.79 | 8.93 | 19.52 | 1.9 | 10.7 | 29.5 |
| 20° | 1.14 | 9.08 | 18.17 | 1.6 | 10.0 | 24.6 |
| 30° | 1.14 | 8.78 | 18.42 | 1.9 | 11.2 | 21.9 |
| 40° | 0.92 | 10.25 | 18.28 | 2.3 | 11.6 | 20.0 |
| 50° | 1.42 | 10.56 | 18.63 | 2.7 | 11.8 | 18.7 |
| 60° | 1.56 | 10.74 | 17.68 | 3.1 | 10.8 | 17.8 |
| 70° | 0.85 | 10.17 | 16.50 | 3.0 | 10.8 | 17.2 |
| 80° | 0.72 | 11.52 | 16.30 | 3.6 | 10.8 | 16.9 |
| 90° | 0.72 | 11.52 | 16.30 | 0.4 | 10.8 | 16.8 |

2.6. Propagation model between TN and UE

Propagation model between TN BS and UE could be referred to section 5.2.2 in TR 38.803 [6].

2.6.1	Path loss

The Path loss model is summarized in Table 2.6.1-1 and the distance definitions are indicated in Figure 2.6.1-1. Note that the distribution of the shadow fading is log-normal, and its standard deviation for each scenario is given in Table 2.6.1-1.

Table 2.6.1-1: Pathloss models

| Scenario | Pathloss [dB], fc is in GHz and d is in meters (6) | Shadow fading std [dB] | Applicability range, antenna height default values |
| --- | --- | --- | --- |
| UMa LOS |  | σSF=4.0 σSF=4.0 | 10m < d2D < d'BP 1) d'BP < d2D <5000m 1.5m ≦ hUT≦ 22.5m hBS = 25 m |
| UMa NLOS |  | σSF =6 | 10 m < d2D < 5 000 m 1.5 m ≦ hUT ≦ 22.5 m hBS = 25 m Explanations: see note 3 |
| UMi - Street Canyon LOS |  | σSF=4.0 σSF=4.0 | 10m < d2D < d'BP 1) d'BP < d2D <5000m 1.5m ≦ hUT≦ 22.5m hBS = 10 m |
| UMi – Street Canyon NLOS |  | σSF=7.82 | 10 m < d2D < 5000m 1.5m ≦ hUT≦ 22.5m hBS = 10 m Explanations: see note 4 |
| InH - Office LOS |  | σSF=3.0 | 1<d3D<100m |
| InH - Office NLOS |  | σSF=8.03 | 1<d3D<86m |
| Note 1: d'BP = 4 h'BS h'UT fc/c, where fc is the centre frequency in Hz, c = 3.0108 m/s is the propagation velocity in free space, and h'BS and h'UT are the effective antenna heights at the BS and the UT, respectively. In UMi scenario the effective antenna heights h'BS and h'UT are computed as follows: h'BS = hBS – 1.0 m, h'UT = hUT–1.0 m, where hBS and hUT are the actual antenna heights, and the effective environment height is assumed to be equal to 1.0 m. In UMa scenario the effective antenna heights h'BS and h'UT are computed as follows: h'BS = hBS – hE, h'UT = hUT – hE, where hBS and hUT are the actual antenna heights, and the effective environment height hE is a function of the link between a BS and a UT. In the event that the link is determined to be LOS, hE=1m with a probability equal to 1/(1+C(d2D, hUT)) and chosen from a discrete uniform distribution uniform(12,15,…,(hUT-1.5)) otherwise. Note 2: The applicable frequency range of the PL formula in this table is 0.8 < fc < fH GHz, where fH = 30 GHz for RMa and fH = 100 GHz for all the other scenarios. It is noted that RMa pathloss model for >7 GHz is validated based on a single measurement campaign conducted at 24 GHz. Note 3: UMa NLOS pathloss is from TR36.873 with simplified formatand and PLUMa-LOS = Pathloss of UMa LOS outdoor scenario. Note 4: PLUMi-LOS = Pathloss of UMi-Street Canyon LOS outdoor scenario. Note 5: Break point distance dBP = 2π hBS hUT fc/c, where fc is the centre frequency in Hz, c = 3.0 108 m/s is the propagation velocity in free space, and hBS and hUT are the antenna heights at the BS and the UT, respectively. Note 6: fc denotes the center frequency normalized by 1GHz, all distance related values are normalized by 1m, unless it is stated otherwise. |  |  |  |

| Definition of d2D and d3D for outdoor UEs | Definition of d2D-out, d2D-in and d3D-out, d3D-in for indoor UEs. Note that d3D-out+d3D-in=d2D-out+d2D-in2+hBS-hUT2 |
| --- | --- |

Figure 2.6.1-1: Distance definitions

2.6.2	LOS probability

The Line-Of-Sight (LOS) probabilities are given in Table 2.6.2-1.

Table 2.6.2-1: LOS probability

| Scenario | LOS probability (distance is in meters) |
| --- | --- |
| UMi – Street canyon | Outdoor users: Indoor users: Use d2D-out in the formula above instead of d2D |
| UMa | Outdoor users: where and Indoor users: Use d2D-out in the formula above instead of d2D |
| Indoor – Open office |  |
| Note: The LOS probability is derived with assuming antenna heights of 3m for indoor, 10m for UMi, and 25m for UMa |  |

2.6.3	O-to-I penetration loss

The Path loss incorporating O-to-I building penetration loss is modelled as in the following:

PL = PLb + PLtw + PLin + N(0, σP2)

where PLb is the basic outdoor path loss given. PLtw is the building penetration loss through the external wall, PLin is the inside loss dependent on the depth into the building, and σP  is the standard deviation for the penetration loss.

PLtw is characterized as:

is an additional loss is added to the external wall loss to account for non-perpendicular incidence;

, is the penetration loss of material i, example values of which can be found in Table 2.6.3-1.

pi is proportion of i-th materials, where ; and

N is the number of materials.

Table 2.6.3-1: Material penetration losses

| Material | Penetration loss [dB] |
| --- | --- |
| Standard multi-pane glass | Lglass=2+0.2⋅f |
| IRR glass | LIRRglass=23+0.3⋅f |
| Concrete | Lconcrete=5+4⋅f |
| Wood | Lwood=4.85+0.12⋅f |
| Note: f is in GHz |  |

Table 2.6.3-2 gives PLtw, PLin and σP  for two O-to-I penetration loss models. The O-to-I penetration is UT-specifically generated, and is added to the SF realization in the log domain.

Table 2.6.3-2 O-to-I penetration loss model

|  | Path loss through external wall: PLtw [dB] | Indoor loss: PLin [dB] | Standard deviation: σP [dB] |
| --- | --- | --- | --- |
| Low-loss model | 5-10log100.3⋅10-Lglass10+0.7⋅10-Lconcrete10 | 0.5d2D-in | 4.4 |
| High-loss model | 5-10log100.7⋅10-LIRRglass10+0.3⋅10-Lconcrete10 | 0.5d2D-in | 6.5 |

d2D-in is minimum of two independently generated uniformly distributed variables between 0 and 25 m for RMa, UMa and UMi-Street Canyon. d2D-in shall be UT-specifically generated.

Both low-loss and high-loss models are applicable to UMa and UMi-Street Canyon.

Only the low-loss model is applicable to RMa.

The composition of low and high loss is a simulation parameter that should be determined by the user of the channel models, and is dependent on the use of metal-coated glass in buildings and the deployment scenarios. Such use is expected to differ in different markets and regions of the world and also may increase over years to new regulations and energy saving initiatives. Furthermore, the use of such high-loss glass currently appears to be more predominant in commercial buildings than in residential buildings in some regions of the world.

The pathloss incorporating O-to-I car penetration loss is modelled as in the following:

PL = PLb + N(μ, σP2)

where PLb is the basic outdoor path loss given in Section 7.4.1. μ = 9, and σP = 5. Optionally, for metallized car windows, μ = 20 can be used. The O-to-I car penetration loss models are applicable for at least 0.6-60 GHz.

Propagation model between NTN BS and TN BS should reference to TS 38.811 which is used for DL-UL cross link interference for S band.

Propagation model between HAPS BS and UE is defined in TR 38.811 [5]

2.7. Transmission power control model

For downlink scenario, no power control scheme is applied.

For uplink scenario, TPC model specified in Section 9.1 TR 36.942 [7] could be applied for TN with following parameters.

Where, Pmax = 23dBm, Rmin = TBD dB, CLx-ile and γ are set as following:

-	CLx-ile = 88 + 10*log10 (200/X) + 11 – Y,

where X is UL transmission BW (MHz) and Y is the BS noise figure

-	γ = 1For uplink scenario, TPC model for NTN is FFS.

2.8. Received power model

The received power in downlink and uplink scenarios is defined as below:

RX_PWR = TX_PWR – Path loss + G_TX + G_RX

Where,

RX_PWR is the received power

TX_PWR is the transmitted power

G_TX is the transmitter antenna gain (directional array gain)

G_RX is the receiver antenna gain (directional array gain).

2.9. Performance metric

For NR,

The average throughput loss and 5%-ile throughput loss should be less than 5%.

For NB-IOT,

The average throughput loss and SNR loss, 5%-ile throughput loss and SNR loss should be according to 36.802

For NTN,

FFS

2.10. Throughput ~ SNR mapping

Use section 5.2.7 of TR 38.803.

The throughput of a modem with link adaptation can be approximated by an attenuated and truncated form of the Shannon bound. (The Shannon bound represents the maximum theoretical throughput than can be achieved over an AWGN channel for a given SNIR). The following equations approximate the throughput over a channel with a given SNIR, when using link adaptation:

Throughput SNIR, bps/Hz =0                                  for SNIR                                         ∝∙SSNIR                     for SNIRMIN≤SNIR<SNIRMAX ∝∙SSNIRMAX               for SNIR ≥ SNIRMAX

Where:

S(SNIR)     Shannon bound, S(SNIR) =log2(1+SNIR)  bps/Hz
                Attenuation factor, representing implementation losses
SNIRMIN    Minimum SNIR of the code set, dB
SNIRMAX   Maximum SNIR of the code set, dB

The parameters α, SNIRMIN and SNIRMAX can be chosen to represent different modem implementations and link conditions.

2.11. Methodology of TN and NTN coexistence simulation

Adopt the following simulation steps:

- Aggressor and victim network are generated.

-	NTN central beam is at satellite nadir, surrounded with 6 co-frequency beams.

-	TN center is randomly generated within the NTN central beam.

-	NTN to TN: 19-cell with wrap around

-	TN to NTN: It is expected that a large number of cells are needed. The exact number of cell should be decided by system level simulation.

2.	UE associations

-	NTN UE is randomly generated within the TN area depending on the NTN UE density.

-	TN UE is randomly generated inside the TN network. TN UEs are associated to each TN sectors based on coupling loss.

3.	Once association is done, round robin scheduling is used. BF weights are adjusted to point to the LOS direction between BS-UE. This is done for both victim and aggressor networks.

4.	Throughput is computed in the victim systems without considering ACI as below:

-	ThputNO ACIbpshz=fSINRICI=fSN+IICI, where IICI is the inter-cell interference.

5.	Throughput is computed considering ACI as below:

-	ThputACIbpshz=fSINRICI+ACI=fSN+IICI+IACI, where IACI is the adjacent channel interference.

6.	RF parameters are determined based on the degradation cause by ACI as below:

-	LossACI=1-ThputACIThputSINGLE.

- Conclusion

It is proposed to use the simulation assumptions in this paper for NTN co-existence study.

- Reference

[1] R4-2100486

[2] R4-2100904

[3] R4-2101105

[4] R4-2101812

[5] R4-2101859

[6] R4-2101964

[7] R4-2102174

[8] R4-2102508