3GPP TSG-RAN WG4 Meeting # 98-bis-e 	R4-2106482

Electronic Meeting, Apr. 12-20, 2021

Source: 	Huawei, HiSilicon

Title: 	Adding 50 MHz CBW for NR band n3

Agenda Item:	7.24.2

Document for:	Approval

1 Introduction

In the last plenary meeting, proposals for adding new existing BWs are approved in the revised WID RP-202062 [1]. It is copied below that for every proposal, RAN4 considers to specify the corresponding RF requirements:

- Analyze and specify requirements: - Reference sensitivity and associated RB allocation. - When needed: - MPR (relative bandwidth criteria) - Additional Maximum Power Reduction (A-MPR) - NS signalling. - Any other RF requirement which might be relevant.

In this contribution we provide discussion and proposals on RF requirement for n3 when introducing 50 MHz CBW.

2 Discussion

2.1 MPR/A-MPR

For n3 50 MHz CBW, relative channel bandwidth = 2.86% which satisfy the FDD 3% criteria. Hence the existing MPR requirements can be reused.

For n3 only NS_01 and NS_100 is defined as shown in below table. Hence A-MPR is not needed for adding 50 MHz CBW.

Table 2.1-1: Mapping of network signalling label

| NR band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n3 | NS_01 | NS_100 |  |  |  |  |  |  |

Proposal 1: no A-MPR will be specified for n3 when introducing 50 MHz CBW.

2.2 REFSENS

For NR band n3, the existing max channel bandwidth is 40 MHz. And 35 MHz and 45 MHz are discussing under the WI NR_FR1_35MHz_45MHz_BW. With the RB limitation, the major interference contribution is 5th IMD products for channel bandwidth <= 45MHz. For 50 MHz CBW, for UL allocation in the channel edge the strong 3rd IMD product will degrade the REFSENS significantly.Hence we suggest to avoid the 3rd IMD impact when setting UL configulation for 50 MHz REFSENS test. It is proposed to adopt the UL configuration in Table 2.2-2 for n3 50 MHz RFESENS.

Table 2.2-1: existing n3 reference sensitivity

| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n3 | 15 | -97.0 | -93.8 | -92.0 | -90.8 | -89.7 | -88.9 | -82.3 | TBD | FDD |
|  | 30 |  | -94.1 | -92.1 | -91.0 | -89.8 | -89.0 | -82.4 | TBD |  |
|  | 60 |  | -94.5 | -92.4 | -91.2 | -90.0 | -89.1 | -82.6 | TBD |  |

- Table 2.2-2: Uplink Configuration for Reference Sensitivity

| Operating Band | SCS kHz | 50 MHz (dBm) |
| --- | --- | --- |
| n3 | 15 | 50x |
|  | 30 | 24 x |
|  | 60 | 10 x |
| NOTE X: For Band 3, for 50 MHz channel bandwidth, in the case of 15 kHz SCS, the UL resource blocks shall be located at RBstart 165, in the case of 30 kHz SCS, the UL resource blocks shall be located at RBstart 75, and in the case of 60 kHz SCS, the UL resource blocks shall be located at RBstart 39. |  |  |

Proposal 2: It is proposed to adopt the UL configuration in Table 2.2-2 for n3 50 MHz RFESENS.

3 Conclusion

In this contribution we provide discussion and proposals on RF requirement for n3 when introducing 50 MHz CBW.

Proposal 1: no A-MPR will be specified for n3 when introducing 50 MHz CBW.

Proposal 2: It is proposed to adopt the UL configuration in Table 2.2-2 for n3 50 MHz RFESENS.

References

[1] RP-210917, “Revised Basket WID: adding new channel bandwidth(s) support to existing NR bands”, Ericsson