3GPP TSG-RAN WG4 Meeting # 98-bis-e 	R4-2106487

# Electronic Meeting, Apr. 12-20, 2021

Source:	Huawei, HiSilicon, CMCC, China Unicom

Title:	Scenarios for support of 1024QAM

Agenda item:	8.15.1

Document for:	Approval

1 Introduction

In RAN#89 e-meeting, a new work item, i.e. introduction of DL 1024-QAM for NR FR1 was approved [1]. The objective is to specify downlink 1024-QAM for NR PDSCH operation in FR1, together with related procedures, signaling and necessary RF requirements. The RAN4 objectives are listed below.

- Specify corresponding UE and BS RF core requirements [RAN4]

  - UE and BS RF core requirements are specified for stationary wireless scenarios with up to 2 layer DL MIMO

  - The cell size(s) and type of stationary wireless scenarios for which UE and BS RF core requirements are defined will be studied and decided by RAN4.

In this contribution, we provide system-level evaluations on the second objective to discuss the cell size and BS type for the support of 1024-QAM.

2 Discussion

To decide the cell size(s) and type of stationary wireless scenarios for which UE and BS RF core requirement are defined, we think the system-level evaluation is needed to identify the deployment scenarios in which supporting 1024-QAM can provide notable performance gain.

The discussion is focused on the class of wide area BS, which corresponds to the macro BS whose typical application scenario is urban macro. Thus in the following part, the performance comparison between schemes with and without 1024-QAM for the scenario are provided. The detailed simulation assumptions are listed in Table A.1 of Appendix A. The evaluation results in terms of gain of cell average spectral efficiency (SE) and the MCS CDF.

Table 2-1 Evaluation results with and without 1024-QAM of full buffer

| Scenario | Urban macro (100% outdoor) |  | Urban macro (80% indoor, 20% outdoor) |  |
| --- | --- | --- | --- | --- |
| Scheme | w/o 1024 | with 1024 | w/o 1024 | with 1024 |
| Gain of cell average SE | - | 0.73% | - | 1.59 % |

Figure 2-1 MCS CDF for Urban Marco (100% outdoor)

Figure 2-2 MCS CDF for Urban Marco (80% indoor, 20% outdoor)

It can be observed from the above table that supporting 1024-QAM can at most provide 1.59% gain in terms of cell average SE under the traffic model of full buffer. The MCS index 23~26 is for 1024-QAM modulation. From figure 2-1 and 2-2 it can be observed that less than 0.5% UE can use 1024-QAM.

Then in the following Table 2-2, the evaluation results in terms of gain of cell average user perceived throughput (UPT) with FTP traffic are given.

Table 2-2. Evaluation results with and without 1024-QAM of FTP traffic

| Scenario | Urban macro (100% outdoor) |  | Urban macro (80% indoor, 20% outdoor) |  |
| --- | --- | --- | --- | --- |
| Scheme | w/o 1024 | with 1024 | w/o 1024 | with 1024 |
| RU | ~25% |  | ~50% |  |
| Gain of average UPT | - | -1.00% | - | -0.6% |

According to the above table, it is observed that the performance gain from 1024-QAM for average UPT is -1.00% and -0.6% for respectively. From figure 2-3 and 2-4, correspondingly, less than 5 % and less than 0.5% UE for 100% outdoor scenario and 20% outdoor scenario can use 1024-QAM, respectively.

Figure 2-3 MCS CDF for Marco Urban(100% outdoor) RU~25%

Figure 2-4 MCS CDF for Marco Urban(20% outdoor) RU~50%

Observation 1: for Macro scenarios, 1024-QAM can not provide notable performance gain

Observation 2: for Macro scenarios, few UEs can use 1024-QAM MCS

The simulation results are quite in line with the observations from the field deployments, the number of UEs received SINR higher than 26 dB is quite limited.

It is also noted that the above results are based on the assumption without transmit power reduction at the BS side. Actually, the macro BS needs to additionally reduce the transmit power in some scale in order to satisfy the Tx EVM requirement of 1024-QAM. By considering such power reduction, the corresponding performance degradation is expected.

Proposal 1: it is proposed that 1024-QAM RF requirements is not defined for Macro BS. And 1024-QAM is only defined for small cell scenarios.

3 Conclusion

In this contribution, we provide consideration on the support of 1024-QAM.

Proposal 1: it is proposed that 1024-QAM RF requirements is not defined for Macro BS. And 1024-QAM is only defined for small cell scenarios.

Reference

- “RAN1 Chairman’s Notes”, RAN1#103-e, October 26th – November 13th, 2020.

- RP-202866, “Revised WID on Introduction of DL 1024-QAM for NR FR1”, Nokia, RAN#90e, December 7-11, 2020.

Appendix A

Table A.1. System-level evaluation assumptions

| Scenario | Urban Macro |
| --- | --- |
| Layout | 7 sites, 3 sector per site |
| Inter-BS distance | 500m |
| Carrier frequency | 4GHz |
| Simulation bandwidth | 100 MHz |
| Numerology | 30KHz |
| Duplex configuration | TDD: DDDSU |
| Channel model | 3D UMa |
| BS total Tx power | 49dBm |
| BS antenna configuration | (M,N,P,Mg,Ng) = (8,8,2,1,1) (dH,dV) = (0.5, 0.7)λ |
| BS antenna height | 25m |
| BS antenna element gain + connector loss | 6.4 dBi |
| UE Tx power | Maximum 23dBm |
| UE antenna configuration | 2Tx4Rx |
| UE antenna gain | 0dBi |
| UE antenna element gain pattern | Omnidirectional |
| UE receiver noise figure | 9dB |
| UE distribution | 10 UEs per sector; 80% indoor+20% outdoor; 100% outdoor |
| UE receiver | MMSE-IRC |
| Channel estimation | Real |
| Traffic model | 1) Full buffer; 2) FTP model with packet size 2.5Mbytes |
| MIMO scheme | SU-MIMO with rank adaptation and MU-MIMO |
| CSI scheme | Reciprocity based; 2T SRS |