3GPP TSG-RAN WG4 Meeting #98-bis-e	R4-2106507

Electronic Meeting, Apr. 12-20, 2021

Source: 	Intel Corporation

Title: 					NR-U PDSCH simulation results

Agenda Item: 			5.1.4.2

Document for:	Discussion

# 1	Introduction

This paper presents initial simulation results considering the parameters agreed during the previous meetings [1].

# 2	Discussion

The simulation results are shown in the figures in Table 2-2 and summarized in Table 2-3 and Table 2-4. The simulation assumptions are shown in Table 2-1

Table 2-1: PDSCH simulation assumptions

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Active DL BWP index |  |  | 1 |
| LBT failure probability |  |  | [0.50, 0.25] |
| Common serving cell parameters | Physical Cell ID |  | 0 |
|  | SSB position in burst |  | The first SSB |
|  | SSB Q factor |  | 8 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | According to DL Transmission Model |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| RB Allocation |  |  | Full BW |
| MIMO Layers |  |  | 2 |
| Number of HARQ Processes |  |  | 8 |
| TX EVM |  |  | 6% |

| DL Transmission Model | Maximum COT Duration | ms | [4] |
| --- | --- | --- | --- |
|  | Idle Time after COT | ms | [1] |
|  | DL Transmission Model Period/ Fixed Frame Period (Note 1) | ms | 5 |
|  | Probability of LBT Failure pLBT |  | [0.25, 0.5] |
|  | Guard Symbols |  | [TBD] |
|  | UL Symbols |  | [TBD] |
|  | Number of slots between PDSCH and corresponding HARQ-ACK information |  | [TBD, HARQ-ACK is sent in any case within the FFP and minimum 1 slot after the end of the allocated Downlink portion] |
|  | Duration of the Downlink portion of the COT | Slots | {2,3,5,6} |
|  | PDSCH Allocation in the last Slot of the Downlink portion of the COT | Symbols | {6,9,12,14} |
| Notes: The Fixed Frame Period denomination applies only for ChannelAccessType-r16 = ‘semistatic’. For ChannelAccessType-r16 = ‘dynamic’ this parameter is identified only as DL Transmission Model Period. |  |  |  |

Table 2-2: PDSCH throughput statistics

Table 2-3: Summary of alignment PDSCH simulation results

| Bandwidth (MHz) |  | 20 |  | 40 |  |
| --- | --- | --- | --- | --- | --- |
| Antenna configuration |  | 2x2 | 2x4 | 2x2 | 2x4 |
| SNR@70% of maxTP(dB) | LBT failure probability = 0 | 10.7 | 5.8 | 11.0 | 5.9 |
|  | LBT failure probability = 0.25 | 10.6 | 5.9 | 11.0 | 5.9 |
|  | LBT failure probability = 0.50 | 10.7 | 5.9 | 11.0 | 5.8 |

Table 2-4: Summary of impairments PDSCH simulation results

| Bandwidth (MHz) |  | 20 |  | 40 |  |
| --- | --- | --- | --- | --- | --- |
| Antenna configuration |  | 2x2 | 2x4 | 2x2 | 2x4 |
| SNR@70% of maxTP(dB) | LBT failure probability = 0 | 12.7 | 7.8 | 13.0 | 7.9 |
|  | LBT failure probability = 0.25 | 12.6 | 7.9 | 13.0 | 7.9 |
|  | LBT failure probability = 0.50 | 12.7 | 7.9 | 13.0 | 7.8 |

# 3	Conclusion

In this contribution we provided PDSCH simulation results for NR-U.

# 4	References

- R4-2103987 “Way Forward on NR-U UE demodulation requirements” Qualcomm, RAN4 #98-e, Feb 2021