3GPP TSG-RAN WG4 Meeting #98-bis-e	R4-2106528

Electronic Meeting, Apr. 12 – 20, 2021

Agenda item:	5.7.2.2.2

Source: 	OPPO

Title: 	Discussion and simulation results for CSI-RS SINR accuracy requirements

Document for:	Discussion

Introduction

In RAN4 #98-e meeting, the impacts of negative and positive timing offset on the CSI-RS based SINR measurement were discussed [1] and the simulation assumptions were updated accordingly [2]. This contribution will provide our simulation results for CSI-RS SINR based on the following assumptions:

- CSI-RS configuration: <X=1,D=3,N=1,NA,noCDM> with bandwidth=48RBs

- Number of samples: 5

- Carrier frequency: 4GHz for FR1 and 30GHz for FR2

- Propagation condition: AWGN for FR1 and TDL-A with 30ns delay spread for FR2

- Relative delay for cell 2: 0, ±CP/2, ±0.9*CP, ±CP

Discussion

SINR accuracy vs time offset

Table 1/2/3 provide the delta CSI-RS SINR of cell 2 for 15kHz, 30kHz and 120kHz subcarrier spacing, respectively, where the side condition of cell 2 is about -6dB.

Table 1: SCS=15 kHz, AWGN channel @ FR1

| Relative delay | 5% (dB) | 50% (dB) | 95% (dB) | 50%-5% | 95%-50% | Relative accuracy |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | -1.521 | -0.168 | 1.026 | 1.353 | 1.194 | 1.353 |
| 0.5*CP | -1.596 | -0.206 | 0.873 | 1.39 | 1.079 | 1.39 |
| 0.9*CP | -1.898 | -0.461 | 0.59 | 1.437 | 1.051 | 1.437 |
| CP | -1.803 | -0.457 | 0.635 | 1.346 | 1.092 | 1.346 |
| -0.5*CP | -2.2 | -0.647 | 0.592 | 1.553 | 1.239 | 1.553 |
| -0.9*CP | -2.778 | -1.14 | 0.174 | 1.638 | 1.314 | 1.638 |
| -CP | -3.092 | -1.231 | 0.069 | 1.861 | 1.3 | 1.861 |

Table 2: SCS=30 kHz, AWGN channel @ FR1

| Relative delay | 5% (dB) | 50% (dB) | 95% (dB) | 50%-5% | 95%-50% | Relative accuracy |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | -1.52 | -0.153 | 0.988 | 1.367 | 1.141 | 1.367 |
| 0.5*CP | -1.663 | -0.216 | 0.877 | 1.447 | 1.093 | 1.447 |
| 0.9*CP | -1.83 | -0.445 | 0.626 | 1.385 | 1.071 | 1.385 |
| CP | -1.811 | -0.423 | 0.615 | 1.388 | 1.038 | 1.388 |
| -0.5*CP | -2.155 | -0.594 | 0.585 | 1.561 | 1.179 | 1.561 |
| -0.9*CP | -2.729 | -1.136 | 0.159 | 1.593 | 1.295 | 1.593 |
| -CP | -2.897 | -1.267 | 0.071 | 1.63 | 1.338 | 1.63 |

Table 3: SCS=120 kHz, TDL-A channel @ FR2

| Channel | Relative delay | 5% (dB) | 50% (dB) | 95% (dB) | 50%-5% | 95%-50% | Relative accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TDL-A with 30ns delay spread | 0 | -4.811 | 0.263 | 4.603 | 5.074 | 4.34 | 5.074 |
|  | 0.5*CP | -4.843 | 0.028 | 4.249 | 4.871 | 4.221 | 4.871 |
|  | 0.9*CP | -4.855 | 0.025 | 3.82 | 4.88 | 3.795 | 4.88 |
|  | CP | -4.602 | -0.254 | 3.813 | 4.348 | 4.067 | 4.348 |
|  | -0.5*CP | -4.871 | -0.07 | 3.972 | 4.801 | 4.042 | 4.801 |
|  | -0.9*CP | -5.095 | -0.513 | 3.256 | 4.582 | 3.769 | 4.582 |
|  | -CP | -5.143 | -0.653 | 3.114 | 4.49 | 3.767 | 4.49 |
| AWGN | 0 | -1.495 | -0.133 | 0.979 | 1.362 | 1.112 | 1.362 |
|  | 0.5*CP | -1.705 | -0.253 | 0.801 | 1.452 | 1.054 | 1.452 |
|  | 0.9*CP | -1.911 | -0.425 | 0.723 | 1.486 | 1.148 | 1.486 |
|  | CP | -1.925 | -0.464 | 0.647 | 1.461 | 1.111 | 1.461 |
|  | -0.5*CP | -2.056 | -0.627 | 0.541 | 1.429 | 1.168 | 1.429 |
|  | -0.9*CP | -2.803 | -1.179 | 0.047 | 1.624 | 1.226 | 1.624 |
|  | -CP | -2.939 | -1.248 | -0.001 | 1.691 | 1.247 | 1.691 |

Based on our simulation results, the following observations are provided.

- The absolute accuracy performance of CSI-RS SINR of fading channel is worse than that of AWGN channel.

- For AWGN channel at FR1, the absolute accuracy performance of CSI-RS SINR can be within ±2.5dB for timing offset within [-0.5*CP, +0.5*CP], and ±4.0dB for timing offset within [-CP, +CP].

- For TDL-A channel at FR2, the absolute accuracy performance of CSI-RS SINR can be within ±5.0dB for timing offset within [-0.5*CP, +0.5*CP], and ±6.0dB for timing offset within [-CP, +CP].

- For AWGN channel at FR1, the relative accuracy of CSI-RS SINR can be within ±1.5dB for timing offset within [-0.5*CP, +0.5*CP], and ±2.0dB for timing offset within [-CP, +CP].

- For TDL-A channel at FR2, the relative accuracy of CSI-RS SINR can be within ±5.5dB for timing offset within [-CP, +CP].

Observation 1: The absolute accuracy performance of CSI-RS SINR can be found:

- For AWGN channel at FR1, ±4.0dB for timing offset within [-CP, +CP]

- For TDL-A channel at FR2 ±6.0dB for timing offset within [-CP, +CP].

Observation 2: The relative accuracy performance of CSI-RS SINR can be found:

- For AWGN channel at FR1, ±2.0dB for timing offset within [-CP, +CP]

- For TDL-A channel at FR2 ±5.50dB for timing offset within [-CP, +CP].

Observation 3: The absolute accuracy performance of CSI-RS SINR with negative timing offset is worse than that of CSI-RS SINR with positive timing offset.

Considering RF margin and different fading channel, we think a bit shorter timing offset should be allowed, e.g., within 0.9 CP, in order to ensure the absolute accuracy performance of CSI-RS SINR can be reused as those of SSB’s. And the same values of time offset can apply for relative accuracy performance.

Proposal 1: To ensure the absolute and relative accuracy performance of CSI-RS SINR can be reused as those of SSB’s, the upper bound of timing offset can be set within 0.9 CP.

Upper limit of Es/Iot

Table 4-6 provide the 90th %-iles of absolute delta CSI-RS SINR of cell 2 for 15kHz, 30kHz and 120kHz subcarrier spacing, respectively, where the side condition of cell 2 ranges from 6dB to 25dB.

Table 4: SCS=15 kHz, AWGN channel @ FR1

| Relative delay | 6dB | 10dB | 12 dB | 15 dB | 18 dB | 20 dB | 25 dB |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0.299 | 0.267 | 0.262 | 0.260 | 0.278 | 0.310 | 0.439 |
| 0.5*CP | 0.514 | 0.827 | 1.129 | 1.864 | 3.039 | 4.104 | 7.669 |
| 0.9*CP | 1.200 | 2.135 | 2.900 | 4.463 | 6.519 | 8.109 | 12.571 |
| CP | 1.356 | 2.395 | 3.227 | 4.901 | 7.046 | 8.681 | 13.209 |
| -0.5*CP | 1.806 | 3.192 | 4.229 | 6.206 | 8.593 | 10.347 | 15.028 |
| -0.9*CP | 3.208 | 5.264 | 6.636 | 9.031 | 11.686 | 13.548 | 18.385 |
| -CP | 3.571 | 5.740 | 7.162 | 9.602 | 12.300 | 14.189 | 19.051 |

Table 5: SCS=30 kHz, AWGN channel @ FR1

| Relative delay | 6dB | 10dB | 12 dB | 15 dB | 18 dB | 20 dB | 25 dB |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0.299 | 0.261 | 0.257 | 0.265 | 0.287 | 0.308 | 0.438 |
| 0.5*CP | 0.496 | 0.811 | 1.117 | 1.852 | 3.304 | 4.098 | 7.670 |
| 0.9*CP | 1.201 | 2.138 | 2.894 | 4.459 | 6.516 | 8.106 | 12.572 |
| CP | 1.352 | 2.395 | 3.224 | 4.899 | 7.045 | 8.680 | 13.209 |
| -0.5*CP | 1.809 | 3.203 | 4.238 | 6.230 | 8.611 | 10.361 | 15.044 |
| -0.9*CP | 3.201 | 5.273 | 6.642 | 9.035 | 11.690 | 13.553 | 18.394 |
| -CP | 3.568 | 5.753 | 7.171 | 9.614 | 12.307 | 14.189 | 19.045 |

Table 6: SCS=120 kHz, TDLA-30NS channel @ FR2

| Channel | Relative delay | 6dB | 10dB | 12 dB | 15 dB | 18 dB | 20 dB | 25 dB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TDL-A | 0 | 3.686 | 3.796 | 3.933 | 4.301 | 4.966 | 5.697 | 8.814 |
|  | 0.5*CP | 3.815 | 4.275 | 4.727 | 5.767 | 7.319 | 8.577 | 12.628 |
|  | 0.9*CP | 4.431 | 5.392 | 6.135 | 7.721 | 9.794 | 11.414 | 15.911 |
|  | CP | 4.509 | 5.596 | 6.426 | 8.097 | 10.233 | 11.891 | 16.440 |
|  | -0.5*CP | 4.082 | 4.814 | 5.478 | 6.827 | 8.646 | 10.163 | 14.559 |
|  | -0.9*CP | 4.875 | 6.211 | 7.168 | 9.074 | 11.382 | 13.119 | 17.865 |
|  | -CP | 5.189 | 6.597 | 7.670 | 9.605 | 11.993 | 13.765 | 18.509 |
| AWGN | 0 | 0.293 | 0.262 | 0.256 | 0.259 | 0.274 | 0.307 | 0.432 |
|  | 0.5*CP | 0.520 | 0.825 | 1.129 | 1.862 | 3.033 | 4.098 | 7.670 |
|  | 0.9*CP | 1.206 | 2.137 | 2.895 | 4.465 | 6.519 | 8.108 | 12.571 |
|  | CP | 1.337 | 2.384 | 3.216 | 4.894 | 7.042 | 8.679 | 13.208 |
|  | -0.5*CP | 1.800 | 3.207 | 4.253 | 6.227 | 8.615 | 10.369 | 15.052 |
|  | -0.9*CP | 3.232 | 5.304 | 6.684 | 9.063 | 11.721 | 13.582 | 18.416 |
|  | -CP | 3.554 | 5.746 | 7.165 | 9.604 | 12.306 | 14.181 | 19.034 |

Based on our simulation results, the following observations are provided.

- Observation 7: For AWGN channel, the relative accuracy of CSI-RS SINR can be within ±3.5dB for timing offset within [0, +CP] with Es/Iot ≤ 10dB.

- Observation 8: For AWGN channel, the relative accuracy of CSI-RS SINR can be within ±4.5dB for timing offset within [0, +0.5*CP] with Es/Iot ≤ 18dB.

In summary, either option 1 or option 2 seems fine for us based on our simulation results. We can also support a new option 3 as compromise based on timing offset within 0.9*CP.

- Option 3:  Es/Iot ≤ [12] dB for the case that timing offset is within 0.9*CP.

Proposal 2: Option 3 as compromise for the upper limit of Es/Iot for CSI-SINR measurement with timing offset(T△):

- Option 3:  Es/Iot ≤ [12] dB for the case that timing offset is within 0.9*CP.

- Issue 2-3-1: The upper limit of Es/Iot for CSI-SINR measurement with timing offset(T△)

The upper limit of Es/Iot for CSI-SINR measurement with timing offset(T△)

    - Option 1: Es/Iot ≤ [10] dB for the case that timing offset is within CP.

    - Option 2: Es/Iot ≤ [18] dB for the case that timing offset is within CP/2.

Conclusion

This paper provides our simulation results for CSI-RS SINR measurements and the following observations.

Observation 1: The absolute accuracy performance of CSI-RS SINR can be found:

- For AWGN channel at FR1, ±4.0dB for timing offset within [-CP, +CP]

- For TDL-A channel at FR2 ±6.0dB for timing offset within [-CP, +CP].

Observation 2: The relative accuracy performance of CSI-RS SINR can be found:

- For AWGN channel at FR1, ±2.0dB for timing offset within [-CP, +CP]

- For TDL-A channel at FR2 ±5.50dB for timing offset within [-CP, +CP].

Observation 3: The absolute accuracy performance of CSI-RS SINR with negative timing offset is worse than that of CSI-RS SINR with positive timing offset.

Proposal 1: To ensure the absolute and relative accuracy performance of CSI-RS SINR can be reused as those of SSB’s, the upper bound of timing offset can be set within 0.9 CP.

Proposal 2: Option 3 as compromise for the upper limit of Es/Iot for CSI-SINR measurement with timing offset(T△):

- Option 3:  Es/Iot ≤ [12] dB for the case that timing offset is within 0.9*CP.

- References

- R4-2104089, WF on accuracy requirements and test cases of CSI-RS based L3 measurement, CATT, OPPO

- R4-2103725, “Updated link-level simulation assumptions for CSI-RS based L3 measurements”, CATT