3GPP TSG-RAN WG4 Meeting #98-bis-e		R4-2106568
Electronic Meeting, 12th Apr – 20th Apr, 2021

Agenda item:	8.1.2.2

Source:	OPPO

Title:	FoM for FR2

Document for:	Approval

1	Introduction

In last meeting of RAN4 #98-e, the conclusion was made on 36 test points for FR2 MIMO OTA testing, as the WF shown [1].

Furthermore, the figure of merit for FR2 MIMO OTA performance requirements has been defined as the MIMO Average Spherical Coverage (MASC) in the meeting of RAN4 #96-e, as shown in the WF of #96-e [2].

However, there is ambiguity on the calculation of MASC. This contribution discusses the ambiguity and presents our proposal on FoM of MASC.

2	Discussion

Regarding the statement of “averaging all the values better than 50% percentile of CCDF”, the ambiguity comes from how many test points taking into account while some points can not reach 70% throughput, or called the missing points, among the 36 test points, i.e. including the missing points in the CCDF curve or excluding the missing points.

An illustration can be shown to specify the difference between the two situations. Assuming there are N points failed to reach 70% throughput, the number of test points joining in the MASC calculation are as below.

For the situation of including the missing points, top 18 values should be used for MASC calculation.

For the situation of excluding the missing points, the number of values should be INT(18-N2) .

Observation 1: excluding the missing points from the CCDF curve results in less test points joining in the MASC calculation. And the more the missing points are, the less number of calculated points there are.

Another illustration can be shown to express the influence on the situation of excluding the missing points. Considering two DUTs, all the 36 test points of DUT A have reached 70% throughput, while DUT B has N (N>0) missing points. Based on the MASC calculation of excluding the missing points, although having more “bad points”, DUT B has a greater probability to get a better result on MASC. Because it need to guarantee the performances on less test points than those of DUT A. apparently, this is not a reasonable FoM to judge the DUT performance.

Observation 2: excluding the missing points from the CCDF curve results in that the DUT having more missing points has a greater probability to get a better result on MASC.

Considering the above observations comprehensively, the calculation of MASC including the missing points in the CCDF curve will drive the mobile device design to optimize the performance on more spherical coverage, which will definitely bring better user experience. That is, averaging the top 18 values of total 36 test points to derive the MASC.

Proposal: The MASC is derived from averaging the top 18 values of total 36 test points.

MASC50%-tile, 70=10 log18110P50%-tile,70,110+110P50%-tile,70,210+…+110P50%-tile,70,1810

3	Conclusion

Proposal: The MASC is derived from averaging the top 18 values of total 36 test points.

MASC50%-tile, 70=10 log18110P50%-tile,70,110+110P50%-tile,70,210+…+110P50%-tile,70,1810

References

- R4-2103913 WF on NR MIMO OTA_v3

- R4-2012707 WF on NR MIMO OTA