3GPP TSG-RAN WG4 Meeting#98-bis-e                                            R4-2106596

E-meeting,12th April– 20th April, 2021

Agenda item:	5.3.2.2.3

Source: 	ZTE Corporation

Title: 	TP to TS 38.xxx-2:  Section 4.2~4.5

Document for:	Approval

Introduction

In the past RAN4#98e meetings, work split has been agreed among companies, therefore in this contribution, we want to share the draft TP for section 4.2~4.5 for further discussion.

Reference

- R4-2103856	WF on IAB conformance specification work split and drafting guidelines, approved.

Annex

- <Start of TP>

4.2	Radiated requirement reference points

Radiated characteristics for IAB type 1-H, IAB type 1-O and IAB type 2-O are defined over the air (OTA) where the operating band specific radiated interface is referred to as the Radiated Interface Boundary (RIB). Radiated requirements are also referred to as OTA requirements. The (spatial) characteristics in which the OTA requirements apply are detailed for each requirement. For IAB type 1-H the requirements are defined for two points of reference, signified by radiated requirements at the RIB and the conducted requirements at transceiver array boundary (TAB). The OTA requirements of IAB type 1-H are tested in the far field (Fraunhofer) region.

General architecture and reference points of IAB type 1-H, IAB type 1-O and IAB type 2-O are presented on the following figures 4.2-1 – 4.2-2.

Figure 4.2-1: General architecture of IAB type 1-H

This specification details only radiated test requirements and hence only requires the radiated reference points.

Figure 4.2-2: General architecture of IAB type 1-O and IAB type 2-O

The transceiver unit array is part of the composite transceiver functionality generating modulated transmit signal structures and performing receiver combining and demodulation.

The transceiver unit array contains an implementation specific number of transmitter units and an implementation specific number of receiver units. Transmitter units and receiver units may be combined into transceiver units. The transmitter/receiver units have the ability to receive/send parallel independent modulated symbol streams.

The composite antenna contains a radio distribution network (RDN) and an antenna array. The RDN is a linear passive network that distributes the RF power between the transceiver array boundary and the antenna array, in an implementation specific way.

4.3	IAB classes

4.3.1	IAB-DU clas

The requirements in this specification apply to Wide Area IAB-DU, Medium Range IAB-DU and Local Area IAB-DU unless otherwise stated. The associated deployment scenarios for each class are exactly the same for IAB-DU with and without connectors.

For IAB type 1-O and 2-O, IAB-DU classes are defined as indicated below:

-	Wide Area IAB-DU are characterised by requirements derived from Macro Cell scenarios with a BS to UE minimum distance along the ground equal to 35 m.

-	Medium Range IAB-DU are characterised by requirements derived from Micro Cell scenarios with a BS to UE minimum distance along the ground equal to 5 m.

-	Local Area IAB-DU are characterised by requirements derived from Pico Cell scenarios with a BS to UE minimum distance along the ground equal to 2 m.

The manufacturer shall declare the intended class of the IAB under test.

4.3.2	IAB-MT clas

The requirements in this specification apply to Wide Area IAB-MT and Local Area IAB-MT classes unless otherwise stated.

For IAB type 1-O, and 2-O, IAB-MT classes are defined as indicated below:

-	Wide Area IAB-MT are characterised by requirements derived from Macro Cell and/or Micro Cell scenarios.

-	Local Area IAB-MT are characterised by requirements derived from Pico Cell and /or Micro Cell scenarios.

4.4	Regional requirements

Some requirements in the present document may only apply in certain regions either as optional requirements, or set by local and regional regulation as mandatory requirements. It is normally not stated in the 3GPP specifications under what exact circumstances that the requirements apply, since this is defined by local or regional regulation.

Table 4.4-1 lists all requirements in the present specification that may be applied differently in different regions.

Table 4.4-1: List of regional requirements

| Clause | Requirement | Comments |
| --- | --- | --- |
| 5.2 | Operating bands | Some NR operating bands may be applied regionally. |
| 6.2.3 | IAB output power: Additional requirements | These requirements may be applied regionally as additional IAB output power requirements. |
| 9.7.2 | OTA occupied bandwidth | The requirement may be applied regionally. There may also be regional requirements to declare the occupied bandwidth according to the definition in present specification. |
| 9.7.4.2 9.7.4.3 | OTA operating band unwanted emissions | Category A or Category B operating band unwanted emissions limits may be applied regionally. |
| 9.7.4.4.1 | OTA operating band unwanted emissions: Limits in FCC Title 47 | The IAB may have to comply with the additional requirements, when deployed in regions where those limits are applied, and under the conditions declared by the manufacturer. |
| 9.7.5.2.2 9.7.5.3.2 | OTA Tx spurious emissions | Category A or Category B spurious emission limits, as defined in ITU-R Recommendation SM.329 [2], may apply regionally. The emission limits for IAB type 1-O specified as the basic limit + X (dB) are applicable, unless stated differently in regional regulation. |
| 9.7.5.2.3 9.7.5.3.3 | OTA Tx spurious emissions: additional requirements | These requirements may be applied for the protection of system operating in frequency ranges other than the IAB operating band. |
| 9.8.2 | OTA transmitter intermodulation | Interfering signal positions that are partially or completely outside of any downlink operating band of the IAB are not excluded from the requirement in Japan in Band n77, n78, n79. |
| 10.7.2 10.7.3 | OTA Rx spurious emissions | The emission limits for IAB type 1-O specified as the basic limit + X (dB) are applicable, unless stated differently in regional regulation. |

4.5	IAB configurations

4.5.1	Transmit configurations

Unless otherwise stated, the radiated transmitter characteristics in clause 6 are specified at RIB, with a full complement of transceiver units for the configuration in normal operating conditions.

Editor's NOTE:	to be aligned with the figures for the RIB interfaces and co-location concept.

Figure 4.5.1-1: Transmitter test interfaces

Figure 4.5.1-2: Transmitter test interfaces for co-location concept

4.5.2	Receive configurations

Unless otherwise stated, the radiated receiver characteristics in clause 7 are specified at RIB, with a full complement of transceiver units for the configuration in normal operating conditions.

Editor's NOTE:	to be aligned with the figures for the RIB interfaces and co-location concept.

Figure 4.5.2-1: Receiver test interfaces

Figure 4.5.2-2: Receiver test interfaces for co-location concept

4.5.3	Power supply options

If the IAB is supplied with a number of different power supply configurations, it may not be necessary to test RF parameters for each of the power supply options, provided that it can be demonstrated that the range of conditions over which the equipment is tested is at least as great as the range of conditions due to any of the power supply configurations.

4.5.4	IAB with integrated Iuant BS modem

Unless otherwise stated, for the tests in the present document, the integrated Iuant BS modem shall be switched OFF.Spurious emissions according to clauses 6.6.5 and 7.6 shall be measured only for frequencies above 30 MHz with the integrated Iuant BS modem switched ON.

- <End of TP>